# 3GPP TSG RAN WG2 Meeting #130      	                           R2-2503711
St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.5.3

Source:	Apple

Title:	Remaining issues on on-demand SIB1

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

The Rel-19 WID of Network Energy Savings (WID RP- RP-250343) was updated in RAN#107 [1]. The WI objective on on-demand SIB1 (OD-SIB1) for UEs in IDLE/INACTIVE state is copied as below:

In this contribution, we share our view on remaining issues from below aspects:

- Remaining issue on feature specfic barring

- Key RRC open issues

- Key 38.304 open issues

# 2 Discussion

## 2.1 Remaining issue on feature specfic barring

In RAN2#129b [2], it was discussed whether to include feature specific barrng bits and CellAccessRelatedInfo in UL WUS configuraiton. But no conclusion can be made and this issue was postponed to next RAN2 meeting.

We understand that the motivation is to avoid UE unnecessary OD-SIB1 request for checking access restriction information of target OD-SIB1 cell. We think it is useful.

Observation 1: On including feature specific barring, the motivation is to avoid UE unnecessary OD-SIB1 request for checking access restriction information of target OD-SIB1 cell.

During online/offline discussion in RAN2#129b [2], the main concerns include:

- The increased signaling overhead to UL WUS configuration, especially CellAccessRelatedInfo with PLMN / NPN information.

- It will make a list of cells associated with UL WUS configuration more difficult.

On concern 1), we provide an analysis on signaling overhead for feature specific barrng bits and CellAccessRelatedInfo in Table.1.

|  | Detailed IEs | Payload (bit) | Assumption |
| --- | --- | --- | --- |
| Feature specific barring common to all PLMN/NPN | cellReservedForOtherUse, cellReservedForFutureUse, cellBarredATG, cellBarred2RxXR, cellBarred-eRedCap1Rx, cellBarred-eRedCap2Rx, cellBarredNES, cellBarredNTN, cellBarredFixedVSAT, cellBarredMobileVSAT cellBarred-RedCap1Rx, cellBarred-RedCap2Rx, halfDuplexRedCapAllowed ncr-Support | 14bit |  |
| cellAccessRelatedInfo per PLMN / NPN | PLMN related access info - plmn-IdentityList (3*25) - TAC (24) - ranac (7) - cellIdentity (36) - gNB-ID-Length (4) - cellReservedForOperatorUse (1) - iab-Support (1) - mobileIAB-Support (1) | 149bit | Assume 3 PLMN or 3 NPN share TAC, RANAC, cellIdentity, etc. |
|  | NPN related access info - npn-IdentityInfoList (3*50) - TAC (24) - ranac (7) - cellIdentity (36) - gNB-ID-Length (4) - cellReservedForOperatorUse (1) - iab-Support (1) - mobileIAB-Support (1) | 224bit |  |
|  | snpn-AccessInfoList (12) | 12 |  |

Table.1 Analysis of signaling overhead for feature specific barrng bits and CellAccessRelatedInfo

According to Table 1, we can see that feature specific barring bits common to all PLMN/NPN (i.e. the UE doesn’t need to check its PLMN/NPN info) actually only have small signaling overhead (totally 14bit) while PLMN-specific and NPN-specific cell access information have large signaling overhead (149bit and 236bit).

Observation 2: Feature specific barring bits common to all PLMN/NPN (i.e. the UE doesn’t need to check its PLMN/NPN info) only have small signaling overhead (totally 14bit) while PLMN/NPN-specific cell access information have large signaling overhead (149bit for PLMN related access information and 236bit for NPN related access information).

As signaling overhead is small, we think at least feature specific barring bits common to all PLMN/NPN can be included in UL WUS configuration.

On concern 2), we think that it can be resolved by making the feature specific barring bits as optional IE, i.e. if NW does not want it, it can be omitted.

Thus, we propose:

Proposal 1: The following feature specific barring bits common to all PLMN/NPN (totally 14 bit) are optionally included in UL WUS configuraiton:

- cellReservedForOtherUse, cellReservedForFutureUse, cellBarredATG, cellBarred2RxXR, cellBarred-eRedCap1Rx, cellBarred-eRedCap2Rx, cellBarredNES, cellBarredNTN, cellBarredFixedVSAT, cellBarredMobileVSAT, cellBarred-RedCap1Rx, cellBarred-RedCap2Rx, halfDuplexRedCapAllowed, ncr-Support

## 2.2 Key RRC open issues

### 2.2.1 How to check OD-SIB1 broadcast status

Agreement

If a UE has SIB1 request configuration of a cell and before transmitting UL WUS,

- If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=12 for FR2, select the following:

  - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

In RAN1#120b [3], it was agreed it is up to UE implementation whether to monitor SIB1 when K_SSB>=24 for FR1 or K_SSB>=12 for FR2:

For the other case (i.e. when K_SSB<24 for FR1 or K_SSB<12 for FR2), it is legacy UE behavior on monitoring CD-SSB and no need of new specification.

Thus, we think it is sufficient to add the following NOTE in Section 5.2.2.3.1 to capture RAN1 agreement (i.e. UE needs to check, but how to check depends on UE implementation according to RAN1 agreement)

“NOTE: It is up to UE implementation on how to check SIB1 is being broadcasted.”

Proposal 2: Add the following NOTE in Section 5.2.2.3.1 to capture RAN1 agreement:

“NOTE: It is up to UE implementation on how to check SIB1 is being broadcasted.”

### 2.2.2 FFS: if list of cells is ARFCN&PCI or only PCI

In RAN2#129b [2], it was agreed that UL WUS configuration can be associated with a list of cells.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

Thus, the latest running RRC CR [4] captured that one UL WUS configuration can be associated with a list of PCIs, and an FFS on if list of cells is ARFCN&PCI or only PCI.

OD-SIB1-CellConfig-r19 ::=              SEQUENCE {

carrierFreq-r19                          ARFCN-ValueNR,

physCellId-r19                      SEQUENCE (SIZE(1..maxPCI-r19))  OF PhysCellId,

od-SIB1-Config-r19                        od-SIB1-Config-r19                               OPTIONAL,        -- Need R

...

}

FFS: if list of cells is ARFCN&PCI or only PCI

If both frequency and PCI can be associated with one UL WUS configuration, it means that multiple inter-frequency NES cells have the same UL WUS configuration. However, please note that absolute frequency of point A is included in UL WUS configuration, as highlighted below. It is impossible that two inter-frequency cells can have same absolute point A.

OD-SIB1-Config-r19  ::= {

sib1-rsrp-ThresholdSSB-r19                       RSRP-Range                                                  OPTIONAL,   -- Need R

prach-RootSequenceIndex-r19        CHOICE {

l839                               INTEGER (0..837),

l139                               INTEGER (0..137)

}                                                                                                             OPTIONAL, -- Need R

msg1-SubcarrierSpacing-r19 ENUMERATED {kHz1dot25, kHz5, kHz15, kHz30, kHz60, kHz120, spare1, spare2} OPTIONAL, -- Need R

sib1-tdd-UL-DL-ConfigurationCommon-r19   TDD-UL-DL-ConfigCommon                                OPTIONAL, -- Cond TDD

sib1-restrictedSetConfig-r19  ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB} OPTIONAL, -- Need R

offsetToCarrier-r19                          INTEGER (0..2199)                                      OPTIONAL, -- Need R

absoluteFrequencyPointA-r19                  ARFCN-ValueNR                                          OPTIONAL, -- Need R

p-Max-r19                                    P-Max                                                       OPTIONAL, -- Need R

ss-PBCH-BlockPower-r19                       INTEGER (-60..50)                                      OPTIONAL, -- Need R

ssb-PositionsInBurst-r19                SEQUENCE {

inOneGroup                          BIT STRING (SIZE (8)),

groupPresence                       BIT STRING (SIZE (8))                                   OPTIONAL  -- Cond FR2-Only

},

Observation 3: Absolute frequency of point A is included in UL WUS configuration. It is impossible that two inter-frequency cells can have same absolute point A, i.e. two inter-frequency cells can’t have same WUS config.

Thus, we think only a list of PCIs can be associated with one UL WUS configuration.

Proposal 3: Only a list of PCIs can be associated with one UL WUS configuration.

### 2.2.3 FFS: how to capture a CONNECTED UE understands its stored SIB1

In RAN2#129b [2], it was agreed that NW ensures that the RRC connected UE has the latest SIB1, and the UE in RRC_CONNECTED state understands the stored SIB1 is the latest SIB1.

Agreements on OD-SIB1

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

Please note that RRC_CONNECTED UE has no mechanism to check whether stored SIB1 is the latest one in OD-SIB1 cell. Thus, we think this requirement should be captured in TS 38.331 [4].

Proposal 4: Capture the RAN2#129b agreement (“The UE supporting OD-SIB1 in RRC_CONNECTED regards the stored SIB1 is the latest SIB1.”) in field description of si-BroadcastStatus.

### 2.2.4 FFS: Co-existence of SBFD and OD-SIB1

Both SBFD and OD-SIB1 are Rel-19 feature. We don’t prefer to introduce optimization on feature combination in the same release, and the benefit is not clear to us. Thus, we propose:

Proposal 5: RAN2 don’t pursue the optimization of co-existence of SBFD and OD-SIB1.

### 2.2.5 FFS: Co-existence of SDT and OD-SIB1

According to company input, it seems that the intention is to allow the UE to trigger OD-SIB1 procedure even if there is SDT ongoing.

In current on-demand SI, if there is no SDT ongoing, the on-demand SI is not allowed. The on-demand SI is performed toward current serving cell and on-demand SIB1 is toward neighbour cell usually. It is not good idea to forbid the UE to perform on-demand SIB1 to neighbour NES cell, e.g., no other cell can be reselected except NES cell, and it will impact the UE mobility.

According to current spec, after cell reselection occurs, the SDT will stop. However, performing on-demand SIB1 does not mean cell reselection will happen to this cell.

The legacy RRC specification doesn’t allow the UE to trigger OD-SIB procedure if there is SDT ongoing. We understand the reason is because only one RACH procedure can be performed in one time instance. As OD-SIB procedure is not that so essential, it is not necessary to specify the UE to terminate the ongoing SDT procedure and trigger OD-SIB procedure. Back to the OD-SIB1 issue. Because there is no hard latency requirement is put on OD-SIB1 procedure, we also don’t see the need to change legacy OD-SIB behaviour.

Proposal 6: RAN2 don’t pursue the optimization to allow the UE to trigger OD-SIB1 procedure when there is SDT ongoing (i.e. no need to change the procedure of current running RRC CR).

## 2.3 Key 38.304 open issues

### 2.3.1 New UE behaviour on legacy excluded cell lists

According to summary reporting on 38.304 running CR [5], the issue is summarized as follows:

Issue 1: FFS whether the UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided, irrespective of whether dedicated excluded cell lists being provided.

[Issue description] Some companies proposed a new UE behavior on handling excluded cell list. Their view is that NW may not configure Rel-19 excluded cell list if there is no excluded cell for NES UE in real deployment. In this case, they think that the NES UE also needs to ignore the legacy excluded cell list because NES Cell is included in legacy IE to bar the legacy UE. Thus, they propose that the NES UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided, irrespective of whether dedicated excluded cell lists being provided.

We disagree with the new UE behaviour:

    - It actually assumes NW will only include NES cells in legacy excluded cell list. We don’t think it is correct because it restricts NW implementation. We assume that the legacy excluded cell list may include NES cells and/or some normal cells (e.g. blacklist cells for all UE access) based on NW implementation. When legacy excluded cell list includes only blacklist cells for all UE, if NES UE ignores legacy excluded cell list, it will wrongly access blacklist cells.

    - As RAN2 agreed that legacy UE is barred via “no-SIB1” indication in MIB, the dedicated excluded cell list itself is an optimization. We don’t prefer to do optimization over optimization.

Thus, we propose to not pursue this optimization.

Proposal 7: RAN2 don’t pursue the enhancement that OD-SIB1 UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided.

# 3 Conclusion

In this contribution, we discuss remaining issues of on-demand SIB1. Our observations are:

Observation 1: On including feature specific barring, the motivation is to avoid UE unnecessary OD-SIB1 request for checking access restriction information of target OD-SIB1 cell.

Observation 2: Feature specific barring bits common to all PLMN/NPN (i.e. the UE doesn’t need to check its PLMN/NPN info) only have small signaling overhead (totally 14bit) while PLMN/NPN-specific cell access information have large signaling overhead (149bit for PLMN related access information and 236bit for NPN related access information).

Observation 3: Absolute frequency of point A is included in UL WUS configuration. It is impossible that two inter-frequency cells can have same absolute point A, i.e. two inter-frequency cells can’t have same WUS config.

Based on observations, our proposals are:

Remaining issue on feature specfic barring

Proposal 1: The following feature specific barring bits common to all PLMN/NPN (totally 14 bit) are optionally included in UL WUS configuraiton:

- cellReservedForOtherUse, cellReservedForFutureUse, cellBarredATG, cellBarred2RxXR, cellBarred-eRedCap1Rx, cellBarred-eRedCap2Rx, cellBarredNES, cellBarredNTN, cellBarredFixedVSAT, cellBarredMobileVSAT, cellBarred-RedCap1Rx, cellBarred-RedCap2Rx, halfDuplexRedCapAllowed, ncr-Support

Key RRC open issues

Proposal 2: Add the following NOTE in Section 5.2.2.3.1 to capture RAN1 agreement:

“NOTE: It is up to UE implementation on how to check SIB1 is being broadcasted.”

Proposal 3: Only a list of PCIs can be associated with one UL WUS configuration.

Proposal 4: Capture the RAN2#129b agreement (“The UE supporting OD-SIB1 in RRC_CONNECTED regards the stored SIB1 is the latest SIB1.”) in field description of si-BroadcastStatus.

Proposal 5: RAN2 don’t pursue the optimization of co-existence of SBFD and OD-SIB1.

Proposal 6: RAN2 don’t pursue the optimization to allow the UE to trigger OD-SIB1 procedure when there is SDT ongoing (i.e. no need to change the procedure of current running RRC CR).

Key 38.304 open issues

Proposal 7: RAN2 don’t pursue the enhancement that OD-SIB1 UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided.

# 4 References

[1] RP-250343, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#129b, Chair Notes.

[3] RAN1#120b, Chair Notes.

[4] R2-250xxxx, Running RRC CR for enhancements for network energy efficiency, Ericsson.

[5] R2-2503709, Summary report of [POST129b][113][NES] 38.304 CR (Apple), Apple (Rapporteur).