3GPP TSG RAN WG2 #130	R2-2503958

St Julian’s, Malta, 19th - 23rd May 2025

Agenda item:	8.8.2

Source: 	ITRI

Title: 	Discussions on the assistance information for supporting location-based SMTC selection

Document for:	Discussion

1	Introduction

Supporting more than 4 SMTCs per frequency was agreed in RAN2#129bis meeting. It remains FFS on what information can be provided from network to help UE to perform SMTC selection for idle/inactive mode. [1]

This contribution discusses the assistance information to help RRC_IDLE/RRC_INACTIVE UE for SMTC selection.

2	Discussion

Regarding the SSB periodicity could be extended up to 160 ms, RAN2 discussed the related impacts and came to the following agreements [1][2]:

Agreements (RAN2 #129): 1. RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer 2. RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame 3. RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) 4. The number of SMTC/gaps a UE needs to consider at any time will not be increased further

Agreements (RAN2 #129bis);

1.	From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

2.	We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

3.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

4.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

The assistance information on the association between SMTC and location to help UE to perform SMTC selection and the detail of location information need further discussion.

2.1 	The assistance information for the association between SMTC and location

Network may provide up to 6 SMTC per frequency layer configured by the network, UE may select 2 or 4 SMTC to perform measurements according to UE capabilities. The SMTC selected by the UE in RRC_IDLE/RRC_INACTIVE states should allow the UE to perform measurements of the neighbour cells in vicinity for cell reselection.

Observation 1: UE in RRC_IDLE/RRC_INACTIVE state needs to measure neighbour cells in vicinity for cell reselection.

Option 1: Serving cell SSB ID as the assistance information

One of the options of the assistance information is the serving cell SSB ID.

Based on RAN2 assumption that the whole NTN cell could be covered by different SSB beams in SSB beam sweeping case. The serving cell SSB ID detected by a UE implies the spatial direction of a NTN cell.

The footprint of a SSB beam may neighbour with more than one surrounding cell that requires different SMTCs (e.g., when the surrounding cells are deployed with different SSB periodicities, or the difference of SMTC offsets > 5 ms).

Figure 1

A UE capable of 2 SMTCs located in the edge of the footprint of a satellite beam (or in the overlapping area of two satellite beams) may only need to measure a part of the neighbour cells adjacent with footprint of the the SSB beam. For example, as shown in Figure 1, UE may select one adjacent neighbour cell associated with each SSB ID to perform measurements for cell reselection.

When the serving cell SSB ID is used as the assistance information and SMTCs are associated with the serving cell SSB ID, the UE would miss the SMTC associated with the surrounding cell adjacent to the UE location.

Observation 2: Use serving cell SSB ID as assistance information is not sufficient to help RRC_IDLE/RRC_INACTIVE UE perform SMTC selection.

Option 2: Reference location(s) as the assistance information

The option of using reference location as reference information could provide one or more reference location to UE.

If the SMTCs are associated with the reference locations, UE in RRC_IDLE/RRC_INACTIVE states could estimate the distance between the UE and the reference locations in order to select SMTCs for the measurements of neighbour cells in the vicinity of the UE.

Observation 3: Use reference location as the assistance information could help RRC_IDLE/RRC_INACTIVE UE to select SMTC associated with the neighbour cells in vicinity for cell reselection measurements.

Proposal 1: RAN2 to adopt reference location as the assistance information to help RRC_IDLE/RRC_INACTIVE UE to perform SMTC selection.

2.2 	The association of assistance information and SMTC

When providing assistance information, the issue needs to be considered is how to associate the reference location and SMTC.

Option 1: Reference location per SMTC

Considering beam hopping could be used for DL coverage enhancements and the SSB periodicity could be extended to 160 ms, the SSB timing offset of adjacent cells served by the same satellite may be more than 5 ms that could not be configured by the same SMTC. On another hand, the coverages of two cells configured by the same SMTC may be far apart. Therefore, associate reference location per SMTC is not sufficient to help UE to perform SMTC selection.

Observation 4:  Providing reference location per SMTC is not sufficient to help RRC_IDLE/RRC_INACTIVE UE to perform SMTC selection.

Option 2: Reference location per neighbour cell

If a reference location is provided per neighbour cell, RRC_IDLE/RRC_INACTIVE UE could determine the neighbour cells in vicinity based on UE location and apply the associated SMTC for measurements.

Observation 5: Reference locations associated with cells could enable UE to select the surrounding cells neighbouring to the UE for the cell reselection measurements.

Proposal 2: Location information is provided per PCI as the assistance information to help RRC_IDLE/RRC_INACTIVE UE to perform SMTC selection.

3	Conclusion

This contribution discussion the assistance information to help RRC_IDLE/RRC_INACTIVE UE for SMTC selection. The following observations and proposals are provided:

For the assistance information for the association between SMTC and location:

Observation 1: UE in RRC_IDLE/RRC_INACTIVE state needs to measure neighbour cells in vicinity for cell reselection.

Observation 2: Use serving cell SSB ID as assistance information is not sufficient to help RRC_IDLE/RRC_INACTIVE UE perform SMTC selection.

Observation 3: Use reference location as the assistance information could help RRC_IDLE/RRC_INACTIVE UE to select SMTC associated with the neighbour cells in vicinity for cell reselection measurements.

Proposal 1: RAN2 to adopt reference location as the assistance information to help RRC_IDLE/RRC_INACTIVE UE to perform SMTC selection.

For the association of assistance information and SMTC:

Observation 4:  Providing reference location per SMTC is not sufficient to help RRC_IDLE/RRC_INACTIVE UE to perform SMTC selection.

Observation 5: Reference locations associated with cells could enable UE to select the surrounding cells neighbouring to the UE for the cell reselection measurements.

Proposal 2: Location information is provided per PCI as the assistance information to help RRC_IDLE/RRC_INACTIVE UE to perform SMTC selection.

4	References

[1] R2-2501702	Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece.

[2] R2-250xxxx	Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China.