3GPP TSG-RAN WG2 Meeting #130                                  R2-2503983

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.3.2

Source:				Samsung

Title:					Discussion on functionality management for UE-side model

Document for:		Discussion & Decision

1	Introduction

For study on the general spec. impact for AI/ML for mobility, the following table was agreed in RAN2#129bis.

Things to continue study for specification and RAN2 impact

|  | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, | No (up to NW implementation) |
| Temporal domain case A | Yes, all sub cases | Yes |
| Temporal domain case B | Yes , all sub cases | Yes |
| Frequency domain | Yes , all sub cases | Yes |
| Spatial domain | Pending | Pending |
| L3 beam level measurement prediction | Yes, | Yes |

In this paper, we would like to discuss the functionality management of UE-side model (i.e., applicability/capability reporting, inference configuration, inference report) for the use cases agreed as in the table above (i.e., except the pending Spatial domain).

2	Discussion

At the last RAN2#129bis, the following agreement was made for the general LCM framework for AI mobility.

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

According to the agreement above, we can discuss the potential spec. impact of AI mobility based on the baseline procedure agreed for beam management (BM) use case and the discussion here can focus on the aspect that can be different from BM use case for AI mobility. Also, considering the agenda given in the chair note, this paper discusses some aspects (i.e., capability/applicability reporting, inference configuration and reporting) of LCM framework for UE-side model case.

2.1 Capability/Applicability reporting

Definition:

In Rel-19 AI/ML for air WI session, RAN2 agreed the definition of “supported functionalities”, “applicable functionalities”, and “Activated functionalities” as below.

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

To facilitate discussion by ensuring we have the same understanding on the definitions in AI/ML for mobility SI, we would like to propose the following.

Proposal. 1: To facilitate discussion in AI/ML for mobility SI/WI, RAN2 adopt the definition of supported/applicable/activated functionalities agreed in Rel-19 AI/ML for air WI.

UE Capability Reporting:

Regarding UE capability reporting, RAN2 made the following agreement in Rel-19 AI/ML for air WI.

Agreements

1.	Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress.   “supported” means that the UE is capable of supporting the functionality and doesn’t mean neccesarily that the UE has the model available.  FFS what functionality refers to.

2.	Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Since we do not see any issue specific to AI/ML for mobility in UE capability reporting procedure, we would like to propose the following.

Proposal. 2: For AI/ML for mobility, UE can report the supported AI/ML-enabled Features/FGs and supported functionalities to NW via UECapabilityInformation message.

Applicability Reporting:

Regarding applicability reporting, RAN2 made the following agreement for BM use case in Rel-19AI/ML for air WI.

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

Since there is no issue specific to AI/ML for mobility in applicability reporting procedure, RAN2 agreed to have the same procedure as in BM use case as the yellow-colored agreement below.

Agreements for AI mobility (in RAN2#129bis) - The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework - For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message. - FFS whether association ID is required and should provide motivation on per use case bases

Meanwhile, there is FFS on whether associated ID is required for AI mobility or not as the green-colored part above. From our understanding, the motivation of having associated ID in BM use case is to provide UE with some NW-side additional condition that is needed by UE for applicability check but can not be explicitly provided. For example, NW can implicitly indicate the consistency of NW’s beam configuration (e.g., antenna pattern, tilting, codebook configuration that can have impact on analog beam pattern of a certain CSI-RS/SSB index) using the associated ID and the UE may assume the similar properties of DL Tx beam with the same associated ID.

Observation. 1: For BM use case, the associated ID is introduced to allow NW to implicitly indicate the consistency of NW’s beam configuration (e.g., antenna pattern, tilting, codebook configuration that can have impact on analog beam pattern of a certain CSI-RS/SSB index).

For AI mobility, the following use cases utilize the beam-level measurement/prediction. And those use cases require the consistency of NW’s beam configuration across training and inference.
- Temporal domain prediction Case A&B with sub-use cases 1/3.
- Frequency domain prediction with sub-use cases 1/3.
- Measurement event prediction with indirect approach using RRM measurement prediction with sub-use case 1/3.
- L3 beam level measurement prediction

Observation. 2: For AI mobility, the following use cases utilize the beam-level measurement/prediction, which requires the consistency of NW’s beam configuration across training and inference.
- Temporal domain prediction Case A&B with sub-use cases 1/3.
- Frequency domain prediction with sub-use cases 1/3.
- Measurement event prediction with indirect approach reusing RRM measurement prediction with sub-use case 1/3.
- L3 beam level measurement prediction

Therefore, we think the associated ID is needed also in AI for mobility especially for the use cases requiring beam-level measurement/prediction.

Proposal. 3: For AI/ML for mobility, associated ID can be required for the use cases utilizing beam-level measurement/prediction to guarantee the consistency of NW’s beam configuration across training and inference.

2.2 Inference & reporting

For UE-side model inference, NW can provide some configuration for inference at UE-side. Then, the UE can perform the inference with the input data that is internally available at UE and report the outcome of the inference to NW. For the spec. impact, we can focus on the inference configuration from NW to UE and the report from UE to NW.

2.2.1 Inference configuration

At the last RAN2#129bis, RAN2 agreed the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework. In our view, the existing RRM measurement configuration consisting of MeasObject/MeasId/ReportConfig can be easily extended/enhanced to support the use cases of AI for mobility. Therefore, in this part, we propose some potential enhancement based on the existing RRM measurement configuration.

Common part:

For the common part across the use cases (i.e., frequency/temporal domain RRM measurement prediction, measurement event prediction, L3 beam-level prediction, Spatial domain prediction), NW can indicate the target MO for which the UE predicts RRM measurement/event prediction. Also, in our view, it is inefficient for UE to perform the prediction for all the cells detected during the measurement for the target MO. Therefore, NW can indicate the list of cells for which the UE should perform the measurement prediction. For example, NW can configure the UE to perform the prediction only for the cells that can be the candidate HO target cells.

Proposal. 4: For all the use cases agreed in AI for mobility, NW can indicate the target MO (Measurement object) for which the UE performs the prediction (e.g., RRM measurement/event prediction) as part of inference configuration.

Proposal. 5: For all the use cases agreed in AI for mobility, NW can indicate the list of target cells for which the UE performs the prediction as part of inference configuration.

Also, as in BM use case, for the consistency across training and inference, NW can provide the NW-side additional condition (e.g., associated ID) which can be used for applicability check.

Proposal. 6: For all the use cases agreed in AI for mobility, NW can indicate NW-side additional condition (e.g., associated ID) for the applicability check as part of inference configuration if needed.

Temporal domain Case A:

For temporal domain Case A, NW can indicate how far the UE should predict the measurement results in time domain (i.e., prediction window length). Then, UE can predict the L3 Cell measurement results of multiple time instances within the prediction window according to the NW configuration.

Proposal. 7: For temporal domain Case A, NW can indicate the prediction window length (i.e., how far the UE should predict the measurement results in time domain) as part of inference configuration.

Temporal domain Case B:

For temporal domain Case B, one issue to discuss is how to determine MRRT (Measurement Reduction Rate in Time domain). Given the trade-off relation between measurement reduction rate and prediction accuracy, NW should have some controllability over MRRT used by UE to guarantee HO performance. Specifically, NW can either directly determine the value and configure it for UE, or provide some requirement on prediction accuracy for UE to select the proper MRRT value that can meet the requirement from NW.

Proposal. 8: For temporal domain Case B, NW can indicate either the MRRT value to use or the accuracy requirement (e.g., tolerable RSRP difference) to meet as part of inference configuration.

Meanwhile, with the same MRRT value, the measurement skipping pattern can vary depending on the UE’s implementation (i.e., depending on how UE-side server trains the AI model), and NW can not be aware of it. However, in our view, NW’s awareness on the skipping pattern is necessary to maximize energy savings at UE-side (i.e., to align the C-DRX cycle, measurement gap timing with the skipping pattern). To achieve this, NW can configure UE to provide its measurement skipping pattern as an assistance data for NW.

Proposal. 9: For temporal domain Case B, NW can configure UE to report its measurement skipping pattern as an assistance data for NW.

Measurement event prediction:

For measurement event prediction, NW can configure UE to initiate the reporting procedure when the entry condition for the target event is predicted to be fulfilled for all measurements during timeToTrigger within PW. For that, NW can include the indication to configure UE to trigger the measurement reporting based on the prediction, target event (e.g., eventId for eventA3), and PW length in the inference configuration.

Proposal. 10: For measurement event prediction, NW can include the indication to configure UE to initiate the measurement reporting procedure when it predicts the target event within PW as part of inference configuration.

Proposal. 11: For measurement event prediction, NW can indicate the prediction window length (i.e., how far the UE should predict the measurement event in time domain) as part of inference configuration.

Proposal. 12: For measurement event prediction, NW can indicate the target event (e.g., eventId for eventAx) for which the UE performs the measurement event prediction as part of inference configuration.

Frequency domain prediction:

For frequency domain prediction, NW can configure UE to predict the measurement results for the cells in a certain frequency based on the measurement results for the cells in different frequency. For that, NW can provide not only the target MO for which the UE performs the prediction but also the input MO for which the UE performs the measurement for input data.

Meanwhile, during SI phase, we assume the scenario where the input and output cells are co-located and also observe the positive correlation between the prediction accuracy and the inter-frequency correlation. This implies that the input and output cells for inter-frequency prediction should be co-located to guarantee a good prediction accuracy. However, the UE can not know which cells are co-located or not and only the NW can be aware of it. Considering that point, we think NW can configure the input and output cells for the inter-frequency prediction.

Proposal. 13: For frequency domain prediction, NW can indicate both the target MO/cell for which the UE perform the prediction and the input MO/cell for which the UE perform the measurement for input data as part of inference configuration.

L3 beam-level prediction:

For L3 beam-level prediction, NW can configure UE to predict the beam-level measurement results for the best candidate cell (e.g., the cell with highest RSRP) for HO within PW. Also, considering that beam-level prediction is to configure a dedicated RACH resource for some potential best beam of the target cell, UE does not need to report the prediction results for all beams. Thus, NW can configure UE to report only the Top-K beam indexes.

Proposal. 14: For L3 beam-level prediction, NW can indicate the prediction window length (i.e., how far the UE should predict the measurement results in time domain) as part of inference configuration.

Proposal. 15: For L3 beam-level prediction, NW can indicate the maximum number of beams to report (i.e., K) as part of inference configuration (to configure UE to report Top-K beam indexes).

2.2.2 Inference report

At the last RAN2#129bis, RAN2 agreed the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework. In our view, the existing measurement reporting mechanism can be simply extended/enhanced for UE to report the inference results of AI for mobility. Therefore, in this part, we propose some potential enhancement with the assumption of reusing the legacy measurement reporting mechanism.

Regarding further details on the potential spec. impact for the measurement (inference) reporting, RAN2 made the following agreements.

- Agreements

For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

With the above as baseline, we can further discuss the potential spec. impact for each use case as below.

Temporal domain Case A:

For temporal domain Case A, UE can initiate the periodical or event-triggered reporting as in legacy, but it can report the predicted results for the multiple time instances within PW for the target cells. Here, the target cells can be determined by the inference configuration from NW. For example, NW can explicitly configure the target cells, or the target cell can be the serving cells and Top-K neighbor cells where the maximum cells numbers to report (i.e., K) is given by NW.

Proposal. 16: For temporal domain Case A, UE can report the predicted results for the multiple time instances within PW for the target cells. The target cells can be determined based on the inference configuration from NW.

Temporal domain Case B:

For temporal domain Case B, we understand that the UE can skip the measurement at some time instances and replace the measurement results for the skipped time instances with the predicted values. And the UE can trigger the measurement reporting and report the measurement results as in the legacy. The only difference is that some of the measurement results used for measurement reporting are not the actual measurement results but the predicted ones. More specifically, for event-triggered reporting, the UE can initiate the measurement reporting procedure when the entry condition applicable for the event is fulfilled by all actual and predicted measurement results during timeToTrigger. And the UE can report the latest (real/predicted) measurement results for the applicable cell. However, from NW’s perspective, there is no need to know whether the reported measurement result is real or predicted if we assume that the UE performs the prediction with good accuracy under RAN4 requirement. Therefore, we do not see any potential enhancement on the measurement report for temporal domain Case B.

Observation. 3: For temporal domain Case B, UE can trigger the measurement reporting and report the latest (predicted) measurement results as in the legacy. The only difference compared to the legacy is that some of the measurement results used for measurement reporting are not the actual measurement results but the predicted ones.

Proposal. 17: For temporal domain Case B, UE can trigger the measurement reporting and report the latest (real/predicted) measurement results as in the legacy. No further enhancement is needed for measurement (inference) reporting.

Measurement event prediction:

For measurement event prediction, UE can trigger the event-triggered reporting based on the prediction. Specifically, the UE can initiate the measurement (inference) reporting procedure when the entry condition applicable for the event is predicted to be fulfilled by all measurement results for one or more applicable cells during timeToTrigger. And the UE can report the latest measurement result and/or the predicted results for the multiple time instances for the cells for which the entry condition is predicted to be fulfilled during timeToTrigger.

Proposal. 18: For measurement event prediction, the UE can report the latest measurement result and/or the predicted results for the multiple time instances for the applicable cells. The applicable cells can be the cells for which the event is predicted.

When the UE predicts the event, it can report some additional information for the predicted event. For the additional information, the UE can report the expected occurrence time (for indirect method) or the probability of event occurrence within PW (for direct method).

Proposal. 19: For measurement event prediction, the UE can report some additional information for the predicted event. The additional information can be the expected occurrence time (for indirect method) or the probability of event occurrence within PW (for direct method).

Meanwhile, the UE can continue the measurement event prediction even after the first event prediction report and the results of event prediction can be updated/changed from “predicted” to “not predicted”. In this case, the UE should be able to cancel the previous inference report to avoid wrong HO decision by NW based on the old inference report. Therefore, we see the need of supporting the inference report cancellation.

Proposal. 20: For measurement event prediction, UE can continue the measurement event prediction after sending the first event prediction report. If the event prediction result is updated compared to the last report, the UE can report the updated result to NW.

Frequency domain prediction:

For frequency domain prediction, UE can initiate the periodical or event-triggered reporting as in legacy and it can report the latest predicted measurement result for the target cells. Here, the target cells can be determined by the inference configuration from NW. For example, NW can explicitly configure the target cells or the target cell can be the Top-K neighbor cells for the target MO given by NW.

Proposal. 21: For frequency domain prediction, UE can report the latest predicted measurement result for the target cells. The target cells can be determined based on the inference configuration from NW.

L3 beam-level prediction:

For L3 beam-level prediction, UE can initiate the periodical or event-triggered reporting as in legacy and it can report the beam-level prediction results for the best candidate cell for the dedicated RACH resource configuration during HO. Here, the best candidate cell can be determined based on the actual or predicted measurement results. Also, the beam-level prediction results can include the predicted Top-K beam indexes and/or the predicted beam-level measurement results for the multiple time instances within PW for the Top-K beams where the maximum beam number to report (i.e., K) is given by NW.

Proposal. 22: For L3 beam-level prediction, UE can report the beam-level prediction results for the best neighbor cell. The best neighbor cell can be determined based on the actual or predicted measurement results.

Proposal. 23: For L3 beam-level prediction, the beam-level prediction results for the best neighbor cell can include the predicted Top-K beam indexes and/or the predicted beam-level measurement results for the multiple time instances within PW for the Top-K beams.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Capability/Applicability reporting:

Proposal. 1: To facilitate discussion in AI/ML for mobility SI/WI, RAN2 adopt the definition of supported/applicable/activated functionalities agreed in Rel-19 AI/ML for air WI.

Proposal. 2: For AI/ML for mobility, UE can report the supported AI/ML-enabled Features/FGs and supported functionalities to NW via UECapabilityInformation message.

Observation. 1: For BM use case, the associated ID is introduced to allow NW to implicitly indicate the consistency of NW’s beam configuration (e.g., antenna pattern, tilting, codebook configuration that can have impact on analog beam pattern of a certain CSI-RS/SSB index).

Observation. 2: For AI mobility, the following use cases utilize the beam-level measurement/prediction, which requires the consistency of NW’s beam configuration across training and inference.
- Temporal domain prediction Case A&B with sub-use cases 1/3.
- Frequency domain prediction with sub-use cases 1/3.
- Measurement event prediction with indirect approach reusing RRM measurement prediction with sub-use case 1/3.
- L3 beam level measurement prediction

Proposal. 3: For AI/ML for mobility, associated ID can be required for the use cases utilizing beam-level measurement/prediction to guarantee the consistency of NW’s beam configuration across training and inference.

Inference configuration:

Common part:

Proposal. 4: For all the use cases agreed in AI for mobility, NW can indicate the target MO (Measurement object) for which the UE performs the prediction (e.g., RRM measurement/event prediction) as part of inference configuration.

Proposal. 5: For all the use cases agreed in AI for mobility, NW can indicate the list of target cells for which the UE performs the prediction as part of inference configuration.

Proposal. 6: For all the use cases agreed in AI for mobility, NW can indicate NW-side additional condition (e.g., associated ID) for the applicability check as part of inference configuration if needed.

Temporal domain Case A:

Proposal. 7: For temporal domain Case A, NW can indicate the prediction window length (i.e., how far the UE should predict the measurement results in time domain) as part of inference configuration.

Temporal domain Case B:

Proposal. 8: For temporal domain Case B, NW can indicate either the MRRT value to use or the accuracy requirement (e.g., tolerable RSRP difference) to meet as part of inference configuration.

Proposal. 9: For temporal domain Case B, NW can configure UE to report its measurement skipping pattern as an assistance data for NW.

Measurement event prediction:

Proposal. 10: For measurement event prediction, NW can include the indication to configure UE to initiate the measurement reporting procedure when it predicts the target event within PW as part of inference configuration.

Proposal. 11: For measurement event prediction, NW can indicate the prediction window length (i.e., how far the UE should predict the measurement event in time domain) as part of inference configuration.

Proposal. 12: For measurement event prediction, NW can indicate the target event (e.g., eventId for eventAx) for which the UE performs the measurement event prediction as part of inference configuration.

Frequency domain prediction:

Proposal. 13: For frequency domain prediction, NW can indicate both the target MO/cell for which the UE perform the prediction and the input MO/cell for which the UE perform the measurement for input data as part of inference configuration.

L3 beam-level prediction:

Proposal. 14: For L3 beam-level prediction, NW can indicate the prediction window length (i.e., how far the UE should predict the measurement results in time domain) as part of inference configuration.

Proposal. 15: For L3 beam-level prediction, NW can indicate the maximum number of beams to report (i.e., K) as part of inference configuration (to configure UE to report Top-K beam indexes).

Inference report:

Temporal domain Case A:

Proposal. 16: For temporal domain Case A, UE can report the predicted results for the multiple time instances within PW for the target cells. The target cells can be determined based on the inference configuration from NW.

Temporal domain Case B:

Observation. 3: For temporal domain Case B, UE can trigger the measurement reporting and report the latest (predicted) measurement results as in the legacy. The only difference compared to the legacy is that some of the measurement results used for measurement reporting are not the actual measurement results but the predicted ones.

Proposal. 17: For temporal domain Case B, UE can trigger the measurement reporting and report the latest (real/predicted) measurement results as in the legacy. No further enhancement is needed for measurement (inference) reporting.

Measurement event prediction:

Proposal. 18: For measurement event prediction, the UE can report the latest measurement result and/or the predicted results for the multiple time instances for the applicable cells. The applicable cells can be the cells for which the event is predicted.

Proposal. 19: For measurement event prediction, the UE can report some additional information for the predicted event. The additional information can be the expected occurrence time (for indirect method) or the probability of event occurrence within PW (for direct method).

Proposal. 20: For measurement event prediction, UE can continue the measurement event prediction after sending the first event prediction report. If the event prediction result is updated compared to the last report, the UE can report the updated result to NW.

Frequency domain prediction:

Proposal. 21: For frequency domain prediction, UE can report the latest predicted measurement result for the target cells. The target cells can be determined based on the inference configuration from NW.

L3 beam-level prediction:

Proposal. 22: For L3 beam-level prediction, UE can report the beam-level prediction results for the best neighbor cell. The best neighbor cell can be determined based on the actual or predicted measurement results.

Proposal. 23: For L3 beam-level prediction, the beam-level prediction results for the best neighbor cell can include the predicted Top-K beam indexes and/or the predicted beam-level measurement results for the multiple time instances within PW for the Top-K beams.