3GPP TSG RAN WG2 #130	R2-2503987

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.17
Source:	Toyota ITC
Title:	Discussion on IoT-NTN TDD mode

Document for:	Discussion and Decision

# Introduction

This contribution presents proposals for integrating the agreements reached thus far into the RAN2 specifications for the IoT-NTN TDD mode work item [1]. Additionally, it highlights specific areas that require further discussion and consensus.

# Discussion

2.1	Impacts to TS 36.304, 36.331 and 36.321

So far, the RAN2 working group has reached the following agreements concerning paging, cell barring, timer value ranges, actions for pre-configured resources, semi-persistent scheduling behavior, and the k-Mac value range.

Agreements:

1.	Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe.

2.	In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell.

3.	Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes

4.	When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe (36.331)

5.	When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

6.	For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

While Agreements 2 and 3 do not require any changes, agreements 1, 4, 5, and 6 are proposed to be incorporated into the specifications as follows.

The Paging Occasion (PO) is defined in 36.304, clause 7.1, and is referenced as such in other specifications. Therefore, it is sufficient that the agreement is captured within the 36.304 specification. Any future changes can be made in this same specification, simplifying feature maintenance and evolution.

## TS 36.304	7.1	Discontinuous Reception for paging

The UE may use Discontinuous Reception (DRX) in idle mode in order to reduce power consumption. One Paging Occasion (PO) is a subframe where there may be P-RNTI transmitted on PDCCH or MPDCCH or, for NB-IoT on NPDCCH addressing the paging message. In P-RNTI transmitted on MPDCCH case, PO refers to the starting subframe of MPDCCH repetitions. In case of P-RNTI transmitted on NPDCCH, PO refers to the starting subframe of NPDCCH repetitions unless subframe determined by PO is not a valid NB-IoT downlink subframe then the first valid NB-IoT downlink subframe after PO is the starting subframe of the NPDCCH repetitions. In case of IoT-NTN TDD mode, if the subframe determined by PO is not a valid NB-IoT downlink subframe then the first valid NB-IoT downlink subframe after PO is the subframe of the NPDCCH addressing the paging message. The paging message is same for both RAN initiated paging and CN initiated paging.

The UE actions upon the PUR resource start subframe are specified in 36.331, clause 5.3.3.20. The following addition is proposed to capture the agreement 4.

#### TS 36.331	5.3.3.20	Maintenance of PUR occasions

The UE configured with pur-Config shall:

1>	consider that the first PUR occasion occurs at the H-SFN/SFN/subframe given by:

-	H-SFN = (H-SFNRef + offset) mod 1024 occuring after FLOOR (offset/1024) H-SFN cycles;

-	SFN and subframe indicated by startSFN and startSubframe,

-	if the startSubframe does not align with the uplink subframes for IoT-NTN TDD mode, the UE shall postpone the PUR resource start subframe to the next valid uplink subframe;

where:

-	offset is given by periodicityAndOffset;

-	H-SFNRef corresponds to the last subframe of the first transmission of RRCConnectionRelease message containing pur-Config, taking into account hsfn-LSB-Info;

-	H-SFN cycle corresponds to the duration of 1024 H-SFNs;

The UE behaviour for UL SPS is specified in 36.321, clause 5.10.2. The agreement 5 is proposed to be captured alongside other exceptions at the end of the clause 5.10.2.

### TS 36.321	5.10.2	Uplink

// Unchanged parts omitted //

For NB-IoT UEs, BL UEs or UEs in enhanced coverage SFNstart time and subframestart time refer to SFN and subframe of the first transmission of PUSCH where configured uplink grant was (re-)initialized.

In the event of a resource conflict between multiple UL SPS configurations configured with Uplink Semi-Persistent Scheduling V-RNTI, the UE behaviour is undefined.

In the event of a resource conflict in the same serving cell between the initial transmision within a configured grant bundle from multiple different UL SPS configurations configured with Uplink Semi-Persistent Scheduling C-RNTI, the UE behaviour is undefined.

In the event of a resource conflict between UL SPS with a non-UL NB-IoT subframe for IoT-NTN TDD mode, the UE shall postpone the UL SPS resource to the next valid UL subframe.

For NB-IoT UEs, a configured uplink grant shall be used only for BSR or SPS confirmation transmission, and skipUplinkTxSPS is implicitly configured.

The support for k-MAC with a value range of up to 1023 ms can be captured with the following extension and a restriction in the field description in SIB31.

#### TS 36.331	SIB31

ServingSatelliteInfo-r17 ::=	SEQUENCE {

ephemerisInfo-r17			CHOICE {

stateVectors				EphemerisStateVectors-r17,

orbitalParameters			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r17			SEQUENCE {

nta-Common-r17					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r17				INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

ul-SyncValidityDuration-r17		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900},

epochTime-r17					SEQUENCE {

startSFN-r17					INTEGER (0..1023),

startSubFrame-r17				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Offset-r17					INTEGER (0..1023),

k-Mac-r17						INTEGER (1..512)				OPTIONAL,	-- Need OP

...

[[	k-Mac-r19					INTEGER (513..1023)				OPTIONAL	-- Need OP

]]

}

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in ms. The extension value k-Mac-r19 is only present for the IoT-NTN TDD mode. If the extension is present otherwise, it shall be ignored by the UE. If the field if absent, the UE uses the (default) value of 0.

The proposed changes are based on the agreements reached thus far; however, several aspects still require discussion. Currently, there is no agreed definition for the IoT-NTN TDD mode, which is essential for all impacted specifications. Additionally, the UE capabilities have not yet been addressed, and it is necessary to establish how this feature relates to NB-IoT and TDD. The network must also inform the UE when the feature is enabled or disabled, necessitating appropriate signalling for this purpose.

Observation 1: There is currently no definition for the IoT-NTN TDD mode, but such a definition is necessary for all impacted specifications.

Observation 2: The UE capabilities have not yet been addressed.

Observation 3: The indication of the enabling/activation and disabling/deactivation of the feature has not been discussed yet.

While the signalling for enabling/activation and disabling/deactivation of the feature from the network side can be easily incorporated into a system information block, it is proposed to specify the indication in a future-proof manner, where restrictions and periodicity N are signalled separately. This approach allows for the independent relaxation or addition of restrictions without altering the periodicity value, and vice versa, enables the introduction of a new periodicity value without modifying the restrictions. Therefore, it is proposed to discuss the following proposals.

Proposal 1: Include the N value in the indication from the network side for the sake of future proofness, even if it has a fixed value in Release 19.

Proposal 2: Separate the indication of restrictions from the N value with its own indicator from the network side.

Proposal 3: Introduce the following definition for IoT-NTN TDD mode in the impacted RAN2 specifications:

IoT-NTN TDD mode: allows use of NB-IoT channels with TDD mode for NTN with fixed values of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period.

## 2.2	Way forward on TS 36.306

2.2.1		Relation with NTN NB-IoT and NTN eMTC features

The WID in [1] indicates the following:

Other related Work Items and dependencies:

| Other related Work Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| FS_NR_nonterr_nw | 750040 | Study on NR to support non-terrestrial networks | The proposed WID will make use of the channel model defined by the FS_NR_nonterr_nw study |
| FS_NR_NTN_solutions | 800099 | Study on solutions for NR to support non-terrestrial networks | The proposed WID will leverage solutions identified in FS_NR_NTN_solutions to address some of the key issues associated to NTN |
| LTE_NBIOT_eMTC_NTN | 920169 | NB-IoT/eMTC support for Non-Terrestrial Networks | This WI enhanced the IoT usage via NTN in Rel-17 LTE |
| IoT_NTN_enh | 941004 | IoT NTN enhancements | This WI enhanced the IoT usage via NTN in Rel-18 LTE |
| IoT_NTN_Ph3 | 1020096 | Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 | This REL-19 WI IoT_NTN_Ph3 is enhancing IoT usage via NTN based on FDD so it is a parallel enhancement to the present WI but no dependencies between WIs are expected |

In addition, the WID states the following:

“This feature allows extending 3GPP NB-IoT NTN operation with support for additional NGSO satellite system.

(…)

The new feature will extend the deployment of NB-IoT NTN, by extending (…).

(…)

With this proposed feature, there is a significant opportunity to further expand global NB-IoT NTN service coverage.

(…)

The work item aims to specify enhancements for NB-IoT NTN (…).”

The UE radio access capability parameters specified in clause 4 (UE radio access capability parameters signalled and corresponding to a field in RRC) that were introduced in TS 36.306 under the Rel-17 WI LTE_NBIOT_eMTC_NTN as applicable in NB-IoT are listed in Table 1 from Annex A (cap1 to cap6).

The Optional features without UE radio access capability parameters specified in clause 6 that were introduced in TS 36.306 under the Rel-17 WI LTE_NBIOT_eMTC_NTN as applicable in NB-IoT are listed in Table 2 from Annex A (OF1 and OF2).

The UE radio access capability parameters specified in clause 4 (UE radio access capability parameters signalled and corresponding to a field in RRC) that were introduced in TS 36.306 under the Rel-18 WI IoT_NTN_enh-Core as applicable in NB-IoT are listed in Table 3 from Annex A (cap7 to cap22).

The Optional features without UE radio access capability parameters specified in clause 6 that were introduced in TS 36.306 under the Rel-18 WI IoT_NTN_enh-Core as applicable in NB-IoT are listed in Table 4 from Annex A (OF3 to OF8).

Given the WID, the suggested way forward for this WI is that all the Rel-17/Rel-18 NTN NB-IoT UE capabilities and optional features that are applicable in NB-IoT are also applicable in IoT-NTN TDD mode. This also includes the “child features Y” that are stated as “A UE indicating support of X shall also indicate the support of Y”. Features that are NTN eMTC-only are excluded from the proposal.

Proposal 4: In TS 36.306, all the Rel-17 and Rel-18 UE capabilities or optional features without UE capability that are applicable in NTN NB-IoT are also applicable in IoT-NTN TDD mode.

2.2.2		About UE capabilities applicable “in TDD”

TS 36.306 includes a number of UE capabilities that are applicable “in TDD”. It is our understanding that those UE capabilities do not apply to IoT-NTN TDD. However, a reader may believe that, since “IoT-NTN TDD” includes the word “TDD”, all the UE capabilities that are applicable in TDD are also applicable  in IoT-NTN TDD. It needs to be clarified that this is not the case.

Proposal 5: Add a statement in TS 36.306 reflecting that the UE capabilities that apply in TDD do not apply in IoT-NTN TDD, unless otherwise stated.

2.2.3		About the UE support for IoT-NTN TDD

Two solutions are possible:

Option 1: UE radio access capability parameters signalled and corresponding to a field in RRC (specified in clause 4 of 36.306).

Option 2: Optional features without UE radio access capability parameters (specified in clause 6 of 36.306).

Proposal 6: RAN2 to agree on one of the two options below for IoT-NTN TDD mode:

Option 1: UE radio access capability parameter signalled and corresponding to a field in RRC (specified in clause 4 of 36.306).

Option 2: Optional feature without UE radio access capability parameters (specified in clause 6 of 36.306).

Note: The current associated draft 36.306 CR in Annex B implements option 1, but this can be changed depending on RAN2 agreement.

Annex B in this document includes a draft CR on TS 36.306 that includes the above proposals.

# Conclusion

This contribution discusses IoT-NTN TDD mode. Observation and proposals are summarized as follows:

Observation 1: There is currently no definition for the IoT-NTN TDD mode, but such a definition is necessary for all impacted specifications.

Observation 2: The UE capabilities have not yet been addressed.

Observation 3: The indication of the enabling/activation and disabling/deactivation of the feature has not been discussed yet.

Proposal 1: Include the N value in the indication from the network side for the sake of future proofness, even if it has a fixed value in Release 19.

Proposal 2: Separate the indication of restrictions from the N value with its own indicator from the network side.

Proposal 3: Introduce the following definition for IoT-NTN TDD mode in the impacted RAN2 specifications:

IoT-NTN TDD mode: allows use of NB-IoT channels with TDD mode for NTN with fixed values of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period.

Proposal 4: In TS 36.306, all the Rel-17 and Rel-18 UE capabilities or optional features without UE capability that are applicable in NTN NB-IoT are also applicable in IoT-NTN TDD mode.

Proposal 5: Add a statement in TS 36.306 reflecting that the UE capabilities that apply in TDD do not apply in IoT-NTN TDD, unless otherwise stated.

Proposal 6: RAN2 to agree on one of the two options below for IoT-NTN TDD mode:

Option 1: UE radio access capability parameter signalled and corresponding to a field in RRC (specified in clause 4 of 36.306).

Option 2: Optional feature without UE radio access capability parameters (specified in clause 6 of 36.306).

# Reference

- RP-243293, “Revised WID for introduction of IoT-NTN TDD mode”, Iridium Satellite LLC, December 9-12, 2024.

# Annex A	 UE capabilities and optional features introduced for NTN NB-IoT Rel-17 and Rel-18

|  | Name | Definition | Applicability | Additional support |
| --- | --- | --- | --- | --- |
| Cap1 | ntn-Connectivity-EPC-r17 | Indicates whether the UE supports NTN access. | Only applicable if the UE supports ce-ModeA-r13 or any ue-Category-NB | If the UE indicates this capability the UE shall support the following enhancements: - General: - handling of cellBarred-NTN-r17 and trackingAreaList-r17 in SystemInformationBlockType1(-NB) as specified in TS 36.331 [5]; - reception of SystemInformationBlockType31(-NB) as specified in TS 36.331 [5]; - derivation of its position based on its GNSS measurements; - reporting of the remaining GNSS validity duration as specified in TS 36.331 [5]; - PDCP: - if the UE supports ce-ModeA-r13, discardTimerExt-r17 as specified in TS 36.331 [5]; - RLC: - t-ReorderingExt-r17 as specified in TS 36.331 [5]; - MAC: - estimation of UE-gNB RTT as specified in TS 36.321 [4]; - delaying the start of the RA response window as specified in TS 36.321 [4]; - delaying the start of the mac-ContentionResolutionTimer as specified in TS 36.321 [4]; - if the UE supports ce-ModeA-r13 or if the UE supports any ue-Category-NB and supports sr-WithoutHARQ-ACK-r15, handling of sr-ProhibitTimerOffset-r17 as specified in TS 36.331 [5]; - extending the length of the (UL) HARQ RTT timer as specified in TS 36.321 [4]; - Physical layer: - calculation of the UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris as specified in TS 36.211 [17]; - calculation of the common TA in RRC_IDLE and RRC_CONNECTED as specified in TS 36.213 [17]; - for TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE specific TA estimation, and common TA calculation) and closed (i.e., received TA commands) control loops; - frequency pre-compensation to counter shift the Doppler experienced on the service link; - timing relationship enhancements using higher layer parameters k-Offset-r17 and k-Mac-r17 as specified in TS 36.213 [22]; - segmented UL transmission using higher layer parameters prach-TxDuration-r17, nprach-TxDurationFmt01-r17, nprach-TxDurationFmt2-r17, pucch-TxDuration-r17 and (n)pusch-TxDuration-r17 as specified in TS 36.331 [5]. except for UEs indicating support of ue-Category-NB and ntn-ScenarioSupport-r17 with value GSO. A UE indicating support of ce-ModeA-r13 and ntn-Connectivity-EPC-r17 shall also indicate support of standaloneGNSS-Location. A UE indicating support for any ue-Category-NB and ntn-Connectivity-EPC-r17 is assumed to have GNSS location capability. |
| Cap2 | ntn-TA-report-r17 | Indicates whether the UE supports Timing advance reporting in NTN cell as specified in TS 36.321 | Only applicable if the UE supports ntn-Connectivity-EPC-r17. | It is mandatory to support TA report during initial access (clause 7.10 of 36.306, other features) for UEs which support ntn-TA-Report-r17 as specified in TS 36.321. |
| Cap3 | ntn-pur-TimerDelay-r17 | indicates whether the UE supports delaying the start of the pur-ResponseWindowTimer for NTN operation as specified in TS36.321 [4]. | Only applicable if the UE supports ntn-Connectivity-EPC-r17. | A UE indicating support of ntn-PUR-TimerDelay-r17 shall also indicate support of pur-CP-EPC-CE-ModeA-r16 or pur-UP-EPC-CE-ModeA-r16 or pur-CP-EPC-r16 or pur-UP-EPC-r16. |
| Cap4 | ntn-OffsetTimingEnh-r17 | Indicates whether the UE supports timing relationship enhancements using Differential Koffset as specified in TS 36.321 [4] and TS 36.213 [22]. | Only applicable if the UE supports ntn-Connectivity-EPC-r17. |  |
| Cap5 | ntn-ScenarioSupport-r17 | Indicates whether UE supports NTN features in GSO or NGSO scenario. |  | The UE indicating ntn-ScenarioSupport-r17 shall also indicate support of ntn-Connectivity-EPC-r17. If a UE does not include this field but includes ntn-Connectivity-EPC-r17, the UE supports the NTN features for both GSO and NGSO scenarios. |
| Cap6 | ntn-SegmentedPrecompensationGaps-r17 | Indicates the supported gap length between segments (for PUSCH and PUCCH required by a UE supporting ce-ModeA-r13 or) for NPUSCH required by a UE supporting ue-category-NB, for TA pre-compensation. | Only applicable if the UE supports either ue-category-NB or ce-ModeA-r13 and also supports ntn-Connectivity-EPC-r17. This field is not applicable for UEs indicating support of ue-Category-NB and ntn-ScenarioSupport-r17 with value GSO. | If a UE does not include this field but includes ntn-Connectivity-EPC-r17, in case of overlapped transmission between successive uplink segments, UE shall follow the procedure specified in TS 36.213 [22]. |

Table 1: UE radio access capability parameters (clause 4, UE radio access capability parameters signalled and corresponding to a field in RRC) introduced in TS 36.306 under the Rel-17 WI LTE_NBIOT_eMTC_NTN as applicable in NB-IoT

|  | Name | Definition | Applicability | Additional support |
| --- | --- | --- | --- | --- |
| OF1 | Cell Reselection Measurement Triggering based on Service Time | For a UE camped in NTN cell to support triggering of early cell reselection measurements based on the service time broadcasted by the cell as specified in TS36.304. | Only applicable if the UE supports ntn-Connectivity-EPC-r17 |  |
| OF2 | Discontinuous coverage | For a UE camped on NTN cell to support discontinuous coverage as specified in TS 36.304. | Only applicable if the UE supports ntn-Connectivity-EPC-r17 |  |

Table 2: Optional features without UE radio access capability parameters (clause 6) that were introduced in TS 36.306 under the Rel-17 WI LTE_NBIOT_eMTC_NTN as applicable in NB-IoT

|  | Name |
| --- | --- |
| Cap7 | ntn-LocationBasedMeasTrigger-EFC-r18 |
| Cap8 | ntn-LocationBasedMeasTrigger-EMC-r18 |
| Cap9 | ntn-TimeBasedMeasTrigger-r18 |
| Cap10 | ntn-RRC-HarqDisableSingleTB-r18 |
| Cap11 | ntn-OverriddenHarqDisableSingleTB-r18 |
| Cap12 | ntn-DCI-HarqDisableSingleTB-r18 |
| Cap13 | ntn-RRC-HarqDisableMultiTB-r18 |
| Cap14 | ntn-OverriddenHarqDisableMultiTB-r18 |
| Cap15 | ntn-DCI-HarqDisableMultiTB-r18 |
| Cap16 | ntn-UplinkHarq-ModeB-SingleTB-r18 |
| Cap17 | ntn-HarqEnhScenarioSupport-r18 |
| Cap18 | ntn-Triggered-GNSS-Fix-r18 |
| Cap19 | ntn-Autonomous-GNSS-Fix-r18 |
| Cap20 | ntn-UplinkTxExtension-r18 |
| Cap21 | ntn-GNSS-EnhScenarioSupport-r18 |
| Cap22 | ntn-UplinkHarq-ModeB-MultiTB-r18 |

Table 3: UE radio access capability parameters (clause 4, UE radio access capability parameters signalled and corresponding to a field in RRC) introduced in TS 36.306 under the Rel-18 WI IoT_NTN_enh-Core as applicable in NB-IoT

|  | Name |
| --- | --- |
| OF3 | Early RLF triggering based on service time |
| OF4 | Neighbour cell measurements based on service start time of the neighbour cell |
| OF5 | UE autonomous release based on service time |
| OF6 | Cell reselection measurements triggering based on location |
| OF7 | Cell reselection measurements triggering based on location for earth moving cell |
| OF8 | GNSS measurements during inactive time |

Table 4: Optional features without UE radio access capability parameters (clause 6) that were introduced in TS 36.306 under the Rel-18 WI IoT_NTN_enh-Core as applicable in NB-IoT

# Annex B. Draft CR to TS 36.306

3GPP TSG RAN WG2#130

# Malta, 19-23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | <CR#> | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of IoT-NTN TDD in TS 36.306 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Toyota ITC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_TDD |  |  |  |  |  | Date: |  |  | 2025-05-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of IoT-NTN TDD in TS 36.306 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - List the UE capabilities that apply in IoT-NTN TDD mode: the UE capabilities that were introduced for Rel-17 and Rel-18 NTN NB-IoT. - List the optional UE features (without capability) that apply in IoT-NTN TDD mode: the optional UE features (without capability) introduced for Rel-17 and Rel-18 NTN NB-IoT. - Add a new UE capability for IoT-NTN TDD: iot-ntn-tdd-r19. - Clarify/specify that, unless otherwise stated, the UE capabilities that apply in TDD do not apply in IoT-NTN TDD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature/mode corresponding to the approved Rel-19 WI on IoT-NTN TDD mode will not be included in the specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 4.3.38.x (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 36.331, 36.321, 36.304, 36.300 CR w, x, y, z. |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unaffected part omitted >

# 4	UE radio access capability parameters

The following clauses define the UE radio access capability parameters and minimum capabilities for MBMS capable UE. Only parameters for which there is the possibility for UEs to signal different values are considered as UE radio access capability parameters. Therefore, mandatory features without capability parameters that are the same for all UEs are not listed here. Also capabilities which are optional or conditionally mandatory for UEs to implement but do not have UE radio access capability parameter are listed in this specification.

E-UTRAN needs to respect the signalled UE radio access capability parameters when configuring the UE and when scheduling the UE.

All parameters shown in italics are signalled and correspond to a field defined in TS 36.331 [5].

For optional features, the UE radio access capability parameter indicates whether the feature has been implemented and successfully tested. For mandatory features with the UE radio access capability parameter, the parameter indicates whether the feature has been successfully tested.

The mandatory features required to be supported by a UE are the same for all UE categories unless explicitly specified elsewhere in the specifications.

Unless otherwise stated, the requirements on the maximum number of transport block bits are applicable for a TTI length of 1 ms. For other TTI lengths, the requirements shall be scaled according to clause 7.1.7 or 11.1 in TS 36.213 [22] in order to get the corresponding requirement.

The UE radio access capability parameters that are specified in clause 4 as applicable in TDD are not applicable in IoT-NTN TDD unless otherwise stated.

The following UE radio access capability parameters specified in clause 4 are applicable in NB-IoT:

-	ue-Category-NB in NB-IoT (clause 4.1C)

-	supportedROHC-Profiles-r13 (clause 4.3.1.1A)

-	maxNumberROHC-ContextSessions-r13 (clause 4.3.1.2A)

-	rlc-UM-r15 (clause 4.3.2.5)

-	multiTone-r13 (clause 4.3.4.55)

-	multiCarrier-r13 (clause 4.3.4.56)

-	twoHARQ-Processes-r14 (clause 4.3.4.62)

-	multiCarrier-NPRACH-r14 (clause 4.3.4.75)

-	multiCarrierPaging-r14 (clause 4.3.4.76)

-	interferenceRandomisation-r14 (clause 4.3.4.80)

-	wakeUpSignal-r15 (clause 4.3.4.113)

-	wakeUpSignalMinGap-eDRX-r15 (clause 4.3.4.114)

-	mixedOperationMode-r15 (clause 4.3.4.115)

-	sr-WithHARQ-ACK-r15 (clause 4.3.4.117)

-	sr-WithoutHARQ-ACK-r15 (clause 4.3.4.118)

-	nprach-Format2-r15 (clause 4.3.4.119)

-	multiCarrierPagingTDD-r15 (clause 4.3.4.134)

-	additionalTransmissionSIB1-r15 (clause 4.3.4.137)

-	npusch-3dot75kHz-SCS-TDD-r15 (clause 4.3.4.177)

-	npusch-MultiTB-r16 (clause 4.3.4.182)

-	npdsch-MultiTB-r16 (clause 4.3.4.183)

-	npusch-MultiTB-Interleaving-r16 (clause 4.3.4.192)

-	npdsch-MultiTB-Interleaving-r16 (clause 4.3.4.193)

-	multiTB-HARQ-AckBundling-r16 (clause 4.3.4.194)

-	groupWakeUpSignal-r16 (clause 4.3.4.195)

-	groupWakeUpSignalAlternation-r16 (clause 4.3.4.196)

-	subframeResourceResvUL-r16 (clause 4.3.4.197)

-	subframeResourceResvDL-r16 (clause 4.3.4.198)

-	slotSymbolResourceResvUL-r16 (clause 4.3.4.199)

-	slotSymbolResourceResvDL-r16 (clause 4.3.4.200)

-	npdsch-16QAM-r17 (clause 4.3.4.222)

-	npusch-16QAM-r17 (clause 4.3.4.223)

-	supportedBandList-r13 (clause 4.3.5.1A)

-	multiNS-Pmax-r13 (clause 4.3.5.16A)

-	powerClassNB-20dBm-r13 (clause 4.3.5.1A.1)

-	powerClassNB-14dBm-r14 (clause 4.3.5.1A.2)

-	dl-ChannelQualityReporting-r16 (clause 4.3.6.37)

-	connModeMeasIntraFreq-r17 (clause 4.3.6.49)

-	connModeMeasInterFreq-r17 (clause 4.3.6.50)

-	accessStratumRelease-r13 (clause 4.3.8.1A)

-	multipleDRB-r13 (clause 4.3.8.5)

-	earlyData-UP-r15 (clause 4.3.8.7)

-	earlySecurityReactivation-r16 (clause 4.3.8.11)

-	coverageBasedPaging-r17 (clause 4.3.8.16)

-	anr-Report-r16 (clause 4.3.12.2)

-	rach-Report-r16 (clause 4.3.12.3)

-	locationInfo-r16 (clause 4.3.12.5)

-	logicalChannelSR-ProhibitTimer (clause 4.3.19.2)

-	dataInactMon-r14 (clause 4.3.19.9)

-	rai-Support-r14 (clause 4.3.19.10)

-	earlyContentionResolution-r14 (clause 4.3.19.14)

-	sr-SPS-BSR-r15 (clause 4.3.19.15)

-	rai-SupportEnh-r16 (clause 4.3.19.22)

-	earlyData-UP-5GC-r16 (clause 4.3.36.9)

-	pur-CP-EPC-r16 (clause 4.3.37.1)

-	pur-UP-EPC-r16 (clause 4.3.37.2)

-	pur-CP-5GC-r16 (clause 4.3.37.3)

-	pur-UP-5GC-r16 (clause 4.3.37.4)

-	pur-CP-L1Ack-r16 (clause 4.3.37.5)

-	pur-NRSRP-Validation-r16 (clause 4.3.37.6)

The following UE radio access capability parameters specified in clause 4 are applicable in NB-IoT or IoT-NTN TDD:

-	ntn-Connectivity-EPC-r17 (clause 4.3.38.1)

-	ntn-TA-Report-r17 (clause 4.3.38.2)

-	ntn-PUR-TimerDelay-r17 (clause 4.3.38.3)

-	ntn-OffsetTimingEnh-r17 (clause 4.3.38.4)

-	ntn-ScenarioSupport-r17 (clause 4.3.38.5)

-	ntn-SegmentedPrecompensationGaps-r17 (clause 4.3.38.6)

-	ntn-LocationBasedMeasTrigger-EFC-r18 (clause 4.3.38.11)

-	ntn-LocationBasedMeasTrigger-EMC-r18 (clause 4.3.38.12)

-	ntn-TimeBasedMeasTrigger-r18 (clause 4.3.38.13)

-	ntn-RRC-HarqDisableSingleTB-r18 (clause 4.3.38.14)

-	ntn-OverriddenHarqDisableSingleTB-r18 (clause 4.3.38.15)

-	ntn-DCI-HarqDisableSingleTB-r18 (clause 4.3.38.16)

-	ntn-RRC-HarqDisableMultiTB-r18 (clause 4.3.38.17)

-	ntn-OverriddenHarqDisableMultiTB-r18 (clause 4.3.38.18)

-	ntn-DCI-HarqDisableMultiTB-r18 (clause 4.3.38.19)

-	ntn-UplinkHarq-ModeB-SingleTB-r18 (clause 4.3.38.29)

-	ntn-HarqEnhScenarioSupport-r18 (clause 4.3.38.30)

-	ntn-Triggered-GNSS-Fix-r18 (clause 4.3.38.31)

-	ntn-Autonomous-GNSS-Fix-r18 (clause 4.3.38.32)

-	ntn-UplinkTxExtension-r18 (clause 4.3.38.33)

-	ntn-GNSS-EnhScenarioSupport-r18 (clause 4.3.38.34)

-	ntn-UplinkHarq-ModeB-MultiTB-r18 (clause 4.3.38.35)

The UE radio access capabilities specified in clause 4 are not applicable in NB-IoT, unless they are listed above.

The following UE radio access capability parameter specified in clause 4 is applicable in IoT-NTN TDD:

-	iot-ntn-tdd-r19 (clause 4.3.38.x)

The following optional features without UE radio access capability parameters specified in clause 6 are applicable in NB-IoT:

-	RRC Connection Re-establishment for the Control Plane CIoT EPS Optimization (clause 6.7.5)

-	System Information Block Type 16 (clause 6.8.1)

-	Enhanced random access power control (clause 6.8.3)

-	MT-EDT for Control Plane CIoT EPS Optimisation (clause 6.8.10)

-	MT-EDT for User Plane CIoT EPS Optimisation (clause 6.8.11)

-	EDT for Control Plane CIoT EPS Optimization (clause 6.8.4)

-	Enhanced PHR (clause 6.8.6)

-	Carrier specific NRSRP thresholds for NPRACH resource selection (clause 6.8.15)

-	Radio Link Failure Report for NB-IoT (clause 6.10.2)

-	SC-PTM in Idle mode (clause 6.16.1)

-	Multiple TB scheduling for SC-PTM in Idle mode for NB-IoT (clause 6.16.2)

-	Relaxed monitoring (clause 6.17.1)

-	DL channel quality reporting in Msg3 for the anchor carrier (clause 6.17.2)

-	Serving cell idle mode measurements reporting (clause 6.17.3)

-	NSSS-Based RRM measurements (clause 6.17.4)

-	NPBCH-Based RRM measurements (clause 6.17.5)

-	RRM measurements on non-anchor paging carriers (clause 6.17.6)

-	NRS presence on non-anchor paging carriers (clause 6.17.7)

-	DL channel quality reporting in Msg3 for non-anchor carrier (clause 6.17.8)

-	Assistance information for inter-RAT cell selection to/from NB-IoT (clause 6.17.9)

-	RRC Connection Re-establishment for the Control Plane CIoT 5GS Optimisation (clause 6.18.3)

-	NB-IoT/5GC (clause 6.18.4)

-	MO-EDT for Control Plane CIoT 5GS Optimisation (clause 6.18.5)

-	AS RAI (clause 6.18.6)

-	SystemInformationBlockType33(-NB) reception in a TN cell (clause 6.19.9).

The following optional features without UE radio access capability parameters specified in clause 6 are applicable in NB-IoT or IoT-NTN TDD:

-	Cell reselection measurements triggering based on service time (clause 6.19.1)

-	Discontinuous coverage (clause 6.19.2).

-	Early RLF triggering based on service time (clause 6.19.3).

-	Neighbour cell measurements based on service start time of the neighbour cell (clause 6.19.4).

-	UE autonomous release based on service time (clause 6.19.5).

-	Cell reselection measurements triggering based on location for (quasi-)fixed cell (clause 6.19.6).

-	Cell reselection measurements triggering based on location for earth moving cell (clause 6.19.7).

-	GNSS measurements during inactive time (clause 6.19.8).

The optional features without UE radio access capability parameters specified in clause 6 are not applicable in NB-IoT, unless they are listed above.

< Unaffected part omitted >

## 4.3	Parameters independent of the field ue-Category and ue-CategoryDL / ue-CategoryUL

< Unaffected part omitted >

### 4.3.38	IoT NTN parameters

#### 4.3.38.1	ntn-Connectivity-EPC-r17

This field indicates whether the UE supports NTN access. This field is only applicable if the UE supports ce-ModeA-r13 or any ue-Category-NB. If the UE indicates this capability the UE shall support the following enhancements:

-	General:

-	handling of cellBarred-NTN-r17 and trackingAreaList-r17 in SystemInformationBlockType1(-NB) as specified in TS 36.331 [5];

-	reception of SystemInformationBlockType31(-NB) as specified in TS 36.331 [5];

-	derivation of its position based on its GNSS measurements;

-	reporting of the remaining GNSS validity duration as specified in TS 36.331 [5];

-	PDCP:

-	if the UE supports ce-ModeA-r13, discardTimerExt-r17 as specified in TS 36.331 [5];

-	RLC:

-	t-ReorderingExt-r17 as specified in TS 36.331 [5];

-	MAC:

-	estimation of UE-gNB RTT as specified in TS 36.321 [4];

-	delaying the start of the RA response window as specified in TS 36.321 [4];

-	delaying the start of the mac-ContentionResolutionTimer as specified in TS 36.321 [4];

-	if the UE supports ce-ModeA-r13 or if the UE supports any ue-Category-NB and supports sr-WithoutHARQ-ACK-r15, handling of sr-ProhibitTimerOffset-r17 as specified in TS 36.331 [5];

-	extending the length of the (UL) HARQ RTT timer as specified in TS 36.321 [4];

-	Physical layer:

-	calculation of the UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris as specified in TS 36.211 [17];

-	calculation of the common TA in RRC_IDLE and RRC_CONNECTED as specified in TS 36.213 [22];

-	for TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE specific TA estimation, and common TA calculation) and closed (i.e., received TA commands) control loops;

-	frequency pre-compensation to counter shift the Doppler experienced on the service link;

-	timing relationship enhancements using higher layer parameters k-Offset-r17 and k-Mac-r17 as specified in TS 36.213 [22];

-	segmented UL transmission using higher layer parameters prach-TxDuration-r17, nprach-TxDurationFmt01-r17, nprach-TxDurationFmt2-r17, pucch-TxDuration-r17 and (n)pusch-TxDuration-r17 as specified in TS 36.331 [5] except for UEs indicating support of ue-Category-NB and ntn-ScenarioSupport-r17 with value GSO.

A UE indicating support of ce-ModeA-r13 and ntn-Connectivity-EPC-r17 shall also indicate support of standaloneGNSS-Location. A UE indicating support for any ue-Category-NB and ntn-Connectivity-EPC-r17 is assumed to have GNSS location capability.

< Unaffected part omitted >

#### 4.3.38.38	satelliteInfoConfigDedicated-r18

This field indicates whether the UE can be configured via dedicated signalling with NTN assistance information (i.e., satelliteId-r18 or ephemeris information in measObjectEUTRA) to measure an NTN cell in RRC_CONNECTED as specified in TS 36.331 [5]. This feature is only applicable if the UE supports ce-ModeA-r13. A UE supporting this feature shall also indicate the support of ntn-Connectivity-EPC-r17.

#### 4.3.38.x	iot-ntn-tdd-r19

This field indicates whether the UE supports IoT-NTN TDD mode. A UE supporting this feature shall also indicate the support of ntn-Connectivity-EPC-r17.

< End of changes >