3GPP RAN WG2 Meeting #130	R2-2504183

St Julian’s, Malta, May 19th – 23rd 2025

Agenda Item:	8.8.3

Source:	InterDigital

Title:	Discussion on Uplink Capacity/Throughput Enhancement for NTN

Document for:	Discussion, Decision

# Introduction

An objective for Rel-19 NR non-terrestrial networks (NTN) is to study impacts and modifications to support uplink capacity/cell throughput improvement. Detailed objectives include the following [1]:

Agreement At least one of the OCC techniques when PUSCH repetitions are used will be specified: - Inter-slot time-domain OCC with OCC length 2 - Inter-slot time-domain OCC with OCC length 2 and 4 - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 and 4 - Note: combination of techniques is not precluded - PUSCH repetition Type B is not considered Agreement RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact. - RAN1 assumes no specification impact for CFO grouping - RAN1 does not pursue closed-loop frequency adjustment commands. RAN1 assumes that RAN4 does not define new UE requirements for CFO. Agreement OCC length and OCC sequence for OCC with CG PUSCH Type 1 is configured by RRC higher-layers. - Up to RAN2 whether to signal this with one or two RRC parameters - Note: OCC lengths and sequences to be provided in the table of RRC parameters to be prepared by the WI rapporteur

Work in RAN1 has started discussing on the OCC aspects [2].

This contribution discusses signalling impact for the support of uplink capacity/cell throughput enhacement.

# Discussion

RAN1 is currelty discussing how to signal OCC length and OCC sequence indications (see [2] subclause 3.1.5 and 3.1.6).

## OCC configuration

For DG PUSCH and CG PUSCH type 2, DCI is involved for scheduling and so we can assume that OCC length and index can be signalled by DCI. However, CG PUSCH type 1 is semi-statically configured by RRC and no DCI involvement and so from RAN2 point of view, it’s reasonable to assume that OCC length and index are configured by RRC signalling and so we propose;

Proposal 1:	RAN2 assumes;

- For DG PUSCH and CG PUSCH type 2, OCC length and OCC index won’t be configured by RRC; and

- For CG type 1, OCC length and OCC index are configured by RRC.

## UE capabilities

RAN1 agreement includes the following note:

Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

Proposal 2: RAN2 define separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

# Conclusion

In this contribution the following observations and proposals are made regarding UL Capacity/Cell throughput enhancement:

Proposal 1:	RAN2 assumes;

- For DG PUSCH and CG PUSCH type 2, OCC length and OCC index won’t be configured by RRC; and

- For CG type 1, OCC length and OCC index are configured by RRC.

Proposal 2: RAN2 define separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

# References

RP-243300 – Non-Terrestrial Networks (NTN) for NR Phase 3 – Thales, CATT

R1-2501402 "Feature lead summary #3 of AI 9.11.3 on NR-NTN uplink capacity and throughput", MediaTek