3GPP TSG-RAN WG2 #130	R2-2504257

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	5.1.3.1

Source:	Ericsson

Title:	PUCCH SpatialRelationInfo Lists

Document for:	Discussion

# 1	Introduction

At RAN2#128bis, potential issues with the ToAddMod and ToRelease lists for spatialRelationInfo in PUCCH-Config were discussed, based on this document:

R2-2502884	Issues on PUCCH Spatial Relation Info Configuration	ZTE, Samsung	discussion	Rel-16

:

Postponed, but we will not consider any ASN.1 changes.

In this document, we provide our view on the need for specification clarifications.

# 2	Discussion

## 2.1	SpatialRelationInfo ToRelease lists

There are three ToRelease lists to release SpatialRelationInfo list entries (see Appendix for ASN1 with our comments):

spatialRelationInfoToReleaseList, spatialRelationInfoToReleaseListSizeExt, spatialRelationInfoToReleaseListExt Lists of spatial relation configurations between a reference RS and PUCCH to be released by the UE.

It is obvious that the ASN.1 of these lists are not designed according to the RRC Guidelines in TS 38.331 A.4.3.6 (the guidelines were apparently introduced after these ToRelease lists were designed). E.g., a particular list entry can be identified and released by more than one of the ToRelease lists. Still, this should not be a problem for implementations. A release of a list entity ID 5 releases entity 5, and release of a list entity ID 35 releases entity 35, no matter which of the ToRelease lists is used, and no matter which of the ToAddMod lists that was used to add the list entry. ASN.1 is just a transfer syntax. There is no “type checking”, as in a programming language, and in the RRC specification we typically do not consider potential implementation aspects of this.

Consequently, in our view there is no need for any clarification in the field description above.

We ask RAN2 to agree to the following proposal:

Any of the spatialRelationInfToReleaseLists can be used to remove list entries per identity value. No need for spec change.

## 2.2	SpatialRelationInfo ToAddModlists

At RAN2#129bis, it was raised that the use of the words “...as a single list...” and “...concatenation of...” are conflicting in the field description (highlighted below) and cause confusion. Our understanding of the principles used in RRC is that the concatenation of two lists results in a single list, there is no conflict in the text below.

spatialRelationInfoToAddModList, spatialRelationInfoToAddModListSizeExt , spatialRelationInfoToAddModListExt Configuration of the spatial relation between a reference RS and PUCCH. Reference RS can be SSB/CSI-RS/SRS. If the list has more than one element, MAC-CE selects a single element (see TS 38.321 [3], clause 5.18.8 and TS 38.213 [13], clause 9.2.2). The UE shall consider entries in spatialRelationInfoToAddModList and in spatialRelationInfoToAddModListSizeExt as a single list, i.e. an entry created using spatialRelationInfoToAddModList can be modified using spatialRelationInfoToAddModListSizeExt (or deleted using spatialRelationInfoToReleaseListSizeExt) and vice-versa. If the network includes spatialRelationInfoToAddModListExt, it includes the same number of entries, and listed in the same order, as in the concatenation of spatialRelationInfoToAddModList and of spatialRelationInfoToAddModListSizeExt. If unifiedTCI-StateType is configured for the serving cell, no element in this list is configured.

No spec changes to spatialRelationInfoToAddModList field description to clarify “single” and “concatenated”.

Although we consider no change to the ToAddMod list description above is needed, we are fine to askRAN2 to consider the following proposal to clarify existing text:

Consider whether the field description for spatialRelationInfoToAddModList should be clarified by adding “...as concatenated into a single list...” in 38331 Rapp CR.

In the field description above, there is also text on a ToReleaseList (highlighted). This text can be deleted, since misplaced (in wrong field description) and even incorrect (contradicts that any ToRelease list can be used to release list entries). This text can be deleted in 38331 Rapp CR.

In the field description for spatialRelationInfoToAddModList, delete text “(or deleted using spatialRelationInfoToReleaseListSizeExt)” in 38331 Rapp CR.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Any of the spatialRelationInfToReleaseLists can be used to remove list entries per identity value. No need for spec change.

Proposal 2	No spec changes to spatialRelationInfoToAddModList field description to clarify “single” and “concatenated”

Proposal 3	Consider whether the field description for spatialRelationInfoToAddModList should be clarified by adding “...as concatenated into a single list...” (in 38331 Rapp CR).

Proposal 4	In the field description for spatialRelationInfoToAddModList, delete text “(or deleted using spatialRelationInfoToReleaseListSizeExt)” in 38331 Rapp CR.

# Appendix

## ASN.1 on spatialRelationInfo ToAddModList and ToReleaseList

maxNrofSpatialRelationInfos             INTEGER ::= 8

maxNrofSpatialRelationInfos-plus-1      INTEGER ::= 9

maxNrofSpatialRelationInfos-r16         INTEGER ::= 64

maxNrofSpatialRelationInfosDiff-r16     INTEGER ::= 56      -- Difference between maxNrofSpatialRelationInfos-r16 and maxNrofSpatialRelationInfos

PUCCH-SpatialRelationInfoId ::=         INTEGER (1..maxNrofSpatialRelationInfos) 1..8

PUCCH-SpatialRelationInfoId-r16 ::=     INTEGER (1..maxNrofSpatialRelationInfos-r16)  1..64

PUCCH-SpatialRelationInfoId-v1610::=    INTEGER (maxNrofSpatialRelationInfos-plus-1..maxNrofSpatialRelationInfos-r16) 9..64

spatialRelationInfoToAddModList         SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfo 1..8 entries

OPTIONAL, -- Need N

1..8 R15 entries, (ID 1..8)

spatialRelationInfoToReleaseList        SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfoId

ID 1..8                                                                                                                 OPTIONAL, -- Need N

spatialRelationInfoToAddModListSizeExt-v1610    SEQUENCE (SIZE (1..maxNrofSpatialRelationInfosDiff-r16)) OF PUCCH-SpatialRelationInfo

OPTIONAL, -- Need N

1..56 R15 entries, (ID 1..8)

spatialRelationInfoToReleaseListSizeExt-v1610   SEQUENCE (SIZE (1..maxNrofSpatialRelationInfosDiff-r16)) OF PUCCH-SpatialRelationInfoId

OPTIONAL, -- Need N

1..56 entries (ID 1..8)

This list is redundant, can only relaese same entries as spatialRelationInfoToReleaseList

spatialRelationInfoToAddModListExt-v1610  SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos-r16)) OF PUCCH-SpatialRelationInfoExt-r16

OPTIONAL, -- Need N

1..64 extended entries (ID 9..64)

spatialRelationInfoToReleaseListExt-v1610    SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos-r16)) OF

PUCCH-SpatialRelationInfoId-r16       OPTIONAL, -- Need N

1..64 entries, ID 1..64

With this list you can release any list entry.
According to RRC Guidance, this list should have 56 entries, ID 9..64

PUCCH-SpatialRelationInfo ::=           SEQUENCE {

pucch-SpatialRelationInfoId         PUCCH-SpatialRelationInfoId,1..8

servingCellId                           ServCellIndex                                                    OPTIONAL,   -- Need S

referenceSignal                         CHOICE {

ssb-Index                               SSB-Index,

csi-RS-Index                            NZP-CSI-RS-ResourceId,

srs                                     PUCCH-SRS

},

pucch-PathlossReferenceRS-Id            PUCCH-PathlossReferenceRS-Id,

p0-PUCCH-Id                             P0-PUCCH-Id,

closedLoopIndex                         ENUMERATED { i0, i1 }

}

PUCCH-SpatialRelationInfoExt-r16 ::=       SEQUENCE {

pucch-SpatialRelationInfoId-v1610         PUCCH-SpatialRelationInfoId-v1610   9..64                           OPTIONAL,   -- Need S

pucch-PathlossReferenceRS-Id-v1610        PUCCH-PathlossReferenceRS-Id-v1610                             OPTIONAL,    --Need R

...

}