3GPP TSG-RAN WG2 Meeting #130	R2-2504273

St Julian’s, Malta, May. 19th – 23rd, 2025

Agenda Item:	 8.13.2

Source:	 Huawei, HiSilicon

Title:	 Relay discovery and (re)selection for multi-hop Relay

Document for:  Discussion and Decision

# 1	Introduction

In RAN2 #129bis meeting, we had made some progress on the Relay discovery and (re)selection procedures for multi-hop relay, and reached the following agreements. The FFSs and open issues are highlighted in yellow [1].

RAN2#129bis meeting agreements [1]:

Agreements on Relay discovery and (re)selection: - The discovery protocol stack figure in 38.300 (currently between remote and relay) should include also the relay-to-relay case for L2 multihop. - If the candidate intermediate relay UE is out of coverage without a PC5 connection to a parent relay UE, it can forward discovery messages based on preconfiguration. If it is in (direct) coverage, or out of coverage but PC5 connected to a parent relay UE (and thus indirectly to the network), it needs SIB12 or dedicated configuration to forward discovery messages. - For discovery model B when there is no established PC5 link between the remote UE and the intermediate relay UE, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold. - For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE. FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first. - For discovery model B, the quality of the PC5 links is assumed already to have been checked when the solicitation messages were forwarded, and no AS criterion is needed for the intermediate Relay UE(s) to forward the response message towards the Remote UE. - For discovery model B, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold. - Notification for Link quality degradation, as distinct from existing notification causes, is not supported (no spec impact expected). - When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE). FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment). This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message. - RAN2 will focus solely on the L2 Multi-hop Relay design within the scope of this work item. This does not exclude that some work (e.g., discovery conditions, transmission resources) could be reused in L3 without specific impact under our WI. - When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE. - FFS detailed information in the indication and child UE handling. - FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count. - FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

In this paper, we discuss the FFSs and open issues that are listed in [2] for the Relay discovery and (re)selection procedures.

# 2	Discussion

## Open issues items related to the Relay discovery and (re)selection

### Open issue RRC-6: Discovery Model B with no PC5 Link in multi hop scenario

In the previous meeting there were discussions on Discovery Model B, specifically scenarios where no PC5 link is established between a parent Relay UE and the Remote UE. It was agreed that in Discovery Model B, when there is no established PC5 link between the Remote UE and the Intermediate Relay UE, the Intermediate Relay UE should forward the solicitation message only if the PC5 RSRP between the Remote UE (or First Relay UE) and the Intermediate Relay UE is above a defined threshold.

However, one open point that needs further discussion is regarding the scenario where there is no established PC5 link between the last relay UE and the intermediate relay UE, whether the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE.

First, we need to determine whether this scenario is even possible, or whether the Intermediate Relay UE always first establishes itself as a Remote UE. One potential scenario is that the intermediate relay UE has just powered on and before establishing itself as a Remote UE, it receives a discovery solicitation message from a nearby Remote UE. If the SD-RSRP of the received discovery solicitation message is above a pre-configured threshold, the Intermediate Relay UE may. send/forward the solicitation message to nearby candidate last relay UEs.

In such case it seems reasonable that the candidate last relay UE should also check if the SD-RSRP of the received discovery solicitation message is above a pre-configured threshold. This ensures that the resulting multi-hop path is of sufficient quality communication to proceed.

Another possible scenario involves a UE that is capable of functioning as an intermediate relay UE (i.e., it supports the Rel-19 multi-hop relay feature), but is not currently operating as a relay in a multi-hop configuration, as illustrated in Figure 1. In this figure, the UE is capable of acting as an intermediate relay UE but is currently connected to a Rel-17 single-hop relay. In such a scenario, if a multi-hop remote UE attempts to discover a valid multi-hop path, the intermediate relay UE can participate in Model B discovery by forwarding the discovery message. This enables the remote UE to identify potential relay paths.

Since the PC5 link between the intermediate relay UE and the last relay UE has not yet been established, the last relay UE will evaluate the AS conditions between itself and the intermediate relay UE before deciding whether to send a response message.

Figure 1. Intermediate Relay UE participating in Model B multi-hop relay discovery.

Observation 1: The scenario where there is no established PC5 link between the last relay UE and the intermediate relay UE, and the intermediate relay UE sends/forwards the discovery solicitation message received from a nearby Remote UE before establishing itself as a Remote UE could happen when

- The intermediate relay UE has just powered on.

- A UE that is capable of functioning as an intermediate relay UE, but not currently operating as a relay in a multi-hop configuration, participates in Model B discovery by forwarding the discovery Solicitation message.

Given that such scenario is possible, the next question is whether the same SD-RSRP threshold (in SL-RelayUE-ConfigMH-r19) — that is used by the Intermediate Relay UE to decide whether to forward the solicitation message—can also be applied by the Last Relay UE when determining whether to send a discovery response message to the Intermediate Relay UE. Based on our understanding, if support for this scenario is required, it would be straightforward for the Last Relay UE to apply the same threshold when deciding whether to send the discovery response message.

Proposal 1: If the Last Relay UE needs to determine whether to send a discovery response message to the Intermediate Relay UE, it is proposed that the same SD-RSRP threshold condition used by the Intermediate Relay UE to decide whether to forward the solicitation message be applied. This ensures consistency and simplifies the decision-making process for discovery response handling.

### Open issue RRC-5: NotificationMessageSidelink message handling

In the previous meeting, there was considerable discussion about sending the NotificationMessageSidelink to child UEs. However, we were still unable to reach a conclusion on whether this message can be sent in all scenarios, and whether it should be a forwarded copy of the original notification (with the same cause) or a regenerated message from the intermediate relay, reflecting its own cause based on its reaction.

In the current TS 38.331 specification, for example, when an L2 U2N Relay UE receives a reconfigurationWithSync, it either notifies the upper layers (to trigger the release of the PC5 unicast link) or sends a NotificationMessageSidelink message to the connected L2 U2N Remote UE(s). It is then up to the remote UE to take appropriate action to recover from the situation.

From TS 38.331

Upon L2 U2N Relay UE receiving reconfigurationWithSync, it either indicates to upper layers (to trigger PC5 unicast link release) or sends NotificationMessageSidelink message to the connected L2 U2N Remote UE(s) in accordance with 5.8.9.10.

Based on our understanding the same principle should be extended to the multi-hop scenario as outlined below.

Upon L2 U2N Relay UE receiving reconfigurationWithSync, it either indicates to upper layers (to trigger PC5 unicast link release with its connected downstream child UE(s)) or sends NotificationMessageSidelink message to the connected L2 U2N Remote UE(s) or to the connected downstream child UE(s) in accordance with 5.8.9.10.

When an intermediate relay UE receives a NotificationMessageSidelink message from the parent, it has two options

Option 1 –  Forward a copy of the original notification (with the same cause) to its child UEs.
Option 2 –  Generate a new notification message and send it to the child UEs, with an updated cause that reflects its own response, such as relay reselection or re-establishment.

Based on our understanding the issue with Option 1 is that it will lead to duplicate notifications being sent by the intermediate relay UE: first, a forwarded copy of the parent’s original notification, and then a second notification reflecting the intermediate relay’s own actions. This redundancy is clearly undesirable as it does not provide the child UE with any additional information.

To avoid this, we suggest adopting Option 2, where the intermediate relay UE generates a new notification message. This message would reflect the intermediate relay's own status or reaction (e.g., relay reselection) and is always sent, allowing the Remote UE to take appropriate action based on the most relevant and current context of its parent UE.

Based on our understanding this approach also aligns with the principles established in Rel-17 and helps minimize the impact on the specifications. Hence we propose

Proposal 2: Upon receiving a NotificationMessageSidelink from the Last Relay UE, the Intermediate Relay UE generates a new notification message reflecting its own status or reaction (e.g., relay reselection). This newly generated message is always forwarded to the downstream child UE, enabling the directly or indirectly connected Remote UE to take appropriate action based on the most current and relevant context of its parent UE.

## Other remaining FFS for Relay discovery and (re) selection

### FFS detailed information in the indication and child UE handling

If we agree to go with option 2 as described above, where the intermediate relay UE generates a new notification message and this message would reflect the intermediate relay's own status or reaction (e.g., relay reselection) we can introduce a new indication Type set to e.g. relayUE-RelayReselection

Proposal 3: Upon receiving a NotificationMessageSidelink from the Last Relay UE, the Intermediate Relay UE generates a new notification message reflecting its own status or reaction (e.g., relay reselection) with the new indication Type set to e.g. relayUE-RelayReselection

The Child UE on receiving the message takes appropriate action and performs a relay reselection to re-establish the connectivity with the network. This does not preclude the Child UE to reselect the same parent Relay UE.

Proposal 4: Upon receiving a NotificationMessageSidelink from the parent Relay UE with the Indication Type set to e.g. relayUE-RelayReselection, the child UE performs a relay reselection to re-establish the connectivity with the network.

### On FFS if there are exceptional cases where the indication can be suppressed

Regarding this FFS item, we believe there is no need to suppress the transmission of the NotificationMessageSidelink from the parent UE, even in cases where reselection occurs within the same serving cell and does not result in a change in hop count. This suppression would be an optimization that is not essential and could introduce complexity related to group UE handling—something that has been intentionally avoided since Rel-17. Therefore, we propose the following:

Proposal 5: The transmission of the NotificationMessageSidelink from the parent UE should not be suppressed, even when reselection occurs within the same serving cell and does not lead to a change in hop count.

### On FFS if the notification message is used or we rely on upper layer signalling

In the current TS 38.331 specification, for example, when an L2 U2N Relay UE receives a reconfigurationWithSync, it either notifies the upper layers (to trigger the release of the PC5 unicast link) or sends a NotificationMessageSidelink message to the connected L2 U2N Remote UE(s).

From TS 38.331

Upon L2 U2N Relay UE receiving reconfigurationWithSync, it either indicates to upper layers (to trigger PC5 unicast link release) or sends NotificationMessageSidelink message to the connected L2 U2N Remote UE(s) in accordance with 5.8.9.10.

Both of these options are also applicable to relay UEs in a multi-hop communication scenario. A relay UE may, based on its implementation, either use upper layer signaling to trigger the release of the PC5 unicast link with its connected downstream child UE(s), or send a NotificationMessageSidelink to its connected L2 Uu-based Remote UE(s) and/or downstream child UE(s).

Proposal 6: A relay UE could, based on its implementation, either use upper layer signaling to trigger the release of the PC5 unicast link with its connected downstream child UE(s), or send a NotificationMessageSidelink to its connected L2 Uu-based Remote UE(s) and/or downstream child UE(s).

## Detailed content of the discovery message for relay selection and reselection

There may be several candidate U2N Relay UE(s) or candidate Intermediate Relay UE(s) near the U2N Remote UE during selection and reselection. According to the current spec, the only information available to the Remote UE regarding these candidate relay UE(s) is the PC5 link quality between the remote UE and candidate relay UE. For multi-hop relay, this information may not be sufficient to select a good relay UE.

Considering the case where the Remote UE discovers the following candidate relay UE(s).

- Relay UE 1: The SD-RSRP between the remote UE and the candidate relay UE is value 1, and the relay UE 1 is connected to the gNB through a U2N Relay UE. Value 1 satisfies the relay selection criterion, as it is above the configured threshold.

- Relay UE 2: The SD-RSRP between the remote UE and the candidate relay UE is value 2, and the relay UE 2 is connected to the gNB through  an Intermediate Relay UE and then a U2N Relay UE. Value 2 satisfies the relay selection criterion, and is larger than value 1.

Based on the PC5 link quality, the Remote UE is likely to select the relay UE 2, as the PC5 link quality between the Remote UE and the relay UE2 is better. However, from the QoS perspective, the Relay UE 1 may be better since the hop number between the relay UE 1 and the network is smaller. Therefore, we propose that the hop number of each candidate Intermediate Relay UE be indicated to the U2N Remote UE during the discovery procedure. This additional information could help the Remote UE to select a better relay UE.

Proposal 7: RAN2 should discuss whether/how to support indicating the hop number of the candidate Intermediate Relay UE to the U2N Remote UE during the discovery procedure in AS layer.

Further, consider the following cases that remote UE discovers the candidate relay UE 3 and candidate relay UE 4:

- Relay UE 3: The SD-RSRP between the remote UE and the candidate relay UE 3 is value 1, and the relay UE 3 is connected to the gNB through an Intermediate Relay UE 5 and a last Relay UE Relay UE 6. Value 1 satisfies the relay selection criterion, as it is above the configured threshold.

- Relay UE 4: The SD-RSRP between the remote UE and the candidate relay UE 4 is value 1, and the relay UE 2 is connected to the gNB through an Intermediate Relay UE 7 and a last Relay UE 8. Value 2 satisfies the relay selection criterion, and is larger than value 1.

In above two cases, remote UE would consider relay UE 3 and relay UE 4 are both good, and select one of them based on the hop count information and the SD-RSRP between the remote UE and the candidate relay UE. However, the sidelink RSRP and Uu RSRP of Intermediate Relay UE 5 and last Relay UE Relay UE 6 may be better than the sidelink RSRP and Uu RSRP of Intermediate Relay UE 7 and last Relay UE Relay UE 8. Actually, it is better to select relay UE 3 since the link quality on each hop over the path of relay UE 3 is better than that of relay UE 4. Therefore, it is better that the remote UE can know the condition of each hop of the path related to a candidate relay UE.

Proposal 8. It is beneficial for the Remote UE to know the link condition (SD-RSRP or SL-RSRP) of each hop over the path to select a better candidate relay UE or path.

## 2.4 Support of multiple RSC (Relay Service Code)

SA2 has agreed that the policy of RSC (Relay Service Code) based on single-hop relay [4] can be reused in multi-hop relay [4].

A 5G ProSe Remote UE and a 5G ProSe UE-to-Network Relay shall set up a separate PC5 unicast links if an existing unicast link(s) was established with a different Relay Service Code or without a Relay Service Code.

NOTE 5:	A single PC5 unicast link is established between a 5G ProSe Layer-2 UE-to-Network Relay and a 5G ProSe Layer-2 Remote UE, as specified in TS 38.300 [12], for supporting PDU sessions of the 5G ProSe Layer-2 Remote UE.

Each PC5 unicast link for 5G ProSe UE-to-Network Relay is associated with a Unicast Link Profile, which includes:

-	User Info ID and Layer-2 ID of 5G ProSe Remote UE; and

-	User Info ID and Layer-2 ID of 5G ProSe UE-to-Network Relay; and

-	Relay Service Code; and

-	In the case of 5G ProSe Layer-3 UE-to-Network Relay, the network layer protocol and the information about PC5 QoS Flow(s).

Based on the above policy, we need to consider two cases for a multi-hop paths:

- All remote UEs on this multi-hop relay path use the same RSC to establish a PC5 connection with the first relay UE on this path.

Case 1

- The remote UEs on this multi-hop relay path use different RSCs to establish their own PC5 connection with the first relay UE on this path.

Case 2

In our views, we need to discuss whether the SA2 policy for RSC has spec impacts on RAN2, especially the Case 2 where the multiple PC5 connections may be established with different RSCs between two relay UEs. Whether RAN2 needs to specify anything to support the Case 2.

Proposal 9: RAN 2 needs to discuss whether support for multiple PC5 connections, each associated with different RSCs, is required between two relay UEs.

# 3	Conclusion

Open issues items related to the Relay discovery and (re)selection.

Open issue RRC-6: Discovery Model B with no PC5 Link in multi hop scenario

Observation 1: The scenario where there is no established PC5 link between the last relay UE and the intermediate relay UE, and the intermediate relay UE sends/forwards the discovery solicitation message received from a nearby Remote UE before establishing itself as a Remote UE could happen when

- The intermediate relay UE has just powered on.

- A UE that is capable of functioning as an intermediate relay UE, but not currently operating as a relay in a multi-hop configuration, participates in Model B discovery by forwarding the discovery Solicitation message.

Proposal 1: If the Last Relay UE needs to determine whether to send a discovery response message to the Intermediate Relay UE, it is proposed that the same SD-RSRP threshold condition used by the Intermediate Relay UE to decide whether to forward the solicitation message be applied. This ensures consistency and simplifies the decision-making process for discovery response handling.

Open issue RRC-6: Discovery Model B with no PC5 Link in multi hop scenario

Proposal 2: Upon receiving a NotificationMessageSidelink from the Last Relay UE, the Intermediate Relay UE generates a new notification message reflecting its own status or reaction (e.g., relay reselection). This newly generated message is always forwarded to the downstream child UE, enabling the directly or indirectly connected Remote UE to take appropriate action based on the most current and relevant context of its parent UE.

Other remaining FFS for Relay discovery and (re) selection

FFS detailed information in the indication and child UE handling

Proposal 3: Upon receiving a NotificationMessageSidelink from the Last Relay UE, the Intermediate Relay UE generates a new notification message reflecting its own status or reaction (e.g., relay reselection) with the new indication Type set to e.g. relayUE-RelayReselection

Proposal 4: Upon receiving a NotificationMessageSidelink from the parent Relay UE with the Indication Type set to e.g. relayUE-RelayReselection, the child UE performs a relay reselection to re-establish the connectivity with the network.

FFS if there are exceptional cases where the indication can be suppressed

Proposal 5: The transmission of the NotificationMessageSidelink from the parent UE should not be suppressed, even when reselection occurs within the same serving cell and does not lead to a change in hop count.

FFS if the notification message is used or we rely on upper layer signalling

Proposal 6: A relay UE could, based on its implementation, either use upper layer signaling to trigger the release of the PC5 unicast link with its connected downstream child UE(s), or send a NotificationMessageSidelink to its connected L2 Uu-based Remote UE(s) and/or downstream child UE(s).

Detailed content of the discovery message for relay selection and reselection

Proposal 7: RAN2 should discuss whether/how to support indicating the hop number of the candidate Intermediate Relay UE to the U2N Remote UE during the discovery procedure in AS layer.

Proposal 8. It is beneficial for the Remote UE to know the link condition (SD-RSRP or SL-RSRP) of each hop over the path to select a better candidate relay UE or path.

Support of multiple RSC (Relay Service Code)

Proposal 9: RAN 2 needs to discuss whether support for multiple PC5 connections, each associated with different RSCs, is required between two relay UEs.

# 4	Reference

- Session Chair Notes on Relay and Positioning RAN2 #129bis;

- R2-2504272 Open issues for Multi hop Sidelink Relay in TS 38.331

- R2-2503078 Fast/Parallel RRC Setup for MH relay	Apple	discussion	Rel-19	NR_SL_relay_multihop

- TS 23.304 V19.3.0, Proximity based Services (ProSe) in the 5G System (5GS).