3GPP TSG-RAN2 Meeting #130	R2-2504289

St Julian’s, Malta, 19 – 23 May 2025 	Revision of R2-2502911

Agenda Item:	8.4.3 RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Source: 	Ericsson

Title:  	LP-WUS and RRM measurements

Document for:	Discussion and Decision

# Introduction

In this contribution the open issues for LP-WUS and RRM measurements are discussed further.

For the RAN2 and RAN4 agreements in previous meetings the reader is referred to the Annex.

# Discussion

## Reduced number of thresholds

RAN2 discussed the need to reduce the number of thresholds, counting up to 40 or 56 [1, 2]. RAN2 agreed that for some use cases the same threshold is used, see Appendix:

- The same threshold is used for OFDM-based WUR measuring LP-SS only and OOK-based WUR measuring LP-SS.

- The same threshold is used for serving cell measurement relaxation (Rel-19) and further neighbour cell measurement relaxation (Rel-19)

Open issues:

- Whether different thresholds are needed w.r.t. WUR type (e.g. OFDM or OOK based) or WURs with different receiver sensitivity, see RAN4 LS [3].

- Whether the same threshold is used for LPWUS monitoring and serving cell offloading

- Whether the same threshold is used for LPWUS monitoring and serving cell relaxation (Rel-19)

- Whether the same threshold is used for LPWUS monitoring and further neighbour cell measurement relaxation (Rel-19)

- RRC-8: FFS on the relationship between the thresholds for serving cell relaxation and offloading (, neighboring cell relaxation and legacy measurement thresholds (s-IntraSearchP/s-NonIntraSearchP))

Open issue 1: is outside RAN2 scope, i.e. RAN2 should wait for RAN1/RAN4 progress.

RAN2 to wait for RAN1/RAN4 progress whether different thresholds are needed for different WUR types (e.g. OFDM or OOK) or WURs with different receiver sensitivity (e.g. 18 or 13.5 dB).

Open issue 2: we think that the same threshold can be used, i.e. the LP-WUS deployment will target to obtain similar coverage for LP-WUS and LP-SS signal. In case only OFDM-based WUR is used, then the coverage is determined by LP-WUS only. Furthermore there is little power saving gains when the UE is only allowed to monitor LP-WUS or only allowed to offload serving cell measurements:

The same entry/exit threshold is used for LP-WUS monitoring and serving cell offloading.

Open issue 3: we think that serving cell offloading and serving cell relaxation should not be configured together, i.e. the notion that when the UE moves from cell centre to cell edge that the UE first offloads the serving cell measurements and when approaching the cell edge applies a scaling factor, seems academic to us and complex for the NW to configure. Furthermore, when a moving UE applies a large scaling factor then this is basically the same as when the UE is not measuring at all because the Rel-19 scaling factors are very long. In case the UE is offloading the serving cell measurements and only LR measurements serving cell measurements are performed then the entry condition for serving cell relaxation can only be based on LR measurements but RAN2 already agreed that the entry condition can include both MR and LR measurements (similar as the entry condition for serving cell offloading):

The same entry/exit threshold can be used for LP-WUS monitoring and serving cell relaxation when serving cell offloading is not configured.

Open issue 4: we think that a moving UE in urban environment shall not use a large scaling factor for intra-frequency measurements, i.e. this may impact cell reselection performance and the UE may miss paging. If configured, the further neighbour cell relaxation threshold should be close to the cell centre, ensuring that when the UE is moving there is no stronger intra-frequency neighbour cell to reselect to. However that would significantly reduce the usability of LP-WUS to a small area.

The same threshold can be used for LPWUS monitoring and further neighbour cell measurement relaxation (Rel-19), however with moving UEs in urban environment the area where LP-WUS can be used is then significantly reduced.

Open issue 5: we think that we cannot assume any relationship between LP-WUS thresholds and legacy measurement thresholds (s-IntraSearchP/s-NonIntraSearch). The legacy parameters apply to both normal and LP-WUS UEs and legacy parameters are not expected to change when LP-WUS is deployed:

There is no coupling or dependency between LP-WUS thresholds and legacy measurements thresholds (s-IntraSearchP/s-NonIntraSearch). The legacy parameters apply to both normal and LP-WUS UEs and the legacy parameters are not expected to change when LP-WUS is deployed.

## The need for continuous intra-frequency neighbour cell measurements

In legacy the neighbour cell measurements are configured using intra-frequency (Sintrasearch) and inter-frequency (SnonIntraSearch) measurement thresholds. Typically the UE has to perform intra-frequency neighbour cell measurements in the complete cell and inter-frequency measurements are only triggered when the UE is near the cell border:

In urban environments the intra-frequency (Sintrasearch) cannot be configured close to the inter-frequency (SnonIntraSearch) measurement threshold, because due to the radio propagation conditions a stronger intra-frequency neighbour cell may be detected close to the cell center:

In urban environments the radio conditions may be also change rapidly and dramatically, e.g. when you turn a corner. In case of multi-beam operation the cell coverage may also be beam dependent, i.e. beams of the same cell provide different quality, which may trigger intra-frequency cell reselection:

There can be scenarios where the UE has to measure intra-frequency neighbour cells continuously and these intra-frequency measurements cannot be relaxed.

For this reason it is beneficial to discuss neighbour cell measurements performed by (OFDM-based) LR, i.e. for the use case when intra-frequency neighbour cell measurements are needed and cannot be relaxed [4]. If the LR performs intra-frequency measurements and ranking then the MR is only woken up when cell reselection is initiated, i.e. the MR is only activated when an intra-frequency neighbour is Q-hyst (db) better than the serving cell based on LR measurements:

RAN2 to discuss the need for continuous intra-frequency neighbour cell measurements especially for a moving UE in an urban environment and the implications that has on the usability of LP-WUS.

When Rel-16 neighbour cell measurement relaxation is configured, then a stationary UE is not required to measure intra-frequency. We think that Rel-16 neighbour cell measurement relaxation is a possible way to increase the usability in an urban environment.

## Rel-16 neighbour cell measurements relaxation (RRC-10)

With Rel-16 neighbour cell measurement relaxation it is possible to allow a stationary UE to relax intra-frequency measurement when it is close the cell border. A moving UE is only allowed to relax intra-frequency measurements when it is above SIntraSearch close to the cell centre:

When the LP-WUS entry condition is fulfilled the UE can determine based on LR serving cell measurements whether low mobility and/or not-at-cell-edge criterion are fulfilled:

When Rel-16 neighbour cell relaxation is configured and the LP-WUS entry condition is fulfilled the UE determines based on LR serving cell measurements when “low mobility” and/or “not-at-cell-edge” criterion are no longer fulfilled and the MR has to start neighbour cell measurements again.

When the UE is inside LP-WUS coverage, i.e. LP-WUS entry condition is fulfilled, it is possible that the MR has to start neighbour cell measurements, e.g. the UE become mobile and the low mobility criterion is no longer fulfilled. Then the LR can continue serving cell measurements while the MR is performing neighbour cell measurements. For example if the UE is required to perform neighbour cell measurements every 3rd DRX cycle then it is advantageous to avoid MR serving cell measurements inbetween the MR neighbour cell measurements:

The low mobility and/or not-at-cell-edge criterion are not part of the LR serving cell exit condition, i.e. the UE is allowed to continue offloading the serving cell measurements from MR to LR while the MR is performing neighbour cell measurements.

# Summary

RAN2 is kindly asked to discuss LP-WUS and RRM measurements:

Observation 1	There can be scenarios where the UE has to measure intra-frequency neighbour cells continuously and these intra-frequency measurements cannot be relaxed.

Proposal 1	RAN2 to wait for RAN1/RAN4 progress whether different thresholds are needed for different WUR types (e.g. OFDM or OOK) or WURs with different receiver sensitivity (e.g. 18 or 13.5 dB).

Proposal 2	The same entry/exit threshold is used for LP-WUS monitoring and serving cell offloading.

Proposal 3	The same entry/exit threshold can be used for LP-WUS monitoring and serving cell relaxation when serving cell offloading is not configured.

Proposal 4	The same threshold can be used for LPWUS monitoring and further neighbour cell measurement relaxation (Rel-19), however with moving UEs in urban environment the area where LP-WUS can be used is then significantly reduced.

Proposal 5	There is no coupling or dependency between LP-WUS thresholds and legacy measurements thresholds (s-IntraSearchP/s-NonIntraSearch). The legacy parameters apply to both normal and LP-WUS UEs and the legacy parameters are not expected to change when LP-WUS is deployed.

Proposal 6	RAN2 to discuss the need for continuous intra-frequency neighbour cell measurements especially for a moving UE in an urban environment and the implications that has on the usability of LP-WUS.

Proposal 7	When Rel-16 neighbour cell relaxation is configured and the LP-WUS entry condition is fulfilled the UE determines based on LR serving cell measurements when “low mobility” and/or “not-at-cell-edge” criterion are no longer fulfilled and the MR has to start neighbour cell measurements again.

Proposal 8	The low mobility and/or not-at-cell-edge criterion are not part of the LR serving cell exit condition, i.e. the UE is allowed to continue offloading the serving cell measurements from MR to LR while the MR is performing neighbour cell measurements.

# References

- R2-2502154, Whether/How to reduce the number of thresholds for LP-WUS monitoring and RRM relaxation/offloading, vivo, DISC, RAN2#129-bis

- R2-2503032, Discussion report on [AT129bis][202][LPWUS], vivo, report, RAN2#129-bis

- R2-2501743, LS on LP-WUS UE RF, LS out, RAN4, To:RAN1, Cc:RAN2, RAN4#114

- R2-250xxxx, Discussion on neighboring cell measurement with LR, InterDigital, Ericsson, Nokia, Sony, Vodafone, KT, Turkcell, DISC, RAN2#130

# Annex: Agreements

## RAN2 agreements (selection)

RRM measurement relaxation

RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any).

RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR mesasuring SSB threhold or both.

It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell.

The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ.

How to define LP-RSRP and LP-RSRQ is up to RAN1.

The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided, details up to running CR rapporteur and companies’ review.

Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

Chair observation: only one company have strong concern on the following discussion point

- Can further discuss Direction 3:

      - Direction 3a. merge LPWUS monitoring and serving cell offloading

      - Direction 3b. merge LPWUS monitoring and Rel-19 RRM relaxation

## RAN4 agreements (selection)

Issue 1-1-10: Considerations on higher priority frequency layer

Agreement:

- Support relaxed higher priority frequency layer search at case 1

Issue 1-1-12: On LR measurement metrics LP-RSRP and LP-RSRQ

Agreement:

- For frequency range 1, the reference point for the LP-RSRP and LP-RSRQ shall be the antenna connector of the UE

Issue 1-2-1: General on LR accuracy requirement

Agreement:

No dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin relative to the threshold in the core requirement.

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements

Agreement:

For FR1:

- [±3.5] dB can be used for core requirements for LP-RSRQ accuracy and [±5.5 or ±6] dB can be used for core requirements for LP-RSRP accuracy, under the side conditions Ês/Iot [x, x = -3] dB

- [±3.5] dB can be used for core requirements of SSB based RSRQ accuracy and [±5.5 or ±6] dB can be used for core requirements for SSB based RSRP accuracy, under the side conditions Ês/Iot [y, y = -3] dB

- Note: [2 or 2.5] dB RF impairment margin is assumed

Issue 1-2-2: Periodicity for LP-WUR RRM requirements

Agreement:

Regardless of  whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

Issue 1-2-4-1: On LR measurement filtering requirement

Agreement:

- The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR.

Issue 1-2-5-2: On MR evaluation requirements

Agreement:

- Reuse legacy Nserv

- Note: the legacy values are Nserv = 4 when DRX = 0.32s and 0.64s, Nserv = 2 when DRX = 1.28 and 2.56s

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR and RAN1 LS R1-2501624

Issue 1-2-6-1: On the number of SSBs

Agreement: Aim to conclude the issue by Tuesday in RAN4 #115.

Issue 1-2-6-2: On SSBs number for different SSB periodicities

Agreement

Same SSB number for different SSB periodicity

Agreement

When SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay.

Note: This above agreement based on the assumption that the wake up duration is defined as the ramping time + SSB number * SSB periodicity

Issue 1-2-7: On LP-WUR cell selection evaluation requirements for case 1

Agreement:

- No need to define the LR-WUR based cell selection criterion evaluation in case 1.