3GPP TSG-RAN WG2 Meeting #130	R2-2504390

St.Julians, Malta,  May 19th – 23rd , 2025

# Agenda item:	8.6.4

# Source:	CMCC

# Title:	Discussion on Conditional LTM

# Document for:	Discussion, Decision

- Introduction

In RAN2#129bis meeting, RAN2 reached the following agreements related to conditional LTM [1]:

Agreements on C-LTM

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

- RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

- For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

In this paper, we further discuss the open issues of conditional LTM.

- Discussion

  - Common issues for L1 and L3 based CLTM

Scenario

For  Rel-18 intra-CU LTM, the following scenarios are supported in [2]:

LTM supports both intra-gNB-DU and intra-gNB-CU inter-gNB-DU mobility. LTM supports both intra-frequency and inter-frequency mobility, including mobility to inter-frequency cell that is not a current serving cell. LTM is supported only for licensed spectrum. The following scenarios are supported: - PCell change in non-CA scenario and non-DC scenario; - PCell and SCell(s) change in CA scenario; - Dual connectivity scenario, PCell and MCG SCell(s) change and intra-SN PSCell and SCG SCell(s) change without MN involvement. LTM for simultaneous PCell and PSCell change is not supported.

Observation 1: intra-CU LTM can be used in DC and non-DC scenario.

However, based on the revised WID [3], intra-CU conditional LTM is limited in non-DC case:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

Besides, the current discussion on conditional LTM mainly focuses on the SA scenario, in which conditional LTM is used to modify UE’s PCell.  And it’s not clear whether it can be supported in SCell or PScell change in CA cases. In our opinion, since RAN2 has made progress in supporting early DL/UL synchronization, while the main difference between legacy intra-CU LTM and conditional LTM is the entity for LTM decision. It seems that there’re no technique issues to support the above scenarios in CLTM.

Therefore, intra-CU conditional LTM can be supported PCell change in non-CA scenario and for PCell and SCell(s)  change in CA scenario..

Proposal 1: Conditional LTM can be supported for PCell change in non-CA scenario and for PCell and SCell(s)  change in CA scenario.

Co-existence with other features

In last meeting, RAN2 agreed to support the co-existence of CLTM and Rel-18 MIMO 2TA, and two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC. For Rel-18 intra-CU LTM, cross-feature issues for intra-CU LTM was discussed, except for MIMO 2TA, Rel-18 intra-CU LTM can also coexist with coverage enhancement, and (e)RedCap. As mentioned above, the main difference of LTM and CLTM is due to the execution decision entity changed from NW to UE, and we don’t see techniques to support the coexistence of CLTM and coverage enhancement, MIMO 2TA and (e)RedCap.

Proposal 2: The coexistence of CLTM and coverage enhancement, and (e)RedCap is supported.

Fallback mechanism

For Rel-18 intra-CU LTM, the UE performs either a RACH-less LTM or RACH-based LTM cell switch depending on the availability of a valid TA value in LTM Cell Switch Command. The RACH resource related parameters for LTM cell switch are not present with TA value simultaneously in LTM Cell Switch Command. If UE fails to access to the target cell via RACH-less LTM, it initiates recovery mechanism. In other words, there’s no fallback mechanism from RACH-less LTM to RACH-based LTM for Rel-18 intra-CU LTM. However, it’s different for CLTM, that the RACH resources are configured in LTM configurations as agreed in RAN2. And there’s no need to introduce the restriction to avoid RACH resources and early TA acquisition for the UE. Based on this, the fallback from RACH-less LTM to RACH-based LTM can be supported.

Proposal 3: Fallback mechanism from RACH-less LTM to RACH-based LTM can be supported if RACH resources are preconfigured in CLMT configuration.

TAT of candidate cells handling

In last meeting,RAN2 discussed how to handle the TATs of candidate cells and target cell. For the candidate cell(s) but not target cell’s TAT, it is agreed as following:

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

In other words, even UE performs CLTM, the TA with running TAT can be considered as valid. Now that UE has valid TA, there’s no need to acquire TA again for the same candidate. However, TAT is configured by UE’s previous serving cell, and the validity of one candidate’s TA is only known by the UE and the cell configured the TAT based on current agreements, while target cell has no information about this. To avoid repeated TA acquisition after one CLTM, the target cell should be indicated the information about the running TAT (and TA) of candidate cells. To achieve this, two potential solutions can be considered, one is that UE can report the valid TA related information to the target cel after access to the target cell; the other way is to exchange the TA related information via Xn interface.

Proposal 4: The valid TA related information can be indicated to the target cell via the following ways:

- UE reports the valid TA related information after access to the target cell;

- The source cell indicate TA related information via Xn interface.

  - L3 based CLTM specific issues

In RAN2#128 meeting, RAN2 agreed to support L3 based CLTM that network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell. For a specific UE, the network can configure it with candidate cell(s) with L1 execution condition(s) and candidate cell(s) with L3 execution condition(s), therefore, during the evaluation of execution conditions of CLTM, there could be following cases:

- More than one candidate cells with L1 execution condition fulfill their execution condition.

- More than one candidate cells with L3 execution condition fulfill their execution condition.

- Candidate cell(s) with L1 execution condition and Candidate cell(s) with L3 execution condition fulfill their execution conditions simultaneously.

For  Case1 and Case 2, it’s rational to leave it to UE implementation for target cell selection. For Case 3, some company propose to make some priorities like candidate cells with L3 execution condition is prioritized to be selected than candidate cells with L1 execution condition, thinking L3 measurement results is more stable than L1 measurement results. However, the motivation to introduce LTM is that it takes the advantages of L1 measurement. Therefore, we don’t think it should be taken as an disadvantage of L1 based CLTM. Besides, if candidate selection prioritization is supported, it will introduce extra complexity of UE behavior. Hence, we propose that when multiple candidate cells satisfy the CLM execution condition, it is up to UE implementation to select a cell as target cell and perform CLTM.

Proposal 5: When multiple candidate cells satisfy the CLM execution condition, it is up to UE implementation to select a cell as target cell and perform CLTM, no matter the execution condition is L1 based or L3 based.

For the case that the selected target cell is a candidate cell with L1 based execution condition, it’s agreed in RAN2#129 meeting [4] that When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.  That means the beam selection for L1 based CLTM is based beams’ quality, that only beams satisfying L1-based CLTM condition is selected. Similarly mechanism is used in legacy RACH-less HO to ensure the robustness of L3-based CLTM, that a RSRP threshold can be provided by the network, and the UE will select the beam with CG resources satisfying the RSRP threshold. If there’s multiple beams satisfying the RSRP threshold, it’s UE implementation to select a one a beam and perform CLTM.

Proposal 6: For L3 based CLTM, similar approach to RACH-less HO can be considered, that the UE selects the beam with CG resources satisfying the RSRP threshold.

Proposal 7:  When multiple candidate beams satisfy the RSRP threshold, it is up to UE implementation to select a beam and perform C-LTM.

  - Co-existence and Collision with other mobility mechanism(s)

Following are the agreements on co-existence of intra-CU LTM and other mobility mechanism:

RAN2#121bis meeting agreements[4]: While configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network. R2 assumes that is could be up to the network to avoid any issue due to the race condition between LTM execution and RRC Reconfiguration (e.g. L3 HO cmd), e.g. avoid sending LTM switch cmd and L3 HO cmd in the same TB. RAN2#123bis meeting agreements[5]: It is assumed that L3 handover may happen while LTM is configured / evaluated / used. P4: RAN2 confirms that during network triggered L3 HO / PSCell change, the UE does not autonomously release the LTM configuration. P5: RAN2 confirms that the RRCReconfiguration message to execute an L3 HO or PSCell change procedure may reconfigure (setup, release) the LTM configuration. RAN2#125 meeting agreements[6] No restriction, LTM and conditional reconfiguration can be configured for the same candidate cell (as current TS). Assume that there will be differentiation in the RRC reconfig confirm, so that the target network can know whether CHO or LTM is used. Align with CHO text regarding CHO L3 HO, on priority of LTM cell switch cmd vs execution of CHO. The coexistence of LTM and DAPS HO is not supported in Rel-18.

Based on the agreements above, network triggered L3 HO, CHO and network triggered PScell change (SN change/SN modification) can be configured with intra-CU LTM, while the coexistence of LTM and DAPS HO is not supported in Rel-18.  therefore, we think CLTM can be configured with triggered L3 HO, CHO and network triggered PSCell change, but cannot be configured with DAPS HO.

Observation 2: Network triggered L3 HO, CHO and network triggered PScell change (SN change/SN modification) can be configured with intra-CU LTM, while the coexistence of LTM and DAPS HO is not supported in Rel-18.

Proposal 8: CLTM can coexist with network triggered L3 HO, CHO and network triggered PScell change, while the coexistence of CLTM and DAPS HO is not supported

Based on [7], if the gNB-DU is not able to accept the UE CONTEXT MODIFICATION REQUEST message for mobility because an LTM command has been triggered to the UE, it shall reply with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value. In other words, if there’s HO and LTM collision, DU may refuse CU’s HO command, in other words, LTM has higher priority than L3 HO.

Observation 3: If L3 HO and LTM almost triggered simultaneously, DU may refuse CU’s UE CONTEXT MODIFICATION REQUEST message for mobility, which means the priority of LTM is higher than L3 HO.

Considering the introduction of conditional LTM, two collision cases should be discussed:

- If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied. In this case, UE follows L3 HO command or LTM cell switch command.

- If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time. In this case, we think LTM may have higher priority like priority of legacy LTM if higher than L3 HO, UE performs LTM.

Proposal 9: If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied, UE follows L3 HO command or LTM cell switch command.

Proposal 10: If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time, LTM may have higher priority than CHO, and UE performs LTM.

- Conclusion

In this contribution, we discuss inter-CU LTM related issue. Following are the proposals and observations made in this contribution:

Observations:

Observation 1: intra-CU LTM can be used in DC and non-DC scenario.

Observation 2: Network triggered L3 HO, CHO and network triggered PScell change (SN change/SN modification) can be configured with intra-CU LTM, while the coexistence of LTM and DAPS HO is not supported in Rel-18.

Observation 3: If L3 HO and LTM almost triggered simultaneously, DU may refuse CU’s UE CONTEXT MODIFICATION REQUEST message for mobility, which means the priority of LTM is higher than L3 HO.

Proposals:

Proposal 1: Conditional LTM can be supported for PCell change in non-CA scenario and for PCell and SCell(s)  change in CA scenario.

Proposal 2: The coexistence of CLTM and coverage enhancement, and (e)RedCap is supported.

Proposal 3: Fallback mechanism from RACH-less LTM to RACH-based LTM can be supported if RACH resources are preconfigured in CLMT configuration.

Proposal 4: The valid TA related information can be indicated to the target cell via the following ways:

- UE reports the valid TA related information after access to the target cell;

- The source cell indicate TA related information via Xn interface.

Proposal 5: When multiple candidate cells satisfy the CLM execution condition, it is up to UE implementation to select a cell as target cell and perform CLTM, no matter the execution condition is L1 based or L3 based.

Proposal 6: For L3 based CLTM, similar approach to RACH-less HO can be considered, that the UE selects the beam with CG resources satisfying the RSRP threshold.

Proposal 7:  When multiple candidate beams satisfy the RSRP threshold, it is up to UE implementation to select a beam and perform C-LTM.

Proposal 8: CLTM can coexist with network triggered L3 HO, CHO and network triggered PScell change, while the coexistence of CLTM and DAPS HO is not supported

Proposal 9: If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied, UE follows L3 HO command or LTM cell switch command.

Proposal 10: If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time, LTM may have higher priority than CHO, and UE performs LTM.

- References

- Report of 3GPP TSG RAN WG2 meeting #129;

- TS 38.300 NR and NG-RAN Overall Description; Stage 2 (Release 18);

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom;

- Report of 3GPP TSG RAN WG2 meeting #121;

- Report of 3GPP TSG RAN WG2 meeting #123bis;

- Report of 3GPP TSG RAN WG2 meeting #125;

- TS 38.473 F1 application protocol (F1AP) (Release 18).