3GPP TSG RAN WG2 Meeting #130	R2-2504397

Malta, May 19th – 23rd, 2025

Agenda Item:	8.11.2

Source:	CMCC

Title:	Discussion on random access in SBFD

Document for:	Discussion

Introduction

In RAN2#129bis meeting, the following agreements have been made [1]:

On RO type signaling for CFRA - 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig. - 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated. When both NW indication on RO type and RSRP threshold are absent, it is up to UE implementation to select the RO type. Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM. NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold. Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources. RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

Then in this contribution, we would like to provide our further views on RAN2 impacts to support random access in SBFD.

Discussion

## 2.1 General

### 2.1.1 SBFD operation in LTM

In RAN2#129bis meeting, the following agreement is made in terms of the RA events that support the SBFD operation [1]:

- Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

It is clear that there is still one open issue, that is, whether to support SBFD operation in LTM. Taking the motivation of introducing SBFD in consideration, we slightly prefer to support that LTM can be performed with SBFD RO, since it helps to improve UE’s handover performance, that is, reduce handover latency. While we have also noticed that there are more specification impacts, rather than just adding an RO type indication to EarlyUL-SyncConfig IE. For example, SBFD-aware UEs also need to know the time-domain and frequency-domain SBFD configuration of the candidate cells. Therefore, we are open to discuss whether to support SBFD operation for LTM.

Observation 1: Supporting SBFD operation for early UL sync can help to reduce UE’s handover latency.

Observation 2: There are more specification impacts, rather than just adding an RO type indication to EarlyUL-SyncConfig IE, if SBFD operation is also supported for LTM.

Proposal 1: We are open to discuss whether to support SBFD operation for LTM.

### 2.1.2 SBFD operation in 2-step random access procedure

In RAN1#120 meeting, the following conclusion has been made [2]:

Conclusion There is no consensus in RAN1 to support Type-2 random access procedure (2-step RACH) in SBFD symbols for SBFD-aware UEs.

Since there is no consensus whether to support 2-step RACH or not in RAN1, RAN2 is asked to confirm that 2-step RACH is not supported in R19 and study the RACH configurations for 4-step CBRA and 4-step CFRA. For a comprehensive view, the detailed analysis of the 4-step CBRA and 4-step CFRA PRACH configuration based on Option 1 with Alt 1-1 or Option 2 with Alt 2-3 is conducted in section 2.2.2 and 2.2.3 respectively.

Observation 3: There is no consensus whether to support 2-step RACH or not in RAN1.

Proposal 2: RAN2 is asked to confirm that SBFD operation in 2-step random access procedure is not supported in R19.

### 2.1.3 Determination of RACH configuration option for CFRA

In RAN1 and RAN2, both RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3 is supported. For CBRA, sbfd-RACH-SingleConfig-r19 IE in the BWP-UplinkCommon indicates whether RACH configuration Option 1 is enabled or not. And if sbfd-RACH-DualConfig-r19 IE is included in the BWP-UplinkCommon, it indicates that RACH configuration Option 2 is enabled, otherwise, RACH configuration Option 2 is not enabled. Further taking the agreement that “Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell”, we think that one SBFD-aware UE can determine the RACH configuration option in one cell based on the CBRA RACH configuration option.

Proposal 3: RAN2 confirms that the SBFD-aware UE can determine the RACH configuration option in one cell based on the CBRA RACH configuration option of that cell.

## 2.2 MAC procedure

For a cleaner description, the analysis is divided into two situations: CBRA and CFRA.

### 2.2.1 CBRA case

#### 2.2.1.1 Initial PRACH transmission attempt

In terms of MAC procedure for selecting RO type, the following working assumption has been made in RAN2#129bis meeting that:

Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

Another option discussed in previous meeting is that the selection of RO type is performed after the selection of RA type. For the sake of convenience in description, we will denote the assumption as option a, and denote the scheme of selecting the RO type after selecting the RA type as option b. In fact, both option a and option b work, but may result in different specification impacts and performances. For option a, it seems to have less specification impacts in TS 38.321, because the SBFD-aware UE can determine whether the MSG1 repetition is applicable and the number of MSG1 repetitions after determining the RO type. However, we are not sure whether it will degrade the performance of RACH, such as early indication, etc., because option a seems to make the priority of the selection based on feature combination lower than that of the selection of the RO type. For example, if RACH configuration with slicing feature is configured only in legacy ROs and a SBFD-aware UE with network slicing selects additional ROs before selecting the set of random access resources, it can only use the set(s) of Random Access resources that are not associated with any feature indication. For option b, the SBFD-aware UE needs to determine the RACH resource of feature combinations on both types of ROs, which increases the specification impacts. However, it may better ensure the performance of the UE early indication. Note that the performance of the early indication can be ensured by configuring RACH resources for a certain feature/feature combination by network in both the additional RO and the legacy RO for Option a. Therefore, we are okay to confirm the working assumption.

Observation 4: Both the scheme that RO type selection before the selection of set of random access resources and the scheme that RO type selection after RA type selection can work, but may result in different specification impacts and performances.

Proposal 4: The work assumption that for SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources, can be confirmed.

#### 2.2.1.2 PRACH transmission re-attempt

For PRACH transmission re-attempt, it has been agreed in RAN2#129bis meeting that [1]:

RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

- Further considering that the fallback from 2-step RACH to 4-step RACH with legacy RO is supported in legacy NR, RAN2 may need to discuss whether the direct fallback from 2-step RACH to 4-step RACH with additional RO is supported or not, and if supported, how SBFD-aware UEs determine whether to fallback to 4-step RACH with additional RO or to 4-step RACH with legacy RO, especially for RACH configuration Option 1 case. For us, the direct fallback from 2-step RACH to 4-step RACH with additional RO may help to reduce the latency of random access procedure compared with 2-step RACH fallback to 4-step RACH with legacy RO and then fallback to 4-step RACH with additional RO.

Proposal 5: RAN2 is asked to discuss whether the direct fallback from 2-step RACH to 4-step RACH with additional RO is supported or not.

### 2.2.2 CFRA case

#### 2.2.2.1 Determination of RO type

In RAN2#129bis meeting [1], the following agreements are made for CFRA:

On RO type signaling for CFRA - 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig. - 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated.

Therefore, we think that the remaining issue of the RO type indication for CFRA is related to whether RAN2 supports SBFD operation in LTM as described in section 2.1.1. As for the SBFD-aware UE’s behavior for determining the RO type for CFRA, we think the they just follow the explicit indication from the network.

Observation 5: The remaining issue of the RO type indication for CFRA is related to whether RAN2 supports SBFD operation in LTM.

#### 2.2.2.2 Fallback from CFRA to CBRA

- In legacy NR, if CFRA resource is indicated, the UE should compare the measured RSRP with the configured threshold to determine whether the CFRA resource can be used. To be specific, if the measured RS-RSRP is above and configured power threshold, the UE shall use the indicated CFRA resource, otherwise, the UE shall use the CBRA resource. And when the current random access procedure is unsuccessfully completed, UE should compare the measured RS-RSRP and configured power threshold to determine whether CFRA resource or CBRA resource shall be used again next time.

- With the introduction of SBFD operation, one SBFD-aware UE can be indicated by the network to use CFRA resource with addition RO or with legacy RO. In this case, a natural issue is which RO type for CBRA resource the SBFD-aware UE should use when the network indicates the CFRA resource with one type of RO but its measured RSRP is not above the configured threshold. From our point of view, there are following two alternatives:

Alt 1: SBFD-aware UE uses the CBRA resource with same RO type as that indicated in CFRA resource.

Alt 2: SBFD-aware UE selects the RO type based on the RO type indication of CBRA or configured RSRP threshold and corresponding rule (below/above the RSRP threshold) or its implementation (if both RO type indication and RSRP threshold are absent).

Alt 1 seems to be a simple and reasonable solution because the SBFD-aware UE only needs to consider the RO type indication of the CFRA resource as that of CBRA resource.

Observation 6: There are two alternatives for the RO type determination when fallback from CFRA to CBRA is performed:

Alt 1: SBFD-aware UE uses the CBRA resource with same RO type as that indicated in CFRA resource.

Alt 2: SBFD-aware UE selects the RO type based on the RO type indication of CBRA or configured RSRP threshold and corresponding rule (below/above the RSRP threshold) or its implementation (if both RO type indication and RSRP threshold are absent).

Proposal 6: RAN2 is asked to support that SBFD-aware UE uses the CBRA resource with same RO type as that indicated in CFRA resource when fallback from CFRA to CBRA is performed.

## 2.3 Issues related to MSG1 repetition

In RAN1#120bis meeting, the following agreement has been made [3]:

It is up to RAN2 to determine the following issues for PRACH transmission with preamble repetition: - RO group selection for initial PRACH transmission attempt in one RACH procedure - RO group switch for PRACH transmission re-attempt in one RACH procedure

For RO group selection for initial PRACH transmission attempt, we think it is performed during the selection of the set random access resources, which is same as that in the legacy NR. Thus, we will give a brief analysis about the RO group switch during one RACH procedure in this section.

### 2.3.1 Increase in number of MSG1 repetition

In legacy NR, the fallback from transmission with lower MSG1 repetition number to higher MSG1 repetition number is supported. Then one open issue is that whether to support the fallback transmission with lower MSG1 repetition number to higher MSG1 repetition number in addition RO. With the consideration that higher MSG1 repetition number in addition RO can also bring better uplink coverage performance for SBFD-aware UEs, the fallback from transmission with lower MSG1 repetition number to higher MSG1 repetition number in addition RO should be supported. While, in order to avoid introducing additional complexity for SBFD-aware UEs, such fallback behavior should be restricted to RACH resources with the same feature/feature combination, which is also stipulated in the existing TS 38.321, Clause 5.1.4.

Observation 7: Higher MSG1 repetition number in addition RO can also bring better uplink coverage performance for SBFD-aware UEs.

Proposal 7: RAN2 is asked to support the fallback from transmission with lower MSG1 repetition number to higher MSG1 repetition number in addition RO.

Proposal 8: The fallback from transmission with lower MSG1 repetition number to higher MSG1 repetition number in addition RO is restricted to RACH resources with the same feature/feature combination.

### 2.3.2 RO type switch with MSG1 repetition

From our point of view, the switch of RO type also needs to be considered along with the change in the number of preamble repetition. To this end, one remaining issue is that whether both RO type switch and increase of preamble repetition number are allowed during one RACH procedure. From our point of view, RAN2 is asked to support this because it can bring better performance to 4-step CBRA. With that, we think that both the switching of RO type and increasing on the preamble repetition number should be based on the existing counter PREAMBLE_TRANSMISSION_COUNTER. And one SBFD-aware UE can determine whether to increase the number of repetitions of Msg1 first or to switch the RO type first based on the values of the configured preambleTransMax-Msg1-Repetition and the maximum number of PRACH transmission attempts with additional RO, denoted as preambleTransMaxRO-Type here.

#### 2.3.2.1 Case 1: RACH Configuration Option 1

- For RACH configuration Option 1, only rsrp-ThresholdMsg1-RepetitionNum2/4/8 can be separately configured. We assume preambleTransMax-Msg1-Repetition is less than preambleTransMaxRO-Type which is configured for RO type switch, here. When the PREAMBLE_TRANSMISSION_COUNTER satisfies certain condition, then one SBFD-aware UE should firstly increase the number of PRACH repetitions, but use same RO type (i.e., legacy RO or additional RO) as the earlier PRACH repetitions. Then when the number of PRACH repetitions equals the preambleTransMaxRO-Type plus 1, SBFD-aware UE use the other RO type. In terms of the repetition number before and after the RO type switch, SBFD-aware UE uses the same MSG1 repetition number. UE can continue increasing the number of MSG1 repetitions until reaches the maximum configured Msg1 repetition number configured for the RO type after RO type switch. Table 1 gives an example about the above analysis.

Table 1. Example of RO type and repetition number selection for RACH configuration Option 1

| Initial attempt | Re-attempt | Re-attempt | Re-attempt |
| --- | --- | --- | --- |
| additional RO Repetition N=2 | additional RO Repetition N=4 | legacy RO Repetition N=4 | legacy RO Repetition N=8 |

#### 2.3.2.2 Case 2: RACH Configuration Option 2

- For RACH configuration Option 2, both the repetition number and RSRP threshold can be separately configured. We assume preambleTransMax-Msg1-Repetition is less than preambleTransMaxRO-Type which is configured for RO type switch, here. When the PREAMBLE_TRANSMISSION_COUNTER satisfies certain condition, then one SBFD-aware UE should firstly increase the number of PRACH repetitions, but use same RO type (i.e., legacy RO or additional RO) as the earlier PRACH repetitions. Then when the number of PRACH repetitions equals the preambleTransMaxRO-Type plus 1, SBFD-aware UE switch to the other RO type if the current Msg1 repetition number or higher Msg1 repetition number is also configured for the type after RO type switch. In terms of the repetition number before and after the RO type switch, the SBFD-aware UE uses the minimum repetition number that is not lower than the MSG1 repetition number before RO type switch and is also configured for the other RO type. SBFD-aware UE can continue increasing the number of MSG1 repetitions until reaches the maximum configured Msg1 repetition number configured for the RO type after RO type switch. The following Table 2 gives an example about the above analysis, with the assumption that repetition number 8 are configured for legacy RO and repetition number 2, 4 and 8 are configured for additional RO in RACH configuration 2.

Table 2. Example of RO type and repetition number selection for RACH configuration Option 2

| Initial attempt | Re-attempt | Re-attempt |
| --- | --- | --- |
| additional RO Repetition N=2 | additional RO Repetition N=4 | legacy RO Repetition N=8 |

Based on the above analysis, we think that a unified description can be used for both RACH configuration Option 1 and RACH configuration Option 2. When RO switch is performed, the SBFD-aware UE uses the minimum repetition number that is not lower than the MSG1 repetition number before RO type switch and is also configured for the other RO type. SBFD-aware UE can continue increasing the number of MSG1 repetitions until reaches the maximum configured Msg1 repetition number configured for the RO type after RO type switch.

Proposal 9: When RO switch is performed, the SBFD-aware UE uses the minimum repetition number that is not lower than the MSG1 repetition number before RO type switch and is also configured for the other RO type.

Proposal 10: After RO type switch, SBFD-aware UE can continue increasing the number of MSG1 repetitions until reaches the maximum configured Msg1 repetition number configured for the RO type.

## 2.4 Msg3-based early indication

Msg1-based early indication in legacy ROs has been agreed not to be supported in RAN2. For Msg3-based early indication, it has been widely discussed in RAN2#128 [4] but no consensus was reached. If Msg3-based early indication is supported, dedicated LCID(s) can be allocated to SBFD-aware UEs. With that, gNB can identify a SBFD-aware UE based on the logical channel ID in Msg3 and thus schedule subsequent uplink transmission, such as: NAS messages within NAS initial access procedure, etc., in SBFD symbol(s) for SBFD-aware UE. In terms of using reserved LCID(s) in present NR, there is almost no influence on the SBFD-aware UE itself and other UEs. However, if Msg3-based early indication is not supported, gNB can only identify a SBFD-aware UE through UE capability transfer which is later than that through Msg3-based early indication.

From our point of view, RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs, because there is no impact on PRACH capacity of non-SBFD-aware UEs and can bring benefits in increasing the performance of SBFD operation.

Observation 8: With Msg3-based early indication, there is almost no impact on PRACH capacity of non SBFD-aware UEs.

Observation 9: If Msg3-based early indication is not supported, network can only identify a SBFD-aware UE through UE capability transfer which is later than that through Msg3-based early indication.

Proposal 11: RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs.

## 2.5 RA-RNTI collision

For RACH configuration Option 2, it is possible that the additional ROs configured by the additional RACH configurations overlap with the legacy ROs. Such case may happen in flexible symbols, because both legacy and additional ROs can be configured in flexible symbols. When this overlapping happens, there is a possibility that RA-RNTI collision occurs. We are open to discuss whether to solve RA-RNTI collision issue. From our point of view, there are many solutions to solve this issue, such as:

- Solution 1: It is up to network implementation to solve this.

- Solution 2: A configurable offset of symbol index (s_id) or frequency index (f_id) can be introduced.

- Solution 3: An additional offset can be introduced to the formula for computing RA-RNTI, such as:

- RA-RNTI= 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 4  (1)

- For solution 1, there is no any spec impact. However, some companies think that solution may not solve RA-RNTI collision issue efficiently at some times. Solution 2 seems to be the most flexible solutions among the above three solutions, but RAN2 needs to discuss which message should be used for indicating the configured offset and design the corresponding IE. In terms of solution 3, RAN2 needs to define a new formula for computing RN-RNTI when addition RO is used by SBFD-aware UE, but do not need to define any new message or IE. Therefore, if RA-RNTI collision issue is agreed to be addressed by RAN2, we prefer to introduce an additional offset to the formula for computing RA-RNTI in additional RO. In addition, when designing the offset value, it is necessary to avoid collisions with the legacy RA-RNTI and MSGB-RNTI, as given in formula (1).

Observation 10: There are many solutions to solve RA-RNTI collision issue:

- Solution 1: It is up to network implementation to solve this.

- Solution 2: A configurable offset of symbol index (s_id) or frequency index (f_id) can be introduced.

- Solution 3: An additional offset can be introduced to the formula for computing RA-RNTI, such as:

Observation 11: When designing the offset value in the formula for computing RA-RNTI in additional ROs, it is necessary to avoid collisions with the legacy RA-RNTI and MSGB-RNTI.

Proposal 12: If RAN2 agrees to address the RA-RNTI collision issue, introducing an additional offset to the formula for computing RA-RNTI in additional RO is preferred.

# Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on random access in SBFD:

Observation 1: Supporting SBFD operation for early UL sync can help to reduce UE’s handover latency.

Observation 2: There are more specification impacts, rather than just adding an RO type indication to EarlyUL-SyncConfig IE, if SBFD operation is also supported for LTM.

Observation 3: There is no consensus whether to support 2-step RACH or not in RAN1.

Observation 4: Both the scheme that RO type selection before the selection of set of random access resources and the scheme that RO type selection after RA type selection can work, but may result in different specification impacts and performances.

Observation 5: The remaining issue of the RO type indication for CFRA is related to whether RAN2 supports SBFD operation in LTM.

Observation 6: There are two alternatives for the RO type determination when fallback from CFRA to CBRA is performed:

Alt 1: SBFD-aware UE uses the CBRA resource with same RO type as that indicated in CFRA resource.

Alt 2: SBFD-aware UE selects the RO type based on the RO type indication of CBRA or configured RSRP threshold and corresponding rule (below/above the RSRP threshold) or its implementation (if both RO type indication and RSRP threshold are absent).

Observation 7: Higher MSG1 repetition number in addition RO can also bring better uplink coverage performance for SBFD-aware UEs.

Observation 8: With Msg3-based early indication, there is almost no impact on PRACH capacity of non SBFD-aware UEs.

Observation 9: If Msg3-based early indication is not supported, network can only identify a SBFD-aware UE through UE capability transfer which is later than that through Msg3-based early indication.

Observation 10: There are many solutions to solve RA-RNTI collision issue:

- Solution 1: It is up to network implementation to solve this.

- Solution 2: A configurable offset of symbol index (s_id) or frequency index (f_id) can be introduced.

- Solution 3: An additional offset can be introduced to the formula for computing RA-RNTI, such as:

Observation 11: When designing the offset value in the formula for computing RA-RNTI in additional ROs, it is necessary to avoid collisions with the legacy RA-RNTI and MSGB-RNTI.

Proposal 1: We are open to discuss whether to support SBFD operation for LTM.

Proposal 2: RAN2 is asked to confirm that SBFD operation in 2-step random access procedure is not supported in R19.

Proposal 3: RAN2 confirms that the SBFD-aware UE can determine the RACH configuration option in one cell based on the CBRA RACH configuration option of that cell.

Proposal 4: The work assumption that for SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources, can be confirmed.

Proposal 5: RAN2 is asked to discuss whether the direct fallback from 2-step RACH to 4-step RACH with additional RO is supported or not.

Proposal 6: RAN2 is asked to support that SBFD-aware UE uses the CBRA resource with same RO type as that indicated in CFRA resource when fallback from CFRA to CBRA is performed.

Proposal 7: RAN2 is asked to support the fallback from transmission with lower MSG1 repetition number to higher MSG1 repetition number in addition RO.

Proposal 8: The fallback from transmission with lower MSG1 repetition number to higher MSG1 repetition number in addition RO is restricted to RACH resources with the same feature/feature combination.

Proposal 9: When RO switch is performed, the SBFD-aware UE uses the minimum repetition number that is not lower than the MSG1 repetition number before RO type switch and is also configured for the other RO type.

Proposal 10: After RO type switch, SBFD-aware UE can continue increasing the number of MSG1 repetitions until reaches the maximum configured Msg1 repetition number configured for the RO type.

Proposal 11: RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs.

Proposal 12: If RAN2 agrees to address the RA-RNTI collision issue, introducing an additional offset to the formula for computing RA-RNTI in additional RO is preferred.

Reference

- “Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others”, RAN2#129bis, Wuhan, China.

- “RAN1 Chair’s Notes”, RAN1#120, Athens, Greece.

- “RAN1 Chair’s Notes”, RAN1#120bis, Wuhan, China.

- “Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD”, RAN2#128, Orlando, USA.