3GPP TSG-RAN2 Meeting #130	R2-2504460

St.Julians, Malta,  May 19th – 23rd , 2025

Source: 			ZTE Corporation

Title: 	Discussion on configuration and report of inference input to the network side model

Agenda item:	8.3.3

Document for: 	Discussion and Decision

- Introduction

In the last meeting, we discussed the LCM and spec impact for AI mobility and achieved the following agreements [1]:

1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management - Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM) - The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference - For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements. FFS details - For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements. FFS details - Baseline is that this applies to all A1-6 events, unless technical problems are identified. Baseline quantity is RSRP.

In this document, we’d like to share our view on the configuration and report of the inference input to network

- Discussion

For the measurement configuration for AI-assist measurement, the existing RRM measurement configuration can be taken as baseline. The network can indicate the UE which frequency that the UE shall perform inference on. And since three RRM sub use cases are considered for cell-level RRM measurement prediction, the network needs to collect different types of measurement results, e.g. L1 filtered beam level measurement results (e.g. for sub use case 1) or L3 filtered/unfiltered cell level measurement result(s) (e.g. for sub use case 2) for different types of AI model. So, the network can configure the UE which type of the measurement results need to be reported.

Proposal 1: For inference of network side model, the network can configure the frequency that the UE shall perform inference on and the type of the reported measurement results (e.g. L1 filtered beam results, L3 filtered cell results) to the UE.

For temporal domain prediction, in the current simulation, the measurement interval and/or prediction interval are fixed. However, based on the current specification, when to perform measurement is up to UE implementation, the UE measurement only needs to satisfy the measurement periodicity requirements provided in the TS 38.133. For the model training and inference, in order to obtain better prediction accuracy, it’s expected to have measurement results with fixed measurement interval. So, in our view, the network can configure an expected L1/L3 measurement interval to the UE, the UE is expected to perform measurement with configured measurement interval.

Proposal 2: For temporal domain prediction case A and case B, the network can configure an expected measurement interval to the UE.

For temporal domain prediction case A, although there are multiple measurement results within the observation, since the partial overlapping between the adjacent observation window, the UE can report the measurement results of only one time instance at one measurement report, the network can obtain the multiple measurement results within observation window using new reported measurement results and historical reported measurement results. For example, in the figure below, at the time instance 2, the network predicts the future measurement results at time instance 3 and 4 based on the historical measurement results at time instance 1 and 2; and then, at time instance 3, since the network already has the measurement results at time instance 2, the UE only needs to report the measurement results at time instance 3 for network side model inference.

Figure: an example for temporal domain prediction Case A

However, for temporal domain case B, since the observation windows for two adjacent prediction are not overlapped, the UE needs to report measurement results at multiple time instance within observation window to the network (in order to obtain the measurement results within observation window). If using existing measurement reporting mechanism, the measurement report only includes the measurement results at only one time instance. Thus, multiple measurement reports are needed. In order to reduce signaling overhead, for AI-mobility, the network can indicate the number of the continuous measurement to the UE, and then the UE reports the measurement results of the continuous measurement to the network via one measurement report. So, the logging of the measurement results shall be supported for data collection for NW-sided model.

Proposal 3: For temporal domain prediction case B, the network can configure the number of continuous measurements to the UE. And the existing measurement framework is enhanced to support the UE to report the measurement results of multiple historical time instances in one measurement report.

Finally, in the existing measurement configuration, the UE always reports the RSRP, RSRQ and available SINR results for the serving cells configured with servingCellMO, which may be not needed for model inference. Regarding reporting input of the inference, in order to save signaling overhead, RAN2 to study the enhancement to allow the UE to not report RSRQ/RSRQ results of all serving cell that configured with servingCellMO.

Observation 1: In the existing measurement report, the measurement report always includes RSRP, RSRQ and available SINR results for the serving cell configured with servingCellMO, which may be not needed for model inference.

Proposal 4: For inference in network side model, enhancement is needed to allow the UE to not report RSRP/RSRQ results of all serving cells that configured with servingCellMO.

- Conclusion and proposals

Observation 1: In the existing measurement report, the measurement report always includes RSRP, RSRQ and available SINR results for the serving cell configured with servingCellMO, which may be not needed for model inference.

Proposal 1: For inference of network side model, the network can configure the frequency that the UE shall perform inference on and the type of the reported measurement results (e.g. L1 filtered beam results, L3 filtered cell results) to the UE.

Proposal 2: For temporal domain prediction case A and case B, the network can configure an expected measurement interval to the UE.

Proposal 3: For temporal domain prediction case B, the network can configure the number of continuous measurements to the UE. And the existing measurement framework is enhanced to support the UE to report the measurement results of multiple historical time instances in one measurement report.

Proposal 4: For inference in network side model, enhancement is needed to allow the UE to not report RSRP/RSRQ results of all serving cells that configured with servingCellMO.

- Reference

- Report of RAN2 129bis meeting.