3GPP TSG-RAN WG2 #130	R2-2504637

St. Julian’s, Malta, 19th – 23rd May, 2025

Agenda Item:	8.1.3

Source:	Ericsson

Title:	NW-side data collection for beam management and positioning

Document for:	Discussion

# 1	Introduction

In this contribution we share our views on remaining open issues listed in [1] and further aspects.

# 2	Discussion on NW-side data collection for beam management

## 2.1 RRC open issues from [1]

### Open issue RRC-18: NW control on retaining logged data at HO

RAN2#129bis agreed the following:

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

Given the agreement above, it should be clarified whether/how the 1-bit indication can be sent during or before HO, taking into account that the source gNB decides whether the data should be kept. It is not possible for the source gNB to add the 1-bit indication during HO, in the same transmission that includes the RRCReconfiguration from the target gNB. Thus, we see the following two possible solutions for this issue:

- The source gNB sends the 1-bit indication to the UE before HO. With this solution the source gNB may need to decide in advance, before it knows which is the target gNB, whether the UE should keep or not the logged data upon a HO. Thus, this gives less flexibility to the source gNB, which may need to take a conservative decision and configure the UE to discard the logged data upon any HO.

- The source gNB decides if the 1-bit indication is needed and, if so, sends it to the target gNB, which includes it in the RRCReconfiguration sent to the UE during HO. This solution is superior to the first one, because it allows the source gNB to decide whether the logged data should be kept or not right at the HO time, when it knows which is the target gNB and can take a more informed decision, thus increasing the likelihood that useful logged data is retained upon HO.

Assuming the second solution, to minimize specification impact, in a first step it is sufficient to support only the intra-vendor case. Namely, the 1-bit indication can signal to retain unretrieved data upon HO completion (otherwise, if UE does not receive this indication, it discards logged data upon HO) and the 1-bit indication is sent from source gNB to target gNB via implementation. This covers a relevant case in practice, where a vendor has incentive to allow the UE to send logged data to the target gNB, if the target gNB is of the same vendor. Otherwise, if the target gNB is of a different vendor, the UE discards the logged data upon HO. No other spec impact is expected in this case, to transmit the 1-bit indication from source gNB to target gNB.

In a second step, inter-vendor signaling could be introduced, where it is specified how the source gNB sends the 1-bit indication to the target gNB.

(RRC-18) The 1-bit indication signals to the UE to retain unretrieved logged data upon HO completion. In absence of this indication, the UE releases unretrieved logged data upon HO completion.

(RRC-18) The 1-bit indication (decided by the source gNB) is sent to the UE in the HO command from the target gNB, i.e. in RRCReconfiguration containing reconfigurationWithSync.

### Open issue RRC-19: Reporting assistance information related to logged measurements

RAN2#129bis agreed that:

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

It is not yet clear if the configurations in otherConfig can be grouped or should be separated for each/some of the UAI indications: full buffer reached, buffer threshold reached, and low power. It is straightforward to have a separate optional field to configure the indication “buffer threshold reached”, because a specific buffer threshold value is needed by the UE to trigger the UAI indication in this case. However, the configuration of the “full buffer” and “low power” indications can be grouped together, i.e. the UE is configured to report both these indications if it is configured to report assistance information for logged data.

Regarding the threshold value, it has been discussed whether this should be a percentage of the total buffer size, or an absolute value (bits/bytes, etc.). In our view both cases can work, but it may be easier to specify percentage values.

(RRC-19) The “buffer threshold reached” indication is configured with a dedicated field in otherConfig, which gives the value of the threshold. The “full buffer being reached” and “low power” indications are configured jointly in otherConfig.

(RRC-19) The buffer threshold value is specified as a percentage of the UE buffer size.

### Open issue RRC-20: Further procedures for UE assistance information related to logging

According to this open issue and the notes in the running CR, it has not yet been discussed whether further procedures for UE reporting assistance information related to logging are need. Such further procedures may be, e.g. prohibit timers, indication that battery state is not low any longer, indication that the memory is not full any longer.

In our view, there should be a mechanism to avoid redundant indications. For instance, the UE should report a low power state when the UE enters this state, but it should not trigger another indication of low power state while it stays in this state. A new low power indication should be triggered only after the UE exits the low power state and enters it again. Similarly, the UE should send an indication to NW when the buffer is first declared full and not all the time during which the buffer stays full. A similar situation occurs for reaching the buffer threshold. Thus, we do not think that prohibit timers are needed, but procedural text needs to clarify that such indications are sent upon state change and not all the time that the UE stays in that state.

Furthermore, the NW should be informed when the UE is no longer in low power state or when the UE’s buffer is no longer full. This is because it was agreed that upon buffer full the UE stops logging and upon low power state it is expected that the NW de-configures/deactivates logging. Thus, the NW should be informed when logging can be resumed. Triggering UAI to report this information can be achieved by configuring the UE via otherConfig to report assistance information for logging, even if no logging configuration is configured/activated.

(RRC-20) The UE indicates “full buffer reached”, “buffer threshold reached” and “low power” only upon entering these states and not while continuing to stay in these states.

(RRC-20) If configured to report assistance information related to data logging, the UE indicates when it is no longer in low power state and when its buffer is no longer full.

### Open issue RRC-21: Time related content of logged data

RAN2#129bis agreed that:

1. For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit

Furthermore, the following related agreements were made:

From RAN2#129bis: „1 The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)“ From RAN2#129: „4. As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.“

Given the agreements above, multiple logging configurations corresponding to multiple sets of resources can be used for logging. Furthermore, the UEInformationResponse contains logged measurement entries in chronological order, which means that measurement entries from different logging configurations can be interleaved in the UEInformationResponse, according to the chronological order in which they were stored in the memory. Thus, in general it cannot be assumed that subsequent measurement entries in UEInformationResponse are spaced at regular time gaps according to a known configured periodicity, even if all the configurations include periodic logging (without any L3 event based logging).

(RRC-21) Multiple logging configurations result in the UE logging interleaved measurement entries from different logging configurations, where subsequent entries are not equally separated in time according to known periodicities.

Thus, to make it clear what the time gap between subsequent logged entries is, the timestamp of each logged measurement entry is needed. The timestamp of a logged measurement is already included in the logged MDT report, as a tool to assist the network in retrieving NW operating conditions at the moment in which the measurement was taken by the UE. In the same way, in the context of AIML, a timestamp would allow the network to correlate a given UE radio measurement with the NW conditions/configurations experienced at the network, which is critical to ensure a proper AIML model setup.

We note that, if the relative timestamp is reported for every taken measurement as for logged MDT, we do not foresee major overhead issues, given the UEInformationRequest/UEInformationResponse that was agreed to be used for fetching data. This framework supports reporting amounts of data that are large enough. Furthermore, it was also agreed that the UE can keep the logged data upon HO (if indicated by NW), so the NW has the possibility to retrieve the remaining data also via the target gNB, if there was not enough time to retrieve it by the source gNB before HO.

(RRC-21) Including relative time stamps for each logged entry is not expected to introduce major overhead issues, given the UEInformationRequest/ UEInformationResponse framework agreed for fetching logged data.

Given the above considerations, we propose adopting a similar approach as logged MDT, i.e. the UE logs in the report the absolute time stamp (as provided by the network at the time of configuring the UE with the NW-side data collection configuration), and for each measurement entry the UE logs the relative time stamp.

(RRC-21) The UE includes in the logged measurement report an absolute time stamp (provided by the network at the time of configuring the UE with the NW-side data collection configuration), and for each measurement entry, a relative time stamp related to the point in time in which the measurement was taken.

### Open issue RRC-22: RAN1 involvement for logged data for NW-side and UE-side data collection

The necessary signalling to configure the UE to perform the NW-side data collection is captured and under discussion in the ASN.1 part of the RRC running CR. In this context, the issue of where/how to capture the procedures for logging data in a variable at the UE in procedural text was raised. In our view, the procedures related to how the UE should perform the L1 measurements and the storing/logging of such measurements (i.e. UE behaviour upon reception of logging configuration, collection of measurement results based on the periodic/event triggered conditions, and storage of the measurement results in a variable in the UE memory) should be captured in RAN1 specifications. This is because, in legacy, all the procedures on how the UE performs the channel measurements on configured CSI-RS/SSB resources are captured in TS 38.214. Hence, it should be up to RAN1 to evaluate if there is any impact on that specification.

(RRC-22) L1 procedures to perform radio measurements and logging for the NW-side data collection purpose are not captured in RAN2 specification. RAN1 to evaluate impact in TS 38.214.

Since all the above assumptions captured in previous proposals are of RAN1 interest, we suggest RAN2 informing RAN1 on that.

(RRC-22) RAN2 to inform RAN1 about any agreements related to UE configuration for NW-side data collection, contents of the report with logged data and needed procedural text for NW-side data collection.

### Open issue RRC-23: Cell ID stored with logged data for NW-side data collection

Firstly, it is important that the UE logs a cell ID of the cell in which it performed measurements for NW-side data collection, so that the NW can clearly identify where the measurements were performed. This is especially needed since the UE can log measured data on different serving cells, and in the report the UE should indicate to which cell the logged measurements relate to. However, the cell ID should not unnecessarily be repeated in the report for every logged entry that was measured on the same serving cell.

Regarding the type of cell ID, it would be sufficient for the UE to log the PCI-ARFCN of the cell in which the measurement was performed. However, since we agreed that the data can be retained at HO, and hence the data can be reported to a gNB different than the gNB that configured the UE, it would be needed to adopt a globally unique cell identifier such as the CGI, as it is done for example in the SON/MDT reports. However, it is noted that the UE is not always mandated to acquire the CGI, particularly if the measurements are performed in an SCell or in a PSCell. Hence our proposal is that the UE logs the CGI of the serving cell on which the UE is taking the measurements, and if the CGI of such cell is not available, the UE logs the PCI-ARFCN of the serving cell on which the UE is taking the measurements, and the CGI of the SpCell to which the UE was connected while performing the measurements.

(RRC-23) When the UE reports logged data, it does not repeat the cell ID for every measured logged entry performed on the same serving cell.

(RRC-23) Within logged data for NW-side data collection, UE includes CGI of the serving cell in which it performs the logged measurements, if available.

(RRC-23) If CGI of the measured cell for logging is not available, UE logs the PCI-ARFCN of the measured serving cell and the CGI of the PCell or PSCell to which the UE is connected while performing the logging of the measured data.

### Open issue RRC-24: Where to include the logging configuration from NW to UE

In our companion paper submitted to RAN2#130 we provide an extensive analysis and argue that it is beneficial to include the CSI measurement configuration for NW-side data collection within the CSI-MeasConfig. Here we take this assumption and we discuss further details of where within the CSI-MeasConfig the logging configuration should be included.

Based on contributions submitted to last meeting, two possible solutions exist:

- Solution-1: Extending the CSI-MeasConfig framework by adding a logging configuration (Figure 1)

- Solution-2: Reusing CSI-ReportConfig IE.

Solution-1: Enhancing the CSI-MeasConfig framework with additional IEs

This solution allows to separate the data collection configuration from the legacy CSI-ReportConfig IE. This avoids creating dependency on the legacy procedures and enables easy integration of potentially new AI for PHY use cases. For example, this choice would avoid the discussion on how to handle the mandatory parameters included in the legacy CSI-ReportConfig that may not be applicable to the data collection scenario.
Specifically, this solution would imply creating an extension of the L1 CSI-MeasConfig outside the CSI report configuration. The L1 CSI measurement configuration would consist of a list of logging configurations, each including the CSI measurement resources that the UE should measure, and optionally the L3 events. This approach would be similar to the way the LTM CSI report configuration is represented in the CSI-MeasConfig.

Solution-2: Reusing CSI-ReportConfig IE

An alternative solution to the above is to reuse existing CSI-ReportConfig IE. We note that the legacy CSI-ReportConfig is used in TS 38.214 to report the L1 measurement results in the UCI, as per the description of the IE CSI-ReportConfig:

“ The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included.“

However, the above is not applicable for the NW-side data collection configuration, since it was agreed in RAN2 that the UE should not transmit the data using UCI, rather it needs to store the collected measurements and transmit them to network using RRC signalling. Furthermore, one major problem if using CSI-ReportConfig is that it contains a mandatory list of report configuration types, i.e., reportConfigType IE, which is not extendable and not applicable to the data collection scenario.

(RRC-24) Reusing CSI-ReportConfig to configure the UE with NW-side data collection configuration leads to impact on legacy beam management procedure and creates additional constraints on the data collection procedure.

(RRC-24) The L1 CSI measurement configurations for the NW-side data collection consist of a list of logging configurations within the legacy CSI-MeasConfig.

(RRC-24) Each logging configuration contains a reference CSI resource configuration (i.e. a reference to a CSI-ResourceConfig IE) on which the UE shall perform radio measurements and the related logging, and if configured, the L3 events (A1/A2 thresholds, TTT).

Figure 1: Framework for NW side data collection

### Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging

It is useful to allow the gNB to activate the NW-side data collection based on L1 measurements received over the UCI. Especially since the NW-side data collection based on L1 events is not supported, then there is the need to give the possibility to the NW to dynamically activate the data collection based on real-time L1 measurements received from the UE. Otherwise, there is the risk with periodic resources that the gNB collects a lot of redundant data, that were already previously collected. The dynamic activation/deactivation of a logging configuration can be performed via a MAC CE and it can be further discussed if a new MAC CE is needed, or whether a legacy MAC CE can be enhanced for this, depending also on the outcome of open issue RRC-24 on where the logging configuration will be transmitted.

(RRC-25) Dynamic activation/deactivation of logging configurations for NW-side data collection is supported. FFS whether to introduce a new MAC CE or to enhance a legacy MAC CE, based on the outcome of open issue RRC-24.

### Open issue RRC-26: Multiplexing legacy logged data and AIML logged data in new SRB

RAN2#129bis agreed to introduce a new SRB for NW-side data collection. It is thus clear from legacy procedures that legacy MDT logged data is transmitted in UEInformationResponse via SRB2, and it was clarified in RAN2 agreements that logged data for NW-side data collection is transmitted in UEInformationRespose via the new SRB. However, it was not yet clarified on which SRB to transmit UEInformationResponse in which legacy logged MDT data and new logged data for NW-side data collection are multiplexed. In our view there are three possible solutions:

- Restrict UEInformationResponse to contain only one of two types of logged data (either legacy MDT or new data for NW-side data collection) and it is thus clear which SRB to use, based on the type of logged data. In our view, this solution is suboptimal, because it does not allow the NW to fetch both types of logged data simultaneously, in the same message. Thus, if there is only a small amount of logged MDT data and a small amount of new data for NW-side data collection at the UE at a time, some UEInformationResponse messages will contain only small amounts of data, when in fact they could carry more data. This would increase the number of UEInformationRequest/ UEInformationResponse exchanges to fetch the data from the UE.

- Allow multiplexing of the two types of logged data in the new SRB. This solution is also not desirable, since it affects the way that legacy logged MDT data is reported, namely on SRB2 if only this type of data is included in the UEInformationResponse and on the new SRB if logged MDT data is multiplexed with new logged data in the same message. Impact on legacy procedures should be avoided.

- Reconsider the agreement on introducing a new SRB and allow logged data for NW-side data collection to be sent in SRB2. This solution is desirable because it allows multiplexing of legacy logged MDT data and logged data for NW-side data collection in the same UEInformationResponse and it does not change legacy procedures for logged MDT data.

(RRC-26) The UEInformationResponse is transmitted via SRB2 when containing logged data for NW-side data collection.

### Open issue RRC-27: How to set logging periodicity

Related to the logging periodicity, we believe that it should be controllable by the network, in order to reduce the amount of data to be logged and the impact on the UE. For example, the CSI-RS/SSB periodicity could be 80 ms as per the CSI resource configuration, but for periodic logging for NW-side data collection, it might be enough to just perform measurements (and hence logging) every 640 ms. And for example, if a certain event is triggered, the periodicity of logging can be increased to e.g. 160 ms, in order to provide more granular results to the network.

(RRC-27) The logging periodicity/interval for both periodic and event-triggered data collection is configured in the L1 CSI measurement configurations for data collection.

### Open issue RRC-28: Handling of configuration to report data availability and low power state during RRCReestablishment, transition to RRC_INACTIVE, etc.

RAN2#129bis agreed the following about logged data:

Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

Given the agreement above about logged data, we do not see reasons why the UE should retain the configuration for reporting related data availability and low power state.

(RRC-28) UE releases the configuration for data availability and low power state indications upon transition to RRC_INACTIVE/IDLE and upon initiating the RRCReestablishment procedure.

### Open issue RRC-29: Whether data availability indication should be sent when the UE has data below the threshold and low power state is sent, and what cause should be included then

In our opinion, this is an optimization and the cases in which it is beneficial are probably rare. First of all, if the UE reports low power state without any data availability indication (e.g. the UE has not yet logged anything, or the logged amount of data is very small), the NW always has the possibility to probe whether the UE logged any data in the buffer, by sending a UEInformationRequest.

The benefit of including a data availability indication (before buffer threshold is reached) together with a low power indication is only that it can save one unnecessary exchange of UEInformationRequest/ UEInformationResponse, if there is no logged data in the buffer at all. Furthermore, it is unclear how often it happens in practice that the UE reports low power state right after receiving a logging configuration and before it has time to log anything at all. Thus, the benefit of this solution seems to be questionable.

(RRC-29) UE does not send data availability indication when it sends low power indication, if the amount of data is below the configured buffer threshold.

# 3	Discussion on NW-side data collection for positioning

Network-side data collection for positioning occurs in the following network-based positioning methods:

- Case 3a: NG-RAN node assisted/LMF-based positioning with gNB side model (direct positioning)

- Case 3b: NG-RAN node assisted/LMF-based positioning with LMF-side model (assisted positioning)

Case 3a and Case 3b both are based on UL measurements performed at the gNB and share similar flows as the legacy UL-TDOA positioning method. As such the impact is primarily in RAN3 domain NRPPa.

Case 3a/3b NW side data collection should be driven by RAN3 as it has primarily NRPPa impact.

RAN2 to wait for RAN3 before checking/working on if there is any impact for RAN2 for case 3a/3b data collection.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	(RRC-21) Multiple logging configurations result in the UE logging interleaved measurement entries from different logging configurations, where subsequent entries are not equally separated in time according to known periodicities.

Observation 2	(RRC-21) Including relative time stamps for each logged entry is not expected to introduce major overhead issues, given the UEInformationRequest/ UEInformationResponse framework agreed for fetching logged data.

Observation 3	(RRC-24) Reusing CSI-ReportConfig to configure the UE with NW-side data collection configuration leads to impact on legacy beam management procedure and creates additional constraints on the data collection procedure.

Observation 4	Case 3a/3b NW side data collection should be driven by RAN3 as it has primarily NRPPa impact.

Based on the discussion in the previous sections we propose the following:

Proposal 1	(RRC-18) The 1-bit indication signals to the UE to retain unretrieved logged data upon HO completion. In absence of this indication, the UE releases unretrieved logged data upon HO completion.

Proposal 2	(RRC-18) The 1-bit indication (decided by the source gNB) is sent to the UE in the HO command from the target gNB, i.e. in RRCReconfiguration containing reconfigurationWithSync.

Proposal 3	(RRC-19) The “buffer threshold reached” indication is configured with a dedicated field in otherConfig, which gives the value of the threshold. The “full buffer being reached” and “low power” indications are configured jointly in otherConfig.

Proposal 4	(RRC-19) The buffer threshold value is specified as a percentage of the UE buffer size.

Proposal 5	(RRC-20) The UE indicates “full buffer reached”, “buffer threshold reached” and “low power” only upon entering these states and not while continuing to stay in these states.

Proposal 6	(RRC-20) If configured to report assistance information related to data logging, the UE indicates when it is no longer in low power state and when its buffer is no longer full.

Proposal 7	(RRC-21) The UE includes in the logged measurement report an absolute time stamp (provided by the network at the time of configuring the UE with the NW-side data collection configuration), and for each measurement entry, a relative time stamp related to the point in time in which the measurement was taken.

Proposal 8	(RRC-22) L1 procedures to perform radio measurements and logging for the NW-side data collection purpose are not captured in RAN2 specification. RAN1 to evaluate impact in TS 38.214.

Proposal 9	(RRC-22) RAN2 to inform RAN1 about any agreements related to UE configuration for NW-side data collection, contents of the report with logged data and needed procedural text for NW-side data collection.

Proposal 10	(RRC-23) When the UE reports logged data, it does not repeat the cell ID for every measured logged entry performed on the same serving cell.

Proposal 11	(RRC-23) Within logged data for NW-side data collection, UE includes CGI of the serving cell in which it performs the logged measurements, if available.

Proposal 12	(RRC-23) If CGI of the measured cell for logging is not available, UE logs the PCI-ARFCN of the measured serving cell and the CGI of the PCell or PSCell to which the UE is connected while performing the logging of the measured data.

Proposal 13	(RRC-24) The L1 CSI measurement configurations for the NW-side data collection consist of a list of logging configurations within the legacy CSI-MeasConfig.

Proposal 14	(RRC-24) Each logging configuration contains a reference CSI resource configuration (i.e. a reference to a CSI-ResourceConfig IE) on which the UE shall perform radio measurements and the related logging, and if configured, the L3 events (A1/A2 thresholds, TTT).

Proposal 15	(RRC-25) Dynamic activation/deactivation of logging configurations for NW-side data collection is supported. FFS whether to introduce a new MAC CE or to enhance a legacy MAC CE, based on the outcome of open issue RRC-24.

Proposal 16	(RRC-26) The UEInformationResponse is transmitted via SRB2 when containing logged data for NW-side data collection.

Proposal 17	(RRC-27) The logging periodicity/interval for both periodic and event-triggered data collection is configured in the L1 CSI measurement configurations for data collection.

Proposal 18	(RRC-28) UE releases the configuration for data availability and low power state indications upon transition to RRC_INACTIVE/IDLE and upon initiating the RRCReestablishment procedure.

Proposal 19	(RRC-29) UE does not send data availability indication when it sends low power indication, if the amount of data is below the configured buffer threshold.

Proposal 20	RAN2 to wait for RAN3 before checking/working on if there is any impact for RAN2 for case 3a/3b data collection.

# References

R2-2504350, “RRC open issues AIML PHY”, 3GPP TSG RAN WG2#130, St. Julian’s, Malta, 19-23 May 2025.