3GPP TSG RAN WG2 #130		R2-2504641

Malta, May 19th – 23rd, 2025

Agenda Item:	8.1.5

Source:	BT Plc, Turkcell, T-Mobile, Deutsche Telekom, Orange

Title:	Operators views for the AI_ML model delivery options

Document for:	Discussion/Decision

# Introduction

During RAN2#129bis, the following agreements were made.

Agreements 1 Dataset/model parameter transfer solution from NW to UE training entity shall follow below principles: - A1 - Size: From RAN2 point of view, aim to define a unified solution (e.g. OTA or non-OTA) to support various sizes of dataset/model parameter transfer (dataset and/or parameter sharing size could range from tens of KBs to hundreds of MBs, but in average around hundreds of MBs); - A2 - Continuity: Service continuity of dataset and/or parameter transfer/delivery during UE mobility needs to be supported; - A3 - Controllability: NW decides on if and when to transfer/delivery the dataset and/or model parameter from NW to UE or UE training entity (a server inside MNO or an OTT server); - A4 - Latency: Relaxed latency requirement and infrequent update; - A5 - Visibility: dataset and model parameter to be understandable by UE/UE-side training entity (a server inside MNO or an OTT server). - A6: Proprietary information of the network and UE should be respected and not disclosed. 2 The following approaches for dataset/model parameter transfer Direction A are agreed to be included in the TR: Alternative 1 (non-OTA approach): gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server) data RAN2 analyzed area dataset/model parameter transfer NW-side dataset/model parameters collection entity (gNB/CN/OAM) for two-sided UE-part model training CSI compression data collection at gNB UE-side training entity for two-sided UE part model training. Alternative 2 (OTA approach): gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server) RAN2 analyzed area CSI compression data collection at gNB data dataset/model parameter transfer NW-side dataset/model parameters collection entity (gNB/CN/OAM) for two-sided UE-part model training UE-side training entity for two-sided UE part model training dataset/model parameter for training. NOTE: The data transfer (up to RAN1 further details on what to transfer) between gNB and NW dataset/model parameters collection entity (OAM/CN) in Alternative 1/2, if needed, is up to RAN3/SA2/SA5.. 3 For non-OTA solutions ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidates. From RAN2 point of view it is assumed they can be supported within Rel-19 existing architecture framework. RAN3, SA2, and SA5 can confirm this assumption (i.e. we can send an LS). It does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below. Option; Impacted WG; Specification impact/Implementation impact. - OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity; SA5, SA3; Up to SA5 (any intermediate node, if any, between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion). - CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity; SA2, SA3; Up to SA2 (any intermediate node, if any, between CN and UE-side OTT server is up to SA2). - gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity; RAN3, SA2, SA5, SA3; Up to RAN3, SA2, SA5 (any intermediate node, if any, between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5). 4 From RAN2 point of view, when gNB is the NW dataset/model parameter collection entity, solution 1b (i.e. gNB -> UE via UP) is not considered as a candidate solution in 5GA Rel-19/20 5 In OTA approach, UE transfers the received dataset/model parameter to UE training entity (an OTT server) transparently to 3GPP network. 6 For OTA solutions ‘gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified following candidate solutions but there’s no consensus on feasibility: - OTA solution 1a: gNB -> UE via CP - OTA solution 2: CN -> UE via gNB - OTA solution 3: OAM -> UE via gNB 7 The following table is captured in the TR Table 2. OTA candidate solutions gNB -> UE via CP; Challenges; - UE is only required to support 45kB RRC buffer size, according to TS 38.306. - Significant specification impact: - Other segmentation beyond RRC layer requires a new SRB protocol stack to perform segmentation, including functions such as handling segmentation, retransmission, etc - UE selection - Challenges to support E2E reliability, considering dataset/model parameter transfer is shared by different gNB/vendors during UE mobility and different RRC state transition - Uu overhead for data collection from UE and dataset/model parameter transfer to UE - Overloading CP with large datasets would disrupt core control message transmission (e.g. service degradation, reliability, etc). Potential suitable scenario; - Small dataset/model parameter size. However, the maximum RRC segment needs to be further studied - Split large dataset/model parameter into small pieces, and potentially send to multiple UEs, then gather by UE training entity. RAN2 has not study the feasibility of split dataset/model parameter to multiple UEs.. CN -> UE via gNB; Challenges; - Same challenges as OTA solution ‘gNB->UE via CP’, if OTA solution ‘CN -> UE via gNB’ via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to CN, then transmit back to gNB. Relaying dataset/model parameter via gNB to UE then back to UE training entity is not desirable. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure if gNB and CN are not from the same NW vendor. Potential suitable scenario; - Feasibility analysis of OTA solution ‘CN -> UE via gNB’ via CP is the same as OTA solution ‘gNB -> UE via CP’ - OTA solution ‘CN -> UE via gNB’ and its feasibility is required to be evaluated by RAN3 and SA2.. OAM -> UE via gNB; Challenges; - Same challenges as OTA solution ‘gNB -> UE via CP’, if OTA solution ‘OAM -> UE via gNB’ via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to OAM, then transmit back to gNB. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure that OAM may share to a second NW vendor. Potential suitable scenario; - Feasibility analysis of OTA solution ‘OAM -> UE via gNB’ via CP is the same as OTA solution ‘gNB -> UE via CP’ - OTA solution ‘OAM -> UE via gNB’ and its feasibility is required to be evaluated by RAN3 and SA5..

In this TDoc, remaining issues will be discussed.

# Discussion

## 3GPP solution for model transfer

The solutions summarized on Table 1 in TR 38.843 [1] can be categorized into three different categories.

- RAN

- Core and LMF

- OAM

Table 1 Need to consider standardization solutions for transferring/delivering AI/ML models

Whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) is unclear from the outcome of the present study

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP). - Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE. Note: The relationships between model transfer/delivery solutions and use cases can be derived from what is captured in clauses 7.3.2, 7.3.3, and 7.3.4.

Since the OTT solution has no impact on 3GPP specifications, we consider there is not need to further discuss and focus the remaining time on other solutions.

Proposal 1: Solution 4a, OTT without 3GPP impact, captured on [1] is no further discussed in RAN2

## Dataset/model parameter transfer solution for one-sided model

For an operator, multiple solutions specified for a single problem is never a good outcome. It framents the market and consequently, difficults the implementation. 3GPP has multiple examples were fragmentation has difficulted the solution adoption. What is more important, there is no technical reason to differentiate the principles agreed in RAN2#129bis for two-sided model and apply them for one-sided model.

For that reason, for one-sided UE model, the aim is to agree the same principles agreed in RAN2#129bis for two-sided models.

Proposal 2: One-sided dataset/model transfer solution shall follow below principles:

- A1 - Size: from RAN2 point of view, aim to support various sizes of dataset/model parameter transfer (FFS on model size);

- A2 - Continuity: service continuity of model transfer/delivery during UE mobility needs to be supported;

- A3 - Controllability: NW decides on if and when to transfer/delivery over the air interface;

- A4 - Latency: relaxed latency requirement and infrequent update;

- A5 - Visibility: dataset and model parameters to be visible and understandable by MNO;

## Principles to transfer/delivery a model

In previous sections, we have discussed how transfer the dataset/model parameters for one-sided model. In this section, it will be discussed the basic principles to transfer/deliver the model once it has been trained on the training entites. Were are allocated this training entities and who control them are out of discussion is this TDoc.

AI/ML functionality has a potential to optimize procedures improving important aspecs as energy consumption and signalling overhead. To achieve it, well-known and tested procuderes will be replaced. For example, measurements will be avoided and with that, all the work already covered by RAN4 and RAN5 associated to them to ensure the expected behaviour. Instead, actions will be taken based on predictions. As any prediction, there are error margins. Well trained models will reduce the error margins in a way that the expections is to achieve the same performance. But it is important to remind ourselves that this is the best possible scenario.

Model training requires data and time to enhance the predictions. In consequence, it is expected that first AI/ML trained models or models for new UEs are not mature enough. That may cause problems on the network. From 3GPP, until RAN4 and RAN5 work on AI/ML for the air interface, there is no guarantee of their performance.

Observation 1: There are no standardized test to validate AI/ML models

It is expected that infrastructure vendors, device vendors and chipset vendors create their own AI/ML models. Hence, without individuals SLAs, MNOs have no control or visibility over the AI/ML model creation.

Observation 2: AI/ML models are expected to be created by infraestructe vendors, device vendors and chipset vendors

Combining the lack of standardized tests and the lack of MNOs control over the models, it is evident that MNOs require mechanisms to activate and deactivate them. MNOs are the only ones looking for the end-to-end performance. A AI/ML model may work perfect under certain circumstances but it is hard to train the models for all the possible situations. That makes the overhal performance may be far from expected. For those situations, it is required that AI/ML models can be activated and deactivated.

Observation 2: AI/ML models may require to be deactivated if the end-to-end performance is impacted

In [2], a list of requirements were captured in proposal 1. Considering the challenges, it is important to adop the following requirements to ensure the end-to-end performance in the network.

Proposal 3: Adopt additional list of requirements for one-sided and two-sided model transfer/delivery.

- Activation: once the AI/ML one-sided or two-sided model is transferred to NW and/or to UE, MNO should be capable to activate and deactivate them at any point in time

- Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic.

- Differentiability: model transfer/delivery traffic should be differentiated from other user traffic.

- Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

- Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one

- Controllability: the NW is in control of if and when to transfer/deliver a model

# Conclusions

During the TDoc, we have made the following observations

Observation 1: There are no standardized test to validate AI/ML models

Observation 2: AI/ML models are expected to be created by infraestructe vendors, device vendors and chipset vendors

Observation 2: AI/ML models may require to be deactivated if the end-to-end performance is impacted

For those reasons, it is proposed

Proposal 1: Solution 4a, OTT without 3GPP impact, captured on [1] is no further discussed in RAN2

Proposal 2: One-sided dataset/model transfer solution shall follow below principles:

	A1 - Size: from RAN2 point of view, aim to support various sizes of dataset/model parameter transfer (FFS on model size);

	A2 - Continuity: service continuity of model transfer/delivery during UE mobility needs to be supported;

	A3 - Controllability: NW decides on if and when to transfer/delivery over the air interface;

	A4 - Latency: relaxed latency requirement and infrequent update;

	A5 - Visibility: dataset and model parameters to be visible and understandable by MNO;

Proposal 3: Adopt additional list of requirements for one-sided and two-sided model transfer/delivery.

	Activation: once the AI/ML one-sided or two-sided model is transferred to NW and/or to UE, MNO should be capable to activate and deactivate them at any point in time

	Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic.

	Differentiability: model transfer/delivery traffic should be differentiated from other user traffic.

	Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

	Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one

	Controllability: the NW is in control of if and when to transfer/deliver a model

# References

- TR 38.843 - Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R2-2500949 - Requirements for Model Transfer/Delivery - T-Mobile USA, Boost Mobile Network, Deutsche Telekom, Orange, Charter Communications Inc., Nokia Corporation