3GPP TSG RAN WG2#130	R2-2503321

St Julian’s, Malta, 19th - 23rd May 2025

3GPP TSG-RAN WG3 Meeting #127-bis	R3-252481

Wuhan, China, 7-11 April, 2025

# Title:	LS on Ambient IoT further progress of RAN3

# Response to:

# Release:	Release 19

# Work Item:	Ambient_IoT_Solutions-Core

Source:	RAN3

To:	SA2, SA5

Cc:	RAN2

Contact Person:

Name:	Yan Wang

E-mail Address:	wangyan7 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	N/A

1. Overall Description:

RAN3 would like to inform SA2 and SA5 about the following RAN3 further progress:

General Aspects:

- Previous working assumption: “Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP” is changed to an agreement.

- Existing NGAP Interface management procedures will be enhanced to support A-IoT.

- RAN3 agreed to define a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. The detailed coding of the new “A-IoT Area” is FFS.

- The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB.

On Inventory:

- Within all Inventory related NGAP messages:

  - The Correlation Identifier is included inside the AIOTF information container.

  - The AIOTF Identifier is included outside the AIOTF information container.

  - Working assumption: The Correlation identifier is also included outside the AIOTF information container.

On Command after Inventory:

- Introduce a “follow-on command indication” in the AIOTF information container in the Inventory Request message.

- Working assumption: The NGAP Command Request procedure concerns a single device only.

- Within all Command related NGAP messages:

  - The AIOTF Identifier is included outside the AIOTF information container.

  - Working assumption: The Correlation identifier is included inside and outside the AIOTF information container.

- RAN3 agreed to introduce per-session per-device device context (“RAN NGAP Device ID”).

  - Upon receiving the “follow-on command indication”, the gNB allocates for each device a “RAN NGAP Device ID” and provides it in the AIOTF information container in the Inventory Report message.

  - The “RAN NGAP Device ID” is included in the AIOTF information container of each Command related NGAP message.

2. Actions:

To SA2 and SA5

ACTION: 	RAN3 asks SA2 and SA5 to take above into account, and provide feedback, if any.

3. Date of Next TSG RAN WG3 Meetings:

RAN3#128	2025-05-19 - 2025-05-23	Malta, MT

RAN3#129	2025-08-25 ~ 2025-08-29	Bangalore, India