3GPP TSG-RAN WG2 Meeting #130	        R2-2504742

St.Julians, Malta,  May 19th – 23rd , 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.314 | CR | 0034 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of number of UEs in RRC_INACTIVE state with data transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Huawei, HiSilicon, ZTE Corporation, Sanechips, CATT, Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the reply LS R2-2501758[S5-250827], SA5 would like RAN2 to specify metrics that can measure the number of RRC_INACTIVE UEs in a cell with ongoing SDT. SA5 notes that the specific definition in the tables of TS 38.314 clause 4.2.1.3 does not identify and restrict that the UE is in the RRC_CONNECTED state, but instead can be used generically for both the connected state and inactive state. In the reply LS R2-2504919/R3-253755, RAN3 kindly requests RAN2 to leave the existing measurement in TS 38.314 as is, and to define a new separate measurement specific for the number of RRC_INACTIVE UEs in a cell with ongoing SDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new Section 4.2.1.3a and sub-sections are added to support measurements of number of UEs in RRC_INACTIVE state with data transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The KPIs on number of UEs in RRC_INACTIVE state with data transmission reqiured by SA5 are not supported in TS 38.314. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2502248; R2-2503035 in principle agreed in RAN2#129bis; R2-2503684 |  |  |  |  |  |  |  |  |

START OF CHANGES

### 4.2.1	Measurements valid for all gNB deployment scenarios

<Irrelevant Texts Omitted>

#### 4.2.1.3a	Number of active UEs in RRC_INACTIVE with ongoing SDT procedure

##### 4.2.1.3a.1	General

The objective of the measurement is to measure the number of active UEs in RRC_INACTIVE with ongoing SDT procedure per QoS level for OAM performance observability. It is intended to be part of a calculation to determine the bitrate UEs achieve when they are active, i.e. when applications are transmitting and receiving data. The measurements are applicable for both non-split gNB and split gNB deployment scenario.

4.2.1.3a.2	Mean number of Active UEs in RRC_INACTIVE with ongoing SDT procedure in the DL per DRB per cell

Protocol Layer: MAC, RLC

Table 4.2.1.3a.2-1: Definition for Mean number of Active UEs in the DL per DRB per cell

| Definition | Mean number of Active UEs in the DL per DRB per cell. The DRBs are mapped with the same 5QI for NR SA. This measurement refers to UEs for which there is data available for transmission for the DL for DRBs. Detailed Definition: M(T,drbid,p)=∀iN(i,drbid)I(T,p)*1010, where explanations can be found in the table 4.2.1.3a.2-2 below. |
| --- | --- |

Table 4.2.1.3a.2-2: Parameter description for Mean number of Active UEs in the DL per DRB per cell

| M(T,drbid,p) | Mean number of Active UEs in the DL per DRB per cell, averaged during time period T. Unit: 0.1. |
| --- | --- |
| N(i,drbid) | Number of UEs for which there is data available for transmission for the DL in MAC or RLC protocol layers for a Data Radio Bearer of traffic class at sampling occasion i. Data available for transmission includes data for which HARQ transmission has not yet terminated. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |
| drbid | The DRBs mapped with the same 5QI for NR SA. |

##### 4.2.1.3a.3	Max number of Active UEs in RRC_INACTIVE with ongoing SDT procedure in the DL per DRB per cell

Protocol Layer: MAC, RLC

Table 4.2.1.3a.3-1: Definition for Max number of Active UEs in the DL per DRB per cell

| Definition | Maximum number of Active UEs in the DL per DRB per cell. The DRBs are mapped with the same 5QI for NR SA. This measurement refers to UEs for which there is data available for transmission for the DL for DRBs. Detailed Definition: M(T,drbid,p)=maxTNi,drbid, where explanations can be found in the table 4.2.1.3a.3-2 below. |
| --- | --- |

Table 4.2.1.3a.3-2: Parameter description for Max number of Active UEs in the DL per DRB per cell

| M(T,drbid,p) | Maximum number of Active UEs in the DL per DRB per cell, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i,drbid) | Number of UEs for which there is data available for transmission for the DL in MAC or RLC protocol layers for a Data Radio Bearer of traffic class at sampling occasion i. Data available for transmission includes data for which HARQ transmission has not yet terminated. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| T | Time Period during which the measurement is performed, Unit: second. |
| drbid | The DRBs mapped with the same 5QI for NR SA. |

##### 4.2.1.3a.4	Mean number of Active UEs in RRC_INACTIVE with ongoing SDT procedure in the UL per DRB per cell

Protocol Layer: MAC

Table 4.2.1.3a.4-1: Definition for Mean number of Active UEs in the UL per DRB per cell

| Definition | Mean number of Active UEs in the UL per DRB per cell. The DRBs are mapped with the same 5QI for NR SA. This measurement refers to UEs for which there is data available for transmission for the UL for DRBs. Detailed Definition: M(T,drbid,p)=∀iN(i,drbid)I(T,p)*1010, where explanations can be found in the table 4.2.1.3a.4-2 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.3a.4-2: Parameter description for Mean number of Active UEs in the UL per DRB per cell

| M(T,drbid,p) | Mean number of Active UEs in the UL per DRB per cell, averaged during time period T. Unit: 0.1. |
| --- | --- |
| N(i,drbid) | Number of UEs for which there is data available for transmission for the UL in MAC or RLC protocol layers for a Data Radio Bearer of traffic class at sampling occasion i This is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. In such case, when DRB cannot be determined at the time of the sampling occasion, gNB can determine DRB after successful reception of data. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |
| drbid | The DRBs mapped with the same 5QI for NR SA. |

##### 4.2.1.3a.5	Max number of Active UEs in RRC_INACTIVE with ongoing SDT procedure in the UL per DRB per cell

Protocol Layer: MAC

Table 4.2.1.3a.5-1: Definition for Max number of Active UEs in the UL per DRB per cell

| Definition | Maximum number of Active UEs in the UL per DRB per cell. The DRBs are mapped with the same 5QI. This measurement refers to UEs for which there is data available for transmission for the UL for DRBs. Detailed Definition: MT,drbid,p=maxTNi,drbid, where explanations can be found in the table 4.2.1.3a.5-2 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.3a.5-2: Parameter description for Max number of Active UEs in the UL per DRB per cell

| M(T,drbid,p) | Maximum number of Active UEs in the UL per DRB per cell, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i,drbid) | Number of UEs for which there is data available for transmission for the UL in MAC or RLC protocol layers for a Data Radio Bearer of traffic class at sampling occasion i This is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. In such case, when DRB cannot be determined at the time of the sampling occasion, gNB can determine DRB after successful reception of data. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| T | Time Period during which the measurement is performed, Unit: second. |
| drbid | The DRBs mapped with the same 5QI for NR SA. |

##### 4.2.1.3a.6	Mean number of Active UEs in RRC_INACTIVE with ongoing SDT procedure per cell

Protocol Layer: MAC, RLC

Table 4.2.1.3a.6-1: Definition for Mean number of Active UEs per cell

| Definition | Mean number of Active UEs per cell. This measurement refers to UEs for which there is data available for transmission for the UL for DRBs, or there is data available for transmission for the DL for DRBs, or both. Detailed Definition: M(T,p)=∀iN(i)I(T,p)*1010, where explanations can be found in the table 4.2.1.3a.6-2 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.3a.6-2: Parameter description for Mean number of Active UEs per cell

| M(T,p) | Mean number of Active UEs per cell, averaged during time period T. Unit: 0.1. |
| --- | --- |
| N(i) | Number of UEs for which there is data available for transmission for the UL or for the DL or for both in MAC or RLC protocol layers at sampling occasion i For UL, this is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |

##### 4.2.1.3a.7	Max number of Active UEs in RRC_INACTIVE with ongoing SDT procedure per cell

Protocol Layer: MAC, RLC

Table 4.2.1.3a.7-1: Definition for Max number of Active UEs per cell

| Definition | Maximum number of Active UEs per cell. This measurement refers to UEs for which there is data available for transmission for the UL for DRBs, or there is data available for transmission for the DL for DRBs, or both. Detailed Definition: M(T,p)=maxTNi, where explanations can be found in the table 4.2.1.3a.7-2 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.3a.7-2: Parameter description for Max number of Active UEs per cell

| M(T,p) | Maximum number of Active UEs per cell, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i) | Number of UEs for which there is data available for transmission for the UL or for the DL or for both in MAC or RLC protocol layers at sampling occasion i For UL, this is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| T | Time Period during which the measurement is performed, Unit: second. |

##### 4.2.1.3a.8	Mean number of Active UEs in RRC_INACTIVE with ongoing SDT procedure per DRB per cell

Protocol Layer: MAC, RLC

Table 4.2.1.3a.8-1: Definition for Mean number of Active UEs per DRB per cell

| Definition | Mean number of Active UEs per DRB per cell. The DRBs are mapped with the same 5QI for NR SA. This measurement refers to UEs for which there is data available for transmission for the UL for DRBs, or there is data available for transmission for the DL for DRBs, or both. Detailed Definition: M(T,drbid,p)=∀iN(i,drbid)I(T,p)*1010, where explanations can be found in the table 4.2.1.3a.8-2 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.3a.8-2: Parameter description for Mean number of Active UEs per DRB per cell

| M(T,drbid,p) | Mean number of Active UEs per DRB per cell, averaged during time period T. Unit: 0.1. |
| --- | --- |
| N(i,drbid) | Number of UEs for which there is data available for transmission for the UL or for the DL or for both in MAC or RLC protocol layers for a Data Radio Bearer of traffic class at sampling occasion i For UL, this is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. In such case, when DRB cannot be determined at the time of the sampling occasion, gNB can determine DRB after successful reception of data. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |
| drbid | The DRBs mapped with the same 5QI for NR SA. |

##### 4.2.1.3a.9	Max number of Active UEs in RRC_INACTIVE with ongoing SDT procedure per DRB per cell

Protocol Layer: MAC, RLC

Table 4.2.1.3a.9-1: Definition for Max number of Active UEs per DRB per cell

| Definition | Maximum number of Active UEs per DRB per cell. The DRBs are mapped with the same 5QI for NR SA. This measurement refers to UEs for which there is data available for transmission for the UL for DRBs, or there is data available for transmission for the DL for DRBs, or both. Detailed Definition: M(T,drbid,p)=maxTNi,drbid, where explanations can be found in the table 4.2.1.3a.9-1 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.3a.9-2: Parameter description for Max number of Active UEs per DRB per cell

| M(T,drbid,p) | Maximum number of Active UEs per DRB per cell, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i,drbid) | Number of UEs for which there is data available for transmission for the UL or for the DL or for both in MAC or RLC protocol layers for a Data Radio Bearer of traffic class at sampling occasion i For UL, this is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. In such case, when DRB cannot be determined at the time of the sampling occasion, gNB can determine DRB after successful reception of data. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| T | Time Period during which the measurement is performed, Unit: second. |
| drbid | The DRBs mapped with the same 5QI for NR SA. |

CHANGE END