3GPP TSG-RAN WG2 Meeting #131 	 R2-2505080

Bangalore, India, August 25th – 29th, 2025

Agenda item:	8.8.4

Source:	vivo

Title:	Remaining Issues on MBS Broadcast Provision in NTN

Document for:	Discussion and Decision

# Introduction

In the RAN2#130 meeting, the following agreements were made for MBS broadcast within intended service area [1]:

Agreements: 1. Do not introduce service area information specific for FSAI in SIB21; 2. Do not introduce service area per neighbour cell in MCCH; 3. RAN2 assumes that the ISA can be valid for all the frequencies providing the MBS session; 4. The new SIBxx containing MBS ISA(s) can be cell or area specific, which is up to NW implementation (No specs change).

In the [Post130][302][R19 NR NTN] RRC CR, 2 FFS were identified for further discussion,

FFS to align RAN2 specifications and use the Ellipsoid Point with uncertainty Circle from TS 37.355 to define the circle shape of the ISA in the new SIBXX. FFS on the terminology misalignment between ‘broadcast service area’ vs. ‘intended service area’ and the potential impact on the MII procedure.

In this contribution, we would like to provide our consideration of the remaining issues.

# Discussion

## 2.1	Targeted service area in Service Announcement

In the SA4 reply LS [2], SA4 confirms that geographical information representing the intended service area in case of NR NTN is included in service announcement.

S4-251099: SA4 has agreed to extend the Service Announcement and add a targeted service area in form of a list of polygon or circle shapes. However, SA4 is still discussing the technical realization and possibly a generalization, so that the target service area can be extended to represents the intended service area in case of NR NTN.

Then it comes to the first FFS, we believe that there is no need to align the area information in SIBxx with that in the Service Announcement. In the current implementation, the area information in USD, generated at the application layer, is indicated in relatively coarse granularity, whereas ISA information in SIBxx provides more refined area information. This enables operators to provide more accurate area information based on specific network deployment scenarios.

Proposal 1: RAN2 confirms that there is no need to align the MBS ISA signaling in SIBxx with that in Service Announcement.

Furthermore, we have identified another issue: whether and how to utilize the targeted service area information in the Service Announcement. Considering the tight Rel-19 timeline and the fact that we have already introduced the ISA in SIBxx, for the sake of simplicity, we propose that the target Service Announcement (SA) should only be used to facilitate UEs in avoiding unnecessary MCCH acquisition when they are outside the targeted service area. This is also aligned with the legacy usage of the cell area list and tracking area list in the USD.

Proposal 2: It is up to UE implementation to use the targeted service area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.

Proposal 3: From RAN2 perspective, targeted service area list in the USD is not used for other purposes.

## 2.2	Signalling details

In the latest running RRC CR, the mapping between an MBS broadcast session (TMGI) and ISA ID(s) is achieved by extending the MBS-SessionInfoList. The signalling details are given below.

MBS-SessionInfoList-r17 ::=      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17

MBS-SessionInfoListExt-r19 ::=   SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfoExt-r19

MBS-SessionInfo-r17 ::=          SEQUENCE {

mbs-SessionId-r17                TMGI-r17,

g-RNTI-r17                       RNTI-Value,

mrb-ListBroadcast-r17            MRB-ListBroadcast-r17,

mtch-SchedulingInfo-r17          DRX-ConfigPTM-Index-r17                      OPTIONAL, -- Need S

mtch-NeighbourCell-r17           BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL, -- Need S

pdsch-ConfigIndex-r17            PDSCH-ConfigIndex-r17                        OPTIONAL, -- Need S

mtch-SSB-MappingWindowIndex-r17  MTCH-SSB-MappingWindowIndex-r17              OPTIONAL  -- Cond MTCH-Mapping

}

MBS-SessionInfoExt-r19 ::=       SEQUENCE {

mbs-SessionAreaList-r19          SEQUENCE (SIZE (1..maxNrofMBS-AreaPerSessionPerArea-r19)) OF MBS-IntendedAreaID-r19 OPTIONAL, -- Need R

...

}

We would like to highlight that broadcast services delivered within a cell may have differing reception areas. For instance, some are intended for the entire cell, while others are restricted to the associated intended service areas. For services requiring reception across the full cell area, the network should configure the corresponding entry in the MBS-SessionInfoListExt as empty (i.e., not configuring explicit area IDs). From the UE point of view, upon association of an empty entry with a broadcast service in the MCCH, the UE should consider that the service is permissible for reception within the entire cell area. Concurrently, the legacy behavior remains applicable.

Proposal 4: If a broadcast session is intended for reception within the entire cell area, the network does not configure the intended area ID(s) associated with that service.

Proposal 5: RAN2 confirms that if no intended area ID is explicitly indicated in MCCH for an MBS broadcast service the UE is interested in, the UE considers the service is allowed for reception within the entire cell area, with legacy behavior applicable.

## 2.3	MBS Interest Indication (MII)

This past is related to the second FFS.

5.9.4.2 Initiation An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell. If the UE does not have the CFR information and subcarrier spacing for MBS broadcast reception on a non-serving cell at the time it sends the MBS Interest Indication, the UE sends another MBS Interest Indication after it has acquired this information from the non-serving cell.

Based on the current spec, we can observe that the RRC_CONNECTED UE may initiate the MII reporting procedure upon entering or leaving the broadcast service area (i.e., cell/tracking area) based on the USD, as highlighted above. We think the same technical logic can be applicable to the Rel-19 case. That is a UE may report MII when entering or leaving the intended service area, as shown in Figure 1. With that, the gNB may provide the UE with an RRC reconfiguration, select a proper target cell, and deprioritize unicast/multicast scheduling, to allow the UE to receive the MBS services the UE is interested in.

Specifically, for CONNECTED UE, MII can be used for scheduling prioritization handling between broadcast and unicast/multicast (e.g. if the UE prioritizes broadcast service(s) over unicast/multicast, the network may release some DRBs/multicast MRBs to save more radio bearer capability for broadcast reception). Then the CONNECTED UE should report it to the network when it goes inside the intended service area so that the network can do the proper scheduling prioritization handling. Furthermore, if the UE goes outside the intended service area, the UE should also indicate the MII since the network has to know it does not need to consider scheduling prioritization handling between broadcast and unicast/multicast anymore for this UE.

Figure 1: MII reporting when moving inside/outside the intended service area

Based on the above, we propose,

Proposal 6: RRC_CONNECTED UE may initiate the MBS Interest Indication reporting procedure when UE is entering or leaving the intended service area.

Proposal 7: The existing term “broadcast service area” in sub-clause 5.9.4.2 of TS 38.331 refers to “intended service area” when the cell provides SIBxx. Clarifying this with a NOTE in the RRC specification.

# 3 Conclusion

In this contribution, we have discussed the remaining issues on MBS broadcast in Rel-19 NTN. And we make the following observations and proposals,

Target service area in Service Announcement:

Proposal 1: RAN2 confirms that there is no need to align the MBS ISA signaling in SIBxx with that in Service Announcement.

Proposal 2: It is up to UE implementation to use the targeted service area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.

Proposal 3: From RAN2 perspective, targeted service area list in the USD is not used for other purposes.

Singalling details:

Proposal 4: If a broadcast session is intended for reception within the entire cell area, the network does not configure the intended area ID(s) associated with that service.

Proposal 5: RAN2 confirms that if no intended area ID is explicitly indicated in MCCH for an MBS broadcast service the UE is interested in, the UE considers the service is allowed for reception within the entire cell area, with legacy behavior applicable.

MII:

Proposal 6: RRC_CONNECTED UE may initiate the MBS Interest Indication reporting procedure when UE is entering or leaving the intended service area.

Proposal 7: The existing term “broadcast service area” in sub-clause 5.9.4.2 of TS 38.331 refers to “intended service area” when the cell provides SIBxx. Clarifying this with a NOTE in the RRC specification.

# 4 Reference

[1] R2-2505002, RAN2#130 Meeting Report, MCC.

[2] S4-251099, Reply on Inclusion of NTN intended service area in the Service Announcement, Ericsson.