3GPP TSG-RAN WG2 Meeting #131				               R2-2505255

Bengaluru, India, August 25th – 29th , 2025

Agenda item:	8.5.4

Source:	Samsung

Title:	Adaptation of common signal transmissions

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss the RAN2 aspects of Paging and PRACH adaptation in time domain.

# Discussion

## 2.1. Paging adaptation

Starting PDCCH monitoring occasion number for adaptive POs

RAN2 has agreed the following

- Introduce a new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter for Rel-19 UEs that support adaptive paging

Its not clear whats the UE behaviour, if firstPDCCH-MonitoringOccasionOfPO-r19 is not configured.

The intention of firstPDCCH-MonitoringOccasionOfPO-r19 is have flexible offset between the POs configured for paging adaptation. If firstPDCCH-MonitoringOccasionOfPO-r19 is not configured, UE does not determine POs based on starting PDCCH monitoring occasion number i.e. UE applies the green highlighted part below.

TS 38.304

“

When firstPDCCH-MonitoringOccasionOfPO is present, the starting PDCCH monitoring occasion number of (i_s + 1)th PO is the (i_s + 1)th value of the firstPDCCH-MonitoringOccasionOfPO parameter; otherwise, it is equal to i_s * S*X.

“

Proposal 1: If firstPDCCH-MonitoringOccasionOfPO-r19 is not configured, UE does not determine POs based on starting PDCCH monitoring occasion number. The starting PDCCH monitoring occasion number of (i_s + 1)th PO is equal to i_s * S*X.

pagingAdaptation-NS-r19 and pagingAdaptationNAndPagingFrameOffset configuration

In the current CR both parameters are optionally configured i.e. network may configure only one of them for paging adaptation. In our understanding, both of them should be configured for paging adaptation. If only one of them is configured, we need additional rule to determine missing parameters of second configuration.

Proposal 2a: If network supports paging adaptation, both pagingAdaptation-NS-r19 and pagingAdaptationNAndPagingFrameOffset are configured.

Proposal 2b: pagingAdaptation-NS-r19,  pagingAdaptationNAndPagingFrameOffset and  firstPDCCH-MonitoringOccasionOfPO are grouped together in  one IE.  pagingAdaptation-NS-r19,  pagingAdaptationNAndPagingFrameOffset are mandatory in this IE. firstPDCCH-MonitoringOccasionOfPO-r19 is optionally configured in the IE.

Paging search space for adaptive POs

PagingSearchSpace can be zero or non zero. For paging search space zero, Ns can be 1 or 2 only. Inorder to have larger value of Ns for adaptive POs, it should be possible to configure non zero paging search space for adaptive POs irrespective of paging search space configuration for legacy POs. To have this flexibility, Paging search space should be separately configured for paging adaptation.

Proposal 3: Paging search space is separately configured for paging adaptation

## 2.3. PRACH adaptation in time domain and CE

RAN2#130 agreement

RAN2 will select one of the following options. (CE here means MSG1 repetition).

- Option-1b: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon.

- Option-1c: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, but rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon. (i.e., if [feature combination including CE / CE-only feature] is included in featureCombinationPreamblesList-r17 as one of the associated feature (-combination)s, the additional RACH is not supposed to be included in this RACH-ConfigCommon for this featureCombinationPreamblesList-r17)

In our view, there is no need for option 1c. It unecessarily restricts network implementation. Option 1b is sufficient and if network wants it may not configure CE and non CE feature in RACH-ConfigCommon including additional ROs.

Proposal 4: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if RACH-ConfigCommon associated with the selected random access resource set includes additional RO(s) and the selected random resource set is associated with MSG1 repetition, UE does not use the additional RO(s) configured in this RACH-ConfigCommon.

# Conclusion

Proposal 1: If firstPDCCH-MonitoringOccasionOfPO-r19 is not configured, UE does not determine POs based on starting PDCCH monitoring occasion number. The starting PDCCH monitoring occasion number of (i_s + 1)th PO is equal to i_s * S*X.

Proposal 2a: If network supports paging adaptation, both pagingAdaptation-NS-r19 and pagingAdaptationNAndPagingFrameOffset are configured.

Proposal 2b: pagingAdaptation-NS-r19,  pagingAdaptationNAndPagingFrameOffset and  firstPDCCH-MonitoringOccasionOfPO are grouped together in  one IE.  pagingAdaptation-NS-r19,  pagingAdaptationNAndPagingFrameOffset are mandatory in this IE. firstPDCCH-MonitoringOccasionOfPO-r19 is optionally configured in the IE.

Proposal 3: Paging search space is separately configured for paging adaptation.

Proposal 4: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if RACH-ConfigCommon associated with the selected random access resource set includes additional RO(s) and the selected random resource set is associated with MSG1 repetition, UE does not use the additional RO(s) configured in this RACH-ConfigCommon.

# References