3GPP TSG RAN WG2 Meeting #131                                                            R2-2505262

# Bangalore, India Aug 25th – 29th, 2025

Agenda item:	8.7.6

Source:	Ofinno

Title:	UL Rate Control for XR

Document for:	Discussion and decision

# Introduction

In this contribution, we cover the following topics about UL Rate Control for XR:

- [MAC-3] QoS Flow Identification: regarding the two options for QoS flow identification left from the last RAN2 meeting, we prefer using a bitmap within the UL Rate Control MAC CE to identify QoS flows. The mapping between the bits in the bitmap and the specific QoS flows is established through a list of {PDU Session ID + QFI} pairs pre-configured by RRC signalling.

- [MAC-5] Partial Transmission of Bit Rate Queries: to address scenarios where available UL-SCH resources are insufficient to accommodate all pending bit rate queries in a single UL Rate Control MAC CE, we propose that the UE be allowed to transmit a subset of the pending queries. In this case, only the bit rate queries that have been transmitted would be cancelled.

- (UE capability-05) UE Capability for Maximum Number of QoS Flows: we proposed introducing a UE capability or UE assistance information to indicate the maximum number of QoS flows for which the UE supports or prefers to have UL Rate Control. This aims to simplify UE implementation by limiting the number of QoS flows that need to be supported for rate control.

# Discussion

## QoS flow Identification in UL Rate Control MAC CE

RAN2#130 Agreements - (MAC-05) XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify. - (MAC-04) We rule out explicit signaling of PDU session ID + QFI - (MAC-04) We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead: - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit) - Implicit, e.g. index or mapping

As specified in TS 38.300 [1], the 5GC can establish one or more PDU sessions for a given UE, with each QoS flow within a PDU session being uniquely identified by a QFI. Consequently, the same QFI value may be assigned to distinct QoS flows belonging to different PDU sessions. Furthermore, at the Access Stratum level, the NG-RAN maps packets associated with different PDU sessions to different DRBs. Multiple QoS flows belonging to the same PDU session can be mapped to the same DRB. Therefore, two options emerge for uniquely identifying a QoS flow:

- (1) PDU session ID + QFI; and

- (2) DRB ID + QFI.

Regarding the explicit methods for indicating the QFI in the UL rate control MAC CE, both alternatives were initially considered. The first option, PDU session ID + QFI, would require the MAC layer to be aware of PDU session information, which would have implications for the F1AP interface, as the CU would need to convey PDU session information to the DU. Moreover, in comparison to the 5-bit DRB ID used in the second option, DRB ID + QFI, the 8-bit PDU session ID would introduce additional overhead over the Uu interface. Consequently, the second option, DRB ID + QFI, was preferred during the last RAN2 meeting.

In addition to the explicit methods for identifying QoS flows, an alternative approach involving implicit mapping has also been proposed. This option suggests employing a shorter and implicit identifier within the MAC CE, with the mapping to the actual QoS flow established through RRC signaling. This shorter identifier could take the form of an index or a bitmap referencing a list of QoS flows pre-configured by the network. The RRC configuration would define the association between this shorter identifier and the complete, unique QoS flow identifiers (e.g., PDU session ID + QFI). In the case of a bitmap, each bit could correspond to a respective QoS flow pre-configured by RRC, with a bit’s value indicating whether a recommended bit rate is being provided for that respective QoS flow.

Therefore, as shown in Figure 1, the options after the last RAN2 meeting were narrowed down to:

- Option 1 (in Figure 1(a)): Explicit DRB ID + QFI in the UL rate control MAC CE.

- Option 2 (in Figure 2(b)): Implicit, e.g., bitmap for QoS flows pre-configured by RRC.

Figure 1: Explicit vs. Implicit Options for QoS flows

Table 1: Comparison between Option 1 and Option 2

|  | Option 1 (Explicit: DRB ID + QFI) | Option 2 (Implicit: Bitmap for QoS flows) |
| --- | --- | --- |
| Pros | - Direct identification: DRB ID + QFI can uniquely identify a QoS flow, which allows the MAC to directly deliver the rate control information to upper layers without needing any remapping. | - Flexibility for Network: The network maintains control over the mapping between the shorter index/bitmap and the actual QoS flows through RRC signaling, allowing for flexible management. - Signaling overhead reduction: This method offers the most substantial reduction in signaling overhead within the MAC CE compared to explicit DRB ID + QFI and provides a more compact and efficient way to signal, needing only a single bit per QoS flow within the bitmap. |
| Cons | - High signaling overhead: The DRB ID (5 bits) + QFI (6 bits) is considered excessive, especially when the UL rate control MAC CE is expected to include information for multiple QoS flows simultaneously. - Latency for QoS flow remapping: if a DRB is released and its QoS flow is remapped to another DRB, new MAC CE with the updated DRB ID would be required to inform about the change, potentially adding latency and signaling overhead. | - Fixed number of QoS flows (for bitmap): The number of QoS flows that can be indicated in a single MAC CE is limited by the size of the bitmap. |

The comparison between Option 1 and Option 2 is summarized in Table 1.

While the explicit approach, DRB ID + QFI (Option 1), provides a straightforward method for identifying QoS flows and is consistent with current MAC layer procedures, it necessitates the inclusion of a dedicated identifier (total 11 bits) for each QoS flow within the MAC CE, which can result in significant overhead, particularly when reporting a large number of QoS flows simultaneously.

The implicit approach, Bitmap for QoS flows (Option 2), directly mitigates this overhead concern through:

- (1) pre-configuring the relevant QoS flows via RRC signaling to utilize their complete and unique identifiers (e.g., PDU Session ID + QFI), thereby establishing a known mapping at both the UE and the network.

- (2) employing a much shorter bitmap (e.g., a single octet for up to 8 QoS flows) efficiently indicates, within the MAC CE, which of these pre-configured QoS flows are subject to rate control .

This implicit approach enables the simultaneous adjustment of multiple QoS flows with minimal per-QoS flow overhead in the MAC CE, which is a critical attribute for handling multi-modal XR traffic characterized by its diverse and dynamic rate requirements. By balancing the need for unique identification with the essential requirement for signaling efficiency, particularly in the context of a dynamic UL rate control mechanism for XR, the Option 2 (Implicit, bitmap for QoS flows pre-configured by RRC) emerges as the more advantageous option.

[MAC-3] A UL Rate Control MAC CE includes a bitmap to identify QoS flows, where the bits of the bitmap are mapping to QoS flows identified via a list of {PDU Session ID + QFI} pairs pre-configured by RRC.

## UL Resources Accommodation for UL Rate Control MAC CE

TS 38.321 [2] ### 5.18.x UL Rate Control The UL Rate Control procedure provides the MAC entity with information on UL physical-layer bit rate available to a QoS flow. The gNB may transmit a UL Rate Control MAC CE (defined in clause 6.1.3.x) to the MAC entity to recommend UL bit rate(s) for one or multiple QoS flows. Upon reception of a UL Rate Control MAC CE, the MAC entity shall indicate the recommended bit rate(s) to upper layers. The MAC entity may transmit a UL Rate Control MAC CE to the serving gNB to request preferred bit rate(s) for one or multiple QoS flows. Upon request by upper layers for a preferred UL bit rate for a QoS flow, the MAC entity shall trigger a bit rate query for the QoS flow, if no other bit rate query is already pending for the same QoS flow. A bit rate query remains pending after being triggered, until it is cancelled. When UL-SCH resources are available for a new transmission, the MAC entity shall: 1> for each QoS flow with a pending bit rate query: 2> if bitRateQueryProhibitTimer for the QoS flow is configured but not running: 3> include the QoS flow and its preferred bit rate in the MAC entity’s list of bit rate queries; 1> if there is at least one entry in the MAC entity’s list of bit rate queries and the UL-SCH resources can accommodate the UL Rate Control MAC CE plus its subheader as a result of logical channel prioritization: 2> instruct the Multiplexing and Assembly procedure to generate the UL Rate Control MAC CE; 1> for each QoS flow whose bit rate query is included in the UL Rate Control MAC CE: 2> start its bitRateQueryProhibitTimer; 2> cancel its bit rate query. Editor’s note: FFS whether a UL Rate Control MAC CE is transmitted only if the available UL-SCH resources can accommodate all the pending queries

Based on the current TS 38.321 running CR [2], upon receiving a request from upper layers for a preferred UL bit rate for a QoS flow, the MAC entity is required to trigger a bit rate query for that specific QoS flow. A bit rate query remains in a pending state after being triggered until it is cancelled. When UL-SCH resources are available for a new transmission, the MAC entity includes the QoS flow and its associated preferred bit rate in a list of bit rate queries, considering only those QoS flows for which a bit rate query is currently pending. If the MAC entity’s list contains at least one entry, and the available UL-SCH resources are sufficient to accommodate the UL Rate Control MAC CE, the UE proceeds to generate/transmit the UL Rate Control MAC CE.

The question that arises is: if the available UL-SCH resources are insufficient to accommodate all the pending bit rate queries in the bit rate queries list within a single UL Rate Control MAC CE, is the UE permitted to transmit a subset of the bit rate queries, or is it obligated to wait until sufficient resources are available to include all pending bit rate queries together in a single UL Rate Control MAC CE?

We believe that, given the RAN2 agreement that a single UL Rate Control MAC CE can comprise multiple bit rate queries for multiple QoS flows, the potential exists for the UL Rate Control MAC CE to become excessively large when numerous bit rate queries are pending for various QoS flows. Requiring the UE to wait until sufficient UL-SCH resources are available to accommodate all the pending bit rate queries in a single UL Rate Control MAC CE may introduce significant delays in the transmission of the bit rate queries, as it necessitates the availability of substantial UL-SCH resources. Furthermore, the UL Rate Control MAC CE is currently assigned a relatively low MAC CE priority within the LCP procedure (i.e., only higher than BSR/SL-BSR for padding), which means that UL-SCH resources are likely to be consumed by other MAC CEs with higher priorities before being allocated to the UL Rate Control MAC CE. These further compounds the difficulty of securing sufficiently large UL-SCH resources at a single instance for a single UL Rate Control MAC CE that includes all the pending bit rate queries.

To facilitate the most expeditious transmission of bit rate queries, it is preferable to allow the UE to include a subset of bit rate queries for a selection of QoS flows in order to generate and transmit the UL Rate Control MAC CE. This approach would enable the UE to transmit the pending bit rate queries in segments, utilizing different UL-SCH resources at different times. When the UE generates/transmits a UL Rate Control MAC CE that includes bit rate queries for specific QoS flow(s), it should cancel the pending bit rate queries only for those QoS flow(s). The remaining bit rate queries would remain pending. The decision on which pending bit rate queries to include in a particular UL Rate Control MAC CE could be left to UE implementation or determined based on, for example, the ascending order of the identifiers of the QoS flows.

[MAC-5] If available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, the UE can include a subset of pending bit rate query(s) in the single UL Rate Control MAC CE, and only cancel the pending bit rate query(s) which has been included in the single UL Rate Control MAC CE.

[MAC-5] If available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, which pending bit rate queries should be included in the single UL Rate Control MAC CE can be up to UE implementation or based on ascending order of the identifiers of the QoS flows.

## UE Capability of Max number of QoS flows for Rate Control

With UL Rate Control, XR applications gain the ability to dynamically adjust their codec bit rate and encoding parameters, leading to more precise uplink congestion management. This is achieved by monitoring prevailing network conditions and adapting the application’s behavior accordingly, for instance, by reducing resolution or frame rate in response to network congestion.

As proposed in [3], a UE can theoretically support up to 960 QoS flows, given that each PDU session can include up to 64 QoS flows, and a UE can support up to 15 PDU sessions. From the UE vendors’ perspective, it is not necessarily the case that all QoS flows should support the UL rate control. For example, the UL rate control might be restricted to only a subset of QoS flows that is related to XR applications or to specific high data rate QoS flows to mitigate potential congestion. Furthermore, different tiers of UEs should possess varying capabilities regarding the number of QoS flows for which they can support the UL rate control. A high-tier UE with greater processing power and memory capacity would be able to support more QoS flows than a low-tier UE.

To simplify the UE implementation, we share the same view as [3] that it is thus advantageous to define a maximum number of QoS flows that can be supported/preferred by the UE for UL rate control. The specific value of this maximum number should be contingent on the UE’s capability or UE’s preference. Therefore, it is more appropriate to define this maximum number as a UE capability or UE assistance information rather than as a fixed constant in the standard applicable to all UE types.

(UE capability-05) Introduce a UE capability or UE assistance information to indicate the maximum number of QoS flows that the UE supports/prefers for UL Rate Control.

# Conclusion

Based on the analysis above, we have made the following proposals:

Proposal 1	[MAC-3] A UL Rate Control MAC CE includes a bitmap to identify QoS flows, where the bits of the bitmap are mapping to QoS flows identified via a list of {PDU Session ID + QFI} pairs pre-configured by RRC.

Proposal 2	[MAC-5] If available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, the UE can include a subset of pending bit rate query(s) in the single UL Rate Control MAC CE, and only cancel the pending bit rate query(s) which has been included in the single UL Rate Control MAC CE.

Proposal 3	[MAC-5] If available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, which pending bit rate queries should be included in the single UL Rate Control MAC CE can be up to UE implementation or based on ascending order of the identifiers of the QoS flows.

Proposal 4	(UE capability-05) Introduce a UE capability or UE assistance information to indicate the maximum number of QoS flows that the UE supports/prefers for UL Rate Control.

# Reference

3GPP TS 38.300 V18.6.0

[POST129bis][502][XR] MAC running CR and open issues (Qualcomm)

R2-2503368 Discussion on XR rate control (Qualcomm, MediaTek)