3GPP TSG-RAN WG2 Meeting #131	R2-2505342

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.13.3

Source:	OPPO

Title:	SRAP configuration for multi-hop U2N Relay

Document for:	Discussion, Agreement

Introduction

This paper is to further discuss the left issue about SRAP configuration for the indirectly connected remote UE at the U2N Relay UE.

Discussion

For how to configure the Remote UE ID and BEARER ID, in RAN2 #130 meeting, the reflective bearer mapping was discussed, and the conclusion is as follows.

Do not implement reflective bearer mapping in the running CR(s) now; interested companies are asked to converge a single detailed TP for final go/no-go decision next meeting.

The motivation/benefit of supporting reflective bearer mapping is not clear. The intention of reflective bearer mapping is as follows:

- To reuse legacy per-remote UE L2 ID configured list, i.e., each indirectly connected remote UE is configured based on the L2 ID of itself;

- To use reflective bearer mapping instead of configuration from the network to determine the DL egress link and the relay UE.

Firstly, For the structure of SRAP configuration, by following legacy principle, SRAP configuration should be used for the egress link determination when a SRAP PDU is received. In legacy, L2 ID-local ID mapping is used for egress link determination, i.e., L2 ID is used to identify a link, and the mapping between L2 ID and local ID actually means the mapping between link and local ID.

In legacy U2N Relay, L2 ID-local ID mapping in SRAP configuration is used for egress link determination, i.e., the mapping between L2 ID and local ID actually indicates the mapping between egress link and local ID.

In the baseline mechanism, the SRAP configuration is based on the L2 ID of the directly connected child UE, which follows legacy egress link determination mechanism.

In the running CR, egress link determination follows legacy mechanism, i.e., egress-link is determined based on SRAP configuration.

For a Relay UE performing SRAP PDU forwarding, the key concerns are identifying the PDU on the receiving side and determining the appropriate egress link or RLC channel on the transmitting side. The L2 ID of an indirectly connected child UE is not relevant to this forwarding process. Therefore, configuring SRAP for indirectly connected remote UEs based on their individual L2 IDs does not fulfill the legacy SRAP configuration purpose - namely, indicating the egress link and bearer mapping. Instead, it introduces unnecessary signaling overhead without functional benefit.

The L2 ID of the indirectly connected child UE is not used for the SRAP PDU handling at all, so there is no point to intentionally waste signaling overhead for useless fields.

In the proposed per-remote UE L2 ID-based SRAP configuration, egress link determination cannot be directly achieved through SRAP configuration as it is in the legacy and baseline mechanisms. Therefore, to support egress link determination under this approach, additional solutions must be introduced.

Proponent of remote UE L2 ID based SRAP configuration also admit that additional solution is needed for egress link determination at the relay UE for DL data transmission.

Two solutions are proposed by interested companies,

- Option-1: to rely on reflective bearer mapping, i.e., relay UE doesn’t need SRAP configuration for the egress link determination;

- Option-2: to add additional configuration “next-hop” to indicate the L2 ID of the directly connected remote UE.

Reflective bearer mapping requires each Relay UE to maintain and update the mapping between Local ID and ingress link, which introduces additional complexity, particularly in multi-hop scenarios where the number of child UEs is higher and dynamic link changes are more frequent.

The reflective bearer mapping requires each relay UE to maintain and update the Local ID to ingress link mapping, which adds additional complexity for the relay UE especially considering multi-hop scenario.

Another issue is error handling, in legacy, the relay UE determines whether a packet is valid by the following two aspect:

- Whether the UE ID and BEARER ID match the SRAP configuration;

- Whether the L2 ID configured associated with the UE ID matches the ingress link.

For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRelay is received except in the case where the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2IdentityRemote of the ingress link is received by U2N Relay UE, the SRAP entity shall: - Discard the received SRAP Data PDU.

The second aspect of error handling cannot be supported by reflective bearer mapping, as there is no valid mapping from UE ID to L2 ID that identifies the ingress link configuration.

Without L2 ID which can identify ingress link configured in reflective bearer mapping mechanism, the handling of unknown, unforeseen, and erroneous protocol data at the relay UE cannot work.

The alternative solution of configuring an additional next-hop indication is conceptually similar to the current baseline mechanism, which establishes an association between the L2 ID (identifying the ingress link) and the UE ID of the remote UE. However, this approach introduces extra signaling overhead due to the inclusion of remote UE L2 IDs, which do not contribute functional value in this context.

Therefore, reflective bearer mapping as an optimization should not be supported considering the above issues and unclear gain.

(SRAP-1) RAN2 not pursue reflective bearer mapping enhancement and adopt the legacy solution as in the running CR (i.e., child UE’s SRAP configuration includes entries for indirect child UE).

Conclusion

We have the following observations:

Observation 1	In legacy U2N Relay, L2 ID-local ID mapping in SRAP configuration is used for egress link determination, i.e., the mapping between L2 ID and local ID actually indicates the mapping between egress link and local ID.

Observation 2	In the running CR, egress link determination follows legacy mechanism, i.e., egress-link is determined based on SRAP configuration.

Observation 3	The L2 ID of the indirectly connected child UE is not used for the SRAP PDU handling at all, so there is no point to intentionally waste signaling overhead for useless fields.

Observation 4	Proponent of remote UE L2 ID based SRAP configuration also admit that additional solution is needed for egress link determination at the relay UE for DL data transmission.

Observation 5	The reflective bearer mapping requires each relay UE to maintain and update the Local ID to ingress link mapping, which adds additional complexity for the relay UE especially considering multi-hop scenario.

Observation 6	Without L2 ID which can identify ingress link configured in reflective bearer mapping mechanism, the handling of unknown, unforeseen, and erroneous protocol data at the relay UE cannot work.

We have the following proposals:

Proposal 1	(SRAP-1) RAN2 not pursue reflective bearer mapping enhancement and adopt the legacy solution as in the running CR (i.e., child UE’s SRAP configuration includes entries for indirect child UE).