3GPP TSG-RAN WG2 Meeting #131	R2- 2505618

Bangalore, India, August. 25th – 29rd, 2025

Agenda Item	: 8.13.3

Source	: LG Electronics Inc.

Title	: Discussion on Control Plane Procedure for multi-hop U2N relay

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-242349 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we mainly investigate the notification message delivery condition considering the other working group spec works (e.g., CT1/SA2).

2.	Discussion

2.1 Notification message delivery condition

RAN2 sends LS to SA2 (R2-2504800) for inform the RAN2 agreement about the notification message. The followings are captured from the LS.

Agreements: The notification message is used for the L2 child UE in case of upstream relay (re)selection triggered by AS layer failure cases (Uu/PC5 link failure/release, using legacy cause value) where the upper layer does not trigger link release towards the child. For reselection triggered by upper layers we rely on the upper-layer notification/release mechanism already defined. Remote UE handling of the notification is up to upper layer implementation. Further handling of any inter-layer issues on this topic can be done in maintenance phase.. Regarding to the notification message mentioned in the above agreements, it is defined in TS 38.331 as NotificationMessageSidelink since Rel-17, in details, the notification message is used to inform L2 or L3 U2N Remote UE(s) when the L2 or L3 U2N Relay UE performs handover, detects Uu RLF, or its Uu RRC connection establishment/resume fails. Upon reception of the notification message, it is up to U2N Remote UE implementation whether to release or keep the unicast PC5 link. If U2N Remote UE decides to release the unicast PC5 link, it triggers the PC5 release procedure and may perform cell or relay reselection.

According to our understanding based on the RAN2 agreement, the notification message can be used for L3 or L2 U2N intermediate Relay UE or Remote UE. If the upper layer keeps maintaining the child link after the AS layer failure cases in the upstream, the AS layer can send the NotificationMessageSidelilnk message to the child UE. When the child UE receives the NotificationMessageSidelilnk message from its parent UE, it’s up to the child UE’s implementation whether to maintain or release the link with the parent UE. In other words, the UE receiving the NotificationMessageSidelilnk message has an opportunity to decide whether to maintain or release the link.

Meanwhile, according to the CT1 spec (TS 24.554), they are describing the case of relay reselection triggering conditions and how to handle the relay reselection procedure. The followings are partly captured from TS 24.554.

8b.2.3.2 Multi-hop UE-to-network relay reselection procedure initiation The 5G ProSe multi-hop remote UE or the 5G ProSe intermediate UE-to-network relay UE shall trigger the multi-hop UE-to-network relay reselection procedure if one of the following conditions is met: a) the UE has received a lower layers indication that the serving 5G ProSe multi-hop UE-to-network relay UE or the serving 5G ProSe intermediate UE-to-network relay UE no longer fulfills the lower layers criteria as specified in 3GPP TS 38.331 [13]; <Omitted> To conduct multi-hop UE-to-network relay reselection process, the UE shall first initiate one of the following procedures or both depending on UE's configuration parameters for 5G ProSe multi-hop UE-to-network relay as specified in clause 5.2.8: a) monitoring procedure for UE-to-network relay discovery as specified in clause 8b.2.1.2.3; or b) discoverer procedure for UE-to-network relay discovery as specified in clause 8b.2.1.3.2. After the execution of the above discovery procedure(s), the 5G ProSe multi-hop remote UE performs the multi-hop UE-to-network relay selection procedure as specified in clause 8b.2.5. For a 5G ProSe intermediate UE-to-network relay UE based on model B discovery, if the 5G ProSe intermediate UE-to-network relay UE detects link failure or link quality degradation, the the 5G ProSe intermediate UE-to-network relay UE may send a relay reselection indication toward the 5G ProSe multi-hop remote UE by using the 5G ProSe direct link modification procedure as specified in clause 8b.2.2.3. If the 5G ProSe multi-hop remote UE receives the relay reselection indication, it may initiate the multi-hop UE-to-network relay reselection procedure.

According to the CT1 spec, the relay UE shall trigger relay reselection when the lower layers do not fulfil the lower layer criteria as specified in 3GPP TS 38.331. The lower-layer criteria to initiate relay reselection triggering at the intermediate Relay UE may be Uu/PC5 link failure/release with the parent UE. When the relay reselection is triggered, the UE initiates the discovery procedure.

If relay reselection is triggered at the intermediate Relay UE, the intermediate Relay UE receives the announcement message from the candidate parent UEs and makes the PC5 connection with one of the candidate parent UEs before forwarding the announcement message to the child UE in the discovery model A. If the child UE receives the announcement message, the child UE can update the changed information via the announcement message, such as hop-count or root information changes, while maintaining the PC5 link with the intermediate Relay UE. According to the CT1 spec, in the discovery model B, the intermediate Relay UE may send a relay reselection indication toward the child UE when relay reselection is triggered. If the child UE receives the relay reselection indication from its parent UE, the child UE initiates relay reselection, i.e., transmitting a solicitation message. If the child UE maintains the PC5 link with the parent UE, the solicitation message can be delivered via the current maintaining PC5 link.

For these procedures, there is no clear description of whether the link between the intermediate Relay UE and child UE is maintained or released when relay reselection is triggered. The intermediate Relay UE may perform the relay reselection procedure while maintaining the child links. The upper layer can perform the link modification, such as hop-count or path change, via the discovery message or relay reselection indication while maintaining the child links. In the multi-hop relay, it takes a longer time to set the multi-hop connection than to set a single-hop connection, including the discovery procedure. So, partly recovering a link while maintaining the downstream links can help save signalling load and reduce connection effort.

Comparing the the NotificationMessageSidelilnk usage to the CT1 spec, the NotificationMessageSidelilnk gives a possibility to the child UE whether to maintain the link with the parent UE or release. The unnecessary propagation of the NotificationMessageSidelilnk message can make unnecessary link release and relay reselection triggering at each intermediate Relay UE or Remote UE.

Observation 1: According to the CT1 spec (i.e., TS 24.554), the upper layer can handle the link modification, such as hop-count or path information changes via discovery message or relay reselection indication, while maintaining the downstream links.

Observation 2: The NotificationMessageSidelilnk message applied to L2 or L3 U2N Relay, and it gives the opportunity whether to maintain or release the upstream link to the intermediate Relay UE or Remote UE.

- When relay reselection is triggered at the intermediate Relay UE

If relay reselection is triggered (e.g., due to RLF or upstream link release) at the intermediate Relay UE and the reselected parent Relay UE is in the same cell as the original parent Relay UE, the intermediate Relay UE doesn’t need to send the NotificationMessageSidelilnk message to the child UE. The AS layer of the child UE doesn’t need to have the possibility to release the link with the current parent Relay UE. Also, there is no reason the NotificationMessageSidelilnk message is propagated along the downstream links. To support this operation, the intermediate Relay UE doesn’t send the NotificationMessageSidelilnk message immediately to the child UE when the relay reselection is triggered. The intermediate Relay UE tries to establish a PC5 connection with a new parent UE for a while. If the PC5 connection is established successfully and the new parent UE is in the same cell as the previous parent UE, the intermediate Relay UE doesn’t send the notification message. Otherwise, the intermediate Relay UE should send a notification message to the child UE.

Proposal 1: If the relay reselection is triggered at the RRC_IDLE/INACTIVE intermediate Relay UE and the new parent Relay UE is in the same cell as the previous parent UE, the intermediate Relay UE doesn’t need to send the NotificationMessageSidelilnk message to the child UE regardless of the hop-count or path information changes.

- When the intermediate Relay UE receives path switching commend (i.e., indirect-to-(in)direct path switching) within the same cell.

If the intermediate Relay UE receives a path switching command, for example, indirect-to-indirect or indirect-to-direct path switching, and the target parent UE belongs in the same cell as the previous parent UE, the intermediate Relay UE doesn’t need to send the NotificationMessageSidelilnk message to the child UE. Because gNB already has the context of all the downstream relay UEs of the intermediate Relay UE, the gNB can configure a proper configuration to the intermediate Relay UE to keep the downstream links. It’s different from the group HO in terms of just changing the path.  Even if the hop-count is changed, it can be handled by the upper layer via discovery message or relay reselection indication.

More specifically, if the intermediate Relay UE receives path switching command from the gNB and the target parent UE is in the same cell as the previous parent UE, the intermediate Relay UE doesn’t send the NotificationMessageSidelilnk message immediately to the child UE. Or, if the intermediate Relay UE receives path switching command from the gNB and the target path is a direct path to the same cell, the intermediate Relay UE doesn’t send the NotificationMessageSidelilnk message immediately to the child UE. In those cases, the intermediate Relay UE doesn’t send the NotificationMessageSidelilnk message to the child UE when the connection establishment to the target path is successfully performed. The change of the upper layer information, such as path information or hop-count change, can be handle at the upper layer. The AS layer sends the NotificationMessageSidelilnk message only when the cause of the NotificationMessageSidelilnk should give the opportunity to the child UE whether to maintain or release the connection with the parent UE.

Proposal 2: If the RRC_CONNECTED intermediate Relay UE receives path switching command and the target parent Relay UE is in the same cell of the previous parent UE, the intermediate Relay UE doesn’t need to send NotificationMessageSidelilnk message to the child UE regardless of the hop-count or path information changes.

Proposal 3: If the RRC_CONNECTED intermediate Relay UE receives path switching commend and the target path is the direct path of the same cell, the intermediate Relay UE doesn’t need to send NotificationMessageSidelilnk message to the child UE.

2.2 Timers

According to the RAN2 agreement at the last meeting, multi-hop remote UE uses the extended timer value for the connection establishment timers, such as T300, T301, and T319. The UE and gNB multiply the signalled timer value by the hop count. gNB will recognize the hop count based on the recursive connection establishment procedure. UE has to receive the hop count from its upper layer.

Proposal 4: The multi-hop Remote UE receives the hop count from its upper layer to extend the timers for connection establishment, such as T300, T301, and T319.

3.	Conclusion

Observation 1: According to the CT1 spec (i.e., TS 24.554), the upper layer can handle the link modification, such as hop-count or path information changes via discovery message or relay reselection indication, while maintaining the downstream links.

Observation 2: The NotificationMessageSidelilnk message applied to L2 or L3 U2N Relay, and it gives the opportunity whether to maintain or release the upstream link to the intermediate Relay UE or Remote UE.

Proposal 1: If the relay reselection is triggered at the RRC_IDLE/INACTIVE intermediate Relay UE and the new parent Relay UE is in the same cell as the previous parent UE, the intermediate Relay UE doesn’t need to send the NotificationMessageSidelilnk message to the child UE regardless of the hop-count or path information changes.

Proposal 2: If the RRC_CONNECTED intermediate Relay UE receives path switching command and the target parent Relay UE is in the same cell of the previous parent UE, the intermediate Relay UE doesn’t need to send NotificationMessageSidelilnk message to the child UE regardless of the hop-count or path information changes.

Proposal 3: If the RRC_CONNECTED intermediate Relay UE receives path switching command and the target path is the direct path of the same cell, the intermediate Relay UE doesn’t need to send NotificationMessageSidelilnk message to the child UE.

Proposal 4: The multi-hop Remote UE receives the hop count from its upper layer to extend the timers for connection establishment, such as T300, T301, and T319.

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNet.