3GPP TSG-RAN WG2 Meeting #131	R2-2505688

Bangalore, India, Aug 25th – 29th, 2025

Agenda item:		8.8.2

Source:		Lenovo

Title:		Some remaining issues for DL-CE in NTN

Document for:		Discussion

- Introduction

The latest RAN2 meetings made the following agreements regarding DL-CE (Downlink Coverage Enhancement) in NTN:

RAN2#129bis Agreements - From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits - We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases). - We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking) - Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode. RAN2#130 Agreements: - The maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue) - To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications) - We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported - No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme) RAN2#130 Working Assumption: We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options: Alt 1: UE provide the closest N reference locations/neighbor cells to network. Alt 2: UE reports the selected SMTCs from configured SMTC set to network. We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

In this contribution we discuss some remaining issues for DL-CE in NTN.

- Discussion

## SMTC enhancements in connected mode

For the SMTC configuration in IDLE/INACTIVE, a location-based SMTC selection procedure was agreed to be introduced, and each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. Although additional complexity is expected, we see some benefit of this scheme to achieve more accurate SMTC selection relying on distance, which is a metric that has been used for NTN since Rel-17. However for the SMTC configuration in CONNECTED mode, we see no such necessity as the gNB can collect necessary information to configure SMTC appropriately. More specifically, regarding the working assumption with condition made in the previous RAN2#130 meeting:

We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options: Alt 1: UE provide the closest N reference locations/neighbor cells to network. Alt 2: UE reports the selected SMTCs from configured SMTC set to network. We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

Both Alt 1 and Alt 2 are proposed based on the assumption that the gNB needs to know which neighbour cell(s) is close to the UE (by the reported reference location, reported cell identity or reported SMTC selected). However, the assumption is not correct as the gNB has already been able to acquire such information through the PDD (Propagation Delay Difference) reporting, wherein the propagation delay differences between serving and neighbour cells clearly reflects the neighbour cell(s) close to the UE, and it is more accurate than the metric of distance to reference location on the ground level.

Proposal 1: To assist the gNB to configure SMTCs in connected mode, PDD report is sufficient as it has reflected the closet neighbour cells. (There is no need to confirm the previous WA)

## Timing of satellite beam activation/deactivation

With the agreement that “beam hopping” applies equally to all the satellite beams of a given cell, the beam activation/deactivation pattern is the same within a cell, which is a mechanism similar to cell DTX/DRX. Due to the large propagation delay and propagation delay difference in NTN, the configuring gNB and the configured UEs may have different understanding about the timing of satellite beam activation/deactivation. For DL satellite beams being activated or deactivated, UE may either miss possible transmission from network or monitor PDCCH too early with unnecessary power consumption.

Proposal 2: If a beam activation/deactivation pattern is configured by gNB, UE shall consider the propagation delay (e.g. half of UE-gNB RTT) when determine the timing of satellite beam activation/deactivation.

- Conclusion

In this contribution we discuss some remaining issues for DL-CE in NTN. The following proposals are given:

Proposal 1: To assist the gNB to configure SMTCs in connected mode, PDD report is sufficient as it has reflected the closet neighbour cells. (There is no need to confirm the previous WA)

Proposal 2: If a beam activation/deactivation pattern is configured by gNB, UE shall consider the propagation delay (e.g. half of UE-gNB RTT) when determine the timing of satellite beam activation/deactivation.