3GPP TSG-RAN WG2 Meeting #131						       	  R2-2505757

Bengaluru, India, 25 - 29 Aug, 2025

Title: 	LCM for UE-sided model for Beam Management and CSI use cases

Source: 	Huawei, HiSilicon

Agenda item:	8.1.2.2

Document for:	Discussion/Decision

# Introduction

This Tdoc is to address the remaining open issues from the running 38.331 CR discussion [1].

# Discussion

## Open issue RRC-1: Cause of inapplicability

Issue description: It is FFS how to define the simple cause value of inapplicability related to model availability and how to capture it in the spec.

Proposed resolution: This open issue was resolved by the RAN2#130 agreement below.

1 Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified)

The reason of introducing this “release configuration” flag is because the inference/applicable configuration will never be used during this connection due to the model related to inference/applicability configuration are unavailable in the local device.  This is also explicitly agreed in the previous RAN#129bis meeting that the simple cause is for model unavailable as highlighted below:

Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

Since the reason is for “model unavailable”, then “release configuration” flag does not seem logical to be applied for subsequent applicability status change from applicable to inapplicable and can only be indicated in the initial inapplicability report via RRCReconfigurationComplete message. Thus, we have proposal as following:

Proposal 1: (RRC-1) The release indication is only in initial inapplicability report via RRCReconfigurationComplete message (i.e. add a condition that the inapplicabilityCause-r19 is only for RRCReconfigurationComplete message).

As on the FFS on specifying the reason for the flag, we think that it can specify as part of the field description for the “release configuration” flag as in the running RRC CR, but with “e.g.” removed as it should be clear of the use of the “release configuration” flag by the network as illustrated below based on the running RRC CR:

releaseConfigurationPreference Indicates the UE’s preference to release the configuration associated to applicabilityReportConfigId (e.g. due to model unavailability), if the applicabilityStatus is set to 'inapplicable'. Editor's Note: FFS whether it is needed to capture the reason for this preference in the spec .

Proposal 2: (RRC-1) Specify the usage of the “release configuration” flag clearly in the field description (i.e. removing the “e.g.” in the running CR).

## Open issue RRC-13: CSI prediction LCM framework

Issue description: For the CSI prediction use case, it is very likely the same framework (e.g., applicability report, inference configuration, data collection, monitoring) for AI based beam management can be used, but it is still upon RAN1/RAN2’s confirmation.

In the WID RP-250792, we have a use case CSI prediction:

- CSI feedback enhancement, encompassing [RAN1/RAN2]:

  - CSI prediction (UE-sided model):

    - Functionality-based LCM leveraged from other use cases, when necessary and applicable,

    - Study, and if necessary, specify consistency of training/inference,

    - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

This use case is only for UE-sided model, and RAN1 has made some progress. Similar to BM use case, RAN2 impacts will be on the LCM procedure below:

- Applicability reporting/inference configuration

- UE request for UE side data collection for training

- Performance monitoring

For applicability reporting, we checked the latest RAN1 progress. Firstly, RAN1 has not made any conclusions on it. Secondly, in their FL summary R1-2503059, there were still some discussions on the details of applicability reporting, option A and option B are still kept in their proposal for future discussion as can be seen from the RAN1 feature discussion.

(clo) Proposal 2.1-1 For CSI prediction using UE-side model, Reuse general signalling procedure on applicable functionality reporting (Step 1~5 in the figure) for beam management UE-sided model in R2-2407848: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - Measurement related information, e.g., CSI-ResourceConfig. - Report related information, e.g., codebookConfig - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 - FFS on whether to down-select between Option A) and B). - Note: whether or not to introduce associated id is separate discussion.

Understanding that RAN1 is yet to complete discussing the applicability reporting for CSI prediction.  we think it is better to wait for more RAN1 progress (including L1 parameter list and the signaling support for Option B), and then RAN2 could check detailed specification impacts.

For data collection configuration for training, we agree with Rapp’s view as following:

(RRC-13) The procedures for UE data collection for the CSI prediction use case are the same as for the beam management use case (start/stop request from UE, candidate configuration provision from NW, request for preferred candidate configurations from UE, etc.).

The procedure is the same as beam management case, which includes the configurations for data collection candidate configurations as agreed for the beam management case and some configurations related with data collection in the RAN1 parameter list can be part of candidate configurations.

For inference and monitoring aspects, we think RAN1 is still discussing them, and RAN2 can wait for the L1 parameter list and then check for possible RAN2 impacts, which should be straightforward.

In summary, the procedure of CSI prediction case on LCM framework can be similar as for beam management case and the exact applicability reporting/inference/data collection/monitoring configuration can follow the L1 parameter list from RAN1. With this, we have the following proposal:

Proposal 3: (RRC-13) The same LCM framework of beam management case can be reused on CSI prediction case as baseline.On the signaling details for applicability reporting, inference, data collection and monitoring for CSI prediction, RAN2 can wait for more RAN1 progress (i.e., L1 parameter list including the signaling support of Option B for applicability reporting).

## Open Issue RRC-15: The time duration for an AI functionality to become available for inference when the UE reports applicability

Issue description: The UE needs to synchronize the time duration for an AI functionality to become available for inference with the network when the UE reports applicability for either a full inference configuration or a set of inference parameters.

This issue RRC-15 is relevant to the RAN4 LS [2], which relates to the activation of the periodic CSI reporting. We assume that RAN4 will handle this time duration for the AI configuration to become available for inference in Step 3 for Option A or in Step 5 for Option B. In the LS, companies in RAN4 have different interpretations about the following RAN2 agreement related to the availability of the periodic CSI reporting configuration: “the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4”.

- Interpretation 1: the applicable functionalities are already activated (i.e., the applicable functionalities activation is completed) before reporting applicable functionalities via RRCReconfigurationComplete in step 4

- Interpretation 2: UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete in step 4

Interpretation 1 means that the applicable periodic reporting configuration is available for inference before the UE sends the RRCReconfigurationComplete in Step 4.  This will mean that the periodic reporting configuration will have to be made available for inference within the RRC message processing time (e.g. 10/16 ms depending on RRC-17).  Whereas for Interpretation 2, making an applicable periodic reporting configuration becoming available for inference is decoupled from the RRC message processing time and RAN4 can decide further the activation delay upon reporting applicable configuration via RRCReconfigurationComplete.

From UE implementation perspective, UE will require some preparation time to make an applicable configuration available for inference, e.g. as in the rapporteur’s explanation on the open issue; UE implementations may use various types of memory to balance cost and performance, and these different memory types and constraints can affect the access speed of stored models. As it is related to UE implementation in terms of memory and access speed, it would be preferable to decouple from the fixed RRC processing time as in Interpretation 1 and hence, our inclination is towards Interpretation 2. This time period value(s) should be discussed in RAN4 and can be defined as part of RAN4 UE capability.

Observation 1: (RRC-15) As the time to make an applicable inference configuration available for inference is related to memory and access speed of UE implementation, it would be preferable to decouple from the fix RRC processing time as in Interpretation 1.

Proposal 4: (RRC-15) Adopt Interpretation 2 from the RAN4 LS [2] (i.e. UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete in step 4).  The time delay/preparation time values to make an inference configuration available for inference should be discussed in RAN4 and is defined as part of RAN4 UE capability.

## Open issue RRC-16: UE behavior when the associated ID is not provided by the network

Issue description: The UE behaviour when the associated ID is not provided by the network is not yet clarified.

In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - the associated ID Note: this doesn’t imply the associated ID is mandatory

From RAN1 #119 agreement highlight above, the NW-side additional condition can be indicated in step 3 optionally in Option A and B for applicability report.

The UE reporting behavior can be as follows depending on whether the network indicates or not the NW-side additional condition in Step 3 in all options or whether the UE indicates back or not in Step 4 for Options B:

- Case 1: If the network indicates the associated ID, the network can indicate the network-side additional condition it is interested in. In this case, the UE reports applicability only to the inference/applicability configuration corresponding to the indicated associated ID(s).

- Case 2: If the network does not indicate the associated ID for the applicability/inference configurations, the UE can determine which of the applicability configuration are applicable from UE perspective considering model availability and UE side additional conditions and report those configurations together with the corresponding associated IDs, if available to the network. Based on this, the network can select and/or reconfigure the selected applicable functionality for inference according to the current network-side addition condition.

For Case 1, NW may indicate the interested associated IDs when requesting the applicable functionality reporting, and then the UE only needs to report the applicability of the inference/applicability configurations associated with the indicated associated IDs. In this case, the UE can do the filtering based on the associated ID information, so that it does not need to report all the applicable functionality information stored.

For Case 2, when the associated ID is not provided, the RRC rapporteurs originally proposed the following for this case:

If the network does not provide the associated ID, the UE reports the applicability (applicable/inapplicable) based on UE-side additional conditions (internally known by UE), model availability in device and the other provided information in the configuration.

We think there are 2 sub-cases to consider:

- Associated ID was not provided during training and is not provided for applicability check:

  - In this case we think the above proposal from rapporteur should apply as the UE has all the information it needs to determine applicability

- Associated ID was provided during the training, but not provided by network for applicability check:

  - Firstly we think this case happens only for option B as for option A the UE is supposed to perform inference based on the configuration right away, so associated ID should be provided in case it was provided by the network during training

  - But for option B this scenario is still valid because the network may want to check under which NW conditions the UE is able to perform inference, then the applicability can be decided by the NW and the NW can then decide on the inference configuration to configure the UE. That is why we think that the UE should be able to report associated IDs matching the applicability report configuration in the applicability report

The benefits for reporting the associatedIDs in the applicable reporting for Option B is that UE reporting the applicability of inference/applicability configurations with the corresponding associated IDs can help network to reduce repeated applicability check procedure when the associated ID change to a new one which is included in the UE applicability report list. The NW can just perform a reconfiguration based on the associated ID reported earlier.  Another advantage is that the NW can reduce the signaling overhead in the RRCReconfiguration if multiple associated IDs are with the same resource configuration.

In summary, when network is not provided associated ID, the UE only checks applicability status by configurations exclude associated ID and UE-side additional conditions and model availability, then reports applicable functionalities together with the supported associated IDs, if available.

Proposal 5: (RRC-16) In case the network does not provide the UE with associated IDs, the UE performs applicability check without considering NW-side additional conditions and reports the (UE-considered) applicable inference/applicability configuration together, and in the case of Option B with their associated IDs, if available.

## Open issue RRC-17: Processing timing requirement of applicability/inapplicability report via RRCReconfigurationComplete

Issue description: According to Section 12 of TS 38.331, there are two processing latency requirements (10 ms vs 16 ms) between reception of RRCReconfiguration and reporting RRCReconfigurationComplete, depending on whether it is related to CA/DC operation. For Option A, it was agreed to report initial applicability report in RRCReconfigurationComplete. However, it is not clear what is its processing latency requirement because the applicability reporting is a new reporting different from CA/DC configuration.

For applicability status check, we agree with the Rapp's view that there will be some challenges for the UE to handle the operation, and the relaxed RRC processing time is friendly to UE, i.e., 16ms. However, when CA/DC operation is handled at the same time, the UE may not have enough time to determine the applicability for all the functionalities. Considering UEs may need various time to decide whether a functionality is applicable or not under different conditions (i.e. UE-side conditions/NW-side conditions), it is not suggested to further relax the RRC processing time in standard for the worst case, which would delay the data transmission.  Instead, when the UE has no enough time to determine the applicability when sending the RRCReconfigurationComplete message, the UE should be allowed to indicate the applicability status has not been determined yet. In this case, UAI can be provided later to update the applicability status after the UE completes the evaluation.

Observation 2: (RRC-17): With the existing RRC processing time (i.e. 16ms), the UE may not be able to decide whether a functionality is applicable or not when sending the RRCReconfigurationComplete message. Further relaxing of RRC processing time will delay the start of data transmission.

Proposal 6: (RRC-17) RRCReconfigurationComplete containing applicability reports has a processing latency requirement of 16 ms. The UE is allowed to indicate the applicability status of a functionality is not decided yet when sending the RRCReconfigurationComplete message.

## Open issue RRC-40: Configuration for Option B for applicability reporting

Issue description: RAN2 is still to confirm the assumption that option B is configured in otherConfig.

RAN2#129bis agreed that:

RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

It is still an assumption that option B in OtherConfig after RAN2 #129bis, until receiving RAN1 RRC parameters list and RAN1 #121 agreement which suggest the parameters in option B as following, we have the reason that option B should in OtherConfig.

RAN1#121 agreed that:

Agreement For option B of applicability check, RAN 1 assumes that at least the following RRC parameters are to be reused: - For both BM-Case 1 and BM-Case 2: - associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19 - For BM-Case 2: - TimeGap-r19, nroftimeinstance-r19, Note: this doesn’t imply the associated ID is always present

Even though most of the Option B parameter set configurations are in CSI-ReportConfig, the UE may start the measurement immediately if it is applicable which is not the intention for Option B unless some additional flag is added in CSI-ReportConfig (which will have RAN1 impact, similar to the dataCollectionPreferenceConfiguration). In addition, the UE behavior for Option A and B are clearly different. For Option A, the UE performs applicability check on a inference configuration in Step 3 but will activate the applicable inference configuration after applicability reporting in Step 4.  For Option B, the UE only performs applicability check on an applicability configuration in Step 3.  The UE does not have the full inference configuration that is applicable until Step 5 when the NW needs to activate it.  Based on these,  the Option B parameter set configurations should be placed in the OtherConfig.

Proposal 7: (RRC-40) RAN2 confirms the agreement that UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B.

## 2.6.1 Whether set of inference related parameters of Option B needs to be serving cell specific/

Compared with to Option A that where the NW will send CSI-Reconfiguration for inference configuration specific to a serving cell, sets of inference related parameters for Option B in OtherConfig can also be UE specific. For example, there are in cases where no associatedID is configured and/or that when the resources used by the set of inference related parameters are common among the serving cells, there may not be a need to indicate the serving cell and can be indicate implicitly to be valid for all the serving cellsexplicitly.  On the other hand, if associatedID is provided by the network and/or that the resources used by the set of inference related parameters are different for each of the serving cell, then there is a need to indicate the serving cell.  We think that making serving cell index as optional may reduce overhead.

Proposal 8: (RRC-40) For the set of inference related parameters configurations in the OtherConfig, the serving cell index is not present if no associatedID is configured and the CSI-Resource is common among all the serving cells. Otherwise, if associatedID is configured or the CSI-Resource is not common among the serving cells, the serving cell index is present.

Proposal 9: Take the draft TP in this contribution into account for stage 3 spec design. The relate change is highlight in yellow.

## Open issue RRC-14: Repeated reports of applicability for configurations which consistently perform poorly

Issue description: UEs may repeatedly report a configuration as applicable based on the applicability determination information (UE-side and NW-side) but still perform poorly based on unforeseen factors, e.g., scenarios not captured in the dataset which trained one or more models supporting the AI/ML configuration. There is nothing we have agreed to so far to prevent a UE from repeating the same indication of applicability every time it re-enters RRC CONNECTED.

One solution is that initial KPI of applicable functionality can be provided as part of applicable functionality reporting. For example, prediction accuracy can be calculated during training by using data for the ground truth as reference against the trained model or UE performs performance monitoring with the prediction accuracy during previous usage of the applicable functionality. This prediction accuracy can be defined as initial KPI. It would be beneficial for the network to know how well the applicable functionality has been trained (by the OTT server) or perform before it configures the UE to perform prediction on the functionality or trigger the data collection for training to improve on the KPI or perform performance monitoring before activating the applicable functionality. Without such initial KPI information of a reported applicable functionality by UE, the network may activate an applicable functionality where it may not meet its expectation and requiring starting the performance monitoring immediately after activation.

Observation 3: (RRC-14) Without initial KPI information of a reported applicable functionality by UE, the network may activate an applicable functionality where it may not meet its expectation and requiring starting the performance monitoring immediately after activation.

Proposal 10: (RRC-14) Initial KPI of trained functionality can be part of applicable functionality reporting.

## Open issues RRC-41, 46: Issues related to associated ID

### the area scope for associated ID (RRC-46)

Issue description: RAN1 did not take any agreement about supporting multi-cell specific associated ID.

This open issue refers to the RAN1#118 agreement:

Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Some companies think that a cell specific associated ID is not sufficient and there is a need for a multi-cell specific associated ID. The multi-cell specific associated ID can also be global (i.e. per PLMN).

The following are possible granularities for associated ID:

- Cell specific level: The associated IDs are unique only within a cell

- Multi-cell level: The associated IDs are unique to a group of cells

- PLMN level: The associated IDs are unique on a PLMN scope

We give some analysis based on the granularities for associated ID as following:

Table 1: Analysis of different options for the granularity of the associated IDs

| Options | Pros | Cons |
| --- | --- | --- |
| Cell specific level | 1) it follows RAN1 agreement 2) the size of the associated ID is small, so the signalling overhead is smaller than other options | UE may need to maintain associated IDs per cell which is complex at UE side. |
| Multi-cell level | 1) Maybe less training effort 2) The applicability reporting may be less than cell specific level during mobility | 1) It is unclear which of network nodes should allocate IDs. 2) Co-ordination efforts on IDs between network nodes 3) Management efforts on IDs, and OAM may be involved 4) May be large size of the associated ID |
| PLMN level | Similar Pros as multi-cell level. | Similar Cons as multi-cell level. Some issues are more significant, e.g. it may involve SA2 if CN is to allocate IDs. |

After comparing the pros and cons of three granularities for associated ID, we can observe main reasons for sticking to cell specific level for associated ID in Rel-19:

Observation 4: (RRC-46)

- Aligning with RAN1 assumption in #117 meeting, cell specific level is considering as the baseline for RAN2 discussion.

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell

- Multi-cell and PLMN level for associated ID needs more coordination and management among network nodes.  It is also unclear which of network node should allocate the associated IDs.

- Multi-cell and PLMN level may require potentially larger size associated ID,

- Whether the training effort will be less with multi-cell or PLMN level is in the scope of RAN1 and not of RAN2.

Based on the observation above, we can conclude the proposal as following:

Proposal 11: (RRC-46) Only cell specific level for associated ID is supported for Rel-19.

### Value length for associated ID (RRC-41)

According to assumption that associated ID is cell specific, then we can analysis the value range for associated ID.

In RAN1 #121 meeting agreement for position case, they analysis the value range for associated ID:

For AI/ML based positioning Case 1, regarding Info #7 in the assistance information from legacy UE-based DL-TDOA, - If implicitly provided, the implicit indication of Info #7 is via associated ID. - For given TRP(s), same associated ID implies that geographical coordinates of the TRP(s) can be understood as consistent by the UE. - The associated ID is not expected to provide the real value of Info #7 (i.e., geographical coordinates of the TRP(s) are not disclosed). - an associated ID is configured per-cell (e.g., NCGI-r15) - UE does not expect to receive different values of associated ID for TRPs belonging to the same NCGI-r15 - Associated ID can be realized by an identifier of N bits (e.g., 8 bits)

From the network perspective, beam-related settings are related to implementation. Generally, there may be many parameters relevant to beam-related settings, and then there may be many combinations based on these parameters. Therefore, we think that the associated ID length can be long enough for good network flexibility. Also considering the agreement of the range for associated ID in position case can be as the reference, 8 bit is reasonable.

Proposal 12: (RRC-41) As a starting point, the length of associated ID is 8 bits.

## Open issue RRC-44: Whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations

Issue description: Based on the RAN2 agreements below, it is not clear whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations stored at the UE, or whether it can contain only updates for the inference configurations for which the UE already reported the applicability status in the past.

This open issue refers to the RAN2#129 agreements:

3. Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. - Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause

In the rapporteur’s understanding, there are two interpretations of the agreements above:

- The RRCReconfigurationComplete contains the applicable/inapplicable status for all inference configurations. Only UAI contains updates.

- The RRCReconfigurationComplete contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message and only applicability changes for those inference configurations received in earlier RRCReconfiguration messages, for which the UE already reported applicable/inapplicable at least once. UAI contains updates.

In RAN2#129, RAN2 agreed  that the UE uses the RRCReconfigurationComplete message to initially report the availability status of inference configurations. Specifically, the UE includes only those configurations whose applicability has not been previously indicated in this message. So,  our understanding of the RAN2 agreement is as follow:.

The RRCReconfigurationComplete contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message. UAI contains updates.

In our view, in view that it is the last meeting before the functional freeze, RAN2 should follow the agreement made in RAN2#129 unless it can be proven that something is broken with the agreement.

Proposal 13: (RRC-44) Confirm the understanding that the RAN2 agreement in ‘Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability’ in RAN2#129 is follow:

The RRCReconfigurationComplete contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message. UAI contains updates.

## Open issue RRC-45: How/where to capture activation of periodic inference CSI-ReportConfig in specifications

Issue description: RAN2 agreed that the UE should activate a periodic inference configuration in CSI-ReportConfig only if it reports that it is applicable. There are several possibilities to capture this in the specifications, as commented by several companies, and it should be discussed how to best do this. Furthermore, it should be clarified if the solution can be captured only in RRC specifications or whether RAN1 should also capture (part of) the solution.

This open issue refers to the RAN2#129 agreement:

- If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

Some solutions suggested by companies for several aspects are:

- Where exactly in RRC to capture the solution:

  - In Section 5.3.5.3 when setting the content of RRCReconfigurationComplete, or

  - In Section 5.3.5.3 when sending RRCReconfigurationComplete to the lower layers

- What to write in RRC specifications?

  - Upon including the applicable status in RRCReconfigurationComplete, indicate to lower layers to activate the configuration, or

  - Submit/do not submit the configuration to lower layers, which would be the equivalent of activation/no activation.

On where to specify the exact text to capture the solution, a) and b) above are just modelling and we believe the UE will implement at the right place to inform or activate the periodic CSI reporting in the lower layers. As b) is closer to the point of activation, we have a slight preference to specify it in ‘In Section 5.3.5.3 when sending RRCReconfigurationComplete to the lower layers’.

On how it should be written, as commented during running CR discussion, we do not see a need to introduce activation/deactivation of the periodic inference configuration. For periodic CSI reporting configuration, if it is applicable during the initial reporting, the RRC just have to provide the configuration to the lower layers to configure it, Otherwise (i.e. not applicable), the lower layer will not be informed and hence will not activate it. Subsequently, when the periodic CSI reporting configuration becomes inapplicable, it can only be removed via deconfiguration by RRC.  Hence, we think (ii) is sufficient.

Proposal 14:  (RRC-45) On how/where to capture activation of periodic inference CSI-ReportConfig in specifications

- Capture in b) In Section 5.3.5.3 when sending RRCReconfigurationComplete to the lower layers

- On how to specify, there is no need to introduce activation/deactivation from RRC perspective, RRC just need to indicate the periodic inference CSI-ReportConfig to lower layer (i.e.  ii) Submit/do not submit the configuration to lower layers, which would be the equivalent of activation/no activation).

## Open issue RRC-48: Applicability reporting for inference related parameter sets after the corresponding full inference configuration is provided

Issue description: Based on current agreements, the UE reports the applicability of both full inference configurations and corresponding inference related parameter sets, even if some of them may correspond to each other.

The issue was raised that once the network provides full inference configuration for applicable inference related parameter set, it would be highly inefficient if UE keeps reporting both full inference configuration ID and inference related parameter set config ID upon change in applicability. One solution is to allow the UE to either provide the applicability of inference related parameter set or inference configuration once the inference configuration is provided based on the applicability of inference related parameter set.

First of all, this issue only arises only when both Option A and B are configured by the network, which may not be common.  Even if both Option A and B are configured, we do not see the need to optimize the signaling since the overhead may not be huge. It also ensures integrity of the UE behavior of both options as we do not need to specify UE behaviour if only Option B is configured and when both Option A and B are configured.

Observation 5: This issue only arises only when both Option A and B are configured by the network, which may not be common.  Even if both Option A and B are configured, we do not see the need to optimize the signaling since the overhead may not be considered huge for dedicated signalling. It also ensures integrity of the UE behavior of both options as we do not need to specify UE behaviour if only Option B is configured and when both Option A and B are configured.

Proposal 15: (RRC-48) No change in the UE behaviour is needed when only Option B is configured or when both Option A and B applicable reporting are configured by the UE (i.e. UE will keep reporting applicability/inapplicability for Option A and B separately if both Option A and B are configured by the NW).

# Conclusion

In this paper, our observations and proposals are summarized as follows listed by the number of RRC open issues:

Open issue RRC-1:

Proposal 1: (RRC-1) The release indication is only in initial inapplicability report via RRCReconfigurationComplete message (i.e. add a condition that the inapplicabilityCause-r19 is only for RRCReconfigurationComplete message).

Proposal 2: (RRC-1) Specify the usage of the “release configuration” flag clearly in the field description (i.e. removing the “e.g.” in the running CR).

Open issue RRC-13:

Proposal 3: (RRC-13) The same LCM framework of beam management case can be reused on CSI prediction case as baseline. On the signaling details for applicability reporting, inference, data collection and monitoring for CSI prediction, RAN2 can wait for more RAN1 progress (i.e., L1 parameter list including the signaling to support of Option B).

Open issue RRC-15:

Observation 1: (RRC-15) As the time to make an applicable inference configuration available for inference is related to memory and access speed of UE implementation, it would be preferable to decouple from the fix RRC processing time as in Interpretation 1.

Proposal 4: (RRC-15) Adopt Interpretation 2 from the RAN4 LS [2] (i.e. UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete in step 4).  The time delay/preparation time values to make an inference configuration available for inference should be discussed in RAN4 and is defined as part of RAN4 UE capability.

Open issue RRC-16:

Proposal 5: (RRC-16) In case the network does not provide the UE with associated IDs, the UE performs applicability check without considering NW-side additional conditions and reports the (UE-considered) applicable inference/applicability configuration together, and in the case of Option B with their associated IDs, if available.

Open issue RRC-17:

Observation 2: (RRC-17): With the existing RRC processing time (i.e. 16ms), the UE may not be able to decide whether a functionality is applicable or not when sending the RRCReconfigurationComplete message. Further relaxing of RRC processing time will delay the start of data transmission.

Proposal 6: (RRC-17) RRCReconfigurationComplete containing applicability reports has a processing latency requirement of 16 ms. The UE is allowed to indicate the applicability status of a functionality is not decided yet when sending the RRCReconfigurationComplete message.

Open issue RRC-40:

Proposal 7: (RRC-40) RAN2 confirms the agreement that UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B.

Proposal 8: (RRC-40) For the set of inference related parameters configurations in the OtherConfig, the serving cell index is not present if no associatedID is configured and the CSI-Resource is common among all the serving cells. Otherwise, if associatedID is configured or the CSI-Resource is not common among the serving cells, the serving cell index is present.

Proposal 9: Take the draft TP in this contribution into account for stage 3 spec design. The relate change is highlight in yellow.

Open issue RRC-14:

Observation 3: (RRC-14) Without initial KPI information of a reported applicable functionality by UE, the network may activate an applicable functionality where it may not meet its expectation and requiring starting the performance monitoring immediately after activation.

Proposal 10: (RRC-14) Initial KPI of trained functionality can be part of applicable functionality reporting.

Open issue RRC-41 and 46:

Observation 4 (RRC-46):

- Aligning with RAN1 assumption in #117 meeting, cell specific level is considering as the baseline for RAN2 discussion.

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell

- For multi-cell level and PLMN level, it is unclear which of network nodes should allocate IDs

- Other area scope for associated ID needs more coordination and management among network nodes and larger size of the associated ID,

- The advantage of the model generation due to UE knowing the network additional conditions the same or different for UE training and data collection is in the scope of RAN1 and not of RAN2.

Proposal 11: (RRC-46) Only cell specific level for associated ID is supported for Rel-19.

Proposal 12: (RRC-41) As a starting point, the length of associated ID is 8 bits.

Open issue RRC-44:

Proposal 13: (RRC-44) Confirm the understanding that the RAN2 agreement in ‘Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability’ in RAN2#129 is follow:

The RRCReconfigurationComplete contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message. UAI contains updates.

Open issue RRC-45:

Proposal 14: (RRC-45) On how/where to capture activation of periodic inference CSI-ReportConfig in specifications

- Capture in b) In Section 5.3.5.3 when sending RRCReconfigurationComplete to the lower layers

- On how to specify, there is no need to introduce activation/deactivation from RRC, RRC just need to indicate the periodic inference CSI-ReportConfig to lower layer (i.e.  ii) Submit/do not submit the configuration to lower layers, which would be the equivalent of activation/no activation).

Open issue RRC-48:

Observation 5: (RRC-48) This issue only arises only when both Option A and B are configured by the network, which may not be common.  Even if both Option A and B are configured, we do not see the need to optimize the signaling since the overhead is not huge.

Proposal 15: (RRC-48) No signaling optimization is needed when both Option A and B applicable reporting are configured by the UE (i.e. UE will keep reporting applicability/inapplicability for Option A and B if both Option A and B are configured by the NW).

# TP for other issue

In this section, we give the examples in Running CR 38.331 to design the IEs contents related with other Issue in section 2.12, which serving cell index is optional in OtherConfig and in ApplicabilityReportList.

### 6.3.4	Other information elements

<Text Omitted>

#### –	OtherConfig

The IE OtherConfig contains configuration related to miscellaneous other configurations.

OtherConfig information element

-- ASN1START

-- TAG-OTHERCONFIG-START

OtherConfig ::=                 SEQUENCE {

delayBudgetReportingConfig  CHOICE{

release                 NULL,

setup                   SEQUENCE{

delayBudgetReportingProhibitTimer   ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30}

}

}                                                                                                     OPTIONAL        -- Need M

}

OtherConfig-v1540 ::=           SEQUENCE {

overheatingAssistanceConfig     SetupRelease {OverheatingAssistanceConfig}                            OPTIONAL, -- Need M

...

}

OtherConfig-v1610 ::=                   SEQUENCE {

idc-AssistanceConfig-r16                SetupRelease {IDC-AssistanceConfig-r16}                       OPTIONAL, -- Need M

drx-PreferenceConfig-r16                SetupRelease {DRX-PreferenceConfig-r16}                       OPTIONAL, -- Need M

maxBW-PreferenceConfig-r16              SetupRelease {MaxBW-PreferenceConfig-r16}                     OPTIONAL, -- Need M

maxCC-PreferenceConfig-r16              SetupRelease {MaxCC-PreferenceConfig-r16}                     OPTIONAL, -- Need M

maxMIMO-LayerPreferenceConfig-r16       SetupRelease {MaxMIMO-LayerPreferenceConfig-r16}              OPTIONAL, -- Need M

minSchedulingOffsetPreferenceConfig-r16 SetupRelease {MinSchedulingOffsetPreferenceConfig-r16}        OPTIONAL, -- Need M

releasePreferenceConfig-r16             SetupRelease {ReleasePreferenceConfig-r16}                    OPTIONAL, -- Need M

referenceTimePreferenceReporting-r16    ENUMERATED {true}                                             OPTIONAL, -- Need R

btNameList-r16                          SetupRelease {BT-NameList-r16}                                OPTIONAL, -- Need M

wlanNameList-r16                        SetupRelease {WLAN-NameList-r16}                              OPTIONAL, -- Need M

sensorNameList-r16                      SetupRelease {Sensor-NameList-r16}                            OPTIONAL, -- Need M

obtainCommonLocation-r16                ENUMERATED {true}                                             OPTIONAL, -- Need R

sl-AssistanceConfigNR-r16               ENUMERATED{true}                                              OPTIONAL  -- Need R

}

OtherConfig-v1700 ::=                   SEQUENCE {

ul-GapFR2-PreferenceConfig-r17          ENUMERATED {true}                                             OPTIONAL, -- Need R

musim-GapAssistanceConfig-r17           SetupRelease {MUSIM-GapAssistanceConfig-r17}                  OPTIONAL, -- Need M

musim-LeaveAssistanceConfig-r17         SetupRelease {MUSIM-LeaveAssistanceConfig-r17}                OPTIONAL, -- Need M

successHO-Config-r17                    SetupRelease {SuccessHO-Config-r17}                           OPTIONAL, -- Need M

maxBW-PreferenceConfigFR2-2-r17         ENUMERATED {true}                                             OPTIONAL, -- Cond maxBW

maxMIMO-LayerPreferenceConfigFR2-2-r17  ENUMERATED {true}                                             OPTIONAL, -- Cond maxMIMO

minSchedulingOffsetPreferenceConfigExt-r17  ENUMERATED {true}                                         OPTIONAL, -- Cond minOffset

rlm-RelaxationReportingConfig-r17       SetupRelease {RLM-RelaxationReportingConfig-r17}              OPTIONAL, -- Need M

bfd-RelaxationReportingConfig-r17       SetupRelease {BFD-RelaxationReportingConfig-r17}              OPTIONAL, -- Need M

scg-DeactivationPreferenceConfig-r17    SetupRelease {SCG-DeactivationPreferenceConfig-r17}           OPTIONAL, -- Cond SCG

rrm-MeasRelaxationReportingConfig-r17   SetupRelease {RRM-MeasRelaxationReportingConfig-r17}          OPTIONAL, -- Need M

propDelayDiffReportConfig-r17           SetupRelease {PropDelayDiffReportConfig-r17}                  OPTIONAL  -- Need M

}

OtherConfig-v1800 ::=                   SEQUENCE {

idc-AssistanceConfig-v1800              SetupRelease {IDC-AssistanceConfig-v1800}                     OPTIONAL, -- Need M

multiRx-PreferenceReportingConfigFR2-r18 SetupRelease {MultiRx-PreferenceReportingConfigFR2-r18}      OPTIONAL, -- Need M

aerial-FlightPathAvailabilityConfig-r18 ENUMERATED {true}                                             OPTIONAL, -- Need R

ul-TrafficInfoReportingConfig-r18       SetupRelease {UL-TrafficInfoReportingConfig-r18}              OPTIONAL, -- Need M

n3c-RelayUE-InfoReportConfig-r18        ENUMERATED {true}                                             OPTIONAL, -- Need R

successPSCell-Config-r18                SetupRelease {SuccessPSCell-Config-r18}                       OPTIONAL, -- Need M

sn-InitiatedPSCellChange-r18            ENUMERATED {true}                                             OPTIONAL, -- Need R

musim-GapPriorityAssistanceConfig-r18   ENUMERATED {true}                                          OPTIONAL, -- Cond musimGapConfig

musim-CapabilityRestrictionConfig-r18   SetupRelease {MUSIM-CapabilityRestrictionConfig-r18}          OPTIONAL  -- Need M

}

OtherConfig-v1830 ::=                   SEQUENCE {

sl-PRS-AssistanceConfigNR-r18           ENUMERATED{true}                                              OPTIONAL  -- Need R

}

OtherConfig-v19xy ::=                   SEQUENCE {

applicabilityReportConfigList-r19                SetupRelease {ApplicabilityReportConfigList-r19}                   OPTIONAL, -- Need M

dataCollectionPreferenceConfig-r19           SetupRelease {DataCollectionPreferenceConfig-r19}              OPTIONAL, -- Need M

loggedDataCollectionAssistanceConfig-r19     SetupRelease {LoggedDataCollectionAssistanceConfig-r19}        OPTIONAL  -- Need M

}

IDC-AssistanceConfig-v1800 ::=          SEQUENCE {

idc-FDM-AssistanceConfig-r18            SetupRelease {IDC-FDM-AssistanceConfig-r18}                   OPTIONAL, -- Need M

idc-TDM-AssistanceConfig-r18            ENUMERATED {setup}                                            OPTIONAL  -- Cond FDM

}

MultiRx-PreferenceReportingConfigFR2-r18 ::= SEQUENCE {

multiRx-PreferenceReportingConfigFR2ProhibitTimer-r18  ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

CandidateServingFreqListNR-r16 ::= SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF ARFCN-ValueNR

MUSIM-GapAssistanceConfig-r17 ::= SEQUENCE {

musim-GapProhibitTimer-r17        ENUMERATED {s0, s0dot1, s0dot2, s0dot3, s0dot4, s0dot5, s1, s2, s3, s4, s5, s6, s7, s8, s9, s10}

}

MUSIM-LeaveAssistanceConfig-r17 ::=     SEQUENCE {

musim-LeaveWithoutResponseTimer-r17     ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, spare2, spare1}

}

MUSIM-CapabilityRestrictionConfig-r18 ::= SEQUENCE {

musim-CandidateBandList-r18               MUSIM-CandidateBandList-r18                                           OPTIONAL, -- Need R

musim-WaitTimer-r18                       ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, spare2, spare1},

musim-ProhibitTimer-r18                   ENUMERATED {s0, s0dot1, s0dot2, s0dot3, s0dot4, s0dot5, s1, s2, s3, s4, s5, s6, s7, s8,

s9, s10}

}

MUSIM-CandidateBandList-r18::= SEQUENCE (SIZE (1..maxCandidateBandIndex-r18)) OF FreqBandIndicatorNR

SuccessHO-Config-r17 ::=                SEQUENCE {

thresholdPercentageT304-r17             ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT310-r17             ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT312-r17             ENUMERATED {p20, p40, p60, p80, spare4, spare3, spare2, spare1}         OPTIONAL, --Need R

sourceDAPS-FailureReporting-r17         ENUMERATED {true}                                                       OPTIONAL, --Need R

...

}

SuccessPSCell-Config-r18 ::=            SEQUENCE {

thresholdPercentageT304-SCG-r18         ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT310-SCG-r18         ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT312-SCG-r18         ENUMERATED {p20, p40, p60, p80, spare4, spare3, spare2, spare1}         OPTIONAL, --Need R

...

}

OverheatingAssistanceConfig ::= SEQUENCE {

overheatingIndicationProhibitTimer    ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, spare3, spare2, spare1}

}

IDC-AssistanceConfig-r16 ::=    SEQUENCE {

candidateServingFreqListNR-r16  CandidateServingFreqListNR-r16                     OPTIONAL, -- Need R

...

}

DRX-PreferenceConfig-r16 ::=          SEQUENCE {

drx-PreferenceProhibitTimer-r16       ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MaxBW-PreferenceConfig-r16 ::=        SEQUENCE {

maxBW-PreferenceProhibitTimer-r16     ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MaxCC-PreferenceConfig-r16 ::=        SEQUENCE {

maxCC-PreferenceProhibitTimer-r16     ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MaxMIMO-LayerPreferenceConfig-r16 ::= SEQUENCE {

maxMIMO-LayerPreferenceProhibitTimer-r16 ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MinSchedulingOffsetPreferenceConfig-r16 ::=   SEQUENCE {

minSchedulingOffsetPreferenceProhibitTimer-r16 ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

ReleasePreferenceConfig-r16 ::=       SEQUENCE {

releasePreferenceProhibitTimer-r16    ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, infinity, spare1},

connectedReporting                    ENUMERATED {true}                                               OPTIONAL  -- Need R

}

RLM-RelaxationReportingConfig-r17 ::= SEQUENCE {

rlm-RelaxtionReportingProhibitTimer   ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, infinity, spare2, spare1}

}

BFD-RelaxationReportingConfig-r17 ::= SEQUENCE {

bfd-RelaxtionReportingProhibitTimer   ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, infinity, spare2, spare1}

}

SCG-DeactivationPreferenceConfig-r17 ::=       SEQUENCE {

scg-DeactivationPreferenceProhibitTimer-r17    ENUMERATED {

s0, s1, s2, s4, s8, s10, s15, s30,

s60, s120, s180, s240, s300, s600, s900, s1800}

}

RRM-MeasRelaxationReportingConfig-r17 ::= SEQUENCE {

s-SearchDeltaP-Stationary-r17             ENUMERATED {dB2, dB3, dB6, dB9, dB12, dB15, spare2, spare1},

t-SearchDeltaP-Stationary-r17             ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}

PropDelayDiffReportConfig-r17 ::= SEQUENCE {

threshPropDelayDiff-r17           ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5, ms6 ,ms7, ms8, ms9, ms10, spare5,

spare4, spare3, spare2, spare1}                OPTIONAL,   -- Need M

neighCellInfoList-r17             SEQUENCE (SIZE (1..maxCellNTN-r17)) OF NeighbourCellInfo-r17         OPTIONAL    -- Need M

}

NeighbourCellInfo-r17  ::= SEQUENCE {

epochTime-r17                  EpochTime-r17,

ephemerisInfo-r17              EphemerisInfo-r17

}

IDC-FDM-AssistanceConfig-r18 ::=        SEQUENCE {

candidateServingFreqRangeListNR-r18     CandidateServingFreqRangeListNR-r18                     OPTIONAL, -- Need R

...

}

CandidateServingFreqRangeListNR-r18 ::= SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF CandidateServingFreqRangeNR-r18

CandidateServingFreqRangeNR-r18 ::=     SEQUENCE {

candidateCenterFreq-r18                 ARFCN-ValueNR,

candidateBandwidth-r18                  ENUMERATED {khz200, khz400, khz600, khz800, mhz1, mhz2, mhz3, mhz4, mhz5,

mhz6, mhz8, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100,

mhz200, mhz300, mhz400}                             OPTIONAL -- Need R

}

UL-TrafficInfoReportingConfig-r18 ::=   SEQUENCE {

pdu-SessionsToReportUL-TrafficInfoList-r18   SEQUENCE (SIZE (1.. maxNrofPDU-Sessions-r17)) OF PDU-SessionToReportUL-TrafficInfo-r18,

ul-TrafficInfoProhibitTimer-r18              ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, spare3, spare2, spare1}

}

PDU-SessionToReportUL-TrafficInfo-r18 ::= SEQUENCE {

pdu-SessionID-r18                        PDU-SessionID,

qfi-ToReportUL-TrafficInfoList-r18       SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI

}

applicabilityReportConfigList-r19 ::= SEQUENCE (SIZE (1.. maxNrofApplicabilityConfig-r19)) OF ApplicabilityReportConfig-r19

ApplicabilityReportConfig-r19 ::= SEQUENCE {

servCellId-r19 							ServCellIndex                           OPTIONAL,

applicabilityConfigId-r19                  ApplicabilityConfigId-r19,

associatedIDforSetA-r19                    AssociatedId-r19 					OPTIONAL,   -- Need R

resourcesForSetA-r19                       CSI-ResourceConfigId 			OPTIONAL,   -- Need R

associatedIDforSetB-r19                    AssociatedId-r19 					OPTIONAL,   -- Need R

resourcesForSetB-r19                       CSI-ResourceConfigId 			OPTIONAL,   -- Need R

reportQuantity-r19 CHOICE {

none                                    NULL,

cri-RI-PMI-CQI                          NULL,

cri-RI-i1                               NULL,

cri-RI-i1-CQI                           SEQUENCE {

pdsch-BundleSizeForCSI                  ENUMERATED {n2, n4}                                         OPTIONAL    -- Need S

},

cri-RI-CQI                              NULL,

cri-RSRP                                NULL,

ssb-Index-RSRP                          NULL,

cri-RI-LI-PMI-CQI                       NULL

} 																			OPTIONAL,   -- Need R

reportConfigType-r19                        CHOICE {

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

} 																		OPTIONAL,   -- Need R

nrofReportedPredictedRS-r19::=             ENUMERATED {n1, n2, n3, n4}  OPTIONAL,   -- Need R

predictionTimeGap-r19                      ENUMERATED {10ms, 20ms, 40ms, 80ms, 160ms}  OPTIONAL， -- Need R

predictionNoFutureTimeInstances-r19        ENUMERATED {1,2,4,8}                        OPTIONAL， -- Need R

...

}

DataCollectionPreferenceConfig-r19 :: = SEQUENCE {

FFS

}

LoggedDataCollectionAssistanceConfig-r19 ::= SEQUENCE {

loggedDataCollectionBufferThreshold-r19                      ENUMERATED {FFS}                                       OPTIONAL, -- Need R

loggedDataCollectionFullBuffer-r19                           ENUMERATED {true}                                      OPTIONAL, -- Need R

loggedDataCollectionPowerLow-r19                             ENUMERATED {true}                                      OPTIONAL  -- Need R

}

-- TAG-OTHERCONFIG-STOP

-- ASN1STOP

### 6.3.2	Radio resource control information elements

<Text Omitted>

#### –	ApplicabilityReportList

The IE ApplicabilityReportList comprises information that the UE reports to gNB related to the applicability of the radio measurement prediction configurations or inference related parameter sets in otherConfig at UE.

ApplicabilityReportList information element

-- ASN1START

-- TAG-APPLICABILITYREPORTLIST-START

ApplicabilityReportList-r19 ::=   SEQUENCE (SIZE (1..maxNrofAppliabilityReports) OF ApplicabilityReport-r19

ApplicabilityReport-r19 ::=    SEQUENCE {

servCellId-r19 							ServCellIndex                           OPTIONAL,

applicabilityReportConfigId-r19          CHOICE {

csi-ReportConfigId                              CSI-ReportConfigId,

applicabilityConfigId-r19                  ApplicabilityConfigId-r19,

spare6                                     NULL,

spare5                                     NULL,

spare4                                     NULL,

spare3                                     NULL,

spare2                                     NULL,

spare1                                     NULL

}

applicabilityStatus-r19                        ENUMERATED {applicable, inapplicable},

releaseConfigurationPreference-r19             ENUMERATED {true}                                                                      OPTIONAL,

...

}

-- TAG-APPLICABILITYREPORTLIST-STOP

-- ASN1STOP

Editor's Note: FFS the structure of the IE ApplicabilityReportList and other content in ApplicabilityReport, e.g. for option B.

# References

- [POST130][026][AI PHY] 38.331 CR (Ericsson)

- R4-2508085, LS on AI/ML functionality activation, Source: RAN4, To: RAN2, Cc: RAN1