3GPP TSG-RAN WG2 Meeting #131	R2-2505775

Bangalore, India, 25 – 29 August 2025

Agenda item:	8.13.3

Source:	Samsung

Title:	Outstanding issues related to MH SRAP design

Document for:	Discussion & Decision

# Introduction

The SRAP design for R19 MH is being finalized, with many key issues now settled and implemented in the draft running CR. We do however believe that a couple of issues (which ended up not being identified as designated open issues) should be further discussed, and we outline them in this submission.

# SRAP design – remaining issues

## Error handling: dealing with errors linked to next hop and previous hop

In the downstream direction, there could be multiple child Relays/UEs, and therefore SRAP packet should be discarded by the U2N Relay UE if the next-hop ID configured for the UE identity matching that of the UE ID field of the received SRAP packet does not match any of the L2 IDs of any of the egress links configured for the U2N Relay UE in question. Essentially, this is the case of unknown/wrong next-hop ID which is not present in legacy systems, i.e. when the next-hop ID configured is not one of the child nodes of the Relay UE in question. Current UE ID error handling looks at the UE ID in the SRAP header, and (in the downstream) compares it to the DST UE. Whereas now (in the MH case), in addition to this check, we need to have a check of the next-hop ID. In other words, even if the match between the DST UE and the SRAP header UE ID is ok (this is the legacy check), there could be a misconfiguration of the next-hop ID.

Rather than assuming that such an error would be prevented by performing RRC reconfiguration, we believe SRAP spec should capture this as an error case:

Proposal 1. RAN2 to capture in the SRAP specification the case of unknown/incorrect next-hop ID on the downstream.

Additionally, in legacy specs (single-hop U2N Relaying), the ingress link connects directly to source Remote UE on the upstream, and therefore it indeed makes sense (as is done currently) to perform a check at the Relay UE whether the UE ID of the packet matches sl-LocalIdentity corresponding to L2 ID of the ingress link (which is inherited by the MH case as per the current draft SRAP CR). For MH however, the source UE could be several hops away and this check is potentially incorrect/needs enhancing, as the UE ID in the SRAP header on the upstream only refers to SRC ID, and not the previous hop UE ID. There now exist the indirect parent/child, and the direct parent/child. The current (legacy) error handling does not make a distinction between the two. We therefore propose:

Proposal 2. RAN2 to capture in the SRAP specification the case of the UE ID of the indirect parent being compared to the sl-LocalIdentity corresponding to L2 ID of the ingress link.

## DL and UL handling of Remote UE’s SRB0 in multi-hop U2N Relay

At the RAN2#130 meeting, the following was agreed:

Agreements:

The intermediate relay UE can have its own traffic acting as a remote UE simultaneously.  Running CRs will be checked to make sure this functionality is supported.

Clarify in the normative text that the UE can be a relay and remote UE simultaneously (to be determined case by case where it needs to be documented).

One action specific to the last Relay UE function in legacy specs is the special SRB0 handling. Special SRB0 handling may not be needed for packets from/for Remote UEs not attaching directly to the Relay UE as this could be the function of the first Relay UE in R19 MH. In line with this reasoning, we propose the following:

Proposal 3. RAN2 to clarify whether legacy U2N Relay UE handling of SRB0 needs to be modified to apply the special SRB0 handling only for SL UEs attaching directly to this last Relay UE.

# Conclusion

RAN2 is kindly asked to discuss and agree the following proposals:

Proposal 1. RAN2 to capture in the SRAP specification the case of unknown/incorrect next-hop ID on the downstream.

Proposal 2. RAN2 to capture in the SRAP specification the case of the UE ID of the indirect parent being compared to the sl-LocalIdentity corresponding to L2 ID of the ingress link.

Proposal 3. RAN2 to clarify whether legacy U2N Relay UE handling of SRB0 needs to be modified to apply the special SRB0 handling only for SL UEs attaching directly to this last Relay UE.