# 3GPP TSG-RAN2 Meeting #131	R2-2505864

# Bengaluru, India, Aug 25-29, 2025

Agenda Item:	8.2.2

Work Item:	Ambient_IoT_Solutions

Source: 	Qualcomm Incorporated

Title: 	Remaining aspects of Ambient IoT Paging

Document for:	Discussion/Decision

# Introduction

RAN#106 approved WI for Ambient IoT in [1]. One of the objectives for RAN2 is listed as follows:

- Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

This document provides details about remaining aspects of Paging for Ambient IoT.

# Paging Message Content

## Indication of Number of access occasions

RAN2#130 agreed

Issue (1-4) For number of access occasions introduce exponential way, 4 bits, value range FFS

Consequently, the MAC running CR has captured that the maximum number of access occasions could be up to 2^15 (i.e., the value when codepoint set to ‘1111’), but needs to be exactly a power of 2. Our companion paper R2-2505818 [4] describes in detail the potential issue of such approach, with a simple example of how there would be confusion about the last set of AOs if the total has to be a power of 2.

Figure 1: X=2, Y=3, 32 total AOs (2^5). The last set would be different from other AO sets.

Observation 1: There would be mismatch between the actual Access Occasions and indicated Access Occasions as some possible Access Occasions are not power of 2.

Interested readers are referred to the details in [4] where we describe various options to solve this problem.

Observation 2: There are various ways to solve the mismatch issues, however all of them need some further specification work.

- Opt a: the last AO set includes lower number of AOs compared to other sets - total true AOs is equal to indicated but further clarification needed for the AOs in the last AO set.

- Opt b: the last AO set which is not a full set is dropped - total true AOs is less than indicated, this needs spec update to disallow device to select the AOs in the last set.

- Opt c: last AO set is also made a full set of AOs - total true AOs is more than indicated, this needs spec update to allow device to select from the additional AOs in the last set, otherwise those are wasted.

- Opt d: revert earlier RAN2 agreement and indicate Number of Access trigger messages/sets instead of ‘Number of Access Occasions’ – needs spec update.

RAN2 should discuss the mismatch between the actual number of access occasions and the indicated number of access occasions by A-IoT paging message and decide on a solution.

Discuss and solve the mismatch between the actual number of access occasions and the indicated number of access occasions by A-IoT paging message.

## Msg1 resource indication and subsequent R2D/Paging message

RAN2#129bis agreed to introduce a new R2D trigger message with the following “Assumption”.

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

About the above ‘assumption’, the proponents argued that having some information about slot number/count down number in the R2D trigger message can help the device for power saving.

We agree power saving for device is important. However, we do not see how this can conclusively help on device power saving. To enable the device to go to ‘sleep’ and only wake up later to check the R2D trigger message and find out the slot number/counter, there would need to be prior knowledge about the periodicity of the R2D triggers. RAN2 hasn’t agreed on such mechanism. Even when the periodicity is known, there is uncertainty about the clock accuracy. Consequently, the device would have to determine when to wake up to check. If the device has ability to make this determination even without the knowledge of periodicity, then it could use the proprietary power saving strategies without such counter. If the device doesn’t have such ability, and there is still a chance to miss the R2D trigger message, then having the counter doesn’t help much.

Therefore, we have the following proposal to confirm the assumption:

RAN2 confirms that R2D trigger message does not include slot number/count down number.

## Indication about subsequent R2D message

The subsequent R2D message after D2R transmission may be a failure indication for Msg3 transmission or could be the acknowledgement like the Msg4 of NR random access. RAN2#128 agreed the following:

Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.

While some scenarios would need the subsequent R2D message (e.g. for acknowledging the reception of the A-IoT device ID and/or A-IoT upper layer data sent in the previous D2R message or for triggering the next round of A-IoT re-access or A-IoT data communication, or for security procedure in case of inventory followed by command for authentication as agreed by SA3), in other scenarios there would be no such subsequent R2D message (e.g. Reader requesting information from temperature sensors and no intention to ACK/NACK the reception).

It is beneficial for A-IoT devices to have correct expectation on potential future steps after transmitting a D2R communication in response to the A-IoT paging. To facilitate the A-IoT device actions, the A-IoT paging message can indicate to the A-IoT device whether a subsequent R2D message would be present, e.g. for a follow-up command.

The A-IoT paging message indicates the presence of follow-up command.

# Summary

Based on the discussion above, we have following observations and proposals:

Observation 1: There would be mismatch between the actual Access Occasions and indicated Access Occasions as some possible Access Occasions are not power of 2.

Observation 2: There are various ways to solve the mismatch issues, however all of them need some further specification work.

- Opt a: the last AO set includes lower number of AOs compared to other sets - total true AOs is equal to indicated but further clarification needed for the AOs in the last AO set.

- Opt b: the last AO set which is not a full set is dropped - total true AOs is less than indicated, this needs spec update to disallow device to select the AOs in the last set.

- Opt c: last AO set is also made a full set of AOs - total true AOs is more than indicated, this needs spec update to allow device to select from the additional AOs in the last set, otherwise those are wasted.

- Opt d: revert earlier RAN2 agreement and indicate Number of Access trigger messages/sets instead of ‘Number of Access Occasions’ – needs spec update.

Proposal 1:	Discuss and solve the mismatch between the actual number of access occasions and the indicated number of access occasions by A-IoT paging message.

Proposal 2:	RAN2 confirms that R2D trigger message does not include slot number/count down number.

Proposal 3:	The A-IoT paging message indicates the presence of follow-up command.

# References

[1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN#106, Dec 2024

[2] RP-240826, Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#103, March 2024

[3] TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, Release 18

[4] R2-2505818, Views on Random Access Aspects of Ambient IoT, Qualcomm Incorporated, RAN2#131, August 2025