3GPP TSG RAN WG2 Meeting #131	R2-2505959

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.9.4

Source:	CMCC

Title:	Remaining issues on support of PWS

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of PWS for NB-IoT and some agreements were achieved as follows [1]:

Agreements:

1.	A UE should be able to camp on an NB-IoT cell (at least for NTN) for the purpose of receiving PWS broadcast, without the cell being a part of UEs PLMNs. Come back in the next meeting to discuss the exact solution to make this possible (i.e. not necessarily introducing an acceptable cell category for NB-IoT)

2.	Regarding support continued reception of PWS segmentation of a message from different cells, can come back in the next meeting with an actual TP. In any case we will not target other scenarios than the intra-gNB case

In this contribution, we would like to provide some further considerations on the remaining issues on support of PWS.

Discussion

(TS 36.304 Open issue 1): Acceptable cell operation for NB-IoT (For PWS reception only)

In last meeting, we discussed the acceptable cell solution to allow UE to camp on an NB-IoT cell to receive PWS without the cell being a part of UEs PLMNs. However the detail solution discussion is postponed to this meeting, and a clarification is made that it is not necessarily to introduce an acceptable cell category due to that companies indicated that we would need to differentiate for the case of PWS support and emergency call (which will only be supported in R20). Based on the TS 36.304 open issues discussion [2], the majority companies agreed to Introduce acceptable cell support for NB-IoT in Rel-19, and for the detail specifications changes, the following two changes will be considered as baseline:

Alt.1 Figure in 5.2.2-2 to be updated.

Alt.2 Changes to 5.2.8a to clarify the PWS support of NB-IoT for acceptable cell

From our perspective, alt.2 is a simpler way to support PWS reception on acceptable cell. What we need to do is just to add one sentence in 5.2.8a to provide the corresponding clarification (e.g. The UE could receive the PWS on a acceptable cell).

Proposal 1: (TS 36.304 Open issue 1) Introduce acceptable cell support for NB-IoT in Rel-19, and add the corresponding clarification (e.g. The UE could receive the PWS on a acceptable cell) in 5.2.8a of TS 36.304.

(RRC-2) SIB1-NB acquisition

In RAN2 #128 meeting, we have agreed to extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. For whether support SIB1-NB acquisition is FFS.

Per the existing LTE mechanism, the UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, however, if etws-Indication or cmas-Indication is included in the Paging message, it would trigger the SystemInformationBlockType1 re-acquisition immediately, i.e., without waiting until the next system information modification period boundary. Then UE read the schedulingInfoList to acquire the System Information Block carrying specific PWS message(SystemInformationBlockType10 and SystemInformationBlockType11 for ETWS, SystemInformationBlockType12 for CMAS) if present.

- 5.2.1.4 Indication of ETWS notification <<omitted>> NOTE: The UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, but Paging message including the etws-Indication triggers the UE to re-acquire schedulingInfoList contained in SystemInformationBlockType1 for scheduling changes for SystemInformationBlockType10 and SystemInformationBlockType11. The UE may or may not receive a Paging message including the etws-Indication and/or systemInfoModification when ETWS is no longer scheduled. <<omitted>> - 5.2.1.5 Indication of CMAS notification <<omitted>> NOTE: The UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, but Paging message including the cmas-Indication triggers the UE to re-acquire schedulingInfoList contained in SystemInformationBlockType1 for scheduling changes for SystemInformationBlockType12. The UE may or may not receive a Paging message including the cmas-Indication and/or systemInfoModification when SystemInformationBlockType12 is no longer scheduled. <<omitted>>

- 5.3.2.3 Reception of the Paging message by the UE <<omitted>> if the etws-Indication is included and the UE is ETWS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary; 2> if the schedulingInfoList indicates that SystemInformationBlockType10 is present: 3> acquire SystemInformationBlockType10; NOTE 2: If the UE is in CE, it is up to UE implementation when to start acquiring SystemInformationBlockType10. 2> if the schedulingInfoList indicates that SystemInformationBlockType11 is present: 3> acquire SystemInformationBlockType11; 1> if the cmas-Indication is included and the UE is CMAS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary as specified in 5.2.1.5; 2> if the schedulingInfoList indicates that SystemInformationBlockType12 is present: 3> acquire SystemInformationBlockType12; <<omitted>>

Some companies provide enhancements solutions that network broadcasts early PWS scheduling message even though there is no PWS notification considering the latency. However, such enhancements may introduce some additional signaling overhead. Therefore, we could just extend the existing RRC procedure to NB-IoT NTN for SIB1-NB acquisition, and no further enhancement is needed.

Proposal 2: (RRC-2) We could just extend the existing mechanism to NB-IoT NTN for SIB1-NB acquisition, and no further enhancement is needed.

(RRC-3) Whether to allow UE to receive and assemble PWS segments from different cells during mobility

Regarding whether allow UE to receive and assemble PWS segments from different cells during mobility. Last meeting agreed to only target the intra-gNB case. Per the existing specification, for UE is ETWS/CMAS capable, upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment, it should discard any previously buffered warningMessageSegment and clear, if any, the current values of messageIdentifier and serialNumber. However, considering that for NTN system, satellites are always moving rapidly, which would lead to that UE may be not able to receive the whole PWS message from the serving cell (as shown in figure 1). And from UE side, it is expected to obtain the whole PWS message as soon as possible. Therefore it is beneficial to enable UE to receive and assemble PWS segments from different cells. And a TP is provided in annex.

Fig.1 The UE may not receive the whole PWS message when served by cell1

Observation 1: Different from TN, for NTN system, satellites are always moving rapidly, which would lead to that UE may be not able to receive the whole PWS message from the serving cell.

Proposal 3: (RRC-3) It is proposed that allow UE to receive and assemble PWS segments from different cells during mobility for intra-gNB case.

Proposal 4: (RRC-3) If P3 is supported, suggest RAN2 to agree the TP in annex.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on broadcast of PWS messages for NB-IoT and have the following observations and proposals:

Proposal 1: (TS 36.304 Open issue 1) Introduce acceptable cell support for NB-IoT in Rel-19, and add the corresponding clarification (e.g. The UE could receive the PWS on a acceptable cell) in 5.2.8a of TS 36.304.

Proposal 2: (RRC-2) We could just extend the existing mechanism to NB-IoT NTN for SIB1-NB acquisition, and no further enhancement is needed.

Observation 1: Different from TN, for NTN system, satellites are always moving rapidly, which would lead to that UE may be not able to receive the whole PWS message from the serving cell.

Proposal 3: (RRC-3) It is proposed that allow UE to receive and assemble PWS segments from different cells during mobility for intra-gNB case.

Proposal 4: (RRC-3) If P3 is supported, suggest RAN2 to agree the TP in annex.

Reference

- R2-130 NR-NTN-IoT-NTN (Sergio)_ EOM

- Draft_R2-250xxxx IoT NTN 36.304 Open Issues-V21_Rapp

- 3GPP TS 36.331 V18.2.0 Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 18)

Annex

  - TP for TS 36.331 based on the latest RRC running CR

  - 5.2.2.4	System information acquisition by the UE

- <<omitted>>

1>	if the UE is ETWS capable:

2>	upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3>	if the ETWS notification was transmitted in non-terrestrial network cells and the cells belongs to the same gNB :

4>	if the UE is an NB-IoT UE:

5>	store any previously buffered warningMessageSegment;

5>	assemble, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11-NB;

3>	else:

4>	discard any previously buffered warningMessageSegment;

4>	clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11(-NB);

2>	when the UE acquires SystemInformationBlockType1(-NB) following ETWS indication, upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3>	if schedulingInfoList indicates that SystemInformationBlockType10(-NB) is present:

4>	if the UE is in CE:

5>	start acquiring SystemInformationBlockType10;

4>	else if the UE is an NB-IoT UE:

5>	start acquiring SystemInformationBlockType10-NB immediately;

4>	else

5>	start acquiring SystemInformationBlockType10 immediately;

3>	if schedulingInfoList indicates that SystemInformationBlockType11(-NB) is present:

4>	start acquiring SystemInformationBlockType11(-NB) immediately;

NOTE 2:	UEs shall start acquiring SystemInformationBlockType10(-NB) and SystemInformationBlockType11(-NB) as described above even when systemInfoValueTag in SystemInformationBlockType1(-NB) has not changed.

1>	if the UE is CMAS capable:

2>	upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3>	if the CMAS notification was transmitted in non-terrestrial network cells and the cells belongs to the same gNB :

4>	if the UE is an NB-IoT UE:

5>	store any previously buffered warningMessageSegment;

5>	assemble, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType12-NB;

3>	else:

4>	discard any previously buffered warningMessageSegment;

4>	clear, if any, stored values of messageIdentifier and serialNumber for SystemInformationBlockType12 associated with the discarded warningMessageSegment;

2>	when the UE acquires SystemInformationBlockType1(-NB) following CMAS indication, upon entering a cell during RRC_IDLE, following successful handover and upon connection re-establishment:

3>	if schedulingInfoList indicates that SystemInformationBlockType12(-NB) is present:

4>	acquire SystemInformationBlockType12(-NB);

NOTE 3:	UEs shall start acquiring SystemInformationBlockType12(-NB) as described above even when systemInfoValueTag in SystemInformationBlockType1(-NB) has not changed.

- <<omitted>>