3GPP TSG-RAN WG2 Meeting #131	R2-2506550

# Bengaluru, India, 25 - 29 August 2025

Title:	LS on not supporting simultaneous LR and MR operation

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	RAN WG2

To:	RAN WG1

Cc: 	RAN WG4

Contact person:	Fangli XU, fangli_xu@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

In RAN2#131 meeting, RAN2 discussed the support of LP-WUS with dual DRX group in CA and the RAN1 agreement on not supporting simultaneous LR and MR operation, and made the following agreements:

LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime.  When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

If secondary DRX group is configured, UE monitors LP-WUS only when both DRX groups are not in DRX active time.

If secondary DRX group is configured, the lpwus-PDCCH-MonitoringTimer configuration for secondary DRX group is different from that for the default DRX group.

The lpwus-PDCCH-MonitoringTimer configuration for secondary DRX group is smaller than or equal to that for the default DRX group.

RAN2 understand that the RAN1 agreement on not supporting simultaneous LR and MR operation is only applicable within one cell group (with or without secondary DRX group configuration).

# 2	Actions

To RAN1:

ACTION: RAN WG2 respectfully asks RAN WG1 to take the above agreements into account in their future work.

# 3	Dates of next TSG-RAN WG2 meetings

TSG-RAN WG2 Meeting #131bis	October 13 – October 17, 2025	Prague, CZ

TSG-RAN WG2 Meeting #132	November 17 – November 21, 2025	Dallas, US