3GPP TSG-RAN WG2 Meeting #131bis	R2-2506867

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda Item:	8.8.2

Source: 	CATT

Title: 	[C005][S024][S025][Z252]Discussion on the mechanism of UE reporting the N closest reference location

Document for: 	Discussion and Decision

Introduction

This contribution discusses RIL [C005], [S024], [S025] and [Z252]. These RILs are proposed to clarify the mechanism of UE reporting the N closest reference location. Specifically, for the first report the N closest reference location, whether the UE shall include the information in the RRCReconfigurationComplete message; or it could report the information in the RRCReconfigurationComplete message as an optional behaviour and could report the information in the UEAssistanceInformation message later for the first time. The RILs are listed in the Annex.

Discussion

In RAN2#131 [1], we achieved the following agreement.

4.	The procedure for UE assisted SMTC configuration in connected mode is follows:

-	For a UE indicating the relevant capability (FFS the name of the capability and it this is merged with some other capability) the network configures the UE via reconfiguration message with a list of reference locations and a parameter N, thereby configuring the UE to report the N closest reference locations (FFS whether also a distance threshold is indicated). If the list of reference locations is not provided, the list of reference locations provided in system information is reused.

-	The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations.

-	The UE can report the N closest reference locations via the RRCReconfigurationComplete message.

-	The network configures the relevant SMTC(s). The maximum number of configured SMTCs is according to legacy UE capability in terms of SMTC support (same as legacy).

We understand that “can” means the UE does not have to report the reference location in the RRCReconfigurationComplete message. However, the current RRC CR [2] appears to require the UE to include the N closest reference locations in the RRCReconfigurationComplete message for the first time. This mismatch between the agreement and the CR was not resolved during the ASN.1 comment phase. This contribution analyses this issue and proposeds several candidate solutions.

1> set the content of the RRCReconfigurationComplete message as follows: [unreleated part is ommitted] 2> if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state: 3> include referenceLocationReport;.

As discussed during the ASN.1 review phase, there are two options to solve this issue.

- Option 1 [C005]: restrict the UE have to report the reference locations in the RRCReconfigurationComplete message for the first time, and remove the “upon being configured to do so” triggered condition under the UEAssistanceInformation message.

- Option 2 [S024][S025][Z252]: make the UE report the reference locations in the RRCReconfigurationComplete message as an optional behaviour, and complete the UE behaviour of “upon being configured to do so” triggered condition under the UEAssistanceInformation message.

Comparing these two options, Option 1 is simpler and could save one signalling. However, as company commented during the ASN.1 review phase, if the NW does not configure reference location in the RRCReconfiguration message and the UE does not have valid SIB2, it needs to read SIB2 firstly which may delay the transmission of the RRCReconfigurationComplete message. Perhaps this issue can be avoided by UE implementation, e.g., maintains valid SIB2. Option 2 is complex, but it is also more flexible. It is propose to down select from these two options firstly.

Proposal 1: Down selection from the following options for UE reporting the N closest reference locations.

- Option 1 [C005]: restrict the UE has to report the reference locations in the RRCReconfigurationComplete message for the first time.

- Option 2 [S024][S025][Z252]: make the UE reports the reference locations in the RRCReconfigurationComplete message as an optional behaviour, and if the UE does not report the reference locations in the RRCReconfigurationComplete message for the first time, report it in the UEAssistanceInformation message.

If option 1 is agreed, adopt the TP in Annex A. If option 2 is agreed, adopt the TP in Annex B, which is slightly different compared with [S025] and [Z252].

Proposal 1a: If option 1 is agreed, adopt the TP in Annex A. If option 2 is agreed, adopt the TP in Annex B.

Conclusion

In this contribution, corrections on N closest reference location report are discussed with the following proposal:

Proposal 1: Down selection from the following options for UE reporting the N closest reference locations.

- Option 1 [C005]: restrict the UE has to report the reference locations in the RRCReconfigurationComplete message for the first time.

- Option 2 [S024][S025][Z252]: make the UE reports the reference locations in the RRCReconfigurationComplete message as an optional behaviour, and if the UE does not report the reference locations in the RRCReconfigurationComplete message for the first time, report it in the UEAssistanceInformation message.

Proposal 1a: If option 1 is agreed, adopt the TP in Annex A. If option 2 is agreed, adopt the TP in Annex B.

Reference

- RAN2#131 ChairNotes.

- R2-2506522  Introduction of NTN Phase 3 enhancements Rel-19 Ericsson	CR	Rel-19

Annex: RILs

# C005 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. C005; NTN; 1; Clarify the meaning of “can” in the following agreement: - The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations. - The UE can report the N closest reference locations via the RRCReconfigurationComplete message.; Yes, R2-250xxxxx; CATT (Da Wang); v008; Duplicate S024. [Description]: From our perspective, “can” means the UE does not have to report the reference location in the RRCReconfigurationComplete message. However, the following text implies that the UE shall report the N closest reference locations in the RRCReconfigurationComplete message for the first time. 1> set the content of the RRCReconfigurationComplete message as follows: [unreleated part is ommitted] 2> if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state: - 3> include referenceLocationReport;. We have two options to solve this issue: - Option 1: restrict the UE have to report the reference locations in the RRCReconfigurationComplete message for the first time, and remove the “upon being configured to do so” triggered condition under the UEAssistanceInformation message. - Option 2: make the UE report the reference locations in the RRCReconfigurationComplete message as an optional behaviour, and complete the UE behaviour of “upon being configured to do so” triggered condition under the UEAssistanceInformation message. For simplicity, we can go with the option 1, i.e., restrict that the UE have to report the N closest reference locations in the RRCReconfigurationComplete message for the first time and the UEAssistanceInformation message can be used for the subsequent reports. The spec can be modified as following. [Proposed Change]: Remove the “upon being configured to do so” for the condition of reporting N closest reference locations in the UEAssistanceInformation message. A UE capable of providing location information for assisted SMTC configuration in RRC_CONNECTED state shall initiate the procedure upon determining that the closest reference location(s) have changed compared with the last reported values. [Comments]: [Samsung]: we share the same view on the issue, as we already mentioned in the CR review but not addressed by the Rapporteur. In our understanding of the agreement, we think Option 2 should be the corresponding UE behaviour, i.e., reporting reference location in RRCReconfigurationComplete message is optional. We propose a change in RIL S024. [vivo]: we also share a similar view on this issue. If the network may not configure refLocList-r19 via RRC configuration message, the UE may take some additional time to acquire the SIB2 meesage containing reference location list. To aviod delaying the transmission of the RRC reconfiguration complete message, the UE should be able to omit the location report in the RRC RRC reconfiguration complete message.

# S024 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. S024; NTN; 1; UE optionally reports N closest reference locations via the RRCReconfigurationComplete message.; Yes, R2-250xxxxx; Samsung (Shiyang); v011; ToDo. [Description]: RAN2 agreement is that - The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations. The UE can report the N closest reference locations via the RRCReconfigurationComplete message. “UE can ..” in our understanding means UE optionally reports in RRCReconfigurationComplete message. If we implement in the current way below, it enforces UE always reports in RRCReconfigurationComplete message, which obviously is not aligned with the agreement! 2> if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state: 3> include referenceLocationReport;. [Proposed Change]: Add “if available” at the end. 2> if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state: 3> include referenceLocationReport, if available; [Comments]: [vivo]: The proposed change is fine to us [Rapp] In our understanding of the agreement, the “can” indicates an alternative way of reporting. Let’s clarify this in the next meeting. We are unsure what is the concern and why “if available” would be applicable. A UE needs to know its accurate location to initiate and maintain its connection to the network. # S025 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. S025; NTN; 1; UE first time reports N closest reference locations using UAI.; Yes, R2-250xxxxx; Samsung (Shiyang); v011; PropAgree. [Description]: As agreed “The UE can report the N closest reference locations via the RRCReconfigurationComplete message.”, if UE has not reported in RRCReconfigurationComplete message or in UAI since it is configured to do so, UE shall report reference location via UAI for the first time. The condition for the first-time report is missing currently. [Proposed Change]: add the condition for the first-time report. 1> if configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state: 2> if the UE did not include referenceLocationReport in a RRCReconfigurationComplete message or in a UEAssistanceInformation message since it has been configured to provide location information; or 2> if the current closest reference locations are different from the ones indicated in the last transmission including referenceLocationReport: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide location information for assisted SMTC configuration; [Comments]: [vivo]: The proposed change is fine to us. [Nokia]: Since we add the clause, we would still suggest to also consider “since last entering connected mode” just to cover all cases.

# Z252 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. Z252; NTN; 1; Triggering condition for reference location report in UAI; Yes, R2-250xxxxx; ZTE (Zhihong); v012; Duplicate S024. [Description]: When discussing the procedure, it is preferred by companies to report first report in RRCReconfigurationComplete message to save one more RRC message for reporting. However there could be the case when refLocList configuration is not provided in RRCReconfiguration but instead UE shall acquire this information from SIB2, which will cause additional delays for UE to compute the closest referenceLocation and put it in RRCReconfigurationComplete message. It is not desired to mandate such beahvior and delay the transmission of RRCReconfigurationComplete message, a compromised way could be leave it to UE implementation whether to report this information in RRCReconfigruationComplete message, if it is the common understanding then the trigger for reference location for UAI report would be update to when configured to do so and UE has not yet report, and upon determining that the closesr reference location(s) have changed... [Proposed Change]: Update the description as below: A UE capable of providing location information for assisted SMTC configuration in RRC_CONNECTED state shall initiate the procedure upon being configured to do so and the first report has not been sent, and upon determining that the closest reference location(s) have changed compared with the last reported values [Comments]: Similar issue as raised by S024. It needs to be further discussed in the meeting. Thanks for proposing a compromise solution.

Annex A: TP for option 1

#### 5.7.4.2	Initiation

[unrelated part omitted]

A UE capable of providing location information for assisted SMTC configuration in RRC_CONNECTED state shall initiate the procedure upon determining that the closest reference location(s) have changed compared with the last reported values.

[unrelated part omitted]

Annex B: TP for option 2

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

[unrelated part omitted]

1>	set the content of the RRCReconfigurationComplete message as follows:

[unrelated part omitted]

2>	if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

3>	include referenceLocationReport ,if available;

[unrelated part omitted]

#### 5.7.4.2	Initiation

[unrelated part omitted]

A UE capable of providing location information for assisted SMTC configuration in RRC_CONNECTED state shall initiate the procedure upon being configured to do so and it did not transmit the closest reference location(s) in the RRCReconfigurationComplete message, and upon determining that the closest reference location(s) have changed compared with the last reported values.

[unrelated part omitted]

1>	if configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

2>	if the UE did not transmit an RRCReconfigurationComplete message with referenceLocationReport or a UEAssistanceInformation message with referenceLocationReport since it was configured to provide the closest reference location(s); or

2>	if the current closest reference locations are different from the ones indicated in the last transmission including referenceLocationReport:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide location information for assisted SMTC configuration;