3GPP TSG-RAN WG2 Meeting #131bis					R2-2506921

Prague, Czech Republic, Oct. 13th-17th 2025

Agenda item:		8.2.2

Source:			Lenovo

Title:		Discussion on max NAS message size

Document for:	 	Discussion and Decision

Introduction

In last RAN2 meeting, A-IoT WI is finished. And this contribution target for discussing the following left open issues that listed by the Rapporteur:

- CT1 LS on max NAS message size (Issue 4-6)

Discussion

CT1 LS on max NAS message size (Issue 4-6)

In [4], CT1 indicated that they are considering the coding aspects of AIoT NAS command messages, and would like to ask RAN2 about maximum supported AIoT NAS container length in both R2D and D2R directions, including segmentation, if supported.

Current size analysis for R2D message and D2R message

From RAN2 perspective, the R2D message relates to the command message is R2D Upper Layer Data Transfer message, and the maximum size is 125 bytes. In this message, the MAC header(s) occupies 6 bytes. This message does not support segmentation.

The D2R message relates to the command response message is D2R Upper Layer Data Transfer message, and the maximum size is 125 bytes. In this message, the MAC header(s) occupies 2 bytes. This message support segmentation, i.e. one command response message could be segmented to multiple segments, and the size of each segment is scheduled by the reader based on e.g. channel quality etc. There has no limitation on the number of segments.

Observation 1: Max size of R2D Upper Layer Data Transfer message and D2R Upper Layer Data Transfer message is 125 bytes. And MAC header size is 6 bytes and 2 bytes respectively.

Reserved bytes for future extension

The R2D/D2R message relates to the command (response) message may further extended to contain more MAC headers in future release, e.g. Rel-20. Thus, besides existing MAC header size, RAN2 is suggest to reserve more bytes for future extension e.g. 3 bytes could be enough.

Proposal 1 (Issue 4-6): RAN2 is suggested to reserved additional 3 bytes for MAC header for future extension when replies to CT1 LS.

So overall, the size of R2D Upper Layer Data Transfer message and D2R Upper Layer Data Transfer messaged is as following

- R2D Upper Layer Data Transfer message: max size 125 bytes, MAC header 6 bytes, future extension 3 bytes, then data SDU size is 116 bytes.

- D2R Upper Layer Data Transfer message: max size 125 bytes, MAC header 2 bytes, future extension 3 bytes, then data SDU size is 120 bytes, and this is for one segment

NAS container

From CT1/SA2 understanding, one NAS container contains one command response. So, if one command response is split to several segments in AS layer, the reader needs to combine them to the one command response again and use one NAS container to transmit to AIoTF. Additionally, in SA3, the security is also performed per command and per command response.

From MAC layer perspective, although there has no limitation for the segments number for D2R Upper Layer Data Transfer message, the scheduling parameter Received Data Size field, which is to indicate the received data size in the reader and schedule the followed segments from the device, may impact the maximum size of command response that can be combined by the reader. For example, the field has 7 bit and can indicate maximum 127 bytes size, which may imply the maximally combined command response size is 127 bytes.

Observation 2: One NAS container contains one command response, i.e. if one command response is split to several segments in AS layer, the reader needs to combine them to the one command response again and use one NAS container to transmit to AIoTF.

Observation 3: The maximum size of command response that can be combined by the reader maybe limited by the Received Data Size field, which has 7 bits and maximum indicated size is 127 bytes.

Proposal 2 (Issue 4-6): RAN2 is suggest to reply the LS from CT1 about following core points considering all above observations and proposals:

- R2D Upper Layer Data Transfer message: Max size 125 bytes, MAC header 6 bytes, future extension 3 bytes, then max data SDU (NAS container) size is 116 bytes.

- D2R Upper Layer Data Transfer message: Max size 125 bytes, MAC header 2 bytes, future extension 3 bytes, then data SDU size is 120 bytes, and this is for one segment. For max size of NAS container, max size is 127 bytes which is limited by the Received Data Size field

Detail draft reply LS could be found in [5].

Conclusion

In this contribution we have discussed left open issues for Rel-19 Ambient IoT and made the following observations and proposals:

Observation 1: Max size of R2D Upper Layer Data Transfer message and D2R Upper Layer Data Transfer message is 125 bytes. And MAC header size is 6 bytes and 2 bytes respectively.

Proposal 1(Issue 4-6): RAN2 is suggested to reserved additional 3 bytes for MAC header for future extension when replies to CT1 LS.

Observation 2: One NAS container contains one command response, i.e. if one command response is split to several segments in AS layer, the reader needs to combine them to the one command response again and use one NAS container to transmit to AIoTF.

Observation 3: The maximum size of command response that can be combined by the reader maybe limited by the Received Data Size field, which has 7 bits and maximum indicated size is 127 bytes.

Proposal 2 (Issue 4-6): RAN2 is suggest to reply the LS from CT1 about following core points considering all above observations and proposals:

- R2D Upper Layer Data Transfer message: Max size 125 bytes, MAC header 6 bytes, future extension 3 bytes, then max data SDU (NAS container) size is 116 bytes.

- D2R Upper Layer Data Transfer message: Max size 125 bytes, MAC header 2 bytes, future extension 3 bytes, then data SDU size is 120 bytes, and this is for one segment. For max size of NAS container, max size is 127 bytes which is limited by the Received Data Size field

Reference

- S2-2507793, LS on AIoT Device Permanent ID Length

- S3-252392, LS on security parameter in paging request message

- S3-252933, Reply LS on paging ID length

- C1-255679, LS on the maximum supported AIoT NAS container length

- R2-2506920, [Draft] Reply LS on the maximum supported AIoT NAS container length