3GPP TSG-RAN WG2 Meeting #131bis                                                       R2-2507601

Prague, Czech Republic, Oct 13th - 17th 2025

Agenda item:	8.20.1

Source: 	ZTE Corporation

Title: 	Consideration on the LBCA Capability Signaling

Document for:	Discussion and Decision

Introduction

About the LBCA capability signaling, a LS was received from RAN4 as follows [1]:

RAN4 has discussed the impact of the low NR band carrier aggregation via switching feature on BandCombinationList and would like to share the following information for RAN2 consideration: - There are CA band combinations (BCs) in TS38.101-1, but there are no legacy UEs supporting those in the market, e.g., CA_n5-n29 and CA_n29-n71. - There are, however, low band CA switching to CA BCs whose traditional CA are already in the market. And, there are UEs supporting only low band CA switching, only traditional CA BCs, or both low band CA switching and traditional CA BCs for the said CA BCs. - Further, when a UE supports both low band CA switching and traditional CA, at least following can be different. - The UE can have different maximum MIMO layers per CC depending on low band CA switching or traditional CA - When specifying the UE requirements, RAN4 has assumed that the network supporting any release of the NR specification should be able to distinguish these UEs

In this paper, we share our views on the capability signaling for the LBCA feature.

Discussion

As shown in the LS, Some UEs may support only low-band carrier aggregation (LBCA) switching, only traditional CA, or both for the specified CA bands. Therefore, backward compatibility must be considered when designing the capability signaling for the LBCA feature. Generally, there are two options:

- Option 1: Introduce a new BC branch for LBCA, as was done for uplink (UL) transmit (Tx) switching.

- Option 2: Introduce an LBCA-specific FeatureSetCombinationID, as was done for dual connectivity (DC).

Option 1 would introduce unnecessary signaling complexity. Furthermore, if a UE supports both traditional CA and LBCA and shares the same per-BC capability reporting for both, it would still need to report two separate BCs, resulting in additional signaling overhead.

The main issue with Option 2 is determining how to set the legacy FeatureSetCombination for a UE that supports only LBCA.

BandCombination ::= SEQUENCE { bandList SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters, featureSetCombination FeatureSetCombinationId,

A similar issue was clarified for CA and DC in reference [2], with the following conclusion:

RAN2#110 => RAN2 confirms that the current UE capability signalling allows the UE to declare band combinations where NR-DC is supported, but NR CA is not supported.

One implementation method is for the UE to set some feature sets to '0'. For example, for a band combination of Band A + Band B where the UE only supports LBCA, it could report the FeatureSetCombination as follows:

.Table 1: FeatureSetCombination for the Traditional CA and the LBCA

| FeatureSetCombination | Feature Set of Band A | Feature Set of Band B |
| --- | --- | --- |
| Legacy FeatureSetCombination | Actual supported capabilities for the single Band A | Set to 0 for both the UL and the DL |
| LBCA FeatureSetCombination | Actual supported capabilities for the band A with LBCA | Actual supported capabilities for the band B with LBCA |

Proposal 1: The LBCA capability can be signaled by introducing an LBCA-specific FeatureSetCombinationID.

For the NR-DC case, both the FeatureSetCombinationID for DC and other per-BC level parameters are added in CA-ParametersNRDC. This field also indicates whether the UE supports NR-DC.

ca-ParametersNRDC If the field (without suffix) is included for a band combination in the NR capability container, the field (without suffix) indicates support of NR-DC. Otherwise, the field is absent. If a version of the field (with suffix) is absent for a band combination, ca-ParametersNR field version in BandCombination corresponding to the ca-ParametersNR-ForDC field version in the field (with suffix) is applicable to the UE configured with NR-DC for the band combination.

For the LBCA case, until now, only the per-FSPC level MIMO layers parameters are identified, which may differ from the traditional CA.

- Further, when a UE supports both low band CA switching and traditional CA, at least following can be different. - The UE can have different maximum MIMO layers per CC depending on low band CA switching or traditional CA

Therefore, there is no need to introduce other LBCA-specific parameters at this stage. As shown in Table 2 (Proposal 1a), the UE can indicate its support for LBCA by using an LBCA-specific FeatureSetCombinationID even it’s the same as the legacy FeatureSetCombinationID.

Table 2: LBCA specific FeatureSetCombinationID

| Case | Present of Legacy FeatureSetCombination | Present of LBCA FeatureSetCombination |
| --- | --- | --- |
| Only support traditional CA | Present | Absent |
| Only support LBCA | Present (with featureset=0 on some bands) | Present |
| Support both cases | Present | Present |

Proposal 1a: The UE can indicate whether it supports the LBCA case by using an LBCA-specific FeatureSetCombinationID. There is no need to add other LBCA-specific parameters at this stage.

Conclusion

Based on the above discussion, we have the following observations and proposals:

Proposal 1: The LBCA capability can be signaled by introducing an LBCA-specific FeatureSetCombinationID.

Proposal 1a: The UE can indicate whether it supports the LBCA case by using an LBCA-specific FeatureSetCombinationID. There is no need to add other LBCA-specific parameters at this stage.

Reference

- R4-2511863  LS on Capability of NR_LBCA_Sw  RAN4 to RAN2

- R2-2004436    Signalling of NR-DC only band combination          Qualcomm Incorporated discussion        Rel-15   NR_newRAT-Core