3GPP TSG-RAN2 Meeting #131bis	R2-2507626

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	8.4.2 RRC issues

Source: 	Ericsson

Title:  	LP-WUS issues (E007, E035, E036, E038, E043, 38304-2, C026)

Document for:	Discussion and Decision

# Introduction

The following LP-WUS issues are discussed in this contribution:

- LP-WUS and NES (C026)

- Type 1 and 2 LR (E036, H053, H054)

- LP-WUS UE capability (E038)

- Move lpwus-Config-r19 into SpCellConfig (E007)

- RX level and cell quality on LR (38304-2)

- RRCReconfiguration of LP-WUS (E043, H050)

- TimeToTrigger (TTT) for RRM relaxation entry (E035)

# Discussion

## LP-WUS and NES (C026)

Paging adaptation

To enable further NW energy saving the gNB can configure every 32th frame in the DRX cycle as a Paging Frame (PF) with Rel-19 paging adaptation. This enables the gNB to sleep for a longer time. To compensate for this sparser paging configuration, the maximum number of POs per PF is increased to 8 and the first PDCCH monitoring occasion for up to 8 POs/PF can be configured:

pagingAdaptation-r19                  SEQUENCE {

pagingAdapt-NS-r19                    ENUMERATED {eight,four, two, one},

pagingAdaptNAndPagingFrameOffset-r19  CHOICE {

oneT                                  NULL,

halfT                                 INTEGER (0..1),

quarterT                              INTEGER (0..3),

oneEighthT                            INTEGER (0..7),

oneSixteenthT                         INTEGER (0..15),

oneThirtySecondT                      INTEGER (0..31)

},

pagingAdaptFirstPDCCH-MonitoringOccasionOfPO-r19  CHOICE {

sCS15KHZoneT                   SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..139),

…

sCS480KHZoneThirtySecondT      SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..143359)

}                         OPTIONAL           -- Need R

}                             OPTIONAL           -- Need R

There is no change to the paging formula in 38.304. And paging adaptation is not early implementable.

Rel-19 UEs that support paging adaptation use the Rel-19 paging configuration (pagingAdaptation-r19), if configured. In the beginning there will be few Rel-19 UEs, and it is likely that legacy paging will wake the gNB up in between two PFs that are 32 frames apart. With the legacy paging configuration, the gNB can configure every 16th frame as a PF and one PF may contain up to 4 POs. In 6G paging adaptation will be there from the beginning, enabling better and effective energy saving.

Paging adaptation and PEI

Rel-17 PEI provides the following functionality:

- PEI can carry up to 8 subgroups, i.e. 8 bits and when all UEs need to wake-up all 8 bits are set

- One PEI occasion is configured before a Paging Frame

- A single PEI can be mapped onto 1 up to 8 POs over up to 2 frames

- The available bits/subgroups in PEI are equally divided over the POs in the PF, e.g. with 2 POs/PF, the first 4 bits/subgroups are mapped onto the first PO and the last 4 bits/subgroups are mapped on the second PO in the PF.

Rel-19 paging adaptation and Rel-17 PEI can be configured and used together. To enable this a separate PEI (pagingAdaptPEI-Config-r19) can be configured that is used for the Rel-19 paging configuration. For example in case paging adaptation is configured without a second Rel-19 PEI and 8 POs/PF are configured, then the last 4 POs in the PF would not have PEI configured.

The number of subgroups for the PEI used with Rel-17 and Rel-19 POs is the same, otherwise there would be impact on UEID-based and CN-assigned subgrouping:

DownlinkConfigCommonSIB ::=     SEQUENCE {

…

pei-Config-r17                  PEI-Config-r17                         OPTIONAL,     -- Need R

…

pagingAdaptPEI-Config-r19       PEI-Config-r17                         OPTIONAL      -- Need R

…

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}            OPTIONAL,  -- Need R

...,

[[

po-NumPerPEI-r19                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r19                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r19                       INTEGER (0..32)

]]

}

Paging adaptation and LP-WUS

Similar as for PEI, it should be possible to configure and use LP-WUS and paging adaptation together:

LP-WUS and paging adaptation can be configured and used together.

When Rel-19 PO configuration (pagingAdaptation-r19) uses 1/32 PF with 8 POs/PF and the legacy PDCCH-config uses 1/16 PF with 4 POs/PF, then:

- The number of POs used by legacy and Rel-19 UEs is the same.

- Every second PF with 4 POs is only used by legacy UEs.

- Every other second PF with 8 POs:

  - first 4 POs are used by both legacy and Rel19 UEs

  - last 4 POs are only used by Rel-19 UEs.

The minimum requirement to enable proposal 1 is to clarify that for LP-WUS and paging adaptation the N and Ns value specified for paging adaptation is used:

The LP-WUS used with Rel-19 POs uses the N and Ns value of paging adaptation.

Similar as with PEI, there is a benefit to enable the parameters settings for the LP-WUS used with Rel-19 POs because the Rel-19 UE population size is different from the legacy population size and a different #MO/LO and #PO/LO could be more efficient:

LowPowerConfig-r19 ::=                    SEQUENCE {

…

lpwus-LoFrameOffsetList-r19               SEQUENCE {

offsetForLongerWakeUpDelay-r19  SEQUENCE (SIZE (1..4)) OF INTEGER (8..200)  OPTIONAL,   -- Need R

offsetForShorterWakeUpDelay-r19 SEQUENCE (SIZE (1..4)) OF INTEGER (8..200)  OPTIONAL    -- Need R

},

lpwus-MoNumPerLo-r19                      ENUMERATED {n1, n2, n3, n4},

lpwus-PoNumPerLo-r19                      ENUMERATED {po1, po2, po4}            OPTIONAL,   -- Need R

…

Most of the parameter values can and should be re-used from the default LP-WUS configuration. But it should be possible to configure a separate time offset, #MO/LO and #PO/LO:

Introduce the possibility to configure another LoFrameOffsetList, MoNumPerLo and PoNumPerLo for LP-WUS used with Rel-19 POs.

## Type 1 and 2 LR (E036, H053, H054)

RAN4 agreed that a WUR implementation may support a Noise Figure (NF) + Implementation Margin (IM) of either 13.6 dB (type 1) or 16 dB (type 2) [1]. This means that the WUR implementation may have different Adjacent Channel Selectivity (ACS), i.e. suffering from adjacent channel interference influencing the WUR performance [2]. If LP-WUS is configured close to the channel edge, then a type 2 implementation may experience co-channel interference and the LP-WUS reception may be impaired [3]:

In H053/H054 it is proposed to introduce separate thresholds for Type 1 and Type 2 UEs. This increases the complexity of the feature, which already suffers from a lot of thresholds. And it is also not clear if different thresholds resolve the issue, i.e. the adjacent channel interference is not resolved when the UE moves.

Type 2 WUR may experience performance degradation when LP-WUS is configured near the channel edge, and hopefully the gNB is able to LP-WUS with sufficient guard band for type 2 WUR. But in case this is not possible, the gNB should have the possibility to prevent WUR type 2 UEs to use LP-WUS. We propose to introduce a flag in SIB to prohibit Type 2 WUR from using LP-WUS when LP-WUS is configured close to the channel edge:

Introduce a flag in SIB to prohibit Type 2 WUR to use LP-WUS in the cell e.g. when LP-WUS cannot be configured with sufficient guard band for type 2 WUR.

## LP-WUS UE capability (E038)

The minimumTimeGap-r19 UE capability parameter was discussed in E038 in UECAP and the following corrections were proposed by the editor [4,5]:

lpwus-SupportedBandList-r19

Indicates whether the UE supports LP-WUS operation in IDLE/INACTIVE mode for a list of frequency bands. The UE shall support UEID based subgrouping for a frequency band if it indicates supporting of LP-WUS operation for the frequency band. The UE supporting this feature shall also indicate the support of at least lpwus-OOK-r19 or lpwus-OFDM-r19.

For each supported band, the capability signalling comprises of the following parameters:

-	supportedBandIndicator-r19 indicates the frequency bands where UE supports LP-WUS operation in IDLE/INACTIVE mode;

…

-	minimumTimeGap-r19 indicates the minimum time gap between LP-WUS reception and UE to start PDCCH monitoring. The values of ‘cap1’, ‘cap2’ and ‘cap3’ for each SSB periodicity are shown in below table:

| SSB periodicity (ms) | cap1 (ms) | cap2 (ms) | cap3 (ms) |
| --- | --- | --- | --- |
| 5/10/20 | 70 | 500 | 900 |
| 40 | 130 | 600 | 1000 |
| 80 | 250 | 800 | 1200 |
| 160 | 490 | 1200 | 1600 |

LPWUS-SupportedBandInfo-r19 ::=      SEQUENCE {

…

minimumTimeGap-r19

ENUMERATED {cap1, cap2, cap3}

}

Agree the LP-WUS corrections in [4, 5].

Rel-19 RRM relaxation and offloading UE capability is captured in chapter 5. It seems that different wording is used for RRM relaxation and offloading in different specifications. There are different flavors of RRM relaxation in idle and connected, which can cause confusion:

38.306

Relaxation of serving cell and further relaxation of neighboring cell RRM measurements and offloading of serving cell RRM measurements

38.300

to relax serving cell measurements on MR, further relax neighbour cell measurements on MR

38.304

UE supporting LP-WUS may perform further relaxed serving cell and further neighbouring cell measurements on MR as specified in clause 5.2.4.x.1 or serving cell measurement offloading from MR to LR as specified in clause 5.2.4.x.3.

5.2.4.x Relaxed measurement and measurement offloading for UE supporting LP-WUS

UE supporting LP-WUS may choose to perform relaxed serving cell and further relaxed neighbouring cell measurements on MR

For the Rel-19 RRM offloading and relaxation capability it is proposed to add a reference to the clause in 38.304 to avoid any confusion:

| Relaxation of serving cell and further relaxation of neighboring cell RRM measurements as specified in clause 5.2.4.x.1 in TS 38.304 [21] and offloading of serving cell RRM measurements as specified in clause 5.2.4.x.3 in TS 38.304 [21]. | It is mandatory to support relaxation of serving cell and neighboring cell RRM measurements and offloading of serving cell RRM measurements if a UE supports reception of LP-WUS in RRC_IDLE/RRC_INACTIVE. A UE supporting this feature shall also indicate the support at least one of lpwus-OOK-r19 and lpwus-OFDM-r19. |
| --- | --- |

Add reference to the clause in 38.304 for Rel-19 RRM relaxation and offloading UE capability for LP-WUS.

## Move lpwus-Config-r19 into SpCellConfig (E007)

Both DCP and LP-WUS can be configured on PCell and PSCell, also referred to as Special Cell (SpCell).

Parameters for the SpCell should be included in the SpCellConfig IE:

spCellConfig Parameters for the SpCell of this cell group (PCell of MCG or PSCell of SCG).

For some reason dcp-Config-r16 did not make it into SpCellConfig IE, but according to the rules lpwus-Config-r19 should be placed there:

Move lpwus-Config-r19 to SpCellConfig IE.

PhysicalCellGroupConfig ::=         SEQUENCE {

…

[[

dcp-Config-r16                      SetupRelease { DCP-Config-r16 }   OPTIONAL,   -- Need M

…

]]

}

…

CellGroupConfig ::=                        SEQUENCE {

…

spCellConfig                               SpCellConfig        OPTIONAL,   -- Need M

…

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=                        SEQUENCE {

…

]],

[[

lpwus-Config-r19                      SetupRelease { LPWUS-Config-r19 }  OPTIONAL   -- Need M

]]

}

## RX level and cell quality on LR (38304-2)

In 38.304 [6] it has been captured that LP-WUS beam selection is left to UE implementation e.g. whether to use strongest beam or average of N beams:

The UE monitors one LO per DRX cycle. An LO is a set of LP-WUS monitoring occasions (LP-WUS MOs). In multi-beam operations, the UE assumes that the same LP-WUS is repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation.

During RAN2#131 issue RRC-13 was discussed [7]:

Rapporteur understands that the below LR measurement based RX level and cell quality value:

-	Qrxlevmeas _lr= current measured cell RX level value of the serving cell based on LR (RSRP).

-	Qqualmeas _lr = current measured cell quality value of the serving cell based on LR (RSRQ).

should be derived by UE implementation in multi-beam operations, i.e. no additional RRC parameters like nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation for LR measurement is needed.

Companies are invited to provide comments on whether LR measurement based RX level and cell quality value should be derived by UE implementation in multi-beam operations.

and the following proposal was discussed online:

Proposal 2: [11/14] LR measurement based RX level and cell quality value should be derived by UE implementation in multi-beam operations.

And it was agreed:

LR measurement based RX level and cell quality value should be derived by UE implementation in multi-beam operations. We assume this conclusion does not impact the cell reselection procedure.

It is not clear whether the agreement re-confirms that beam selection is up to UE implementation. But it was captured in 38.304 [6]:

In multi-beam operations, measured cell RX level value and measured cell quality value of the serving cell based on LR is up to UE implementation.

The sentence can also be read that how the UE determines the Rx level (LR-RSRP) and cell quality (LR-RSRQ) from the LR measurements is up to UE implementation. We do not think that it is up to UE implementation how to measure LR-RSRP/LR-RSRQ. Furthermore LR-RSRP/LR-RSRQ measurements do not differ in case of multi-beam and single-beam operations. Unless it can be clarified what the intention is we propose:

Remove “In multi-beam operations, measured cell RX level value and measured cell quality value of the serving cell based on LR is up to UE implementation” from 38.304.

## RRCReconfiguration of LP-WUS (E043, H050)

In H050 it is proposed that the UE via UAI signalling can indicate a preference to the gNB to disable/de-configure LP-WUS. The motivation given is that the reported MR measurements may be good but the LR reception is bad e.g. due to adjacent-channel interference (e.g. for Type 2 LR see section 2.2).

In our understanding the MR and LR reception are correlated, i.e. when the LR reception is bad this will eventually also show up in the MR measurement reporting of the UE. And we think that adjacent-channel interference should be handled in a different way, see E053. The adjacent-channel interference is of a permanent nature and the LR should not be used by Type 2 WUR when there is not sufficient guard band configured.

Using UAI to disable LP-WUS increases the signalling load. When the prohibit timer is running the UE is not able send UAI. UAI introduces a signalling delay during which LP-WUS reception may be bad. It is better that the UE immediately stops using LP-WUS when the reception is bad. To obtain predictable UE behavior it is beneficial if the NW can configure an exit condition when the UE stops using LP-WUS. Such exit condition can be based on RLM/BFD measurements which the UE periodically performs in connected mode, even when there is no traffic and drx-OnDurationTimer or lpwus-PDCCH-MonitoringTimer is not started:

NW can configure an exit condition for LP-WUS monitoring in connected mode based on RLM/BFD measurements.

## TimeToTrigger (TTT) for RRM relaxation entry (E035)

RAN2 agreed that there is no separate exit condition for RRM relaxation. There is only an entry threshold, similar as with the legacy measurement thresholds SintraSearch and SnonIntraSearch. This means there is no hysteresis, i.e. the exit is when the entry is not fulfilled. Thus there is a risk of ping-pong, i.e. based on small measurement fluctuations the UE starts/stops relaxing RRM measurements. More importantly once the UE is in relaxation state and applies a scaling factor 16 to the serving cell measurements on MR, i.e. only measures the serving cell every 20 sec and fluctuations below the entry threshold during these 20 sec may go unnoticed:

The UE may enter RRM relaxation state based on a single measurement peak. While the UE is in RRM relaxation state, the UE may drop below the RRM relaxation entry threshold for 20 seconds that goes un-noticed.

To avoid that the UE enters relaxation state on a single measurement peak it should be possible to configure a TimeToTrigger (TTT) during which the MR has to be measured above threshold before the entry condition is fulfilled:

NW can configure a TimeToTrigger (TTT) with the RRM relaxation entry condition.

# Summary

RAN2 is kindly asked to discuss LP-WUS issues:

Observation 1	The UE may enter RRM relaxation state based on a single measurement peak. While the UE is in RRM relaxation state, the UE may drop below the RRM relaxation entry threshold for 20 seconds that goes un-noticed.

Proposal 1	LP-WUS and paging adaptation can be configured and used together.

Proposal 2	The LP-WUS used with Rel-19 POs uses the N and Ns value of paging adaptation.

Proposal 3	Introduce the possibility to configure another LoFrameOffsetList, MoNumPerLo and PoNumPerLo for LP-WUS used with Rel-19 POs.

Proposal 4	Introduce a flag in SIB to prohibit Type 2 WUR to use LP-WUS in the cell e.g. when LP-WUS cannot be configured with sufficient guard band for type 2 WUR.

Proposal 5	Agree the LP-WUS corrections in [4, 5].

Proposal 6	Add reference to the clause in 38.304 for Rel-19 RRM relaxation and offloading UE capability for LP-WUS.

Proposal 7	Move lpwus-Config-r19 to SpCellConfig IE.

Proposal 8	Remove “In multi-beam operations, measured cell RX level value and measured cell quality value of the serving cell based on LR is up to UE implementation” from 38.304.

Proposal 9	NW can configure an exit condition for LP-WUS monitoring in connected mode based on RLM/BFD measurements.

Proposal 10	NW can configure a TimeToTrigger (TTT) with the RRM relaxation entry condition.

# References

- R1-2501703, LS on LP-WUS UE RF, LS out, To: RAN1, Cc: RAN2, RAN4#114

- R4-2511904, dCR to 38.101-1, LPWUS ACS, CR 38.101-1, QC, RAN4#116

- R4-2511427, On blocking performance of WUR, Ericsson, DISC, RAN4#116

- R2-2507586, Corrections on Rel-19 RAN1/4 UE capability, Xiaomi, CR 38.306, RAN2#131bis

- R2-2507587, Corrections on Rel-19 RAN1/4 UE capability, Xiaomi, CR 38.331, RAN2#131bis

- R2-2506608, Introduction of LP-WUS in TS 38.304, CATT, CR 38.304, RAN2#131

- R2-2505393, Discussion summary and list of RRC open issue for LP-WUS WUR, vivo, DISC, RAN2#131