3GPP TSG-RAN2 Meeting #131bis	R2-2507628

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	8.4.4 Other issues

Source: 	Ericsson

Title:  	LP-WUS and low mobility (E008, 38304-1 and -3)

Document for:	Discussion and Decision

# Introduction

In this contribution LP-WUS and low mobility is discussed (38304-1 and -3). If a moving LP-WUS UE does not measure neighbour cells up to 20 seconds, then the paging/cell reselection performance may be impacted. Therefore it is proposed to add the low mobility criterion to the entry/exit condition for Rel-19 RRM relaxation. To limit the scope of the changes it is proposed to keep the restriction that offloading is only allowed above SintraSearch/ SnonIntraSearch (E008). The proposed changes do not impact RAN4, i.e. only the entry/exit condition in 38.304 is changed. Similar approach is proposed in [1].

# Discussion

When the entry condition for Rel-19 RRM relaxation is fulfilled, then the LP-WUS UE may apply a scaling factor 16 to serving/neighbour cell measurements. However if a moving LP-WUS UE does not measure neighbour cells up to 20 seconds (16 x 1.28 sec), then the paging/cell reselection performance may be impacted:

RAN2 confirms that paging and cell reselection may be impacted when a moving UE does not measure neighbour cells for up to 20 seconds.

During RAN2#131 it has been discussed offline that impact on paging/cell reselection can be prevented by taking the UE mobility into account. If the UE did not detect a stronger neighbour cell while it was moving, then the UE can refrain from neighbour cell measurements when it stopped moving until it starts moving again. However there were different views on how to enable this:

- Leave it to UE implementation (e.g. the UE uses whatever means it has to detect mobility, e.g.  positioning, and does not apply the scaling factor when mobility is detected).

- Introduce low mobility criterion with LP-WUS (e.g. the UE only applies the scaling factor when low mobility is detected).

Paging and cell reselection performance is a critical KPI for the operator and therefore this issue cannot be left to UE implementation. It is proposed to add the low mobility criterion to entry/exit condition for Rel-19 RRM relaxation. This means that the MR measurements are used to determine when the UE enters low mobility state and the LR measurements are used to detected when the UE is no longer in low mobility state i.e. started moving again:

Introduce low mobility criterion to the entry/exit condition for Rel-19 RRM relaxation for LP-WUS UE.

TPs are provided for further discussion and agreement in the Annex.

# References

- R2-2507618, Discussion on open issues in 38.304 for LP-WUS, Nokia, DISC, RAN2#131bis

# Summary

RAN2 is kindly asked to discuss two critical issues for LP-WUS:

Proposal 1	RAN2 confirms that paging and cell reselection may be impacted when a moving UE does not measure neighbour cells for up to 20 seconds.

Proposal 2	Introduce low mobility criterion to the entry/exit condition for Rel-19 RRM relaxation for LP-WUS UE.

# TP

38.306

| Relaxation of serving cell and neighboring cell RRM measurements and offloading of serving cell RRM measurements | It is mandatory to support relaxation of serving cell and neighboring cell RRM measurements, offloading of serving cell RRM measurements and low mobility criterion for LP-WUS if a UE supports reception of LP-WUS in RRC_IDLE/RRC_INACTIVE. A UE supporting this feature shall also indicate the support at least one of lpwus-OOK-r19 and lpwus-OFDM-r19. |
| --- | --- |

38.304

Short recap of the Rel-16 low mobility criterion:

- The UE drops out of low mobility state when the signal strength has dropped more than delta dB.

- The UE enters low mobility state when the signal strength did not drop more than delta dB for a time period Tsearch.

- Immediately after cell (re-)selection the UE is not in low mobility state, i.e. there is a delay Tsearch before the UE can enter low mobility state.

The main changes are highlighted below:

- The low mobility criterion is re-used, but either MR or LR serving cell measurements are used for evaluation, dependent on whether entry/exit for RRM relaxation is fulfilled.

- When the UE enters/exits RRM relaxation and changes measurement type the reference value is reset.

##### 5.2.4.9.x	Low mobility criterion for UE supporting LP-WUS

Serving cell measurements on LR are used to evaluate the low mobility criterion, when the entry condition for RRM relaxation is fulfilled and the UE chooses to start relaxed serving cell and neighbouring cell measurements on MR. Serving cell measurements on MR are used to evaluate the low mobility criterion, when the entry condition for RRM relaxation is no longer fulfilled and the UE stops relaxed serving cell and neighbouring cell measurements on MR.

The low mobility criterion for UE supporting LP-WUS is fulfilled when for a period of TSearchDeltaPLPWUS the condition below is fulfilled, otherwise the low mobility criterion is not fulfilled:

-	(SrxlevRef – Srxlev) < SSearchDeltaP,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB on MR, dBm on LR).

-	SrxlevRef = reference Srxlev value of the serving cell (dB on MR, dBm on LR), set as follows:

-	After selecting or reselecting a new cell, or

-	After the entry or exit condition for RRM relaxation is fulfilled and the UE chooses to start or stop relaxed serving cell and neighbouring cell measurements on MR, or

-	If (Srxlev - SrxlevRef) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP:

-	The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell.

#### 5.2.4.x Relaxed measurement and measurement offloading for UE supporting LP-WUS

In multi-beam operations, measured cell RX level value and measured cell quality value of the serving cell based on LR is up to UE implementation.

##### 5.2.4.x.1 Relaxed measurement rules

UE supporting LP-WUS may choose to perform relaxed serving cell and neighbouring cell measurements on MR according to requirements specified in TS 38.133 [8] if the relaxed measurement criterion in clause 5.2.4.x.2 is fulfilled.

##### 5.2.4.x.2 Relaxed measurement criterion

The relaxed measurement criterion for serving cell and neighbouring cell measurement relaxation on MR is fulfilled when:

-	Srxlev > SLP_WUS_RelaxThresholdP_MR, and,

-	Qrxlevmeas_lr > QLP_WUS_RelaxThresholdP_LR, if QLP_WUS_EntryThresholdP_LR is configured, and,

-	Squal > SLP_WUS_RelaxThresholdQ_MR, if SLP_WUS_RelaxThresholdQ_MR is configured, and

-	Qqualmeas_lr > QLP_WUS_RelaxThresholdQ_LR, if QLP_WUS_EntryThresholdQ_LR is configured, and

-	Low mobility criterion in clause 5.2.4.9.x is fulfilled, if lowMobilityEvaluationLPWUS is configured.

Where:

…

38.331

SIB2 ::=                            SEQUENCE {

…

relaxedMeasurement-r16              SEQUENCE {

lowMobilityEvaluation-r16           SEQUENCE {

s-SearchDeltaP-r16                  ENUMERATED {

dB3, dB6, dB9, dB12, dB15,

spare3, spare2, spare1},

t-SearchDeltaP-r16                  ENUMERATED {

s5, s10, s20, s30, s60, s120, s180,

s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                   OPTIONAL,       -- Need R

…

]],

[[

relaxedMeasurementForServingAndNeighboringCell-r19                  SEQUENCE {

cellEdgeEvaluationOnMR-ForLR-OnSSB-r19   SEQUENCE {

s-SearchThresholdP3-r19                 ReselectionThreshold,

s-SearchThresholdQ3-r19                 ReselectionThresholdQ                   OPTIONAL        -- Need R

}                                                                                  OPTIONAL,    -- Cond SupportLR-OnSSB

cellEdgeEvaluationOnMR-ForLR-OnLPSS-r19   SEQUENCE {

s-SearchThresholdP4-r19                 ReselectionThreshold,

s-SearchThresholdQ4-r19                 ReselectionThresholdQ                   OPTIONAL        -- Need R

}                                                                                   OPTIONAL,    -- Cond SupportLR-OnLPSS

cellEdgeEvaluationOnLR-ForLR-OnLPSS-r19   SEQUENCE {

rsrpThresholdLR-r19               ThresholdP-LR-r19,

rsrqThresholdLR-r19               ThresholdQ-LR-r19                             OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

cellEdgeEvaluationOnLR-ForLR-OnSSB-r19   SEQUENCE {

rsrpThresholdLR2-r19               ThresholdP-LR-r19,

rsrqThresholdLR2-r19               ThresholdQ-LR-r19                            OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

lowMobilityEvaluationLPWUS-r19           SEQUENCE {

s-SearchDeltaP-LPWUS-r19                  ENUMERATED {

dB3, dB6, dB9, dB12, dB15,

spare3, spare2, spare1},

t-SearchDeltaP-LPWUS-r19                  ENUMERATED {

s5, s10, s20, s30, s60, s120, s180,

s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                   OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

…

]]

}

s-SearchDeltaP-LPWUS Parameter "SSearchDeltaPLPWUS" in TS 38.304 [20]. Value dB3 corresponds to 3 dB, dB6 corresponds to 6 dB and so on.

t-SearchDeltaP-LPWUS Parameter "TSearchDeltaPLPWUS" in TS 38.304 [20]. Value in seconds. Value s5 means 5 seconds, value s10 means 10 seconds and so on.

lowMobilityEvaluationLPWUS Indicates the criteria for a UE supporting LP-WUS to detect low mobility, in order to perform serving cell measurement offloading (see TS 38.304 [20], clause 5.2.4.x.3).

38.300

…

Power saving in RRC_IDLE and RRC_INACTIVE can also be achieved by allowing UEs supporting LP-WUS to relax serving cell measurements on MR, further relax neighbour cell measurements on MR and/or offload serving cell measurements from MR to LR. Conditions for further relax neighbour and serving cell measurements are based on MR and optionally LR measurements as specified in TS 38.304 [10]. Entry condition for offloading serving cell measurements from MR to LR is based on MR and optionally LR measurements as specified in TS 38.304 [10]. Exit conditions for offloading serving cell measurements from MR to LR are based on LR measurements and whether low mobility criterion for UE supporting LP-WUS is fulfilled, if configured as specified in TS 38.304 [10].

…

38.113 (no impact)

#### 4.8.2.4 Measurements of intra-frequency NR cells for UE with LP-WUR

For a UE supporting LP-WUR capability and not configured with eDRX_IDLE cycle, the requirements in clause 4.2.2.3 for FR1 and FR2-1 apply except for the requirements specified in this clause when the relaxed measurement criterion defined in 5.2.4.x.2 in TS 38.304 [1] is fulfilled.

…

#### 4.8.2.5	Measurements of inter-frequency NR cells for UE with LP-WUR

##### 4.8.2.5.1	Introduction

This clause contains the requirements for measurements on inter-frequency NR cells for UE with LP-WUR when the UE is configured with any of following measurement criteria:

-	Relaxed measurement criterion for UE with LP-WUR defined in 5.2.4.x.2 in TS 38.304 [1],

-	Serving cell measurement offloading criterion for UE with LP-WUR defined in TS 38.304 [1],

…