3GPP TSG-RAN WG2 Meeting #132	R2-2508034

Dallas, USA, November 17-21, 2025

Agenda item:	10.3.1.3

Source:	Qualcomm Incorporated

Title:	Discussion on scheduling in 6G

WID/SID:	FS_6G_radio

Document for:	Discussion and decision

# Introduction

In this paper, we discuss methods in the MAC layer for reducing scheduling latency and how coordination between HARQ and ARQ can help reduce end-to-end latency.

# Discussion

## UL scheduling

SR/BSR/DSR

At RAN2#132 meeting, there was discussions on using contention-based UL transmission (CBUL) to send SR/BSR to reduce UL access latency.

At present, details on how CBUL works have not been discussed yet. However, certain properties can be deduced from what is known. First, being a contention-based access scheme, its access latency is inherently not deterministic and may depend on factors such as periodicity of CBUL resource occasions, number of UEs sharing these resources, etc. To achieve low access latency for CBUL, network would need to configure frequent resource occasions and small number of UEs sharing it. This, however, would make it essentially the same as type-1 configured grant.

Because of these observations, we do not think CBUL is the optimal choice for SR/BSR triggered by delay-sensitive data and should not be the only option for sending SR/BSR. Other mechanisms, such as legacy configured grants and PUCCH based SR, should still be considered as viable options in 6G.

Observation 1. 	Contention-based UL transmissions should not be the only option for sending SR/BSR, as its latency is not deterministic and can depend on several factors, such as periodicity of transmissions occasions, contention levels, etc.

Among all the available options for sending SR/BSR, we believe each of them has its own most applicable use case:

- Configured grant. It can be a good option for sending SR/BSR with low latency. However, as its resources are dedicated or near dedicated, it may not as spectrally efficient as others. In addition, because its transmission parameters are RRC configured and not dynamically scheduled, it may not be the most reliable when the link quality fluctuates. Perhaps because of these reasons, configured grants are rarely deployed in 5G networks.

- CBUL. As analysed above, we think it can a good option for data without strict delay requirements, because it can achieve high spectral efficiency when shared by many UEs with infrequent request for UL grants. But it is probably not a good option for delay-critical data to use.

- SR over PUCCH. It is still a dependable option for UE to request UL grants, as confirmed by wide deployment. It can perform reliably in various radio conditions and can be flexibly configured for both delay-sensitive and delay-insensitive data. The perception of PUCCH as a “slow” option is largely because operators opt not to configure short periodicities for SR. That choice is primarily due to the limited presence of delay-sensitive traffic in 5G networks, instead of any intrinsic limitation.

Based on the above analysis, we think that which type of UL resources to use to send SR/BSR depends on multiple factors such as priority of the triggering data, how network would allocate its UL resources between different purposes, etc. Therefore, it is more sensible to leave that choice to network, instead of limiting it to CBUL.

This is not to say that the legacy mechanisms are good enough for 6G. It remains important to make SR configurations more resource efficient. Potential areas to explore may include dynamic de-/activation of SR configurations for high-priority (delay-sensitive) data, multi-bit SR to indicate the nature of SR trigger so that SR resources do not need to be partitioned among different types of triggers.

Proposal 1.  	Which type of UL resources (e.g. CBUL vs CG vs PUCCH) should be used to for transmitting SR/BSR can be left to network configuration. Study techniques that can reduce transmission latency of SR over different types of UL resources.

A few designs in the legacy BSR procedure may become shortcomings for 6G use cases.

In legacy, when UE buffer is not empty, BSR is triggered only when higher priority data arrives. This requirement can potentially delay UE from reporting its buffer size in a timely way. For example, consider a logical channel with very bursty traffic and a large variation in burst sizes (e.g. XR, immersive communications). When a large burst arrives after a few small or average sized bursts, gNB scheduler cannot know the much larger amount of data in UE’s buffer and adjust its UL grant allocation accordingly, until it receives a periodic BSR later. Configuring a short period for periodic BSR is not an efficient solution to this problem, because that would unnecessarily trigger too many BSRs.

Given that XR and immersive communications in 6G can have much bustier traffic than legacy, We think it is necessary to study new triggering conditions for buffer information reporting in the new unified BSR/DSR procedure. The new triggers should be designed to enable more timely and adaptive reporting, thereby reducing UL scheduling latency. At the same time, the new design should keep signaling overhead low by not generating an excessive number of reports.

Observation 2. Existing BSR triggers do not always timely report changes in UE’s buffer status, e.g. when traffic in a logical channel has wide-varying burst sizes.

We have observed in field data that a large percentage of BSRs do not report accurate buffer status to gNB. For example, when UE has only one DRB, which is a very common configuration in the field, only short BSR is used. As short BSR uses a BSR table that has a small maximum value, a large percentage of short BSR reports the code point 31 (i.e. >150KB). If a gNB scheduler allocates UL grant conservatively (e.g. to avoid wasting PUSCH resources), it would schedule UE in multiple UL grants with small sizes (e.g. 150KB), resulting in unnecessary scheduling delays.

In Rel-18, long BSR can be used even if UE has only one DRB, if the buffer size is in the range of the refined BSR table. However, as the refined BSR table is defined specifically for XR traffic, all not applications in 6G may benefit from this enhancement.

Therefore, we believe it is useful to improve the accuracy of buffer information reporting in 6G, so that network scheduler can timely schedule right amount of PUSCH resources.

Observation 3. 	Field data shows that inaccuracy in buffer size reporting may also result in delay in UL scheduling.

Proposal 2.  	Study techniques that enable timely and accurate buffer information reporting for the unified B/DSR in 6G.

#### LCP

#### The scheduling algorithm used in NR’s LCP procedure is mainly based on weighted fair queueing, which aims to maximize throughput while ensuring fairness between different QoS flows. It has worked well in the past because most traffic in the field is eMBB. However, as delay critical traffic from applications such as XR and immersive communications is expected to have increased presence in 6G, it may no longer be sufficient to support a more diverse mix of traffic. This is because scheduling priority for delay critical traffic needs to not only depends on the delay budget defined/configured for the entire flow, but also the remaining time of individual PDUs, which is dynamic.

#### Rel-19 introduced dynamic LCP priority, which increases the priority of a logical channel when it has data with a remaining time below a configured threshold. While this enhancement helps address the shortcoming with delay critical data to some extent, it is not a native solution to the problem.

#### 6G should have a scheduling algorithm that can natively support different types of traffic. Instead of developing new algorithms from scratch, the study can focus on how to integrate existing scheduling algorithm with different performance objectives (e.g. throughput vs delay) in a cohesive way.

#### Observation 4.	The LCP procedure in NR may no longer be sufficient in supporting more diverse traffic in 6G, due to increased presence of delay sensitive traffic.

#### In NR, parameters used in the LCP procedure are semi-static (RRC configured). New use cases in 6G may require them to be more adaptive. For example, to support application context awareness or adaptive QoS (see R2-2508033), logical channel priority or LCP parameters such as PBR and BSD need to be dynamically adjusted when QoS attributes of a traffic flow change. This can be implemented by either allowing UE to autonomously switching between preconfigured LCP parameters or introducing new signaling for the purpose.

#### Observation 5.	New 6G use cases such as application context awareness and adaptive QoS require LCP to be more adaptive than NR.

#### Proposal 3. 	Study new LCP designs that can support multiple performance objectives (e.g. throughput vs delay) in a cohesive way and be more adaptive than NR to support new use cases such as application context awareness and adaptive QoS.

#### HARQ

#### In NR and even earlier releases, HARQ is agnostics to QoS differentiation in upper layers. Data from any logical channels can be multiplexed in a TB, as long as it is permitted by LCP restriction, which does not depend on QoS. This approach works because traffic in legacy networks have similar QoS requirements. The remaining difference can be handled by RLC, while HARQ is optimized to achieve high spectral efficiency.

#### In 6G, with applications like XR, which can generate multiple flows with quite different QoS requirements, this approach may warrant a reconsideration. For example, for flows with strict delay requirements, it is more efficient to rely on HARQ alone to achieve both delay and reliability requirements, because a HARQ retransmission has much lower latency than a RLC ARQ. On the other hand, for flows that are throughput-centric and delay tolerant, it is more spectrally efficient to have less HARQ retransmissions and more RLC ARQ. Therefore, it makes more sense to serve these two types of flows by separate HARQ processes.

#### This separation is easy to implementation. For example, a scheduling DCI can indicate which subset of logical channels the UL grant is intended for. On the UE side, LCP restriction can ensure only data from the intended logical channels gets multiplexed in this UL grant.

#### There may be some loss in UL resources because of the separation in HARQ processes. However, we do not expect it to be significant enough to be of concern. First, there will not be many separate types of traffic at HARQ level. For example, two may be enough, i.e. one for throughput-centric flows and another for delay sensitive flows. Second, more accurate buffer information report (see Proposal 2) can help network schedule more efficiently and avoid waste of resources.

#### Observation 6. 	It is more efficient to serve QoS flows with different QoS requirements (e.g. throughput vs delay) by separate HARQ processes.

#### Proposal 4. 	Support finer granularity for HARQ and allow network to schedule different logical channels by separate HARQ processes.

## ARQ

During last meeting, L2 retransmission was briefly discussed with following agreement reached:

Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ. Study should identify enhancements needed for HARQ and L2 ARQ.

In our view, it is useful to first agree on the L2 retransmission baseline, then discuss optimizations and measures that can reduce latency and improve general efficiency of the protocol.

### Baseline L2 Retransmission Mechanism

5G-NR relied on a multi-level retransmission structure to trade-off the speed of HARQ retransmissions with the reliability of RLC status reports. The HARQ mechanism in MAC layer targets fast retransmissions by providing implicit or explicit feedback to the transmitter after each TB transmission, at cost of transmission resources such as (buffering, processing, control overhead, memory, etc.). Thus, it is typical of networks to target a HARQ feedback error of 0.1-1% and rely on the RLC ARQ mechanism to correct residual HARQ errors.

In 6G, we think that this fundamental multi-level retransmission structure should be preserved. In 5G-NR, ARQ and HARQ were developed as completely separate mechanisms to preserve layer modularity. However, with experience from NR, there was a realization that often, the transmitter can observe HARQ feedback and infer an ARQ error needing retransmission earlier than the RLC status report arrival, which may be subject to timers (e.g. t-Reassembly), and this represents an opportunity for minimizing latency by having the RLC transmitter observe both ARQ and HARQ feedback.

On the other hand, 5G NR ARQ design was built on a very important principle: That it can operate completely independently from HARQ errors, i.e., if the NW chooses not to perform HARQ at all, ARQ can still correct all channel errors using the Sequence Number based recovery mechanism (which is how some NTN networks are deployed). It is important to maintain this principle for two reasons:

- To give NW the ability to trade-off the different design options discussed above (latency, spectral efficiency, power, memory, processing, etc.) by setting the HARQ and ARQ reliability targets and not have 6G operating with more limitations than NR.

- To guarantee any reliability target needed by the application independent of HARQ variations in residual errors and HARQ RTT; both of which exhibit significant variation in 5G-NR.

Observation 7.	Reliance on HARQ coordination may speed up the RLC recovery loop. However, the RLC Sequence Number-window based mechanism guarantees that eventually all HARQ errors are corrected independent of HARQ degradation, and gives NW flexibility to trade off different design KPIs (latency, spectral efficiency, power, overhead, processing, etc.)

Thus, before discussing the several optimizations that may speed up ARQ by having it take HARQ events into account, RAN2 should agree to preserve the 5G-NR principle of enabling ARQ to correctly operate independent of HARQ. Note that this need not lead to redundancies in feedback reporting with correct design, i.e., in a NW with very HARQ reliable feedback, the RLC receiver would only occasionally (when polled) trigger ARQ status report positively acknowledging the whole window, incurring very little actual ARQ overhead. We anticipate that with correct design of ARQ-HARQ coordination scheme, feedback redundancies would be minimal.

Proposal 5.  	Sequence number-based L2 recovery mechanism is the baseline for ARQ. ARQ to retain the requirement to achieve up to 100% reliability independent of HARQ.

### Uplink

#### On UL, a RLC transmitter performs retransmission of a RLC PDU only after the corresponding status report is received. The minimum amount of delay in getting this status report after end of its last HARQ transmission is at least the time it takes to send a PDU over DL by HARQ. On the other hand, if UE is able to determine that the HARQ process for a TB has failed, it can immediately trigger RLC retransmission for the RLC PDUs contained in that TB. This removes the need to wait for the status report, thereby reducing latency.

#### Observation 8. 	If UE can determine a UL HARQ process has failed, it can immediately trigger ARQ retransmission of RLC PDUs in the failed TB. That helps reduce UL latency.

This determination can be either made by UE itself or based on some indication from network.

- In the former option, UE may do that by estimating the likelihood of HARQ failure based on the number of HARQ retransmissions it has already performed. This option avoids the need for any new network signalling; however, the UE may make a mistake in its estimation and perform unnecessary RLC retransmissions.

- In the latter option, network can send an indication to UE after it decides to terminate a failed UL HARQ process. This indication then can be used by UE to perform a RLC retransmission. This option is more deterministic and would not cause unnecessary retransmission. And its signaling can be designed such that it will not cause much impact to the DL capacity. We therefore believe it is a more preferrable option.

#### Proposal 6. 	Network indicates a failed UL HARQ transmission to UE. This indication triggers the UE to initiate an ARQ retransmission of the RLC PDUs contained in the failed TB.

### Downlink

In Downlink 5G-NR ARQ, the UE observes the sequence numbers and Segments received in the Rx buffer. Upon detecting a Sequence Number hole (or missing segment), UE starts t-Reassembly to give HARQ a chance to recover the missing TB. If t-Reassembly expires without the missing PDU or segment being received, a status report is triggered to inform gNB of the Rx window status. gNB can then retransmit the missing PDUs or segments at the RLC level.

In 5G-NR HARQ, UE transmits explicit HARQ feedback to gNB on PUCCH. Thus, if HARQ feedback is error-less, gNB would track the TB HARQ status after every HARQ feedback transmission. However, PUCCH occasionally incurs two types of errors:

- Nack-to-Ack error: gNB erroneously decodes a HARQ NACK as an ACK and terminates HARQ process. In this case, gNB would still maintain RLC transmission buffer until receiving a status report. The status report would indicate an RLC Nack of the TB content triggering an RLC retransmission.

- Ack-to-Nack error: gNB erroneously decodes a HARQ ACK as a NACK. In this case, gNB decides to retransmit the TB contents redundantly. It is up to the gNB whether to retransmit TB contents on HARQ or ARQ level. There may be a maximum HARQ retransmission number that determines the feedback-level, but this is not specified in standards. gNB may also wait for RLC status report to trigger ARQ retransmission. In this case, Ack-to-Nack would only cause redundant HARQ retransmission, potentially.

Another source of residual HARQ errors (that may need RLC level retransmission) are the DTX-to-Ack errors: In this case, UE may miss the DCI indicating a downlink assignment and subsequent HARQ feedback. After the DL transmission which the UE would not attempt to decode, the gNB may erroneously infer the UE lack of PUCCH transmission (DTX) as a HARQ Ack.

Observation 9.  In DL, PUCCH errors and missed DCIs contribute to DL RLC errors requiring UE to correct using an RLC Status report, triggering an RLC retransmission at gNB.

ARQ-HARQ coordination in downlink should start by studying which of the aforementioned HARQ errors can be detected fast (i.e. faster than a typical duration of t-Reassembly timer) at the UE and reported back to the network.

Observation 10. At the UE, detecting certain HARQ events may indicate a potential DL RLC error. This detection can be faster than the typical duration of t-Reassembly timer.

Having identified HARQ events that lead to DL RLC errors, we think that detecting those events are heavily influenced by DCI and PUCCH design. Since DCI and PUCCH design will have to be handled by RAN1, RAN2 cannot progress on the HARQ events’ detection mechanisms themselves but can design a holistic signalling mechanism for ARQ that includes both legacy triggers (i.e., t-reassembly, polling, etc.) and new yet-to-be defined triggers (e.g. HARQ event detection).

Proposal 7.  	On Downlink, RAN2 to send an LS requesting RAN1 to investigate how the UE can best detect the following events with low latency:

- HARQ Nack-to-Ack error

- DTX-to-Ack error, i.e., due to missing DL DCI, UE does not transmit HARQ feedback while network detects DTX as ACK.

RAN2 may continue to discuss ARQ state-machine including signalling, retransmission triggers, etc. based on RAN1 progress.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

SR/BSR/DSR

Observation 1. 	Contention-based UL transmissions should not be the only option for sending SR/BSR, as its latency is not deterministic and can depend on several factors, such as periodicity of transmissions occasions, contention levels, etc.

Proposal 1.  	Which type of UL resources (e.g. CBUL vs CG vs PUCCH) should be used to for transmitting SR/BSR can be left to network configuration. Study techniques that can reduce transmission latency of SR over different types of UL resources.

Observation 2. 	Existing BSR triggers do not always timely report changes in UE’s buffer status, e.g. when traffic in a logical channel has wide-varying burst sizes.

Observation 3. 	Field data shows that inaccuracy in buffer size reporting may also result in delay in UL scheduling.

Proposal 2.  	Study techniques that enable timely and accurate buffer information reporting for the unified B/DSR in 6G.

#### LCP

#### Observation 4.	The LCP procedure in NR may no longer be sufficient in supporting more diverse traffic in 6G, due to increased presence of delay sensitive traffic.

#### Observation 5.	New 6G use cases such as application context awareness and adaptive QoS require LCP to be more adaptive than NR.

#### Proposal 3. 	Study new LCP designs that can support multiple performance objectives (e.g. throughput vs delay) in a cohesive way and be more adaptive than NR to support new use cases such as application context awareness and adaptive QoS.

#### HARQ

#### Observation 6. 	It is more efficient to serve QoS flows with different QoS requirements (e.g. throughput vs delay) by separate HARQ processes.

#### Proposal 4. 	Support finer granularity for HARQ and allow network to schedule different logical channels by separate HARQ processes.

ARQ

Observation 7.	Reliance on HARQ coordination may speed up the RLC recovery loop. However, the RLC Sequence Number-window based mechanism guarantees that eventually all HARQ errors are corrected independent of HARQ degradation, and gives NW flexibility to trade off different design KPIs (latency, spectral efficiency, power, overhead, processing, etc.)

Proposal 5.  	Sequence number-based L2 recovery mechanism is the baseline for ARQ. ARQ to retain the requirement to achieve up to 100% reliability independent of HARQ.

#### Observation 8. 	If UE can determine a UL HARQ process has failed, it can immediately trigger ARQ retransmission of RLC PDUs in the failed TB. That helps reduce UL latency.

#### Proposal 6. 	Network indicates a failed UL HARQ transmission to UE. This indication triggers the UE to initiate an ARQ retransmission of the RLC PDUs contained in the failed TB.

Observation 9.  In DL, PUCCH errors and missed DCIs contribute to DL RLC errors requiring UE to correct using an RLC Status report, triggering an RLC retransmission at gNB.

Observation 10. At the UE, detecting certain HARQ events may indicate a potential DL RLC error. This detection can be faster than the typical duration of t-Reassembly timer.

Proposal 7.  	On Downlink, RAN2 to send an LS requesting RAN1 to investigate how the UE can best detect the following events with low latency:

- HARQ Nack-to-Ack error

- DTX-to-Ack error, i.e., due to missing DL DCI, UE does not transmit HARQ feedback while network detects DTX as ACK.

RAN2 may continue to discuss ARQ state-machine including signalling, retransmission triggers, etc. based on RAN1 progress.