3GPP TSG-RAN WG2 Meeting #132 	 R2-2508038

Dallas, USA, Nov. 17th - 21st, 2025

Agenda item:	9.7.2

Source:	vivo

Title:	Discussion on Support of IMS Voice over NB-IoT NTN

Document for:	Decision

# Introduction

In the RAN#108 meeting, a new WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4 was approved [1]. This WID was further revised in the RAN#109 meeting [2]. The objectives of this WID are listed as follows,

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives: - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

In the RAN2#131bis meeting, RAN2 has disccused three SA LSs regarding GEO voice and provided the reply LS to SA groups. The related Agreement are as followings [3].

Agreements regarding the reply LS to SA2:

1.	Regarding SA2 Q1, RAN2 is not able to make any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC, and thinks the implementation of ROHC functions (e.g., compression and decompression) falls outside of RAN2 scope

2.	Regarding SA2 Q3, RAN2 responds to SA2 that scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues.

3.	Regarding SA2 Q6, RAN2 responds to SA2 that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

4.	Regarding SA2 Q9 and Q10, RAN2 responds to SA2 that it is technically feasible to introduce SRB with RLC UM for voice packet

5.	We add a sentence in the same reply LS saying that majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary.

Agreements regarding the reply LS to SA4:

1.	RAN2 assumes the MAC header size may be 1 to 3 bytes. However, in most common scenario, the total MAC header size is likely to be 3 bytes.

2.	For transmitting voice packets via the UP solution RAN2 expects 1 byte for PDCP + 1 byte for RLC UM + the MAC header size

3.	For transmitting voice packets via the CP solution RAN2 expects 2 bytes for RRC + 2 bytes for RLC AM (the only mode currently specified for CP) or 1 byte for RLC UM (if RLC UM will be introduced for SRB in case a CP based solution will be selected) + the MAC header size

4.	RAN2 will provide further feedback on the expected average RoHC header size

5.	We indicate to SA4 that RAN2 has not started work on SPS for voice but thinks that SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 would require additional specification work in RAN2 to resolve the issues

In this contribution, we will focus on discussing the following three parts from the RAN2 perspective.

- Part 1: Down-selection between CP and UP solutions;

- Part 2: RAN Scheduling for packet with variable size;

- Part 3: ROHC header size;

# Discussion

## Down-selection between CP and UP solutions

SA2 has been working on supporting GEO voice over NB-IoT for several meetings, and the candidate solutions have been documented in SA2 TR 23.700-19. In SA2#171 meeting, the interim conclusion has been agreed as follows (see S2-2509293[4]):

The following interim conclusions for Key Issue #1 support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made: - The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

IMS voice service includes IMS signaling and voice packet. Currently, SA2 has agreed that the UP solution is adopted for voice packet transmission. However, no decision has been made regarding whether CP or UP solution should be used for IMS signaling transmission.

Observation 1: SA2 has agreed the UP solution is adopted for voice packet transmission while no decision has been made for IMS signalling transmission.

Below, relevant analysis and proposed solution for IMS signalling transmission will be provided from the RAN perspective.

NB-IoT supports both CP and UP solutions: the CP solution is mandatory while the UP solution is optional UE capability. Currently, SA2 has agreed the voice packet shall be transported over NB-IoT UP architecture. This means that any NB-IoT UE supporting GEO voice shall implement the UP solution, so the CP solution holds no advantage over the UP solution in terms of commercial deployment. Additionally, in TN design, IMS signalling and voice packet are both transmitted via DRBs, making it a spontaneous logic to reuse this TN design principle in NTN scenario.

Observation 2: Considering IMS signalling and voice packet are both transmitted via DRBs in TN, it is a spontaneous logic to reuse such design in NTN scenario, especially under the situation that UP solution has been adopted for GEO voice packet transmission.

If CP solution is adopted for IMS signalling transmission, RAN should discuss which SRB is used for this purpose. In NB-IoT, only SRB1bis is supported for RRC messages (may include a piggybacked NAS message carrying NB-IoT service data) as well as for NAS messages prior to the activation of AS security. Once AS security is activated, RRC messages shall be transmitted using SRB1. Considering IMS signalling has different QoS requirements with the legacy NB-IoT service and RRC message, new SRB for IMS signalling transmission should be introduced. This raises a subsequent question: when to establish this new SRB.  For example, if IMS signalling transmission is expected during RRC connection establishment procedure (e.g., via MSG5), negotiation between UE and RAN shall be performed for SRB establishment for IMS signalling during this process, which may require the UE to send an indication requesting SRB establishment. If IMS signalling transmission is performed after RRC connection establishment procedure, the establishment of new SRB can be triggered by CN, similar to how DRB establishment for IMS signaling is handled in TN. Additionally, in legacy design, CN only provides QoS information for DRB to RAN, while SRB management is governed by RAN policies. Thus, if SRB is adopted for IMS signalling transmission, RAN should discuss how to perform QoS management for SRB carrying IMS siganlling, which may need coordination with SA. In summary, adopting the CP solution (i.e., SRB) for IMS siganlling transmission imposes significantly more workload on the RAN compared to UP solution (i.e., DRB) for IMS signalling transmission.

Observation 3: If CP solution is adopted for IMS signalling transmission, more RAN works are needed in compared with UP solution for IMS siganlling transmission, e.g., introduce new SRB, and establishment and QoS management of such new SRB.

Proposal 1: From RAN2 perspective UP solution (i.e., via DRB) is adopted for both IMS signalling and voice packet transmission over NB-IoT via GEO.

According to the description of candidate solutions in SA2 TR 23.700-19, besides the traditional UP IP based soluiton, UP non-IP based solution is also proposed for voice packet transision. Specifically, the UE sends the voice packet with RTP info (e.g., 1 byte SN), whereas UDP/IP is removed considering static characteristics. When the PGW receives the uplink voice packet with non-IP type transmission, the PGW converts the voice packet with RTP info to voice data with a full RTP/UDP/IP header. It should be noted that the exact length of the reduced RTP header needs further discussion by SA2.Under UP non-IP based solution, NW shall be aware that the ROHC mechanism is not applied. To achieve this, explicit/implicit indication should be provided by CN, which depends on SA2 work. Considering the main work for UP non-IP solution is in the scope of SA2. It is proposed that whether/How to further down-select between UP non-IP and UP IP for voice packet is up to SA2.

Proposal 2: Whether/How to further down-select between UP non-IP and UP IP for voice packet is up to SA2.

## RAN Scheduling for variable size packet

In RAN2#131bis meeting, RAN2 has discussed the RAN scheduling for variable size packet, and achieved the following agreement.

Regarding SA2 Q3, RAN2 responds to SA2 that scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues.

During an IMS voice call, packet size may vary due to the talk-silence transition. In the TN, voice packet is generated every 20ms in talk period, while SID (Silence Insertion Descriptor) packet is generated every 160ms in scilence period. Typically in TN, the SPS resource are reserved with maximum expected packet size, e.g. SPS TBS is based on voice packet rather than silence packets. When entering the silence period, RAN will deactivate the SPS resource and use dynamic scheduling for SID packet. However, in the GEO scenario, dynamic scheduling is not suitable for periodic SID packets, considering the long propagation delay and limited radio resources. A feasible candidate solution is to adopt two separate SPS configurations, which are respectively tailored for packet transmission with different periodicity and packet sizes during the talk period and the silence period. Under this mechanism, RAN should discuss how to dynamic switch between two SPS configurations. For example, when SID packets with padding in MAC PDU via SPS resource for voice packet is received, RAN can determine that scilence period is entered and decide to activate the SPS for SID packets. When segemented voice packet via SPS resource for SID packet is received, RAN can determine that talk perido is entered and decide to activate the SPS for voice pacet. The detailed method for dynamic switch between two SPS configuration can be further discussed.

Proposal 3: For case that variable size packet due to talk-scilence transition, two SPS configurations can be candidate solution. FFS how to dynamic switch between two SPS configurations.

When UP IP based solution is adopted for voice packet transmission, packet size may also vary due to the ROHC state transition. In the TN, SPS resource are typically reserved based on the packet generated in ROHC SO state. When ROHC state transition occurs, RAN will schedule dynamic grant to accommodate the transmission of large packets, e.g. IR packet, or FO packet. Considering that ROHC state transition has no periodic characteristic, dynamic grant is also a feasible supplementary solution on the top of SPS resource in GEO scenario. However, this will bring additional 540ms transmission delay (e.g., BSR+ DCI) for voice packet. In TN, the delay requirement for voice packet is 200ms. It is not sure whether such delay requirement will be relaxed in GEO voice scenario, which should be coordinated with SA2.

Proposal 4: For case that variable-sized packet due to ROHC state transition, RAN2 sends an LS to SA2 for asking the delay requirement for voice packet in GEO scenario.

## ROHC header size

In RAN2#131bis meeting, RAN2 has discussed and responded to the SA4 LS on RAN simulation assumptions for ULBC. Regarding the protocol header size, RAN2 provides the following responses. The remaining unresolved issue is the length of ROHC header.

Question 1: the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option. RAN2 Answer: For transmitting voice packets via the UP solution RAN2 expects 1 byte for PDCP + 1 byte for RLC UM + the MAC header size. For transmitting voice packets via the CP solution RAN2 expects 2 bytes for RRC + 2 bytes for RLC AM (the only mode currently specified for CP) or 1 byte for RLC UM (if RLC UM will be introduced for SRB in case a CP based solution will be selected) + the MAC header size. RAN2 may provide further feedback on the expected average RoHC header size later.

For voice packet with RTP/UDP/IP, ROHC framework is used to perform compression/decompression of RTP/UDP/IP protocol header. The implementation of the functionality of the ROHC framework is specified in RFC 3095. According to the RFC 3095, a ROHC packet has the following general format as shown in Fig.1. Padding is any number (zero or more) of padding octets. Either of feedback or Header must be present in a ROHC packet. Header is either a profile-specific header or an IR/IR-DYN header.

Fig.1 General ROHC packet format

All Header packet type have the following general format shown in Fig.2.  The CID (i.e., ROHC context identifier) is sent along with Header packet type in order to maintain ROHC context  (i.e., which is associated with each compressed flow) between compressor and decompressor. Header either:1) does not carry any CID information (i.e., indicating CID=0, That means a small CID with the value 0 is represented using zero bits); 2) includes one Add-CID Octet (as shown in Fig.2); 3) contains embedded CID information of length one or two octets. For GEO voice scenario, it is assumed that no any CID information (i.e., assume CID=0) in Header packet considering that no multiple service flows are multiplexed in same DRB.

Fig.2 General format of Header packet type

Thus, ROHC packet can be classified into following three categories:

- ROHC feedback

  - Feedback carries information (e.g., ACK/NACK) from decompressor to compressor.

- IR/IR-DYN packet

  - IR/IR-DYN packet is used to initializes and refresh the ROHC context. The overall protocol header size is 40 Bytes if IPV4 is used. The overall protocol header size of IR-DYN depends on the length of dynamic field carrying in this packet, which is related to ROHC implementation.

- Profile-specific packet, i.e., carrying ROHC packet type 0/1/2

  - Packe type 0: R-0, R-0-CRC and UO-0. This, the minimal, packet type is used when parameters of all SN-functions are known by the decompressor. It can be understand that this kind of packet is used in ROHC SO sate.

  - Packe type 1: R-1, R-1-ID, R-1-TS, UO-1, U0-1-ID and UO-1-TS. This packet type is used when the parameters of the SN-fucntion for RTP TS or IP-ID change. It can be understand that this kind of packet is used in ROHC FO sate.

  - Packe type 2: UOR-2, UOR-2-ID, and UOR-2-TS. This packet type can be used to change the parameter of any SN-function. It can be understand that this kind of packet is used in ROHC FO sate.

Table.1 Summary of ROHC Pakcet type 0/1/2

| Packe type 0 | Packet type 0 in R-mode: 1~2 Bytes |
| --- | --- |
|  | Packet type 0 in U/O-mode: 1Byte |
| Packe type 1 | Packet type 1 in R-mode: 2 Bytes |
|  | Packet type 1 in U/O-mode: 2 Byte |
| Packe type 2 | Packet type 2 in U/O/R-mode: 3 Byte |

It can be found that ROHC header packet type 0/1/2 is 1 to 3 bytes. As mentioned above, there is no need to carry CID information (i.e., zero bits) in ROHC header packet type in GEO voice scenario. Thus, the Profile-specific packet (packet type 0/1/2) is 1 to 3 bytes. If UDP checksum is enabled, UDP checksum field cannot be compressed and should be sent along with each packet. Then, additional 2 Bytes should be considered. Moreover, as mentioned above, the IR/IR-DYN packet is used to initializes and refresh the ROHC context. That means IR/IR-DYN packet is used when ROHC context is not establish or is broken, which is not a frequent events‌. Thus, it is suggested to focus on the Profile-specific packet (packet type 0/1/2) and RAN2 can provide SA4 feedback that RAN2 assumes the ROHC header size may be 1 to 5 bytes (i.e, considering 2 Bytes UDP checksum).

Observation 4: When ROHC context is established or not broken, the ROHC header size may be 1 to 3 bytes without considering the 2 bytes UDP checksum (i.e., UDP checksum is disabled).

Proposal 5: RAN2 provides SA4 feedback that RAN2 assumes the ROHC header size may be 1 to 5 bytes.

# Conclusion

In this contribution, we have analyzed down-selection between CP soluition and UP solution and the remaining issue regarding SA LS. All the observations and proposals are listed below,

Dwon-selection between CP and UP solutions

Observation 1: SA2 has agreed the UP solution is adopted for voice packet transmission while no decision has been maded for IMS signalling transmission.

Observation 2: Considering IMS signalling and voice packet are both transmitted via DRBs in TN, it is logic to reuse such design in NTN scenario, especially under the situation that UP solution has been adopted for GEO voice packet transmission.

Observation 3: If CP solution is adopted for IMS signalling transmission, more RAN works are needed in compared with UP solution for IMS siganlling transmission, e.g., introduce new SRB, and establishment and QoS management of such new SRB.

Proposal 1: From RAN2 perspective UP solution (i.e., via DRB) is adopted for both IMS signalling and voice packet transmission over NB-IoT via GEO.

Proposal 2: Whether/How to further down-select between UP non-IP and UP IP for voice packet is up to SA2.

RAN scheduling for varible size packet

Proposal 3: For case that variable size packet due to talk-scilence transition, two SPS configurations can be candidate solution. FFS how to dynamic switch between two SPS configurations.

Proposal 4: For case that variable-sized packet due to ROHC state transition, RAN2 sends an LS to SA2 for asking the delay requirement for voice packet in GEO scenario.

ROHC header size

Observation 4: When ROHC context is established or not broken, the ROHC header size may be 1 to 3 bytes without considering the 2 Bytes UDP checksum (i.e., UDP checksum is disabled).

Proposal 5: RAN2 provides SA4 feedback that RAN2 assumes the ROHC header size may be 1 to 5 bytes.

# Reference

- RP-251867, New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4, vivo.

- RP-252473, Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4, vivo.

- RAN2#131bis chairman note.

- S2-2509293 Interim conclusions on KI#1 Support of Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC.

# Annex 1: Draft Reply LS to SA2

3GPP TSG RAN WG2 Meeting #132	R2-250xxxx

Dallas, USA,  Nov. 17th – 21st, 2025

Title:	Draft Reply LS on the support of IMS voice over NB-IoT NTN connected to EPC

Response to:	R2-2506747/S2-2507636

Release:	Release 20

Work Item:	IoT_NTN_Ph4-Core

Source:	vivo [to be RAN WG2]

To:	SA WG2,

Cc:	RAN WG1, CT WG1

Contact Person:

Name:	Xiaodong Yang

E-mail Address:	yangxiaodong5g@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN2 would like to thank SA2 for their LS on the support of IMS voice over NB-IoT NTN connected to EPC.

RAN2 has further discussed the following SA2 questions, and RAN2’s responses are as follows,

SA2 Q3: Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

RAN2 answer: RAN2 has further discussed the solution for handling voice packet with variable size due to ROHC state transition. RAN2 assume dynamic shcuedling is one feasible solution to handle this case. However, for UL, this will bring additional 540ms transimission delay. RAN2 kindly ask SA2 whether the delay requirement for voice packet can be satisfied under this case.

2. Actions:

To SA2:

ACTION: RAN2 kindly inform SA2 to take the above response into account and provide feedback if any.

3. Dates of Next RAN2 Meetings:

RAN2#133		  9th to 13rd Feb. 2026                 Gothenburg Metropolitan Area, SE

# Annex 2: Draft Reply LS to SA4

3GPP TSG RAN WG2 Meeting #132	R2-250xxxx

Dallas, USA,  Nov. 17th - 21st, 2025

Title:	Draft Reply LS on RAN Simulation Assumptions for ULBC

Response to:	R2-2505068/S4-251550

Release:		Release 20

Work Item:		IoT_NTN_Ph4-Core

Source:		vivo [to be RAN WG2]

To:		SA WG4,

Cc:		RAN WG1, CT WG1, SA WG2

Contact Person:

Name:	Xiaodong Yang

E-mail Address:	yangxiaodong5g@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	none

1. Overall Description:

RAN2 would like to thank SA4 for their LS on the RAN simulation assumptions for ULBC.

Regarding SA4 questions, RAN2 further discuss the ROHC header size and RAN2’s responses are as follows,

SA4 kindly asks SA2 and RAN2 to comment on

- the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and

RAN2 answer: RAN2 assumes the ROHC header size may be 1 to 5 bytes.

2. Actions:

To SA4:

ACTION: RAN2 kindly inform SA4 to take the above response into account.

3. Dates of Next RAN2 Meetings:

RAN2#133		  9th to 13rd Feb. 2026                   Gothenburg Metropolitan Area, SE