3GPP TSG RAN WG2 #132			    R2-2508064
Dallas, U.S., Nov 17th – 21st, 2025

Agenda item:		10.3.3.2

Title:						6G Radio: Energy efficiency by default

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion

# Introduction

The transition to 5G NR has led to remarkable improvements in capacity, latency, and flexibility, but it has also introduced new challenges regarding energy consumption. Compared to LTE, 5G NR requires a denser deployment of base stations, higher frequency bands, and advanced hardware like massive MIMO, all of which significantly increase the energy demand of the radio access network (RAN). Even under low traffic conditions, gNBs must remain partially active to ensure connectivity and support periodic signaling, resulting in significant baseline power usage. As energy costs and environmental concerns grow, it has become critical to explore techniques that allow the RAN to operate more efficiently without compromising performance. Among the techniques introduced in 3GPP Release 18 is cell DTX/DRX (Discontinuous Transmission/Reception), which enables a base station to periodically discontinue certain downlink transmissions at the cell level without impacting the transmission of common signals. Notably, the current standardization of cell DTX is targeted at the connected mode, leaving a gap in energy-saving strategies for idle-mode cells.

In parallel, Connected Mode Discontinuous Reception (C-DRX) has been a well-established mechanism, allowing UEs in RRC_CONNECTED to periodically power down their receivers and reduce their own energy consumption. While C-DRX and cell DTX serve complementary purposes, one on the UE side, the other on the gNB side, they have been standardized in different contexts and release cycles. For maximal RAN energy efficiency, these mechanisms must be coordinated in both design and operation, allowing the network and UE to enter low-power states without compromising synchronization or service quality.

# Cell DTX/DRX design for 6G

Cell DTX is one of the most critical mechanisms for improving the energy efficiency of the 6G radio access networks. By allowing the gNB to periodically mute downlink transmissions when there is no active traffic and power down certain elements and enter sleep modes, the system can significantly reduce its baseline power consumption, which would otherwise remain high due to continuous transmission of signals.

Observation 1: Cell DTX is a key mechanism for improving the energy efficiency of the RAN, allowing significant reductions in baseline power consumption.

Proposal 1: RAN2 to study enhancements to the cell DTX framework for 6G RAN as the technique can offer substantial energy savings.

However, one of the current limitations is that the concept has been standardized for the connected state only, without considering inactive and idle states. The gNB can only make extra energy savings if no signal is sent, regardless the RRC states of different UEs. Therefore, 6G basic design should consider cell DTX to operate seamlessly across all UE states, enabling the cell to enter sleep modes even when serving a mix of connected, inactive, and idle UEs. From a higher-level perspective, this can be realized using larger transmission frames or cycles, during which the gNB dynamically decides when to remain inactive (sleep mode) and when to transmit.

Observation 2: NR specifications of cell DTX address the RRC_CONNECTED state only, without considering UEs in RRC_INACTIVE and RRC_IDLE states. This limits the applicability of cell DTX, as the gNB does not fully track each UE’s RRC state during periods of low activity.

Proposal 2: RAN2 to study enhancements to the cell DTX framework enabling operation across RRC_CONNECTED, RRC_INACTIVE, and RRC_IDLE states, allowing cells to dynamically transit to sleep or active modes regardless of the UE state composition.

The on-duration within each cycle should be adaptive, so that the gNB can signal to the UEs when to listen for possible data or control information and when to skip monitoring to save energy. This dynamic signaling helps the network avoid unnecessary activations while maintaining responsiveness. To fully exploit this mechanism, Connected Mode Discontinuous Reception (C-DRX) must be closely coordinated with cell DTX, so that the UE only activates its receiver when the gNB is transmitting.

Proposal 3: RAN2 to investigate coordination mechanisms between cell DTX and C-DRX to ensure that gNB transmission activity and UE reception cycles are optimally aligned, minimizing unnecessary wake-ups and maximizing energy savings on both network and UE sides.

In 5G NR, one of the biggest limitations to significant energy savings is the periodic transmission of common signals, such as SSBs and SIBs, which are typically sent every 20 milliseconds. This fixed periodicity effectively limits how long a gNB can remain in a sleep state, as it must wake up at least once every cycle to transmit these signals, which are essential for network discovery and synchronization. As a result, the duty cycle of a 5G base station - that is the fraction of time it must remain active - cannot be reduced below this 20 ms granularity, restricting potential energy savings, especially during low-traffic periods.

Observation 3: The periodic transmission of common signals in 5G NR limits energy savings.

To overcome this constraint, 6G introduces the opportunity to operate with longer and more flexible frame structures, within which the gNB can dynamically determine its active and inactive transmission periods. By extending the frame duration, the network can define larger scheduling windows where energy-saving cycles (i.e., the sleep and active phases) can occur without disrupting synchronization or coverage. Within such a framework, the gNB could adopt an adaptive duty cycle, where the on-duration shortens or lengthens according to real-time load, service demand, or QoS requirements. This allows the base station to stay silent for extended intervals during low demand and wake up only when transmission is necessary. These long durations need to be designed in a way which handles gNB EE, UE EE and initial access. Such aspects are further discussed in our accompanying contribution [2].

Proposal 4: RAN2 to study longer transmission frames or cycles, within which the gNB dynamically determines active (on) and inactive (sleep) periods including flexible adaptation of the on-duration according to real-time load and traffic conditions.

# Conclusions

The following observations and conclusions have been made in this contribution:

Observation 1: Cell DTX is a key mechanism for improving the energy efficiency of the RAN, allowing significant reductions in baseline power consumption.

Proposal 1: RAN2 to study enhancements to the cell DTX framework for 6G RAN as the technique can offer substantial energy savings.

Observation 2: NR specifications of cell DTX address the RRC_CONNECTED state only, without considering UEs in RRC_INACTIVE and RRC_IDLE states. This limits the applicability of cell DTX, as the gNB does not fully track each UE’s RRC state during periods of low activity

Proposal 2: RAN2 to study enhancements to the cell DTX framework enabling operation across RRC_CONNECTED, RRC_INACTIVE, and RRC_IDLE states, allowing cells to dynamically transit to sleep or active regardless of the UE state composition.

Proposal 3: RAN2 to investigate coordination mechanisms between cell DTX and C-DRX to ensure that gNB transmission activity and UE reception cycles are optimally aligned, minimizing unnecessary wake-ups and maximizing energy savings on both network and UE sides.

Observation 3: The periodic transmission of common signals in 5G NR limits energy savings.

Proposal 4: RAN2 to study longer transmission frames or cycles, within which the gNB dynamically determines active (on) and inactive (sleep) periods including flexible adaptation of the on-duration according to real-time load and traffic conditions.

# References

- R1-2508477, “6G Study on Energy Efficiency”, Fraunhofer IIS, Fraunhofer HHI, 3GPP RAN1 Meeting #123 Dallas, U.S., November 17th – 21st, 2025.

- R2-2508061, “Discussion on 6G Radio Initial and System Access”, 3GPP RAN2 Meeting #132, Dallas, USA, November 17-21, 2025