3GPP TSG RAN WG2 Meeting #132         	                          R2-2508078

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.2.3

Source:	Xiaomi

Title:	General considerations on RAN2 6G design

Document for:	Discussion and Decision

# Introduction

During RAN2#131bis meeting, RAN2 discussed the design principles, services, architecture and requirements for 6GR. However further discussion/clarifications are still needed on:

Issue 1: What mandatory features should be considered from RAN2 perspective?

Issue 2: What design principle should be considered in RAN2 6G design, especially considering RAN requirements on Target scalable and forward compatible design for diverse device types?

Issue 3: What services should be considered/supported in the baseline design?

Issue 4: What E2E architecture should be used as baseline when we discuss features related to E2E architecture, e.g., AI, sensing, UP, mobility, paging, capability handling?

In this contribution, we discuss and provide our view on these aspects.

# Discussion

## 2.1 Supported feature/basic design for diverse device types

The support of diverse device types has been discussed in RANP, RAN1 and has been reflected in the SID[1] [2] as:

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] g) Target scalable and forward compatible design for diverse device types.

In RAN1#122 meeting, the following device type related agreements were achieved.

Agreement Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any Agreement - Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

In RAN#109, the support of diverse device types was discussed based on [3], with following proposals:

Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110. Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type The following proposal was also discussed, but didn’t reach consensus offline: Proposal 2: 6GR aims to specify a set of mandatory baseline functionalities for all UEs, with details to be further studied, by taking into account diverse device types for scalable and forward compatible design, and the requirements, as necessary, as in the 6G SID. Further study whether there is a need to mandate base stations for certain requirements/functionalities. Conclusion: proposal 1 is endorsed, proposal 3 & 4 are endorsed for RAN only (no WG discussion)

Based on RAN1/RANP discussion, all candidate factors to be considered for device type are related to physical layer or service, i.e., the higher layer factors are missing.

Issue 1: What mandatory features should be considered from RAN2 perspective?

We need to understand what use cases should be addressed in 6G for diverse device types, before discussing what mandatory features should be considered from RAN2 perspective. In table 1, we summarize the use cases and key factors of different device types defined in 4G and 5G.

Table 1: Use cases and key factors of different device type (4G/5G)

| Device type | Use cases | Peak date rate | Battery life | Coverage/Range | Cost |
| --- | --- | --- | --- | --- | --- |
| NBIOT-like | Meter reading | < 150kbps | 10 years | 164dB MCL/15km (+20dB vs. GPRS) | Very low |
| eMTC-like | Meter reading | < 1Mbps | 10 years | 156dB MCL/11km (+15dB vs. FDD LTE) | Low |
| RedCap-like | Industrial sensors | <2 Mbps; | Few years |  | Low |
|  | Video surveillance | 7.5-25 Mbps | - |  | Medium |
|  | Wearable | DL 150 Mbps, UL 50 Mbps | 1-2 weeks |  | Medium |
| NR |  | Gbps | - |  | High |

Note: RedCap like device/use cases should be considered as the lowest end device in 6G since eMTC/NB-IOT require individual system design based on the experience in 4G.

Based on above table, the key factors to be considered for different device type are

- Peak data rate

- Battery life

- Coverage

- Cost

These factors will impact the design of the basic supported features for diverse device type.

- The peak data rate and the cost requirements: 6G needs to consider the relevant impacts on Max BW, Rx/Tx antennas number, Duplex, MIMO, modulation, L2 buffer, advanced features, e.g., CA, DC, etc

- The battery life requirement: 6G needs to consider low power consumption in system design;

- Coverage requirement: 6G needs to consider coverage enhancement mechanisms in system design;

In table 2, we list the mandatory features for different device types in NR. Clearly, besides Max BW, Rx/Tx antennas number, Duplex, MIMO, modulation, higher layer related features, e.g., size of L2 SN, L2 buffer, number of DRBs, are also part of the mandatory features.

Table 2: mandatory features for different device types (NR)

| Mandate features | R15 NR | NR Redcap | NR eRedCap | Remark |
| --- | --- | --- | --- | --- |
| UE Rx/Tx antennas number | 2Rx FDD 4Rx TDD FR1 2Rx TDD FR2 | 1 or 2Rx FR1, 2Rx FR2 | 1 or 2Rx FR1 |  |
| Max. BW | 100 MHz FR1 400 MHz FR2 | 20MHz FR1 100 MHz FR2 | RF 20 MHz + BB 5MHz FR1 Not support FR2 |  |
| FDD-operation | Duplex FDD | Type A HD-FDD | Type A HD-FDD |  |
| Max. MIMO layer | 2 layers FDD FR1 4 layers TDD FR1 2 layers FR2 | FR1: 1 DL layer (if 1 Rx branch) 2 DL layers (if 2 Rx branches) FR2: 1 or 2 DL layers (2 Rx branches are always supported) | 1 DL layer (if 1 Rx branch) 2 DL layers (if 2 Rx branches) |  |
| Max. Modulation | DL 256QAM (mandatory) for R15 UE | DL 256QAM (optional) FR1 | DL 256QAM (optional) FR1 |  |
| Peak data rate | ~1Gbps | ~150Mbps | ~10Mbps |  |
| DRB number | 16 | 8 | 8 | DRB number is related to the supported QoS requirement which depends on the supported use case/traffic type |
| PDCP SN | 18 | 12 | 12 | PDCP/RLC SN size is related to the width of the PDCP/RLC sliding window size. 12bits can satisfy the IOT peak date rate requirement |
| RLC SN | 18 | 12 | 12 |  |
| L2 buffer size |  |  | Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction (3.2 ) DL/UL peak data rate of 10 Mbps corresponding to vLayers·Qm·f = 3.2. - If UE supporting this feature also indicates eRedCapNotReducedBB-BW-r18, this component is not applicable. | For most IOT applications, especially those do not require high data rates, the L2 buffer size can be reduced. |
| CA, MR-DC, DAPS, CPAC, IAB (i.e., the RedCap UE is not expected to act as IAB node), and NCR (i.e., the RedCap UE is not expected to act as NCR-MT) related UE features and corresponding capabilities are not supported by RedCap UEs. |  |  |  |  |

Proposal 1: In RAN2 6G study, RedCap and eMBB requirements should be used as baseline for RAN2 system design, i.e., peak data rate (10Mbps-Gbps), battery life (few years), coverage, and low cost.

Proposal 2: Regarding the minimum mandatory feature for diverse device types, higher layer related features, e.g., size of L2 SN, L2 buffer, number of DRBs need to be considered (besides Max BW, Rx/Tx antennas number, Duplex, MIMO, modulation).

Issue 2: What design principle should be considered in RAN2 6G design, especially considering RAN requirements on Target scalable and forward compatible design for diverse device types?

As described in [4], Internet-of-Things (IoT) is developed for many years and the relative branches of technologies have been evolving for several releases in 3GPP. In 4G LTE and 5G NR, IoT technologies includes NB-IoT, LTE MTC, LTE Cat. 1/1b/4, NR RedCap, NR eRedCap.

To support diverse device type, there are two approaches:

Approach 1: 6G basic system should be designed based on eMBB requirements, with additional enhancements for IoT

Approach 2: 6G basic system should be designed based on minimum requirements, i.e., IOT, with additional enhancements for eMBB

Approach 1 is exactly same as what we did in previous generations. In 4G and 5G, IoT technologies were designed from Day-2, which means that the basic system is firstly designed to support normal UE, and then the enhancement, such as complexity reduction, coverage enhancement and power consumption reduction, were designed based on the basic system to support IoT UE. This procedure can cause potential fragmentation and complication of the system design, and limitation of IoT design.

Lesson learnt from 5G (1): RedCap features were introduced at late stage (Rel-17, Rel-18) on top of the system designed for commercial UEs which led the complicated design for backward combability, and impacted the deployments of RedCap features.

It is important to take IoT device into account in 6G Day-1 design, targeting a common system design for both IoT and non-IoT devices.

Figure 1: Support of IoT devices

From extensible, future proof perspective, 6G should make a harmonized system supporting different device types, e.g., co-existence of normal UE and IoT device, i.e., different device types can access the same cell. To achieve this purpose, the basic system shall be designed based on the smallest maximum capability (mandatory) of the 6GR lowest-tier IoT device.

Proposal 3: RAN2 should aim to support diverse device types, including IoT devices, with single framework. The basic system shall be designed based on the smallest maximum capability (mandatory) of the 6GR lowest-tier IoT device.

For RRC_IDLE state, the network has no idea what the device can support. Therefore, the features in RRC_IDLE shall be common for IOT and eMBB devices, for instance:

- Cell search procedure

- System information handling

- Cell (re)selection

- Paging

- Initial access procedure

- UAC

- Coverage enhancement

- Energy saving

For RRC_CONNECTED state, the network has UE capability. But based on the design requirement [1][2], “a)	Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.”, common design is still preferred if possible. For instance,

- Connection (CP/UP)

- Data transmission, including L2 design (need to consider low cost and low peak data requirement)

- Security

- Network controlled mobility, including measurements

- Energy saving

However, the support of lowest end of IoT device in 6G shall not impact the performance of commercial UEs. Therefore the parameter range for the same feature may be different for IOT and eMBB devices, e.g., BW, size of PDCP SN.

Proposal 4: Following RAN2 basic system features shall be common for IOT and eMBB devices, taking into account Low IOT device requirements (low peak data rate, long battery life, low cost):

RRC_IDLE state:

- Cell search procedure

- System information handling

- Cell (re)selection

- Paging

- Initial access procedure

- UAC

- Coverage enhancement

- Energy saving

RRC_CONNECTED state:

- Connection (CP/UP)

- Data transmission, including L2 design (need to consider low cost and low peak data requirement)

- Security

- Network controlled mobility, including measurements

- Energy saving

Note: Parameter range may be different for IOT and eMBB devices.

## 2.2 Supported services in RAN2 design

So far, one of the main topics in RAN SI [1] is “requirements of new and existing services”. During RAN#109 meeting, based on [5], RANP discussed the supported services and the corresponding requirements.

Proposal 1: 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT Proposal 2: For 6G Massive Communication (IoT), the minimum number of UE receive/transmit antenna is 1 for lowest-tier device - Reflect this agreement in the device type section of the TR Moderator proposal 1: It is agreed to capture following into TR 38.914 section 5.4: - Mobile broadband - Immersive Communication - Massive Communication (IoT) - Sensing - AI - Voice - Regulatory services Moderator proposal 2: It is agreed that ‘Observability’ will be discussed in architecture requirements. Moderator proposal 3: Whether to capture following into TR 38.914 section 5.4 will be further discussed: - [Non-Terrestrial Network] - [Aerial] - [FWA] - [Vehicles] - [Mission Critical Communication] - [Positioning and/or Navigation] - [TSN] - [HRLLC] Moderator proposal 4: - 6GR should study the sensing modes, including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic. - 6GR should study cooperative sensing/multi-static sensing mechanism involving multiple UEs and TRPs. Conclusion: proposal 1 and 2, moderator's proposals 1, 2, 3 (without [ ]) and first bullet of moderator's proposal 4 are endorsed

As agreed in last RAN2 meeting, we should avoid the duplicated discussion in RAN2 and RAN.

- We should not duplicate discussions from RAN

Therefore, RAN2 should focus on the services have been agreed in RANP.

For the services Mobile broadband, Immersive Communication, Massive Communication (IoT), Sensing, AI, Voice, Regulatory services, most of them have been supported in NR, except sensing, new requirements for AI service and Immersive Communication. As agreed in last RAN2 meeting, all of these aspects have been considered in RAN2 study.

	RAN2 will start sensing study in April 2026

Agreements on UP

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

Proposal 5: Requirements of new and existing services shall only be discussed in RANP. RAN2 should focus on the services which have been agreed in RANP (Mobile broadband, Immersive Communication, Massive Communication (IoT), Sensing, AI,	Voice, Regulatory services). Requirements on new services, e.g., Digital twin, Non-Terrestrial Network, Aerial, FWA, Vehicles, Mission Critical Communication, Positioning and/or Navigation, TSN, HRLLC, shall be discussed in RANP.

## 2.3 Architecture assumption in RAN2 study

The design of some RAN2 features is related to E2E architecture. For instance, network AI data collection, sensing, mobility will be impacted by CN architecture, RAN architecture. Paging, capability handling may also be impacted by CN architecture. UP design may be impacted by RAN architecture.

So far, RAN3 is still discussing whether to support CU/DU split, CU-CP, CU-UP, and also P2P, SBI with CN. SA2 is also discussing the CN architecture, e.g., SBA, 5GC evolution or resolution, etc.

It is expected that other WGs may not have final conclusion before the completion of the SI. Therefore RAN2 can use general architecture, i.e., UE->RAN->CN as baseline when discussing AI data collection, sensing, mobility, paging, UP, capabilities, etc, without considering CU/DU split, P2P, SBI, new core.

Figure 2: Assumption on E2E architecture in 6G

Proposal 6: For features related to E2E architecture, e.g., network AI data collection, sensing, mobility, paging, UP, capability etc, general architecture, i.e., UE->RAN->CN is used as baseline for RAN2 study without repeating the discussion in other WGs on RAN architecture/CN architecture, e.g., CU/DU split, P2P, SBI, new core, etc.

# Conclusion

Based on the discussion, we have following proposals:

Proposal 1: In RAN2 6G study, RedCap and eMBB requirements should be used as baseline for RAN2 system design, i.e., peak data rate (10Mbps-Gbps), battery life (few years), coverage, and low cost.

Proposal 2: Regarding the minimum mandatory feature for diverse device types, higher layer related features, e.g., size of L2 SN, L2 buffer, number of DRBs need to be considered (besides Max BW, Rx/Tx antennas number, Duplex, MIMO, modulation).

Lesson learnt from 5G (1): RedCap features were introduced at late stage (Rel-17, Rel-18) on top of the system designed for commercial UEs which led the complicated design for backward combability, and impacted the deployments of RedCap features.

Proposal 3: RAN2 should aim to support diverse device types, including IoT devices, with single framework. The basic system shall be designed based on the smallest maximum capability (mandatory) of the 6GR lowest-tier IoT device.

Proposal 4: Following RAN2 basic system features shall be common for IOT and eMBB devices, taking into account Low IOT device requirements (low peak data rate, long battery life, low cost):

RRC_IDLE state:

- Cell search procedure

- System information handling

- Cell (re)selection

- Paging

- Initial access procedure

- UAC

- Coverage enhancement

- Energy saving

RRC_CONNECTED state:

- Connection (CP/UP)

- Data transmission, including L2 design (need to consider low cost and low peak data requirement)

- Security

- Network controlled mobility, including measurements

- Energy saving

Note: Parameter range may be different for IOT and eMBB devices.

Proposal 5: Requirements of new and existing services shall only be discussed in RANP. RAN2 should focus on the services which have been agreed in RANP (Mobile broadband, Immersive Communication, Massive Communication (IoT), Sensing, AI, Voice, Regulatory services). Requirements on new services, e.g., Digital twin, Non-Terrestrial Network, Aerial, FWA, Vehicles, Mission Critical Communication, Positioning and/or Navigation, TSN, HRLLC, shall be discussed in RANP.

Proposal 6: For features related to E2E architecture, e.g., network AI data collection, sensing, mobility, paging, UP, capability etc, general architecture, i.e., UE->RAN->CN is used as baseline for RAN2 study without repeating the discussion in other WGs on RAN architecture/CN architecture, e.g., CU/DU split, P2P, SBI, new core, etc.

# Reference

[1] RP‑251395, “New SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

[2] RP-252912, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator)

[3] RP-252873	Moderator's summary on 6G diverse device types (incl. minimum UE, Apple (moderator)

[4] RP-252197, Discussion on the requirements for IoT towards 6G, Xiaomi

[5] RP-252943	Moderator's summary for RAN led 6G SI: Requirements of new and existing services, CMCC