3GPP TSG RAN WG2 Meeting #132	R2-2508079

Dallas, USA, Nov. 17th - 21st, 2025

Agenda Item:	10.3.2.1

Source:	Xiaomi

Title:	Discussion on RRC states and functionalities

Document for:	Discussion and Decision

# Introduction

This contribution discusses the RRC states and basic RRC functionalities for 6G RRC protocol based on the agreement last meeting.

# Basic RRC functionalities for IDLE mode

For an IDLE mode UE, the main functionality is to ensure the NW reachability of the UE, as well as support UE autonomous mobility to ensure camping on an appropriate cell to perform timely access to the NW upon traffic arrival. Referring to the functionalities of RRC_IDLE in 5G NR in Annex 1, the following Table 1 summarizes the high-level basic RRC functionalities should be supported in 6G Day-1 RRC protocol for IDLE mode.

Table 1. Analyses on High-level functionalities for IDLE mode in 6G based on 5G NR RRC_IDLE [1]

| High-level functionality in 5G NR | Functionality description and/or intended UE behaviour | Basic functionality for 6G Day-1? (Y/N) |
| --- | --- | --- |
| Paging (with DRX monitoring) | CN paging with DRX based paging monitoring, | Y |
| System information | UE acquisition of system information broadcast including: - NAS common information, - Information applicable for UEs in RRC_IDLE (e.g. cell (re-)selection parameters, neighbouring cell information) - ETWS notification, CMAS notification | Y |
| Short message | UE monitors Short Messages transmitted with P-RNTI over DCI | Y |
| UE controlled mobility | UE performs neighboring cell measurements and cell (re-)selection; | Y |
| Access barring | Bar UE’s camping on a cell based on e.g. barring indication, UAC info, PLMN related info, UAC related info, etc. in system information | Y |
| MBS related CP functionalities | - If configured by upper layers for MBS multicast reception, monitors a Paging channel for CN paging using TMGI; - If configured by upper layers for MBS broadcast reception, acquires MCCH change notification and MBS broadcast control information and data; | N |
| Positioning specific system information | UE acquisition of the positioning assistance information | N |
| SL Relay related CP functionalities | Measurements on L2 U2N Relay UEs and relay (re-)selection; | N |
| SON/MDT related CP functionalities | logging of available measurements together with location and time for logged measurement configured UEs | N |

It is proposed that RAN2 agrees to support at least the high-level basic RRC functionalities for IDLE mode in 6G Day-1 as indicated in above Table 1; FFS the specific UE behaviour for each functionality.

Proposal 1: RAN2 agrees to support at least the high-level basic RRC functionalities for IDLE mode in 6G Day-1 RRC protocol as indicated in Table 1:

- Paging (including DRX based CN Paging monitoring by the UE);

- System information acquisition/update via broadcast signalling;

- Notification via Short Message;

- Access barring;

- UE controlled mobility (i.e. cell (re)selection) based on network configuration, including neighbour cell measurement for cell (re)selection.

FFS on the intended UE behaviours for each functionality.

# Basic RRC functionalities for CONNCETED mode

Taking the 5G NR RRC as the reference (in Annex 1), this section discusses the high-level RRC functionalities that may need be supported/studied in the 6G RRC protocol. Referring to the functionalities of RRC_CONNECTED in 5G NR, the following Table 2 summarizes the high-level basic RRC functionalities should be supported in 6G Day-1 RRC protocol for CONNECTED mode.

Table 2. Analyses on High-level functionalities for CONNECTED mode in 6G based on 5G NR RRC_CONNECTED [1]

| High-level functionality in 5G NR | Functionality description and/or intended UE behaviour | Basic functionality for 6G Day-1? (Y/N) |
| --- | --- | --- |
| System information | Including the system information acquisition based on broadcast signaling and based on dedicated signaling, | Y |
| Short message | UE monitors Short Messages transmitted with P-RNTI over DCI | Y |
| RRC Connection control | Can include at least: - Establishment/reestablishment/release of an RRC connection, - initial activation of AS security, - assignment/modification of UE identity - establishment/release of signalling radio bearers. - UAC | Y (NOTE: Connection suspension/resume is pending whether INACTIVE mode is supported or not. See Table 5) |
| RRC Connection reconfiguration | - Addition/modification/release of the dedicated signalling/data radio resource configuration/QoS control, etc. | Y |
| NW controlled Mobility management | Can include e.g.: - RRC connection mobility including e.g. intra-frequency and inter-frequency handover, associated AS security handling, i.e. key/algorithm change, specification of RRC context information transferred between network nodes - Inter-RAT mobility including e.g. AS security activation, transfer of RRC context information; | Y (Note: for inter-RAT mobility, support at least 5G-6G inter-RAT) |
| Measurement configuration and reporting | Can include e.g.: - Establishment/modification/release of measurement configuration (e.g. intra-frequency, inter-frequency and inter- RAT measurements); - Setup and release of measurement gaps; - Measurement reporting. | Y |
| Radio link monitoring | Detection and recovery from radio link failure for the RRC connection, e.g. RLM/RLF | Y |
| CA related CP functionalities | In case of CA, cell management including e.g. addition/modification/release of SCell(s); | Y (NOTE: Generally needed for career/spectrum aggregation function, pending whether CA framework is reused) |
| Other essential functionalities | Generic protocol error handling, transfer of dedicated NAS information, transfer of UE radio access capability information, etc. | Y |
| SL Relay related CP functionalities | - Path switch from a PCell to a target L2 U2N Relay UE or from a L2 U2N Relay UE to a target PCell or from a source L2 U2N Relay UE to a target L2 U2N Relay UE - In case of MP, path management including e.g. addition/modification/release of indirect path; - Configuration of SRAP entity and Uu/PC5 Relay RLC channels for the support of L2 U2N relay. - Configuration of SRAP entity and PC5 Relay RLC channels for the support of L2 U2U relay operation. | N |
| SON/MDT related CP functionalities | - Performs immediate MDT measurement together with available location reporting; | N |
| QoE related CP functionalities | Support of transfer of application layer measurement configuration and reporting. | N |
| NCR related CP functionalities | Configuration of side control information for NCR-node. | N |
| IAB related CP functionalities | Configuration of BAP entity and BH RLC channels for the support of IAB-node. | N |

It is proposed that RAN2 agrees to support at least the high-level basic RRC functionalities for CONNECTED mode in 6G Day-1 as indicated in above Table 2; FFS the specific UE behaviour for each functionality.

Proposal 2: RAN2 agrees to support at least the high-level basic RRC functionalities for CONNECTED mode in 6G RRC protocol as indicated in Table 2:

- System information acquisition via broadcast signalling or dedicated signalling;

- Notification via Short Message;

- RRC Connection Establishment/Reestablishment/Release (including initial AS security activation, SRB establishment, etc.);

- RRC Connection Reconfiguration;

- NW controlled mobility;

- Measurement configuration and reporting;

- Radio Link monitoring and failure recovery;

- Cell management for Carrier/Spectrum aggregation;

- Other basic CP functionalities, including at least generic protocol error handling, transfer of dedicated NAS information, transfer of UE radio access capability information.

FFS on the intended UE behaviours for each functionality.

# “INACTIVE” or “sub-state”?

Based on the agreement in last meeting [2], a key issue left for the RRC state discussion was whether in 6G there needs to be an RRC_INACTIVE state or it is likely to depend on a specific sub-state in e.g. RRC_CONNECTED state, in order to meet the low transition delay requirement of 20 ms as specified in [3] and simultaneously ensures low-power operations at the UE.

Agreements

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

Note that the 20 ms refers to the transition time from a “battery efficient” state to the start of "continuous data transfer" as defined in [3].

### 5.1.8 Control plane latency Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state). The minimum requirement for control plane latency is 20 ms.

In the rest of this section, how the fast transition and power-efficient UE operation in 5G NR RRC_INACTIVE is first reviewed. Then, based on the comparison between RRC_CONNECTED and RRC_INACTIVE in 5G NR from the functionality perspective, we discuss the possibility to the “fast transition”, “low-power operation” by RRC_CONNECTED state, and try to conclude the need of a separate RRC_INACTIVE state or not.

Note that as per the requirement in [3] cited above, the transition time cares only about how fast the UE can start the “continuous data transfer” from the “battery efficient” state. For an RRC_CONNECTED UE, such transition latency only concerns the latency for scheduling the first UL/DL data transmission when the UE currently has no on-going traffic (i.e. referring to the "No-traffic" state in the later discussion), as afterward the network has already been aware of the traffic arrival and its characteristics and can then perform the scheduling for continuous data transfer as needed. Specifically, for an RRC_CONNECTED UE in "No-traffic" state:

- For uplink, upon service initiated by the upper layers, it performs SR/BSR procedure to request UL resources or directly depend on the configured uplink grant for UL transmission.

- For downlink, it needs to monitor DL scheduling periodically from the NW, in order to receive the DL data scheduled upon service triggered by the NW. It is also assumed that the UE periodically monitor DL scheduling with C-DRX, in order to be notified on the scheduling for the first DL data arrival and balance the UE power consumption.

Therefore, the RRC_CONNECTED UE in "No-traffic" state concerned here is an RRC_CONNECTED UE "which is currently having no on-going UL or DL traffic and configured with power saving mechanism, e.g, monitoring the NW scheduling with C-DRX configured".

To facilitate further discussion, RAN2 should first have a clear definition on the meaning of "sub-state" which is proposed as follows.

Proposal 3: In RAN2 study, "sub-state" in RRC_CONNECTED refers to the "No-Traffic state" which refers to an RRC_CONNECTED UE "which is currently having no on-going UL or DL traffic and configured with power saving mechanism, e.g, monitoring the NW scheduling with C-DRX configured".

## Experience/Lesson from 5G NR

On Transition latency

Taking 5G NR as the starting point, below Table 3 compares the transition latency needed by an RRC_INACTIVE UE and RRC_CONNECTED UE in "No-traffic" state.

Table 3. Comparison on transition latency between an RRC_INACTIVE UE and an RRC_CONNECTED UE in “No-Traffic” state

| RRC states | Definition of “Transition” | Transition Latency Components |  |
| --- | --- | --- | --- |
|  |  | For UL data arrival | For DL data arrival |
| RRC_ INACTIVE UE | RRC_INACTIVE RRC_CONNCTED (CP State transition) | - Latency due to “RRC Resume” procedure; - Latency due to “UE AS context Fetch” between gNBs, in the case of serving gNB change; | - Additional delay due to “RAN paging monitoring” (i.e. ~ one paging cycle). - Total transition latency for UL data arrival (as in the left-hand side). |
| RRC_CONNECTED UE with “no on-going traffic” | “No-Traffic” state “Continuous Data transfer” state (No state transition) | - Latency due to UL scheduling e.g. SR/RACH and/or BSR; - Latency due to CONNECTED mobility, if the UL data arrival happens during HO/LTM cell switch. | - Waiting time for PDCCH monitoring due to C-DRX cycle - Latency due to CONNECTED mobility, if DL data just arrives during HO/LTM cell switch. |

Based on the table, some observations can be derived as follows:

- State transition in RRC_INACTIVE (RRC_INACTIVE to RRC_CONNECTED):

  - It should be noted that the RRC_INACTIVE actually can meet the targeted 20 ms transition latency only at some very special occasions for UL data arrival, e.g. when the last serving gNB is not changed, as otherwise the UE AS context fetch among gNBs may introduce delay already exceeding 20 ms delay bound. For DL data arrival, it is almost impossible to reach this 20 ms latency, as the delay due to RAN paging monitoring (with a Paging cycle of hundreds of milliseconds) is already much larger than 20 ms.

Observation 1: The state transition latency from RRC_INACTIVE to RRC_CONNECTED (which enables the continuous data transfer) cannot meet 20 ms latency in most cases, e.g. DL data arrival, last serving gNB change due to UE mobility, etc.

- State transition in RRC_CONNECTED ("No-Traffic" state to "continuous data transfer"):

  - In case the of UL/DL data arrival w/o UE mobility, the transition latency needed for the UL data arrival is obviously shorter than that needed in RRC_INACTIVE, as there is no need for the CP procedures for state transition. For DL data arrival, it is seen that the C-DRX cycle for an RRC_CONNECTED UE can share similar value as RAN paging cycle [1], whereas the onDuration time can be set to include a similar number of PDCCH occasions to be monitored as paging occasions as to RRC_INACTIVE. Therefore, with C-DRX being configured to afford similar PDCCH occasions as RAN paging (with power saving consideration), the transition delay due to DL data arrival in RRC_CONNECTED is at least no worse than that due to RAN paging in RRC_INACTIVE.

  - Even if we consider UE mobility, the latency due to HO/LTM cell switch can be much shorter than the total latency of “RRC Resume” + “UE AS context Fetch” in RRC_CONNECTED. Also, there is more probability for UE AS Context Fetch in RRC_INECTIVE than the mobility in CONNECTED, leading more latency needed by the RRC_INACTIVE. So, total transition latency due to UL/Data arrival in RRC_CONNECTED is much shorter than the state transition time in RRC_INACTIVE.

Observation 2: In 5G NR, compared with the state transition latency from "RRC_INACTIVE": transition delay from "No-Traffic" state to "continuous data transfer" state for an RRC_CONNECTED UE can be much shorter upon UL data arrival, and is at least no worse for DL data arrival (even with C-DRX configured for power saving).

On UE Power Consumption

Below Table 4 compares the power consumption an RRC_ONNECTED UE in “No traffic” state to that needed by an RRC_INACTIVE. Based on Table 4 the following lessons/observations can be derived from 5G NR:

    - The power consumption to monitor PDCCH for the RAN scheduling of the DL data arrival can be similar between the two, by configuring similar DRX cycles/PDCCH occasions for C-DRX and Paging DRX;

    - For the power consumption needed by the measurement and reporting:

- How frequently a measurement on an intra-/inter-frequency, upon being triggered, needs to be performed is dependent on the DRX cycle, and is possibly made into similar levels for both RRC_INACTIVE and RRC_CONNECTED UEs.

- For measurements in RRC_CONNECTED, the NW can depend on the S-measure related configuration to control the trigger of measurements, and possibly enable the CONNECTED measurements on each configured frequency to be triggered as infrequently as that for RRC_INACTIVE measurement, if the NW wants (e.g., being triggered only upon serving cell link degradation to some extent).

- NW can control how frequently the UE measurement reporting needs to be performed, and it is possible for the report to be made when the UE only approaches the cell border, thus reducing the power consumption needed for measurement reporting signalling. Also, by applying the Network controlled UE based mobility (e.g. CHO/C-LTM), the power consumption of measurement reporting can be further reduced.

    - The power consumption due to UE access to the target cell during mobility is the extra component for the power consumption in RRC_CONNECTED, but is needed to avoid resource wastage that the NW keeps reserving resources in target cells without awareness of when UE really comes.

Observation 3: In 5G NR, for an RRC_CONNECTED UE in “No-traffic” state, the power consumption stems mainly from (1) PDCCH monitoring for DL data arrival, (2) measurement and reporting and (3) UE access to target cell during CONNECTED mobility.

- For power consumption from (1), it may be made similar to paging monitoring in RRC_INACTIVE, by setting a C-DRX cycle similar to RAN Paging cycle.

- For power consumption from (2), it may be aligned with RRC_INACTIVE measurement to a similar level, with NW setting similar DRX cycle for C-DRX and Paging and controlling the trigger for measurements in CONNECTED, e.g. via S-measure (which can be subject to other RRM requirements by RAN4). Also, the power consumption for measurement reporting part can be even reduced by applying the NW controlled UE based mobility (e.g. CHO/C-LTM).

Table 4. Comparison on Power consumption between an RRC_INACTIVE UE and an RRC_CONNECTED UE in “no-traffic” state (NOTE 0)

|  | Power Consumption Components (Total power = sum of the components) | Factors that impact the related power consumption components |
| --- | --- | --- |
| RRC_ INACTIVE UE | Component 1: Power consumed for RAN paging monitoring for DL data | Paging DRX Config (e.g. RAN paging for INACTIVE): - Paging cycle (several hundreds to thousands of milliseconds [1]) - UE specific Paging occasions. |
|  | Component 2: Power consumed for Measurement and INACTIVE mobility | Factor 1: Trigger of per carrier measurement - Always trigger for higher-priority frequency - Only triggered upon serving cell link degradation for equal-/lower-priority frequency |
|  |  | Factor 2: RRM requirements on measurement interval/period, if triggered (w.r.t. Paging DRX cycle): - For serving cell, measurement required at least once per 1 or 2 DRX cycle ([4], subclause 4.2.2.2) - For intra-frequency, measurement required every several (e.g. 1, 2, or 4) DRX cycles ([4], subclause 4.2.2.3) - For inter-frequency, measurement required every several (e.g. 1, 2, or 6) DRX cycles ([4], subclause 4.2.2.4): - for higher-priority frequencies; - for equal-priority frequency or lower priority frequency, only if triggered ([4], subclause 4.2.2.4). (NOTE 1) |
| RRC_CONNECTED UE with “no on-going traffic” | Component 1: Power consumption for PDCCH monitoring for DL traffic arrival | C-DRX config: - C-DRX cycle (can share similar values as Paging cycle, e.g. 320 ms) - OnDuration time |
|  | Component 2: Power consumed for Measurement and Reporting | Factor 1: Trigger of per carrier measurement (e.g. S-measure) |
|  |  | Factor 2: RRM Requirements on measurement interval/period, if triggered (w.r.t. C-DRX cycle): - Several times of C-DRX cycles for the intra-frequency, with or w/o gap ([4], subclause 9.2.6.3, 9.2.5.2) - Several times of C-DRX cycles for the inter-frequency, with or w/o gap ([4], subclause 9.3.9.2, 9.3.5]) (NOTE 2) |
|  |  | Factor 3: Trigger of UL measurement reporting: - Reporting period - Reporting Events |
|  | Component 3: Power consumed for UE access to target cell during CONNECTED mobility | Channel/Signal to be used for UE access (e.g. RACH based vs. RACH less, etc.) |
| NOTE 0: The table takes FR1 as an example for analyses. NOTE 1: See Annex 2 for related citation from TS 38.133. For simplicity, the discussion is based on the DRX cycle for paging and w/o HST related features considered. NOTE 2: See Annex 3 for related citation from TS 38.133. Consider SSB based measurement to be comparable with Inactive measurement. |  |  |

## Proposals to 6G RRC state study on “INACTIVE”

As to 6G RRC state discussion, inspired by the above experience/observations from 5G NR in 4.1, we may consider the possibility to depend on CONNECTED mode to realize the “fast transition” and “lower-power operation” targets. If this is possible, then another INACTIVE mode is not necessarily defined. Specifically, RAN2 should focus on a CONNECTED UE in a "No-traffic" state which is defined as in Proposal 3. Since it is in CONNECTED, such a CONNECTED UE in "No-traffic" state should take the high-level basic functionalities for CONNECTED UEs as in above Proposal 2.

Then, the study on how to enable the fast transition and low power operation can be carried out for such a CONNECTED UE in a "No-traffic" state based on the above observations in 4.1. From 4.1, what can be observed are as follows:

- Regarding the support of fast transition, since 6G will specify advanced scheduling mechanism enabling lower-latency data transfer than 5G NR, the latency from “No-traffic” state to “continuous data transfer” state by an 6G CONNECTED UE must be even shorter than that in 5G NR and thus even much better than the state transition delay than 5G NR INACTIVE. So, there is definitely no problem for an 6G CONNECTED UE to support the 20-ms transition delay required by [2] (especially in case 5G NR INACTIVE was already regarded as sufficient to satisfy this requirement).

Observation 4: 6G is likely to support scheduling for data transfer with lower-latency than 5G NR. Thus the transition latency from “No-traffic” state to “continuous data transfer” state by an 6G CONNECTED UE must be much better than 5G NR RRC_INACTIVE state transition time (that was regarded as sufficient to support the 20 ms transition delay).

- Regarding support of low power consumption, 6G may further study whether/how to enable a low-power consumption operation for an CONNECTED UE in “No-traffic” state from below perspectives:

  - Whereas the support of DRX can largely save the power consumed for PDCCH monitoring and measurements for a UE in CONNCTED, further consideration may need to be taken on other possible RRM requirements that impact the CONNECTED measurement power consumption. This needs coordination with RAN4.

  - In RAN1, there is on-going discussions on the support of WUS in 6G: if there can be well-performed WUS mechanism without mandatory high hardware requirement, it is also helpful to save CONNECTED power consumption.

  - For the measurement reporting, it is worth considering if it is more possible to rely on the NW-controlled UE-based mobility mechanism to reduce the power consumption and signalling overhead needed for reporting.

  - Also, consideration can be made to reduce the power consumption of UE transmit power for access notification during handover/LTM cell switch

Therefore, it is proposed that RAN2 study the feasibility and possible enhancements to enable the low-power operation for a CONNECTED UE with No on-going traffic, to see whether the target of low-latency, low power operation has already been able to be achieved by the CONNECTED state in 6G.

Proposal 4: A CONNECTED UE in "No-traffic" state supports the high-level basic functionalities of RRC_CONNECTED as in Proposal 2.

Proposal 5: Study how to enable a CONNECTED UE in "No-Traffic" state with low power consumption compatible to 5G RRC_INACTIVE, considering the aspects/mechanisms that (at least) have impact on:

- Power consumption for monitoring of scheduling, especially for DL data arrival, e.g. DRX, WUS, etc.;

- Power consumption for measurement and reporting, e.g. DRX/WUS, RRM requirements, trigger of measurements, reporting reduction by NW controlled UE based mobility, etc.

- Power consumption for UE access during CONNECTED mobility, e.g. channel/signal used for UE access.

If RAN2, based on the study, concludes that a low-power operation for a CONNECTED UE is feasible and sufficient for the low transition latency, low power operation, a separate INACTIVE mode is not needed in 6G. Otherwise, a separate "INACTIVE" state still needs to be considered. In case RAN2 finally decides to still support a separate “INACTIVE” state similar in 6G, it is necessary to study the potential enhancements on INACTIVE mode, taking 5G NR RRC_INACTIVE operations as a baseline.

Below Table 5 summarizes the high-level functionalities of 5G NR RRC_INACTIVE state based on subclauses 4.2.1/4.4 in [1] (cited in Annex 1,), and further shows our views on which high-level RRC functionalities for INACTIVE mode, if supported, are the basic ones that should be supported in 6G Day-1 RRC protocol.

Table 5. Analyses on High-level functionalities for “IANCTIVE” mode in 6G (if supported) based on 5G NR RRC_INACTIVE [1]

| High-level functionality in 5G NR | Functionality description and/or intended UE behaviour | Basic functionality for 6G Day-1? (Y/N) |
| --- | --- | --- |
| Paging (with DRX monitoring) | RAN paging and CN paging with DRX based paging monitoring, | Y with RAN paging FFS (See below Proposal 3) |
| UE context storage and connection suspension/resume | Storage of UE Inactive AS context at both UE and NW sides, RRC connection suspension with suspendConfig and RRC resume procedure to resume the connection. | Y |
| System information | UE acquisition of system information broadcast including: - NAS common information, - Information applicable for UEs in RRC_INACTIVE (e.g. cell (re-)selection parameters, neighbouring cell information) - ETWS notification, CMAS notification | Y |
| Short message | UE monitors Short Messages transmitted with P-RNTI over DCI | Y |
| UE controlled mobility | UE performs neighboring cell measurements and cell (re-)selection; | Y |
| Access barring | Bar UE’s camping on a cell based on e.g. barring indication, UAC info, PLMN related info, etc. in system information | Y |
| RAN based notification area update | UE Performs RAN-based notification area updates periodically and when moving outside the configured RAN-based notification area | FFS (See below proposal 7) |
| MBS related CP functionalities | - If configured by upper layers for MBS multicast reception, while SDT procedure is not ongoing, UE monitors a Paging channel for paging using TMGI; - If configured by upper layers for MBS broadcast reception, UE acquires MCCH change notification and MBS broadcast control information and data; - If configured for MBS multicast reception in RRC_INACTIVE, UE acquires multicast MCCH change notification and MBS multicast control information and data; | N |
| Positioning specific system information | UE acquisition of the positioning assistance information | N |
| SDT related CP functionalities | - While T319a is running, monitors control channels associated with the shared data channel to determine if data is scheduled for it; - While SDT procedure is ongoing and T319a is not running, if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI except if the UE is acting as a L2 U2N Remote UE; | N |
| SL Relay related CP functionalities | Measurements on L2 U2N Relay UEs and relay (re-)selection; | N |
| SON/MDT related CP functionalities | logging of available measurements together with location and time for logged measurement configured UEs | N |

On top of the above basic high-level functionalities, 6G should study how to overcome the pain points that prevents the 5G NR RRC_INACTIVE being deployed in the practice. Specially, with the lack of interests in deployment on Xn interface, the UE AS context fetch across gNBs is usually not achievable, which limits the applicability of the RRC_INACTIVE state [5][6][7]. The RNA planning is complicated and the RNA update mechanism also leads to excessive signalling overhead. At the UE side, support of both RAN paging and CN paging also leads to extra UE implementation complexity.

Therefore, it is proposed that if RAN2 decides to support a separate INACTIVE mode, at least the high-level basic RRC functionalities as indicated in above Table 5 should be supported in 6G Day-1 RRC protocol, with the specific UE behaviours to be FFS. RAN2 also study an enhanced "INACTIVE" state in 6G to extend its applicability, compared with 5G NR RRC_INACTIVE, with the enhancement ease implementation at both NW side (e.g. relaxing dependency on Xn deployment, simplified location area management/planning, etc.) and UE side (e.g. simplified paging mechanism, access procedure, etc.)

Proposal 6: Separate INACTIVE state is needed, only if the low-power operation for CONNECTED UE with "No-Traffic" state by Proposal 3/5 cannot be justified. If INACTIVE state is regarded as needed, RAN2 agrees to support at least the high-level basic RRC functionalities for INACTIVE mode in 6G Day-1 RRC protocol as indicated in Table 5:

- Paging, including DRX based paging monitoring

- System information acquisition via broadcast signalling;

- Notification via Short Message;

- Access barring;

- UE controlled mobility based on network configuration, including neighbour cell measurement and cell (re)selection.

- UE Inactive AS context storage at NW and UE with connection suspension/resume.

FFS the specific UE behaviours for each functionality.

Proposal 7: Study enhancements on INACTIVE mode in 6G to extend its applicability (than 5G NR RR_INACTIVE), with the aim to ease both NW implementation (e.g. relaxing/removing dependency on Xn deployment, simplified location area management/planning, etc.) and UE implementation (e.g. simplified paging mechanism, access/resume procedure, etc.).

# Conclusion

This contribution discusses RRC states and basic RRC functionalities for 6G. Proposals are as follows.

Proposal 1: RAN2 agrees to support at least the high-level basic RRC functionalities for IDLE mode in 6G Day-1 RRC protocol as indicated in Table 1:

- Paging (including DRX based CN Paging monitoring by the UE);

- System information acquisition/update via broadcast signalling;

- Notification via Short Message;

- Access barring;

- UE controlled mobility (i.e. cell (re)selection) based on network configuration, including neighbour cell measurement for cell (re)selection.

FFS on the intended UE behaviours for each functionality.

Proposal 2: RAN2 agrees to support at least the high-level basic RRC functionalities for CONNECTED mode in 6G RRC protocol as indicated in Table 2:

- System information acquisition via broadcast signalling or dedicated signalling;

- Notification via Short Message;

- RRC Connection Establishment/Reestablishment/Release (including initial AS security activation, SRB establishment, etc.);

- RRC Connection Reconfiguration;

- NW controlled mobility;

- Measurement configuration and reporting;

- Radio Link monitoring and failure recovery;

- Cell management for Carrier/Spectrum aggregation;

- Other basic CP functionalities, including at least generic protocol error handling, transfer of dedicated NAS information, transfer of UE radio access capability information.

FFS on the intended UE behaviours for each functionality.

Proposal 3: In RAN2 study, "sub-state" in RRC_CONNECTED refers to the "No-Traffic state" which refers to an RRC_CONNECTED UE "which is currently having no on-going UL or DL traffic and configured with power saving mechanism, e.g, monitoring the NW scheduling with C-DRX configured".

Proposal 4: A CONNECTED UE in "No-Traffic" state supports the high-level basic functionalities of RRC_CONNECTED as in Proposal 2.

Proposal 5: Study how to enable a CONNECTED UE in "No-Traffic" state with low power consumption compatible to 5G RRC_INACTIVE, considering the aspects/mechanisms that (at least) have impact on:

- Power consumption for monitoring of scheduling, especially for DL data arrival, e.g. DRX, WUS, etc.;

- Power consumption for measurement and reporting, e.g. DRX/WUS, RRM requirements, trigger of measurements, reporting reduction by NW controlled UE based mobility, etc.

- Power consumption for UE access during CONNECTED mobility, e.g. channel/signal used for UE access.

Proposal 6: Separate INACTIVE state is needed, only if the low-power operation for CONNECTED UE with "No-Traffic" state by Proposal 3/5 cannot be justified. If INACTIVE state is regarded as needed, RAN2 agrees to support at least the high-level basic RRC functionalities for INACTIVE mode in 6G Day-1 RRC protocol as indicated in Table 5:

- Paging, including DRX based paging monitoring

- System information acquisition via broadcast signalling;

- Notification via Short Message;

- Access barring;

- UE controlled mobility based on network configuration, including neighbour cell measurement and cell (re)selection.

- UE Inactive AS context storage at NW and UE with connection suspension/resume.

FFS the specific UE behaviours for each functionality.

Proposal 7: Study enhancements on INACTIVE mode in 6G to extend its applicability (than 5G NR RR_INACTIVE), with the aim to ease both NW implementation (e.g. relaxing/removing dependency on Xn deployment, simplified location area management/planning, etc.) and UE implementation (e.g. simplified paging mechanism, access/resume procedure, etc.).

# Reference

[1]	TS 38.331

[2]	RAN2#131bis Meeting Minutes

[3]	TR 38.391

[4]	TS 38.133

[5]	R2-2507072	Controlling the 6G access stratum

[6]	R2-2507172	Views on Control Plane for 6G

[7]	R2-2507232	Considerations for 6G Control Plane

# Annex 1: 5G NR RRC functionalities and UE behaviours [1]

### 4.2.1 UE states and state transitions including inter RAT A UE is either in RRC_CONNECTED state or in RRC_INACTIVE state when an RRC connection has been established. If this is not the case, i.e. no RRC connection is established, the UE is in RRC_IDLE state. The RRC states can further be characterised as follows: - RRC_IDLE: - A UE specific DRX may be configured by upper layers; - At lower layers, the UE may be configured with a DRX for PTM transmission of MBS broadcast; - UE controlled mobility based on network configuration; - The UE: - Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5); - Monitors a Paging channel for CN paging using 5G-S-TMSI, except if the UE is acting as a L2 U2N Remote UE; - If configured by upper layers for MBS multicast reception, monitors a Paging channel for CN paging using TMGI; - Performs neighbouring cell measurements and cell (re-)selection; - Performs measurements on L2 U2N Relay UEs and relay (re-)selection; - Acquires system information and can send SI request (if configured); - Performs logging of available measurements together with location and time for logged measurement configured UEs; - Performs idle/inactive measurements for idle/inactive measurement configured UEs; - If configured by upper layers for MBS broadcast reception, acquires MCCH change notification and MBS broadcast control information and data. - RRC_INACTIVE: - A UE specific DRX may be configured by upper layers or by RRC layer; - At lower layers, the UE may be configured with a DRX for PTM transmission of MBS broadcast and/or a DRX for PTM transmission of MBS multicast; - UE controlled mobility based on network configuration; - The UE stores the UE Inactive AS context; - A RAN-based notification area is configured by RRC layer; - Transfer of unicast data and/or signalling to/from UE over radio bearers configured for SDT. - The UE: - Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5); - While T319a is running, monitors control channels associated with the shared data channel to determine if data is scheduled for it; - While SDT procedure is ongoing and T319a is not running, if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI except if the UE is acting as a L2 U2N Remote UE; - While SDT procedure is not ongoing, monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI, except if the UE is acting as a L2 U2N Remote UE; - If configured by upper layers for MBS multicast reception, while SDT procedure is not ongoing, monitors a Paging channel for paging using TMGI; - Performs neighbouring cell measurements and cell (re-)selection; - Performs measurements on L2 U2N Relay UEs and relay (re-)selection; - Performs RAN-based notification area updates periodically and when moving outside the configured RAN-based notification area; - Acquires system information and, while SDT procedure is not ongoing, can send SI request (if configured); - While SDT procedure is not ongoing, performs logging of available measurements together with location and time for logged measurement configured UEs; - While SDT procedure is not ongoing, performs idle/inactive measurements for idle/inactive measurement configured UEs; - If configured by upper layers for MBS broadcast reception, acquires MCCH change notification and MBS broadcast control information and data; - If configured for MBS multicast reception in RRC_INACTIVE, acquires multicast MCCH change notification and MBS multicast control information and data; - Transmits SRS for Positioning. - RRC_CONNECTED: - The UE stores the AS context; - Transfer of unicast data to/from UE; - Transfer of MBS multicast data to UE; - At lower layers, the UE may be configured with a UE specific DRX; - At lower layers, the UE may be configured with a DRX for PTM transmission of MBS broadcast and/or a DRX for MBS multicast; - At lower layers, the UE may be configured with a cell specific cell DTX/DRX; - For UEs supporting CA, use of one or more SCells, aggregated with the SpCell, for increased bandwidth; - For UEs supporting DC, use of one SCG, aggregated with the MCG, for increased bandwidth; - Network controlled mobility within NR, to/from E-UTRA, and to UTRA-FDD; - Network controlled mobility (path switch) between a serving cell and a L2 U2N Relay UE, or vice versa, or between a source L2 U2N Relay UE and a target L2 U2N Relay UE; - Network controlled MP operation. - The UE: - Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5), if configured; - Monitors control channels associated with the shared data channel to determine if data is scheduled for it; - Provides channel quality and feedback information; - Performs neighbouring cell and/or L2 U2N relay measurements and measurement reporting; - Acquires system information; - Performs immediate MDT measurement together with available location reporting; - If configured by upper layers for MBS broadcast reception, acquires MCCH change notification and MBS broadcast control information and data. ## […] ## 4.4 Functions The RRC protocol includes the following main functions: - Broadcast of system information: - Including NAS common information; - Information applicable for UEs in RRC_IDLE and RRC_INACTIVE (e.g. cell (re-)selection parameters, neighbouring cell information) and information (also) applicable for UEs in RRC_CONNECTED (e.g. common channel configuration information); - Including ETWS notification, CMAS notification; - Including positioning assistance data. - RRC connection control: - Paging; - Establishment/modification/suspension/resumption/release of RRC connection, including e.g. assignment/modification of UE identity (C-RNTI, fullI-RNTI, etc.), establishment/modification/suspension/resumption/release of SRBs (except for SRB0); - Access barring; - Initial AS security activation, i.e. initial configuration of AS integrity protection (SRBs, DRBs) and AS ciphering (SRBs, DRBs); - RRC connection mobility including e.g. intra-frequency and inter-frequency handover, path switch from a PCell to a target L2 U2N Relay UE or from a L2 U2N Relay UE to a target PCell or from a source L2 U2N Relay UE to a target L2 U2N Relay UE, associated AS security handling, i.e. key/algorithm change, specification of RRC context information transferred between network nodes; - Establishment/modification/suspension/resumption/release of RBs carrying user data (DRBs/MRBs); - Radio configuration control including e.g. assignment/modification of ARQ configuration, HARQ configuration, DRX configuration; - In case of DC, cell management including e.g. change of PSCell, addition/modification/release of SCG cell(s); - In case of CA, cell management including e.g. addition/modification/release of SCell(s); - In case of MP, path management including e.g. addition/modification/release of indirect path; - QoS control including assignment/ modification of semi-persistent scheduling (SPS) configuration and configured grant configuration for DL and UL respectively, assignment/ modification of parameters for UL rate control in the UE, i.e. allocation of a priority and a prioritised bit rate (PBR) for each RB of UE and logical channel of IAB-MT. - Recovery from radio link failure. - Inter-RAT mobility including e.g. AS security activation, transfer of RRC context information; - Measurement configuration and reporting: - Establishment/modification/release of measurement configuration (e.g. intra-frequency, inter-frequency and inter- RAT measurements); - Setup and release of measurement gaps; - Measurement reporting. - Configuration of BAP entity and BH RLC channels for the support of IAB-node. - Configuration of SRAP entity and Uu/PC5 Relay RLC channels for the support of L2 U2N relay. - Configuration of SRAP entity and PC5 Relay RLC channels for the support of L2 U2U relay operation. - Other functions including e.g. generic protocol error handling, transfer of dedicated NAS information, transfer of UE radio access capability information. - Support of self-configuration and self-optimisation. - Support of measurement logging and reporting for network performance optimisation, as specified in TS 37.320 [61]; - Support of transfer of application layer measurement configuration and reporting. - Configuration of side control information for NCR-node.

# Annex 2: Reference on INACTIVE RRM requirement in 5G NR [4]

#### 4.2.2.2 Measurement and evaluation of serving cell The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; where: - M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second, - otherwise M1=1. […] Table 4.2.2.2-1: Nserv DRX cycle length [s]; Scaling Factor (N1); Nserv [number of DRX cycles]. FR1; FR2-1Note1; FR2-2 Note2. 0.32; 1; 8; 12; M1*N1*4. 0.64; 5; 8; M1*N1*4. 1.28; 4; 6; N1*2. 2.56; 3; 5; N1*2. NOTE 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. NOTE 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length.. […] #### 4.2.2.3 Measurements of intra-frequency NR cells The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5) for intra-frequency cells that are identified and measured according to the measurement rules. […] Table 4.2.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). FR1; FR2-1Note1; FR2-2 Note2. 0.32; 1; 8; 12; 11.52 x N1 x M2 (36 x N1 x M2); 1.28 x N1 x M2 (4 x N1 x M2); 5.12 x N1 x M2 (16 x N1 x M2). 0.64; 5; 8; 17.92 x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 4; 6; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 3; 5; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). NOTE 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. NOTE 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length. NOTE 3: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected.. […] #### 4.2.2.4 Measurements of inter-frequency NR cells […] When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell. A UE not configured with eDRX_IDLE cycle shall measure SS-RSRP or SS-RSRQ at least every Kcarrier * Tmeasure,NR_Inter + Kcarrier_HST * Tmeasure,NR_Inter_HST (see table 4.2.2.4-1, table 4.2.2.4-2 if UE declares support of idle mode inter-frequency measurement enhancement when configured with highSpeedMeasInterFreq-r17 in FR1; or table 4.2.2.4-2a if UE declares support of idle mode inter-frequency measurement enhancement when configured with highSpeedMeasFlagFR2-r17 in FR2; else see Table 4.2.2.4-1) for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell. […] Table 4.2.2.4-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_Inter [s] (number of DRX cycles). FR1; FR2-1Note1; FR2-2 Note2. 0.32; 1; 8; 12; 11.52 x N1 x 1.5 (36 x N1 x 1.5); 1.28 x N1 x 1.5 (4 x N1 x 1.5); 5.12 x N1 x 1.5 (16 x N1 x 1.5). 0.64; 5; 8; 17.92x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 4; 6; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 3; 5; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). NOTE 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. NOTE 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length.. […]

# Annex 3: Reference on CONNECTED RRM requirement in 5G NR [4]

#### 9.2.6.3 Intra-frequency Measurement Period The requirements in this clause apply when a measurement gap is provided or when an activated Pre-MG is provided without any pre-MG status changed during the measurement period. The measurement period for FR1 intra-frequency measurements with gaps is as shown in table 9.2.6.3-1. […] Table 9.2.6.3-1: Measurement period for intra-frequency measurements with gaps (FR1) DRX cycle; T SSB_measurement_period_intra. No DRX; max(200 ms, ceil(5 x Kgap )x max(MGRP, SMTC period)) x CSSFintra. DRX cycle≤ 320 ms; max(200 ms, ceil(1.5x 5 x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320 ms; Ceil(5 x Kgap ) x max(MGRP, DRX cycle) x CSSFintra. NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer.. #### 9.2.5.2 Measurement period The measurement period for intra-frequency measurements without gaps is as shown in table 9.2.5.2-1, 9.2.5.2-2, 9.2.5.2-3 (CC with deactivated SCell), 9.2.5.2-4 (CC with deactivated SCell), 9.2.5.2-8 (deactivated SCG applicable for PSCell) or 9.2.5.2-9 (deactivated SCG applicable for PSCell). When highSpeedMeasFlag-r16 is configured, TSSB_measurement_period_intra is specified in table 9.2.5.2-5. When UE highSpeedMeasFlagFR2-r17 is configured, if SMTC ≤ 40ms, TSSB_measurement_period_intra is given in table 9.2.5.2-7; otherwise, TSSB_measurement_period_intra is given in table 9.2.5.2-2. For power class 6 UE supporting measEnhCAInterFreqFR2-r18 when highSpeedMeasFlagFR2-r17 is configured, the TSSB_measurement_period_intra is given in table 9.2.5.2-7 (if SMTC ≤ 40ms) and table 9.2.5.2-2 (if SMTC > 40ms) shall apply for SCC. […] Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1) DRX cycle; TSSB_measurement_period_intra. No DRX; max(200 ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320 ms; max(200 ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320 ms; ceil( 5 x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. […] #### 9.3.9.2 Measurement period The UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in clauses 10.1.4, 10.1.5, 10.1.9, 10.1.10, 10.1.14 and 10.1.15, respectively, as shown in table 9.3.9.2-1 and 9.3.9.2-2, if UE supports inter-frequency measurement without measurement gaps. When highSpeedMeasInterFreq-r17 is configured and UE supports measurementEnhancementInterFreq-r17, T SSB_measurement_period_inter is specified in table 9.3.9.2-3. Table 9.3.9.2-1: Measurement period for inter-frequency measurements without gaps ((FR1) DRX cycle; T SSB_measurement_period_inter. No DRX; max(200 ms, ceil(Mmeas_period_inter x Kp) x SMTC period)Note 1 x CSSFinter. DRX cycle≤ 320 ms; max(200 ms, ceil(1.5x Mmeas_period_inter x Kp) x max(SMTC period,DRX cycle)) x CSSFinter. DRX cycle>320 ms; ceil( Mmeas_period_inter x Kp ) x DRX cycle x CSSFinter. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. […] ### 9.3.5 Inter-frequency measurements When measurement gaps are provided for inter frequency measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in clauses 10.1.4, 10.1.5, 10.1.9, 10.1.10, 10.1.14 and 10.1.15, respectively, as shown in table 9.3.5-1 and 9.3.5-2. When highSpeedMeasInterFreq-r17 is configured, and UE supports measurementEnhancementInterFreq-r17, TSSB_measurement_period_inter is specified in table 9.3.5-3. When SCG is deactivated, TSSB_measurement_period_inter is specified in table 9.3.5-4 applies for inter-frequency carrier configured by SCG and not configured by MCG and table 9.3.5-2 applies for inter-frequency carrier configured by both SCG and MCG. Regardless of whether the SCG is activated or deactivated, table 9.3.5-2 applies for an inter-frequency carrier configured only by MCG. Table 9.3.5-1: Measurement period for inter-frequency measurements with gaps (FR1) Condition NOTE1,2; TSSB_measurement_period_inter. No DRX; Max(200 ms, Ceil(8 * Kgap) Max(MGRP , SMTC period)) CSSFinter. DRX cycle ≤ 320 ms; Max(200 ms, Ceil(8 1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320 ms; Ceil(8 * Kgap) DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent measurement GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured.. […]