3GPP TSG-RAN WG2 Meeting #132	R2-2508093

# Dallas, Texas, USA, 17 – 21 November 2025

Agenda item:		10.3.1.2

Source:	       Xiaomi

Title:	        	Consideration on 6G UP QoS and Service-awareness

Document for:		Discussion and Decision

# Introduction

In RAN#108 meeting, RAN WG study item “Study on 6G Radio” [1] was approved with the following scope highlighted in yellow regarding to 6G user plane:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In RAN2#131bis meeting, it was agreed that:

=> Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic). Understand the issues and shortcoming with the current NR QoS. Aim to look at a general framework.

In this contribution, we discuss 6G UP QoS and service-awareness.

# Discussion

## AI Service requirement analysis

### AI token

AI tokens are used extensively in AI models. Before discussing AI tokens, it is helpful to review the state-of-the-art usage of AI tokenization for image / video AI tasks (note that image / video is focused here as they consume much more bandwidth compared with text). There are mainly two types of image models: image generation models which generates text / image / video based on user prompts, and image understanding models (e.g. Vision Language Model, VLM) which can answer questions based on user input (text prompt / image / video).

Image generation model

Taking an image generation AI model (Qwen Image [2]) as example, its architecture is as follows:

Figure 1: Architecture of one image generation AI model [2]

In this architecture, image tokenizer is VAE (variational autoencoder) Encoder, which maps image into a distribution within the latent space. Such VAE Encoder can be used for an input image to generate another image based on user's prompt.

In current usage of AI models, AI tokenization is performed at the server side. For example, most AI client implements OpenAI API [4], which is RESTful API (also used by SBI in NR 5GC). For an image editing, the input image is directly uploaded to the server as part of HTTP POST request, and image tokenizer (VAE Encoder) at the server is used to process the input image.

One potential use case is to offload the AI tokenizer from the server to client. First aspect to analyze is whether tokenizer reduces the image size. Taking 512×512 image as an example. Original image without compression / tokenization takes up 512×512×3 (RGB channels) = 786432 bytes. Output of VAE Encoder is latent vector, whose typical dimension in diffusion models is 4×64×64. Each latent value is typically of 4 bytes (as float32), therefore the total output size of VAE Encoder is 4×4×64×64 = 65536 bytes. In comparison, a typical JEPG image of medium quality has about 1:10 to 1:12 compression ratio, and the corresponding JPEG image file has size of 65~79 KB. In summary, the output of VAE Encoder has similar size as medium quality JPEG file. Therefore, moving AI tokenization to UE side does not reduce the traffic compared with transmitting image / video directly.

On the other hand, the gain of processing reduction when moving AI tokenization to UE side is negligible. AI processing is mainly proportional to parameter size. In the same image generation AI model [2] cited above, VAE Encoder has 54M parameters, while main image generation part (MMDiT) has 20B parameters, which means that VAE encoder is only 0.27% of total parameters. It is obvious that VAE encoder only consists of a tiny fraction of total processing requirements.

Moving AI tokenization from server side to client increases the complexity for the AI service provider. Every time there is an update of VAE Encoder, the client side should be updated. Assuming that each parameter of VAE Encoder uses float32, the 54M parameter VAE Encoder in discussion above means that the update is around 216 MB. There are also issues to resolve like version compatibility between client and server.

In addition to “continuous” tokenizers like VAE tokenizer discussed above, there are also “discrete” image tokenizers. For example, compared with traditional image formats (e.g. JPEG), One-D-Piece tokenizer [5] can reduce the transmitted image size while having better perceptual quality at the cost of high distortion, as shown below in red box (the lower the rFID, the better the perceptual quality; the higher the PSNR, the lower the distortion):

In above table, PSNR of JPEG is only shown for the selected image size. Typical PSNR of different quality of JPEG images are shown below. It can be seen that typical PSNR (30~42) of JPEG image is much higher than the discrete tokenizer (18~19) [5] even for medium quality like 50.

Table 1: JPEG image quality and PSNR

| JPEG Quality | Typical Compression Ratio | Typical PSNR Range | Image Quality Description |
| --- | --- | --- | --- |
| 50 | 10:1 ~ 20:1 | 30.0 - 32.5 dB | Visible artifacts, detail loss |
| 60 | 8:1 ~ 15:1 | 32.5 - 34.5 dB | Noticeable artifacts, acceptable |
| 70 | 6:1 ~ 12:1 | 34.5 - 36.5 dB | Minor artifacts, web-suitable |
| 80 | 4:1 ~ 8:1 | 36.5 - 38.5 dB | Near-original, subtle artifacts |
| 90 | 2:1 ~ 4:1 | 38.5 - 42.0 dB | High fidelity, visually lossless |

The distortion of the tokenizer [5] is shown below by comparing the original image with the full 256 token (distortion is highlighted for beak, eye, feature, background).

Original image                               256 tokens

Figure 2: Comparison of image distortion

State-of-the-art image generators (e.g. Qwen Image, Stable Diffusion/SDXL/SD3, DALL·E 3, Imagen, PixArt, Flux) overwhelmingly use a continuous VAE-style latent autoencoder as the tokenizer. Discrete tokenizers (like VQVAE, VQGAN etc.) are only used for autoregressive transformer (e.g. DALL·E 1). The reason is that VAE encoders has better reconstruction fidelity and scalability, and do not have codebook issues of discrete tokenizers, and continuous latent pairs naturally with diffusion/flow transformers. One example is that DALL·E 1 uses discrete tokenizer and later upgrades to VAE in DALL·E 3. In the technical report of Qwen Image [2] as shown below, the state-of-the-art models all use VAE.

Observation 1: Compared with traditional image formats like JPEG, discrete image tokenizers can reduce the transmitted image size while having better perceptual quality, at the cost of high distortion. Discrete image tokenizers are rarely used in state-of-the-art image generation models, which mostly use VAE for image tokenizer.

Image understanding model

Typical tokenizer used in image understanding model is vision transformer (ViT), which decomposes an input image into a series of patches (rather than text into tokens). Note that ViT in image understanding model is typically uses continuous tokenization, which is different from ViT based discrete tokenizer used in [5] above.

Similar to above analysis for VAE models, we analyze whether ViT tokenizer can reduce the image size. Still taking 512×512 image as an example. The image is split into patches with size of 16×16 pixels. The total number of patches are (512×512) / (16×16) = 1024. Each patch is represented by one token. Each token has the size of D×bytes per value. D is embedding dimension, and can be 768, 1024, 1280. “bytes per value” is related to data type, and can be 4 (float32, which is most common) or 2 (float16). Therefore after ViT tokenizer, the output size can be 1,572,864 ~ 5,242,880 bytes, which is much larger than medium quality JPEG image (65~79 KB). This implies that transmitting tokens instead of raw images in air interface for an image understanding model increases overhead significantly.

Regarding the size of ViT relative to the whole AI model, open model Qwen2.5-VL [3] can be an example. It has three variants with total number of parameters of 3B, 7B, and 72B. It is estimated that the ViT has parameters of 632 M (based on 1280 hidden size and 32 layers). Typically 3B and 7B models are mainly used for personal devices and server deploys the largest model with size of 72B. ViT tokenizer is only 0.87% of total parameters of 72B model. It is obvious that ViT tokenizer only consists of a tiny fraction of total processing requirements.

Observation 2: In current usage of AI models, AI tokenization is performed at the server side. For image and video, moving AI tokenization to UE side does not reduce the traffic compared with transmitting image / video directly for image generation model while increase the traffic significantly for image understanding model, and the gain of processing offloading is negligible. There is increased complexity like update of tokenizer model and version compatibility between client and server.

Proposal 1: RAN2 should carefully evaluate the pros and cons (e.g. transmission efficiency, quality distortion, UE memory and processing requirements, complexity for service provider) before considering AI tokenizer approach in RAN.

### AI traffic trace

RAN2#131bis meeting has the following agreements: "Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic)." One of the key new services to support for 6G is the mobile AI traffic which is gaining fast momentum in recent years. Below is a traffic trace captured for a session of communication with an AI chatbot which provides text responses as well as image responses based on user prompts. Each line represents a packet, with positive and negative y values representing DL and UL traffic, respectively. Different color represents different remote server IP addresses.

Figure 3: Traffic characteristics of AI

In above trace, there are actually 106 server IP addresses and many colors are reused several times. Below is a list of server IP addresses (masks are used for the two MSB bytes for privacy):

| C.J.140.219 | C.J.144.187 | F.E.90.159 | G.B.2.180 | G.B.2.195 | G.B.3.67 |
| --- | --- | --- | --- | --- | --- |
| H.D.14.153 | I.L.23.150 | I.M.69.228 | K.A.10.5 | K.A.10.159~254 |  |

It can be seen that even within a single session of AI communications, different server IP addresses are used. for different tasks (e.g. text and image) and even for the same task. And we observe that all the packets targeting different IP addresses are actually mapped to the same DRB. The reason might be in two-folds:

- It is challenging for the network to configure suitable IP packet filters to map the IP flows into QoS flows;

- Network may not have interaction with the AI service server to acquire the QoS information, which is the most common case in current commercial environment.

Observation 3: One single AI session can contain Internet traffic generated by many servers of different IP addresses. Different IP addresses may be used for different types of tasks and even for the same type of task, and packets targeting different IP addresses are mapped to the same DRB by the network, partially because that network has no interaction with the service server to acquire the QoS information.

The above observation is not only valid for AI traffic, but valid for many other use cases. It is observed that smart phones are almost configured with only one DRB all the times, even there are a lot of service flows targeting different IP addresses.

### UE driven QoS framework

Given the above typical situation, the 4G/5G assumption that network can typically know the QoS requirement of user services is no longer valid. Thus, the 4G/5G QoS framework established on this assumption should be revisited.

We think, in 6G, we should shift to UE driven QoS framework, as UE is more able to know the QoS requirement of its traffics. For example, UE can easily know to which app the traffic belongs. And based on the application interaction with the Kernel, UE is able to know the packet characteristics, e.g. image or video. Thereby, UE is able to infer the QoS information. The QoS information acquired from UE can then be applied by network to guarantee the QoS.

Under this paradigm change, HOL blocking issue we are typically facing nowadays can be easily fixed. As shown in the figure below, as all the traffics in practice are mapped to the same DRB, according to the current specification, a single PDCP/RLC queue will be used for the data transmission. If the packet at the queue head is suffering from retransmission, all the subsequent packets are blocked. When UE driven QoS framework is used, UE will first identify packets with different QoS requirement, and then map them to different L2 queues with a flow ID indicated in the L2 header. As each queue can deliver its packets to the LCH based on its own Tx window, HOL issue is resolved at transmitter side. To ensure QoS fulfilment per flow and fairness among flows, UE LCP can dynamically decide how much UL resources are allocated to each queue based on their respective QoS requirement. To facilitate network scheduling, UE can provide the QoS information to network. At receiver side, gNB filters the received packets into different Rx queues based on the flow id in the L2 header, and deliver the packet to upper layer in sequence per flow. To avoid out-of-order delivery impact to upper layer (e.g. TCP/QUIC), the transmitter side can establish a queue per TCP connection or QUIC flow.

Figure 4: Paradigm change from network driven to UE driven QoS framework

Proposal 2: RAN2 should study how to ensure the service QoS when core network cannot provide the QoS information to RAN, due to e.g. no interaction with application server, e.g. by considering a paradigm shift of network driven QoS framework to UE driven QoS framework.

## Service-awareness

As discussed above, UE driven QoS framework is mainly used for the case that core network cannot acquire the service QoS information from the server. In some cases, it is still possible that operator can cooperate with service provider and thereby acquire the QoS information from the server. In this case, 5G QoS framework can still be used. But a finer granularity might be considered in 6G.

PDU set based QoS handling enhancement was proposed early in SA2’s Rel-19 XR as captured in TR 23.700-70 and people discussed whether, what and how PDU Set based handling (e.g. new standardized 5QI, enhancements to Alternative QoS profiles, FEC, etc.) and PDU Set information (including Control Plane and/or User plane information) provided by the AF/AS are enhanced. Because of the time limits, PDU set based QoS handling was not enhanced much compared with the legacy 5G QoS framework. In TR 23.700-70, "Commercial XR split rendering and cloud gaming services use Application Layer Forward Error Correction (FEC)."as confirmed by SA4 was also discussed where source data packets are used to generate repair packets which help in the detection and correction of errors in the data stream. In some scenarios where source and repair bits are sent as different blocks of data packets and the NW can detect whether each block of data packets carries source or repair data respectively. In other scenarios where source and repair bits are mixed in all PDUs of the PDU Set and a ratio of PDUs at least of PDU Set are needed for NW to dynamically detect the success ratio. Both cases require differential protection, i.e. source packets selectively require a higher level of reliability than the other packets.

In 6G, we should investigate if new functionality in 6G QoS framework is required, considering, e.g., the emerging new traffic characteristics and application needs. Currently, despite introducing PDU Set-level metrics in Release 18 for enhanced XR support, the QoS flow remains the smallest configurable unit which forces uniform treatment of heterogeneous data units within a single flow as PDU Set-level QoS metrics (e.g., PDU Set Error Rate, PDU Set Delay Budget) are provided to gNB on a per-QoS flow basis in semi-static manner. The only QoS differentiation of PDU sets within the same QoS flow is the PDU set based discard based on PSI, in case of congestion. In normal communication (i.e. other than congestion), gNB does not apply different QoS treatment of different PDU sets within the same QoS flow not to mention the different QoS treatment of different packets within the same packet set.

Currently, the protocol stack exhibits multiple points of incompatibility with the new services:

MAC

Current MAC-layer scheduling mechanisms, particularly Logical Channel Prioritization (LCP), treat all packets within a single logical channel as equally important. Even though PDU set and PSI can be identified by CN and UE for DL and UL per packet set, the per-packet/per-packet set deadlines are not considered in the scheduling as intra-LCH prioritization is not considered in Rel-18 XR. This creates critical failures during network congestion scenarios. For instance, in conversational AI applications, user command requiring sub-20ms latency may be queued behind non-urgent generative content simply due to their arrival sequence. Hence, high-priority XR PDU Sets miss their application-layer deadlines despite the flow-level PDB being satisfied, resulting in perceptible frame freezes and synchronization errors. This occurs because MAC schedulers lack any mechanism to prioritize packets based on their intrinsic deadlines.

RLC

RLC ARQ configurations apply uniformly across all packets in a flow, resulting in either excessive retransmissions for error-tolerant content or insufficient protection for critical data.

PDCP

The PDCP layer compounds this problem through PDU Set level-wide discard timers that configured by the NW and cannot reflect the PSDB of each packet set. And when pdu-SetDiscard is enabled, loss of one packet triggers discard of the entire PDU set.

Observation 4: The current protocol stack in 5G doesn't allow differentiated QoS fulfilment within a QoS flow.

Since the traffic request is naturally varying data burst by data burst or even packet by packet, the QoS requirements may change frequently. The existing UE/NW triggered QoS adjustment mechanism is based on multiple steps including CN signaling and/or gNB RRC reconfiguration which would often be too slow to react to adaptations. Dynamic QoS should also be considered as one of the potential directions for QoS control mechanism.

Observation 5: The current protocol stack in 5G cannot support dynamic QoS for bursty applications with existing QoS adjustment suffers from prohibitive latency.

To fundamentally address 5G's limitations, 6G must establish an integrated QoS framework enabling simultaneous intra-flow differentiation and dynamic adaptations. First, a native intra-flow prioritization engine should be embedded across the protocol stack, requiring UEs to identify criticality metadata (e.g., I/B-frame tags), thereby allowing UE to implement deadline-aware LCP scheduling at MAC, adaptive ARQ configurations and selective PDCP discard timers aligned with PDU Set Delay Budget variance while maintaining packet sequencing within a single DRB. Second, a dynamic QoS control plane must support microsecond-scale reconfiguration through more dynamic signaling. In order to achieve this, a cross-domain negotiation framework with SA2/CT1 collaboration is needed to standardize an application-RAN interface for real-time QoS attribute exchange. This architecture ultimately transforming 6G into a context-aware network.

Proposal 3: 6G should implement an integrated QoS framework enabling simultaneous intra-flow differentiation and dynamic adaptations.

5G discussed multi-modal systems integrating inputs/outputs across multiple sources/destinations for richer experiences. For example in immersive XR applications, input can come from multiple sources (e.g. movement of AI glasses, haptic / sensor input from wearables like neuro band), and output can go to multiple destinations (e.g. audio / video of AI glasses, haptic feedback to wearables). Inter-modal synchronization is essential and is already defined in SA1 requirements TS 22.261. In 5G Rel-19, only basic multi-modality awareness is supported via the provisioning of Multi-modal Service ID (MMSID) from 5GC to NG-RAN when establishing and/or updating the corresponding QoS Flows. However, the inter-flow synchronization threshold is not specified. As a result, MMSID solution can only work in the case that inter-flow synchronization threshold is larger/equal to latency KPI of the application, e.g. by ensuring the latency of each flow doesn't exceed the latency KPI. If the inter-flow synchronization threshold is smaller than latency KPI of the application (as showcased in TS 22.261 below), MMSID alone cannot work well.

TS 22.261: For immersive multi-modal VR applications, synchronization between different media components is critical in order to avoid having a negative impact on the user experience (i.e. viewers detecting lack of synchronization), particularly when the synchronization threshold between two or more modalities is less than the latency KPI for the application. Example synchronization thresholds [41] [42] [43] [44] are summarised in table 6.43.1-1. Table 6.43.1-1: Typical synchronization thresholds for immersive multi-modality VR applications Media components; synchronization threshold (note 1). audio-tactile; audio delay: 50 ms; tactile delay: 25 ms. visual-tactile; visual delay: 15 ms; tactile delay: 50 ms. NOTE 1: for each media component, “delay” refers to the case where that media component is delayed compared to the other..

Observation 6:  5G cannot well support the synchronization dependency among QoS flows.

We also notice that the new use cases with requirements on coordination and synchronization are proposed in 3GPP TR 22.870, the existing stage 1 requirement of 3GPP, especially the QoS aspect, some examples of potential new requirements as follows:

- [PR 9.4.6-4]	Subject to operator policy and privacy considerations, the 6G system shall support means to coordinate data flows from UEs (e.g. cameras/wearables/sensors), associated with a specific place (e.g. a classroom, meeting room, office), to enable e.g. an AI/ML model to improve the user's perception of and interaction with an immersive scene, e.g. to more accurately predict scene changes or object movements within that specific place.

- [PR 9.6.2-1]	Subject to operator policy, the 6G system shall support the synchronization of independent traffic flows of one or more applications, to be delivered to more than one device (i.e. UE or tethered devices).

- [PR 9.12.6-2]	Subject to operator policy and privacy considerations, the 6G system shall provide a means to synchronize media/flow and data for rendering in time and in XR space (e.g. coherent gestures, facial expressions the personalized avatars of the group of participants.

Observation 7:  6G system shall provide a means to synchronize multiple data flows within an acceptable threshold.

Proposal 4: RAN2 should study mechanisms to better support multi-flow synchronization.

## AQM and ECN enhancements

Active Queue Management (AQM) is a proactive approach to drop packets or ECN-mark packets before a queue becomes full. It should be noted that PDCP discard was proposed as an AQM mechanism as from R2-074689. AQM has different types of algorithms on when to drop a packet and which packet to drop, e.g. Controlled Delay (CoDel, RFC 8289), PIE (Proportional Integral control Enhanced, RFC 8033), ARED (Adaptive Random Early Detection).

ECN, a congestion control mechanism used in IP, TCP, SCTP, and QUIC, can be also used for AQM and L4S. In NR, there is general statement on ECN support, as in TS 38.300: "The gNB and the UE support of the Explicit Congestion Notification (ECN) is specified in clause 5 of [27]". ECN marking for L4S was also specified in TS 38.300 clause 16.16.

Whether to support discarding mechanisms in AQM needs clear motivation as SDU discarding has similar functionality as AQM and several user plane features are defined based on SDU discard mechanism, e.g. PDCP SN gap reporting, DSR, remaining time based polling / RLC retransmission ECN marking can be supported but AS layer enhancements for ECN marking should be justified. In NR, enhancements for ECN marking was discussed (e.g. on R2-1913888) and was not accepted. At that time the enhancement is mainly motived for CU-DU split architecture. In 6G study, whether to support CU-DU split architecture is pending RAN3 discussion and the checkpoint is TSG#112 (June/2026) [1].

Observation 8: SDU discarding has similar functionality as AQM and several user plane features are defined based on SDU discard mechanism.

Proposal 5: AS layer enhancements to support AQM discarding mechanism should be justified over existing SDU discard mechanism. Discussion on potential ECN enhancements depends on RAN3 discussion on CU-DU split.

# Conclusion

In this contribution, we discuss 6G UP QoS, QoE and service-awareness, and have the following observations and proposals:

Observation 1: Compared with traditional image formats like JPEG, discrete image tokenizers can reduce the transmitted image size while having better perceptual quality, at the cost of high distortion. Discrete image tokenizers are rarely used in state-of-the-art image generation models, which mostly use VAE for image tokenizer.

Observation 2: In current usage of AI models, AI tokenization is performed at the server side. For image and video, moving AI tokenization to UE side does not reduce the traffic compared with transmitting image / video directly for image generation model while increase the traffic significantly for image understanding model, and the gain of processing offloading is negligible. There is increased complexity like update of tokenizer model and version compatibility between client and server.

Proposal 1: RAN2 should carefully evaluate the pros and cons (e.g. transmission efficiency, quality distortion, UE memory and processing requirements, complexity for service provider) before considering AI tokenizer approach in RAN.

Observation 3: One single AI session can contain Internet traffic generated by many servers of different IP addresses. Different IP addresses may be used for different types of tasks and even for the same type of task, and packets targeting different IP addresses are mapped to the same DRB by the network, partially because that network has no interaction with the service server to acquire the QoS information.

Proposal 2: RAN2 should study how to ensure the service QoS when core network cannot provide the QoS information to RAN, due to e.g. no interaction with application server, e.g. by considering a paradigm shift of network driven QoS framework to UE driven QoS framework.

Observation 4: The current protocol stack in 5G doesn't allow differentiated QoS fulfilment within a QoS flow.

Observation 5: The current protocol stack in 5G cannot support dynamic QoS for bursty applications with existing QoS adjustment suffers from prohibitive latency.

Proposal 3: 6G should implement an integrated QoS framework enabling simultaneous intra-flow differentiation and dynamic adaptations.

Observation 6:  5G cannot well support the synchronization dependency among QoS flows.

Observation 7:  6G system shall provide a means to synchronize multiple data flows within an acceptable threshold.

Proposal 4: RAN2 should study mechanisms to better support multi-flow synchronization.

Observation 8: SDU discarding has similar functionality as AQM and several user plane features are defined based on SDU discard mechanism.

Proposal 5: AS layer enhancements to support AQM discarding mechanism should be justified over existing SDU discard mechanism. Discussion on potential ECN enhancements depends on RAN3 discussion on CU-DU split.

# References

[1] RP-252912, NTT DOCOMO (Moderator), “New SID: Study on 6G Radio”

[2] Qwen-Image Technical Report, Qwen team, https://arxiv.org/pdf/2508.02324

[3] Qwen2.5-VL Technical Report, Qwen team, https://arxiv.org/pdf/2502.13923

[4] OpenAI Open API, https://github.com/openai/openai-openapi

[5] K. Miwa, et al. One-D-Piece: Image Tokenizer Meets Quality-Controllable Compression, ICML Tokenization Workshop, 2025.