3GPP TSG-RAN WG2 Meeting #132	R2-2508107

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	10.3.1.3

Source:	OPPO

Title:	Discussion on 6G scheduling and (re-)transmission schemes

Document for:	Discussion, Decision

# Introduction

In last RAN2 meeting, the following agreements were made for 6G user plane.

Agreements on UP

1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In this contribution, we would like to further provide our considerations on scheduling and HARQ/ARQ enhancements for 6G.

# Discussion

## Scheduling enhancement

In last RAN2 meeting, there were discussions to enhance scheduling in 6G and the agreements were made as follows:

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

In 5G, when there is UL data arrival, the UE triggers BSR/DSR to request for UL grant (i.e., DG). If there is no UL grant available for transmitting the BSR/DSR MAC CE, there are two cases for the UE to obtain UL grant for BSR/DSR MAC CE transmission:

Case1: The UE initiates SR using the dedicated PUCCH resources. In response to the SR, the NW allocates the UL grant for BSR/DSR MAC CE transmission.

Case2: If there is no valid PUCCH resource configured for the triggered SR, the UE triggers RACH to acquire the UL grant for BSR/DSR MAC CE transmission, i.e., UL grant in RAR or MsgA PUSCH.

In either case, several signalling interactions between UE and NW are required before the UE acquires the UL grant for BSR/DSR MAC CE transmission, which further increases data transmission latency.

Observation 1: In NR, several steps/signalling interactions are required to acquire the UL grant for BSR/DSR MAC CE transmission, i.e., via SR or RACH, which increases UL transmission latency.

According to the agreement, more efficient scheduling schemes can be considered to reduce SR/BSR/DSR latency in 6G. One potential solution is to provide the UE with UL grant for direct BSR/DSR MAC CE transmission. To balance latency and resource efficiency, contention-based CG resources can be introduced for BSR/DSR MAC CE transmission. The general procedure is illustrated as in the figure below.

Figure 1: BSR/DSR transmission based on contention-based CG resource

The main purpose of reducing BSR/DSR latency is to minimize the latency for uplink data/signalling transmission. In addition to BSR/DSR transmission, the use of contention-based CG resources can be extended to data/signalling transmission to improve UL transmission efficiency. For example, the CB-CG resources can also be used for initial access if TA is valid. Compared to triggering RACH, Msg1/2 efforts can be saved which is beneficial for latency and signalling overhead reduction.

Proposal 1: To reduce UL latency, RAN2 to support contention-based CG resources at least for direct BSR/DSR MAC CE transmission.

## HARQ feedback-based ARQ

In last RAN2 meeting, there were interests to improve L2 ARQ latency/efficiency in 6G and the agreement was made as follows:

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In NR, ARQ procedure is handled by AM RLC entity based on the status reporting from its peer RLC entity:

- For transmitting side: The RLC entity re-transmits the RLC SDU or RLC SDU segment for which a negative acknowledgement is received in the STATUS reporting. For faster ARQ retransmission, the AM RLC entity can poll its peer to trigger STATUS reporting transmission.

- For receiving side: The AM RLC entity sends STATUS reporting to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them), based on which the transmitting side determines to perform ARQ retransmission.

Observation 2: In NR, ARQ mechanism relies on the STATUS reporting, i.e., for AM RLC entity, the UE re-transmits the RLC SDU or RLC SDU segment for which a negative acknowledgement is received in the STATUS reporting from its peer AM RLC entity.

According to the agreement, RAN2 should study how to improve L2 ARQ latency/efficiency in 6G based on tight coordination with HARQ. In 5G, HARQ operations for DL data and UL data are handled as follows:

- DL data: the UE sends HARQ-ACK to the NW based on the reception status of DL data. Since both data transmission and HARQ feedback are fully controlled by the NW, it can be up to NW implementation on how to use the existing HARQ feedback, e.g., either (re-)transmitting the TB or trigger ARQ. That is, for DL data transmission, ARQ enhancement based on HARQ can be already achieved by NW implementation.

- UL data: For UL data transmission, HARQ is based on TB new/re-transmission scheduled by the NW. In current mechanism, HARQ (i.e., TB-level (re-)transmission) and ARQ (i.e., SDU/segment-level (re-)transmission) are separately defined in specifications even if the ACK/NACK for a TB can also reflect the transmission status of the contained SDU(s)/segment(s). To achieve more efficient ARQ reaction for UL data transmission, it is promising to study the ARQ enhancement based on HARQ feedback.

Based on these considerations, we would like to clarify that for L2 ARQ enhancement based on tight coordination with HARQ as illustrated above, RAN2 study should focus on UL data transmission.

Proposal 2: For L2 ARQ latency/efficiency enhancement based on tight coordination with HARQ, RAN2 study focuses on UL data transmission.

In NR, there is no explicit HARQ-ACK feedback for UL data transmission. The transmission status of the TB can be deduced by the NDI included in the DCI for UL grant scheduling while it is not sufficient to make a proper decision:

- If NDI is not toggled, the UE performs TB retransmission which implies that the TB is not successfully received by the NW.

- If NDI is toggled, the UE performs TB new transmission and the previous TB will be overridden. In this case, the UE has no knowledge of whether the previous TB is successfully received by the NW, i.e., as one possibility, the new transmission is scheduled due to successful reception of previous TB; while as another possibility, this occurs because the retransmission of previous TB has been abandoned.

Observation 3: In NR, there is no explicit HARQ-ACK feedback for UL data transmission. The existing indication in the DCI for UL grant scheduling is not enough to make a proper decision on the transmission status of the TB.

To achieve ARQ enhancement by coordinating with HARQ feedback, the UE should be able to distinguish whether the TB is successfully received by the NW, based on which the UE makes ARQ decision. Taking NR mechanism as baseline, additional indications in the DCI are required to enable more precise HARQ feedback as shown in the table below, along with our considerations for HARQ-based ARQ.

|  | Case1: UL TB is successfully received | Case2: UL TB is not successfully received |  |
| --- | --- | --- | --- |
| HARQ feedback | - NDI toggled - TB successful reception indication | - NDI toggled - TB unsuccessful reception indication | - NDI not toggled |
| UE behaviour | - ARQ is not required and the UE can perform new transmission as scheduled by the NW | - UE performs new transmission - Trigger ARQ for (partial) SDU(s)/segment(s) included in the NACKed TB. Further study is needed on how to select the SDU(s)/segment for retransmission e.g., at least for AM SDU/segment. | - UE performs TB re-transmission as scheduled by the NW - UE can trigger ARQ for (partial) SDU(s)/Segment(s) included in the NACKed TB. FFS on other conditions, e.g., the UE triggers ARQ when the re-transmission of the TB is scheduled for a number of times. |

How it works is further demonstrated in Figure2.

Figure 2: RLC retransmission triggered by HARQ NACK

Proposal 3: To support HARQ feedback-based ARQ, HARQ feedback for UL data transmission is enhanced to indicate whether the TB has been successfully received when the NDI is toggled.

Proposal 4: To improve ARQ latency/efficiency for UL data transmission, the UE triggers ARQ based on HARQ feedback as follows:

- If the TB is successfully transmitted, no ARQ is triggered.

- If the TB is not successfully transmitted, the UE triggers ARQ for (partial) SDU(s)/segment(s) included in the NACKed TB. FFS on how to select the SDU(s)/segments for retransmission. FFS for other conditions, e.g., timer-based/count-based.

If HARQ feedback-based ARQ is introduced, another issue is whether to maintain current ARQ retransmission mechanism based on STATUS reporting. Although faster ARQ can be achieved by tightly interworking with HARQ, the mechanism has its drawbacks at least from the following aspects:

- HARQ feedback is lack of reliability, which may lead to a misalignment in data transmission status between UE and network. For example, the DCI may be incorrectly decoded or the DCI for new/re- transmission scheduling may be missed. In such cases, the UE may make a wrong decision on data transmission status.

- HARQ status is vulnerable and HARQ continuity is not always feasible, e.g., during HO, the UE needs to performs MAC reset in which procedure the HARQ status will be also be reset.

Based on the above discussion, data retransmission managed by the upper layer is still necessary to address the limitations of HARQ feedback-based ARQ. Therefore, it is proposed to retain ARQ retransmission based on status reporting to ensure reliability.

Observation 4: L2 ARQ enhancement by tightly interworking with HARQ is not always feasible: 1/ HARQ feedback is lack of reliability. 2/ In handover case, HARQ buffers will be flushed.

Proposal 5: ARQ retransmission based on STATUS reporting is supported.

  - Head-of-line blocking issue

While 5G NR has made significant strides in flexibility and performance, certain architectural decisions, inherited and evolved from LTE, continue to exhibit head-of-line (HOL) blocking under specific conditions. This issue manifests at both the PDCP and RLC layers, primarily due to their in-order delivery mechanisms:

### HOL blocking at PDCP receiving side

Figure 3: Head-of-line blocking at PDCP reordering window

The HOL problem is driven by the receive window and its reordering function in downlink as illustrated in Figure 3. The PDCP receiver must deliver packets to the upper layers in strict sequence number order. If a single packet with a specific Sequence Number (SN) is missing at the receiver, the PDCP entity cannot deliver any subsequent packets with higher SNs, even if they have been successfully received. All these subsequent packets must be held in the reordering buffer until the missing packet arrives, creating a receiver-side HOL block that introduces latency for all data flows within that bearer.

A common scenario exacerbates this issue: multiple non-GBR QoS flows are often mapped to the same DRB. When a packet from one QoS flow is lost, it is unfair that all other QoS flows mapped to the same DRB are also blocked, despite having successfully received packets.

Mechanisms like t-Reordering at the receiver can, upon expiry, advance the window to skip a persistently lost PDU. However, this is a recovery mechanism with its own latency penalty and results in a gap in the sequence, which may be undesirable for some upper-layer protocols (e.g., TCP).

Observation 5: The current PDCP design, which enforces strict in-order delivery using a single sequence number space, render 5G NR systems vulnerable to head-of-Line blocking. This introduces undesirable latency and performance degradation.

Proposal 6: RAN2 to consider solutions to resolve head-of-line blocking issue inherent in the PDCP reordering window mechanism, particularly when multiple QoS flows share one DRB.

### HOL blocking at RLC transmitting side

Figure 4: Head-of-line blocking at RLC AM transmitting window

In the uplink, a distinct yet related form of HOL blocking occurs in RLC Acknowledged Mode (AM). As shown in Figure 4, if a single packet near the front of the transmitting window fails to be ACKed, the entire window becomes blocked and cannot advance. Consequently, even if subsequent packets have been successfully transmitted and ACKed, the transmitting side is unable to transmit any new data packets, irrespective of the nature or priority of the upper-layer data flows being multiplexed. This results in reduced data rate and increased latency.

Observation 6: The PUSH-style operation of the RLC AM transmission window is susceptible to head-of-line blocking, which degrades data transmission speeds and increases latency.

Proposal 7: RAN2 to consider solutions to resolve head-of-line blocking issue caused by the RLC AM PUSH-style transmitting window.

# Conclusion

Based on the discussion in section2, we have following observations and proposals:

Scheduling enhancement

Observation 1: In NR, several steps/signalling interactions are required to acquire the UL grant for BSR/DSR MAC CE transmission, i.e., via SR or RACH, which increases UL transmission latency.

Proposal 1: To reduce UL latency, RAN2 to support contention-based CG resources at least for direct BSR/DSR MAC CE transmission.

HARQ feedback-based ARQ

Observation 2: In NR, ARQ mechanism relies on the STATUS reporting, i.e., for AM RLC entity, the UE re-transmits the RLC SDU or RLC SDU segment for which a negative acknowledgement is received in the STATUS reporting from its peer AM RLC entity.

Proposal 2: For L2 ARQ latency/efficiency enhancement based on tight coordination with HARQ, RAN2 study focuses on UL data transmission.

Observation 3: In NR, there is no explicit HARQ-ACK feedback for UL data transmission. The existing indication in the DCI for UL grant scheduling is not enough to make a proper decision on the transmission status of the TB.

Proposal 3: To support HARQ feedback-based ARQ, HARQ feedback for UL data transmission is enhanced to indicate whether the TB has been successfully received when the NDI is toggled.

Proposal 4: To improve ARQ latency/efficiency for UL data transmission, the UE triggers ARQ based on HARQ feedback as follows:

- If the TB is successfully transmitted, no ARQ is triggered.

- If the TB is not successfully transmitted, the UE triggers ARQ for (partial) SDU(s)/segment(s) included in the NACKed TB. FFS on how to select the SDU(s)/segments for retransmission. FFS for other conditions, e.g., timer-based/count-based.

Observation 4: L2 ARQ enhancement by tightly interworking with HARQ is not always feasible: 1/ HARQ feedback is lack of reliability. 2/ In handover case, HARQ buffers will be flushed.

Proposal 5: ARQ retransmission based on STATUS reporting is supported.

Head-of-line blocking issue

Observation 5: The current PDCP design, which enforces strict in-order delivery using a single sequence number space, render 5G NR systems vulnerable to head-of-Line blocking. This introduces undesirable latency and performance degradation.

Proposal 6: RAN2 to consider solutions to resolve head-of-line blocking issue inherent in the PDCP reordering window mechanism, particularly when multiple QoS flows share one DRB.

Observation 6: The PUSH-style operation of the RLC AM transmission window is susceptible to head-of-line blocking, which degrades data transmission speeds and increases latency.

Proposal 7: RAN2 to consider solutions to resolve head-of-line blocking issue caused by the RLC AM PUSH-style transmitting window.