3GPP TSG-RAN WG2 Meeting #132	R2-2508127

Dallas, USA, 17 – 21 November 2025

Agenda item:	10.3.1.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On scheduling, retransmissions and uplink scheduling information

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

RAN2 131-bis made following agreements on scheduling and retransmissions:

Agreements on UP

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In this document, we expand on the brief overview from [1] and discuss in more detail our proposed next steps for studying configured grand (CG), contention-based PUSCH (CB-PUSCH), unified BSR/DSR and possible improvements for HARQ and ARQ retransmissions.

# 2	Discussions

## 2.1	UL scheduling schemes

Configured Grant

In 5G NR, both dynamic grant (DG) and configured grant (CG) are supported. While DG is predominantly used, CG is an essential part of NR design. The CG with 1ms periodicity (and many enhancements over multiple Releases) has been introduced to support low latency requirements. With similar requirements imposed on 6GR, CG framework is a natural candidate solution to fulfill those.

Two types of CG exist:

- Type 1 CG: Fully configured by RRC, immediate activation by the UE upon RRC (re)configuration, can only be modified or released by RRC (no dynamic modification/activation);

- Type 2 CG: Partially configured by RRC, activation/deactivation via DCI (with remaining configuration), flexible modification using DCI.

Both Type 1 and Type 2 CGs can provide periodic uplink resources to the UE. The resources are allocated at regular intervals, allowing the UE to transmit data at these predefined times. This dual approach addressed similar problems but added redundant complexity. Similar, 5G NR CG framework has been enhanced to support both licensed and unlicensed band operations, often creating tailored solutions for a specific band (ex. HARQ behavior).

Observation 1: CG framework in 5G NR has been designed sub-optimally with multiple solutions for similar problems.

For 6GR, we propose to study a unified CG framework by combining the beneficial parts of both CG types into the single flexible framework. In our view, 6G CG framework shall meet the following (minimum) requirements:

- RRC-based configuration

- Flexible CG activation: immediate upon RRC configuration and/or via DCI

- Flexible modification using DCI

During study item, when assessing the minimum set of features to be supported by 6GR CG framework, we propose focusing only on the features necessary for licensed-band operation. Features introduced in 5G NR for NR-U operations may not need to be a part of an initial 6G release. At later point in the Study Item, RAN 2 may check on feature-by-feature basis which CG enhancements introduced during 5G lifecycle should remain and form as a baseline for 6G CG design.

Proposal 1: Study a unified CG framework that satisfies at least the following requirements: 1) RRC-based configuration, 2) Flexible CG activation, 3) Flexible modification using DCI

CB PUSCH
In NR, the uplink transmission procedure involves a UE transmitting a Scheduling Request (SR), waiting for an initial uplink grant, transmitting a Buffer Status Report (BSR), and finally sending the first payload on the granted PUSCH resources. The impact of SR/BSR/DSR latency on system performance varies depending on traffic characteristics:

-	Continuous, large volume data transfer: Initial scheduling latency is negligible with respect to the total transmission time as the UE continuously updates the gNB on buffer status using BSR/DSR MAC CEs during subsequent uplink grants;

-	Bursty, periodic data transfer: SR/BSR latency can be significant when using dynamic grants. The CG framework effectively limits scheduling latency;

-	Bursty, aperiodic data transfer: SR/BSR latency is significant for most aperiodic bursts. The CG framework is not suitable for reducing latency. Network-based solutions (e.g., proactive uplink grants, larger initial uplink grants) may reduce latency but may be limited by the accuracy of buffer status prediction thus work only in favorable (predictable) scenarios;

Proposed scope of CB-PUSCH Study
During RAN2 #131-bis, CB-PUSCH has been proposed as a solution to reduce scheduling latency. However, different assumptions on the content of data transmitted over CB resources were shared. We propose to initially limit the study of CB-PUSCH to connected-mode UEs and BSR/DSR messages only, as they directly impact scheduling latency. Further, these messages can be transmitted opportunistically over CB resources without requiring UE-specific processing, iterative/blind decoding, or contention resolution mechanisms. Proposals to transmit UE-specific data over contention-based resources need to be well justified considering additional processing requirements, such as decoding complexity or retransmissions.

Proposal 2: Focus the initial study of CB-PUSCH solely on RRC connected mode and BSR/DSR messages.

Performance Study of CB-PUSCH
In our opinion, in the first step of a study item, RAN 2 shall ask RAN1 to perform a simulation study of CB PUSCH based on following minimum requirements:

The study shall evaluate potential limitations of CB-PUSCH, especially in mid- to highly-loaded scenarios, where multiple users access CB resources simultaneously. The envisioned latency reduction via CB-PUSCH is susceptible to the assumption of successful BSR transmission on the first attempt. In the event of a collision, BSR transmission must be re-attempted. This may result in similar or even higher latency compared to 5G NR, when multiple collisions may happen.

The long tail of delay performance may be reduced by a naïve approach of over-dimensioning the CB-PUSCH resources. However, such a strategy may not be desirable in mid- to highly-loaded cells, where resources are limited and the collision probability remains non-negligible.

The study shall therefore assess load, resource utilization, and latency trade-offs, and compare them against 5G SR. Discussion on simulation scenarios together with analysis of potential high-level solutions such as random backoff, dynamic adaptation of CB resource size, and contention resolution shall be left up to RAN1.

Based on the results of a simulation study, RAN 2 may decide on the potential changes to the SR/BSR design. The final solution shall preferably avoid duplicating mechanisms for the same problem and support SR/BSR across all use cases.

Proposal 3: Delegate RAN1 to simulate CB-PUSCH focusing on load/resource utilization/latency trade-offs.

## 2.2	Retransmissions

In Rel-19 XR, two distinct mechanisms were introduced for RLC AM operation, specifically driven by the latency requirement for XR:

-	Avoiding unnecessary retransmissions by stopping transmission when PDCP indicates discard; and allowing the receiver to discard and positively acknowledge missing PDUs after a configured duration.

-	Ensuring timely retransmissions by triggering retransmission or polling when the remaining lifetime of an RLC SDU falls below configured thresholds.

The benefit of those mechanisms is reducing latency of L2 retransmission while not consuming capacity unnecessarily, which will be beneficial for a wider range of services in 6G. Moreover, the term 'Immersive Communication' has been described by ITU-R as a use case that extends the eMBB, which would imply that many functionalities originally developed for XR will need to serve as fundamental components in 6G.

Proposal 4: The RLC in 6G should be built on Rel-19 RLC, incorporating the Rel-19 XR mechanisms of timely retransmission and avoiding unnecessary retransmissions.

The investigation of HARQ and L2 interaction to improve latency and efficiency of RLC ARQ has been identified as a study area envisioned for 6G. RLC ARQ and HARQ differ in their operation and recovery speed, but they are both fundamental retransmission mechanisms. Therefore, a basic design principle for any 6G L2 enhancement involving HARQ interaction must be to avoid unnecessary functional redundancy, and they should be complementary rather than overlapping. In this regard, the following questions need to be answered:

-	What level of reliability can be assumed for 6G HARQ feedback, and is this reliability sufficient for RLC to use as a retransmission trigger?

-	What are the target scenarios requiring faster L2 retransmission beyond those provided by Rel-19 RLC enhancements and HARQ retransmissions?

The 6G HARQ design is to be driven by RAN1, and the detailed work in RAN1 is not expected to start before next year. This timeline would be a challenge for RAN2: We cannot reasonably design a HARQ-ARQ interaction mechanism that assumes high-reliability HARQ feedback. In addition, it will be difficult to predict when a HARQ retransmission will be successful, making it difficult to decide a proper time point to trigger L2 retransmission without overlap to HARQ retransmission and resulting in unavoidable redundant retransmissions across layers.

Given these challenges, RAN2 would need to focus on the mechanisms that complement HARQ rather than operate in parallel with it. Instead of triggering L2 retransmission based on a potentially unreliable HARQ feedback or while HARQ retransmission is still ongoing (e.g., triggering L2 retransmission based on retransmission grant, which is based on the received HARQ NACK), it would be more effective to focus on the case when HARQ retransmission is unavailable (e.g., the network stops HARQ retransmission due to NACK-to-ACK error) or has failed (e.g., the network gives up further retransmission after a number of attempts). In this way, L2 ARQ will act as a complementary mechanism for the scenarios where HARQ performance is potentially limited.

We must not forget that one of 6G design principles is the minimization of memory requirements. Since HARQ operates at the TB level, where multiple logical channels are multiplexed, an ARQ mechanism interacting with HARQ feedback might require tracking which logical channels or even which SDUs are multiplexed into the TB, which would increase the memory requirements. Therefore, any proposed solution must be carefully evaluated for its impact on memory requirements.

Proposal 5: RAN2 study L2 ARQ mechanism with HARQ interaction, with limited memory requirements, that can complement poorly performing HARQ when HARQ retransmission is no longer available.

## 2.3	Scheduling information reporting

For scheduling information reporting, BSR has been supported since LTE Rel-8 and NR Rel-15 and DSR with additional delay information was introduced in Rel-18 to enable the UE to report delay-critical status information for certain LCGs which was then enhanced further in Rel-19 to report finer granularity of different remaining time thresholds.

Interaction between DSR and BSR procedures have been proposed in 3GPP since Rel-18, though not specified considering the backward compatibility and specification complexity. Separate BSR and DSR triggering and cancellation conditions and independent reports using different MAC CEs could result in information reported in a DSR and BSR be either partially or totally overlapping depending on the thresholds configuration and the buffer status. Since we are designing everything from scratch for 6G, it makes sense to simplify the procedure with unified buffer status and delay status report that can provide the gNB with configuration flexibility while reducing the signaling overhead and simplifying the specification and implementation of the corresponding features.

How to handle the AI/ML data, sensing data in the scheduling information report and LCP can be decided later after the discussion on those becomes clearer. However, the aim should target a unified procedure for buffer status and delay status reporting covering also the report of these new types of data for 6G. Different periodicity for triggering, report granularity etc. can be considered if seen needed.

Proposal 6: For scheduling information report, we aim at defining unified procedure for buffer status and delay status reporting, covering also the report of new types of data for 6G.

Besides, PHR is essential scheduling information for the scheduler as well. Type 1 PH for PUSCH, Type 2 PH for PUCCH and Type 3 PH for SRS have been supported for LTE and NR, which types are to be supported for 6G are to be discussed and decided by RAN1. PHR also has MIMO and CA specific features which depend on RAN1 progress.

Proposal 7: PHR is essential information for the scheduler which shall be supported for 6G. The details on the PHR mechanism could be discussed later after the CA framework as well as the RAN1 requirement on MIMO become clearer.

# 3	Conclusion

This document discussed scheduling, retransmissions and scheduling information reporting with the following proposals proposed:

Proposal 1: Study a unified CG framework that satisfies at least the following requirements: 1) RRC-based configuration, 2) Flexible CG activation, 3) Flexible modification using DCI

Proposal 2: Focus the initial study of CB-PUSCH solely on RRC connected mode and BSR/DSR messages.

Proposal 3: Delegate RAN1 to simulate CB-PUSCH focusing on load/resource utilization/latency trade-offs.

Proposal 4: The RLC in 6G should be built on Rel-19 RLC, incorporating the Rel-19 XR mechanisms of timely retransmission and avoiding unnecessary retransmissions.

Proposal 5: RAN2 study L2 ARQ mechanism with HARQ interaction, with limited memory requirements, that can complement poorly performing HARQ when HARQ retransmission is no longer available.

Proposal 6: For scheduling information report, we aim at defining unified procedure for buffer status and delay status reporting, covering also the report of new types of data for 6G.

Proposal 7: PHR is essential information for the scheduler which shall be supported for 6G. The details on the PHR mechanism could be discussed later after the CA framework as well as the RAN1 requirement on MIMO become clearer.

# Reference

[1] R2-2508310, UP functionalities and requirements, Nokia, Nokia Shanghai Bell