3GPP TSG-RAN WG2 Meeting #132	R2-2508136

Dallas, USA, 17th –21th November, 2025

Agenda Item:	10.3.1.1

Source:	Huawei, HiSilicon

Title:	Discussion on 6GR UP functions and related requirements

Document for: Discussion and Decision

1	Introduction

In last meeting, RAN2 discussed several aspects of the User Plan for 6G and agreements as below have been reached. [1]

Agreements on UP - RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality. Discussions should focus on standalone architecture, until told otherwise by plenary. When discussing functionalities companies should focus on identifying the problems being address or new requirements. - UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer. - 6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB) - Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic). Understand the issues and shortcoming with the current NR QoS. Aim to look at a general framework. - Support at least the following scheduling schemes: dynamic grant and configured grant. Further study configured grant. - Study need for scheduling enhancements to address the SR/BSR/DSR latency - Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ. Study should identify enhancements needed for HARQ and L2 ARQ.

In this paper, we further discuss the UP functions based on the agreements above.

2	Discussion

2.1	Overall user plan functions analysis

According to the agreements, RAN2 should first analyze the functionalities required for the 6G protocol stack. The table below shows the main functions supported by the 5G L2 user plane protocol (sidelink and MBS are not considered for now).

Table 1. 5G user plane functions.

| PDCP | RLC |
| --- | --- |
| - maintenance of PDCP SNs; - header compression and decompression using the ROHC protocol; - header compression and decompression using the EHC protocol; - uplink data compression and decompression using the UDC protocol; - ciphering and deciphering; - integrity protection and integrity verification; - timer based SDU discard; - PDU Set discard; - PSI based SDU discard; - for split bearers and DAPS bearer, routing; - duplication; - reordering and in-order delivery; - out-of-order delivery; - duplicate discarding. | - error correction through ARQ (only for AM data transfer); - segmentation and reassembly of RLC SDUs (only for UM and AM data transfer); - re-segmentation of RLC SDU segments (only for AM data transfer); - duplicate detection (only for AM data transfer); - RLC SDU discard (only for UM and AM data transfer); |

To give a better understanding, we analyze the usage and benefits of the above functionalities. In the first step, we focus solely on the function itself, without considering the protocol layer in which the function resides.

2.1.1	Compression functionalities

With the limited radio resources, data compression is an effective way to enhance network capacity. Additionally, compression is also effective for enhancing the cell coverage, particularly important for the UE at the cell edge. Several compression functions have been supported in 5G, including RoHC and EHC for header compression and UDC for data compression. These can be applied to different scenarios, as further analyzed below.

- Header compression (RoHC and EHC)

Both RoHC and EHC perform packet header compression, but they target different protocol headers. RoHC is used for IP/RTP/UDP/TCP protocol headers (Clause 5.7.1 in TS 38.323 [2]), while EHC is used for Ethernet headers (Clause 5.12 in TS 38.323 [2]). RoHC and EHC can also be used together when both Ethernet header and IP/RTP/UDP/TCP protocol headers are present in the PDCP payload.

Header compression can reduce the user plane packet header overhead and improve spectrum efficiency, especially when the packet payload is small. This is particularly relevant for services such as Voice and HRLLC. Additionally, header compression helps provide better network coverage by reducing the traffic size at the network edge. In 6G, maintaining robust header compression support remains essential for ensuring QoS guarantees in low-latency, small-packet critical services.

- Uplink data compression

UDC is important for improving uplink spectrum efficiency in scenarios with uplink-intensive services, such as video uploads and cloud applications, or for UE at the cell edge where channel condition is non-ideal. It leverages the potential redundancy in uplink packets to significantly reduce their size. In 6G, immersive communications and AI-driven services will require much more uplink transmission. UDC could be a good choice to help improve the network capacity for such services.

Based on the analysis above, the compression functionalities are considered useful for 6G services from the perspective of enhancing spectrum efficiency and network coverage.

Additionally, to make full use of these functionalities, the RAN may also consider potential extensions or enhancements of use cases. For instance, to support AI natively in 6G, the data plane is being heavily discussed. It is assumed that there will be huge amounts of data to be transmitted over the data plane. To reduce resource consumption, the compression functionalities could be applied to the data plane.

However, performing the compression functionalities will also contribute to additional processing complexity. It is necessary to make a trade-off between resource consumption and processing complexity. For example, when there are sufficient radio resources, the gain to perform compression might be marginal considering the pain for UE processing complexity.

Observation 1:	Header compression and data compression functionalities are useful for improving spectrum efficiency and network coverage, but also causes additional processing complexity.

2.1.2	Data delivery to upper layer

After receiving the data in L2, the user plane packets would be delivered to upper layers. Both in-order delivery and out-of-order delivery are supported in 5G, to satisfy different requirements.

- In-order delivery

In-order delivery is driven by upper-layer requirements, especially to avoid TCP performance degradation. The TCP protocol supports fast retransmission and congestion avoidance mechanisms. Specifically, if the TCP receiver receives out-of-order packets, it will trigger retransmission procedure by sending three duplicate ACKs to the sender. Upon receiving these duplicate ACKs, the TCP sender will conclude that there is network congestion and reduce the transmission window size, which can impact traffic throughput.

- Out-of-order delivery

Some services have stringent requirements on latency and may use UDP-based protocols instead of TCP. For example, real-time services based on the RTP/UDP protocol or services provided by QUIC do not require in-order delivery from the network.

For in-order delivery, to prevent the TCP layer performance degradation, the 5G protocol stack ensures that packets are delivered to upper layers in the correct order through reordering. For 6G, the TCP protocol is expected to remain the primary choice for most services, making it essential to support for in-order delivery.

For out-of-order delivery, real-time services, such as immersive communication, will continue to be important services supported by 6G. The 6G RAN protocol should maintain the capability to provide low latency by supporting out-of-order delivery. The choice between in-order and out-of-order delivery can be determined by the network based on the specific requirements of the services.

With the above considerations in mind, the 6G protocol stack should continue to support both in-order and out-of-order delivery to the upper layer.

Observation 2:	Both in-order delivery and out-of-order delivery are needed due to the requirements from TCP/UDP.

With the support of in-order delivery above and up-surge of services transported by QUIC, further enhancements can also be investigated in the RAN. For example, it could be studied whether and how to address the Head-Of-Line (HOL) blocking issue when multiple upper-layer flows or QUIC streams are mapped to the same radio bearer that requires in-order delivery, i.e., the delivery of packets from one flow may be blocked due to the loss of packets from another flow. We are also aware that SA2 is currently working on this issue, e.g., how to let the UPF be able to identify the QUIC traffic.

Observation 3:		HOL blockage might be an issue for multiple QUIC streams mapped to a single DRB that supports in-order delivery.

2.1.3	Functionalities for L2 reliability

Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability. Within the 6G TR 38.914 [3], the following requirements have been specified

### 5.1.10 Reliability Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability. Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality. The minimum requirement for the reliability is 1-10-5 success probability of transmitting a layer 2 PDU (protocol data unit) of 32 bytes within 1 ms in channel quality of coverage edge for the Urban Macro-HRLLC [test environment], assuming small application data (e.g. 20 bytes application data + protocol overhead).

To achieve the requirement, the following functionalities can be considered.

- Error correction through AQR

The 5G network ensures lossless transmission through the ARQ mechanism by correcting the residual error HARQ in the MAC/PHY layer. For each transmitted packet, the transmitting side verifies its correct transmission based on feedback from the receiving side. If a packet is not transmitted successfully, the transmitting side will continue to retransmit the packet or its segments until it is received successfully. This is particularly crucial when the upper layer is based on the TCP protocol, as TCP requires a reliable connection. If the RAN cannot guarantee lossless transmission, it will lead to frequent TCP retransmissions, significantly degrading network performance.

RLC ARQ mechanism was enhanced in 5G R19. The transmitting RLC side is now allowed to cease retransmissions of a packet when it is determined to be out of date. This avoids unnecessary consumption of radio resources and window stalling that prevents the transmission of subsequent packets. This enhancement improves ARQ performance in terms of resource efficiency and latency. It is particularly beneficial for services that require low latency, high reliability, and high throughput, as it provides retransmission opportunities for packets within the delay budget while avoiding waste of resources due to out-of-date packets.

For 6G networks, high reliability remains a key requirement for immersive and HRLLC, and the performance of TCP, making it essential to support the ARQ mechanism. The ARQ enhancements introduced in R19 could be further refined to enhance flexibility.

Observation 4:	ARQ mechanism ensures high reliability for data transmission. Further improvements can be considered to improve its flexibility.

- Duplication

L2 duplication was introduced in R15, motivated by the requirements of URLLC, which demands extremely high reliability and very low latency. In R15, the network can duplicate packets and transmit the duplicated packets over two different radio paths under CA duplication or DC duplication. As long as the receiver successfully receives one of the duplicated packets, the transmission is considered successful. In R16, the maximum number of legs was increased from 2 to 4 to further improve reliability, to meet the high reliability requirement of 1-10-5 success probability, primarily driven by IIoT.

Given the same high reliability requirement for HRLLC in 6G, L2 duplication could be worth supporting. However, it is important to note that L2 duplication significantly increases radio resource consumption. The RAN should therefore balance the need for reliability with the efficient use of resources.

Observation 5:	L2 duplication is useful to provide high reliability within short delay budget. But the resource consumption needs to be considered.

2.1.4	L2 buffer management

The L2 protocol stack needs to manage the transmitting and the receiving buffer to avoid issues such as buffer overflow or redundant buffering. To achieve the purpose, the following functionalities are supported.

- SDU discard at the Tx side

At the transmitting side, when a packet is received and transmitted, it must be buffered in memory to prepare for possible retransmissions. However, if packets are kept indefinitely, it will eventually lead to memory bloat. To avoid such issues, the protocol stack must discard packets at an appropriate time.

A timer-based discard mechanism is supported in 5G. When the transmitting side receives a packet from the upper layer, it can start a discard timer for the packet. When the timer expires, the corresponding packet is considered out-of-date and can be discarded. Additionally, the 5G protocol also supports discard based on acknowledgement feedback; that is, a packet can be discarded once it is confirmed to have been transmitted successfully.

- Duplicate detection and discarding at the Rx side:

The receiving side may receive some duplicated packets due to the retransmission during handover/recover or duplication over the radio interface. To avoid redundant buffering and unnecessary processing for these packets and provide correct data delivery for application layer, duplicate detection and discarding is supported at the receiver.

The above discard functionalities are essential for the protocol stack to manage the L2 buffer. Both the Tx side and the Rx side should support the corresponding functionality in 6G.

Observation 6:	Packet discard at the Tx side and duplicate detection at the Rx side are essential for buffer management.

2.1.5	Functionalities based on service awareness

To support the emerging XR services, there are also some user plane functionalities developed based on service awareness. These functionalities consider the specific characteristics and requirements of XR services and utilize the PDU set information to improve the performance. The following functionalities are considered.

- PDU set discard

The PDU set discard mechanism is enhanced based on the timer-based SDU discard mechanism, motivated by the integrity transmission requirement of XR services. When integrated transmission is required, all PDUs within the same PDU set must be transmitted within a specified time duration. If any single PDU is lost, the entire PDU set is considered useless. With this concept in mind, the entire PDU set can be discarded as soon as the discard timer of any PDU within the PDU set expires, since the entire PDU set is then considered out-of-date.

- PSI based discard

PSI-based discard is also enhanced based on the timer-based discard mechanism. It leverages the PDU set importance information, which assigns different importance levels to different PDU sets. When the network is congested, it is assumed that PDU sets with lower importance can be discarded first to save radio resources for PDU sets with higher importance. To achieve this effect, a shorter discard timer can be enabled for lower importance PDU sets during congestion.

According to the following agreements excerpted from the RAN1#122bis meeting, one of the important services in 6G, i.e., immersive communication, will use XR traffic models as its starting point. Therefore, the functionalities related to XR awareness, including PDU set discard and PSI-based discard, could still be supported for immersive communication.

Study traffic modelling for evaluations related to immersive communication services including but not limited to advanced XR [e.g., TR22.870] and haptics services, • XR traffic models (in TR 38.838) are considered as starting point. o FFS the detailed modifications on the parameters to the XR traffic model, e.g., higher packet size, higher packet arrival rate, higher packet size deviation, PDB, etc.

Observation 7:	PDU set and PSI based discard are built upon XR awareness, and could be further studied in coordination with SA2/4 and RAN1 progress for 6G immersive communication.

2.1.6	Security aspects

The user plane protocol is required to provide secure transmission by supporting the following functionalities.

- Ciphering

Before transmitting a packet, the transmitter performs ciphering to protect the packet from being intercepted. This is crucial for ensuring the privacy and security of user data.

- Integrity protection

Integrity protection prevents packets from being tampered with or forged. The transmitter applies integrity protection, and the receiver verifies the packet upon receipt. If the verification fails, the packet is discarded to eliminate the risk of an attack.

Security is a critical aspect in 6G networks and the security requirements are primarily discussed in SA3. This can be further evaluated based on the progress in SA3 on the requirements for security functionalities.

Observation 8:	Security functionalities can be studied in coordination with SA3 progress.

2.1.7	Segmentation and reassembly

The AS protocol stack takes SDUs as input and transmits them over the air interface. Given the variable size of the SDUs and the limited radio resources, it is challenging to always transmit complete SDUs without wasting resources. To optimize resource usage, SDU segmentation and reassembly are supported. However, segmentation and reassembly could introduce some processing complexity to the devices and cause some delay. For low capability devices or delay-critical traffics, RAN may consider to disable the functionality of segmentation.

Based on the above analysis, we can identify some essential functionalities for the 6G protocol stack, as well as some potential functions which can be considered for specific services requirements. The following proposals summarize our findings.

Proposal 1:	Support the following functionalities for 6GR UP:

- Header compression (RoHC and EHC) and uplink data compression (UDC)

- In-order and out-of-order delivery

- ARQ with R19 enhancements and duplication for reliability

- SDU discard at the Tx side and duplicate detection at the Rx side

- Segmentation and reassembly

Proposal 2:	Study the following functionalities for 6GR UP in coordination with the other groups:

- PDU-set and PSI-based discard in coordination with RAN1/SA2/SA4 progress

- Ciphering and integrity protection in coordination with SA3 progress

2.2	Reduction of processing complexity and memory requirement

In 5G NR, the user plane stack operates on per IP-packet basis, and the data flow through the user plane Layer 2 protocols for NR is shown below in Figure1. On one hand, the headers are added at each of L2 layers including SDAP, PDCP, RLC, and MAC. On the other hand, security-algorithm invocations, including the Integrity protection is performed for each packet at PDCP layer.

Figure1. Packet construction in Layer 2

With respect to L2 header overhead, taking the voice packet and IPv4 ACK as examples, the L2 header overhead ratio can be up to 19.4% for a voice packet with size of 25 bytes, and the L2 header overhead ratio can be up to 13% for an IPv4 ACK with size of 40 bytes. Hence, for these types of traffics, characterized by very small packet sizes, the L2 header overhead constitutes a significant amount of air interface resources.

The current L2 header overhead can be summarized as below:

| Layer | Overhead |
| --- | --- |
| SDAP | 0 bytes, or 1 byte carrying QFI |
| PDCP | 2 or 3 bytes with 12 bits or 18 bits SN |
| RLC | Up to 5 bytes for different transmission modes with segment or not |
| MAC | 1 byte to 5 bytes containing mainly the F field, L filed and LCID (eLCID) field |

Table2. Summary of the header overhead in layer2

Furthermore, in terms of security algorithm, integrity protection for DRB has been introduced in NR. Specifically, the MAC-I field with 32 bits length is added at the end of each PDCP SDU for DRB configured with integrity protection. Adding MAC-I field for each packet will consume more resources, especially for the case where the packet size is small. Additionally, both ciphering and IP algorithm currently need to be initiated for each packet since the COUNT value in the PDCP is one input for both algorithms. This results to high processing complexity.

In our understanding, PDCP concatenation can potentially enhance the resource efficiency and reduce the security algorithm processing complexity. Multiple packets can be concatenated into one single PDCU SDU through PDCP concatenation, so that the header consumption from each L2 over the air can be reduced, and the integrity protection can be performed based on the concatenated PDCP SDUs instead of adding MAC-I for each of the packets. Also, security algorithms only need to be initiated once for concatenated PDCP SDUs instead of for each packet. Hence, we propose the following

Proposal 3:	RAN2 to study the benefit and mechanism for PDCP concatenation.

In NR, the receiver-side determines the end of one MAC subPDU and the start of the next MAC subPDU based on the contents of MAC subheader. Specifically,

- For variable-sized MAC CE and MAC SDU, the MAC subheader includes the L field to indicate the length of the corresponding MAC SDU or MAC CE. The UE can directly determine the end of the MAC subPDU and the start of the next MAC subPDU based on this L field.

- For fixed-sized MAC CEs, the L field is not carried in MAC subheader, so the UE cannot directly determine the length of the fixed-sized MAC CE based on the MAC subheader alone. For fixed-sized MAC CE, UE needs to maintain the mapping between LCID and the length of the fixed-sized MAC CE to obtain the size of MAC subPDU.

The issue is that, upon receiving a fixed-sized MAC CE, UE needs to look up the maintained mapping relationship to determine the length of the fixed-sized MAC CE. This introduces additional processing complexity. More importantly, the UE would need to continuously update the maintained mapping relationship to add newly defined f MAC CEs after each protocol version update, regardless of whether UE supports these newly defined fixed-sized MAC CEs. However, this is difficult to achieve, especially if the mapping relationship is maintained through hardware.

Based on the above analysis, we can know that not including the L field in the MAC subheader for the fixed-sized MAC CE saves only a small bit air-interface overhead but significantly increases the processing and implementation complexity. In 6G, to simplify processing and implementation complexity, we can consider to carry the L field in all MAC subheaders.

Observation 9:	Not including the L field in the MAC subheader for the fixed-sized MAC CE saves only a small bit overhead but significantly increases the processing and implementation complexity.

In addition, for the MAC PDU format, there is another issue related to pipe-line processing. Let's first review the evolution of the MAC PDU format from LTE to NR. In LTE, all MAC headers are located at the beginning of the entire MAC PDU as shown in the Figure 2. By contrast, in NR, the MAC PDU format is changed to the interleaved structure as shown in Figure 3. The intention to introduce the new MAC PDU structure in NR is to enable pipe-line processing, which facilitates the handling of MAC subPDUs on the receiver side and the transmitter side. At the transmitter side, pre-processing is allowed so that the transmitting side is able to generate MAC subPDUs before the UL grant. Meanwhile, the receiver side can directly parse and process a MAC subPDU right after this MAC subPDU is received, instead of having to wait for the whole MAC PDU to be received as in LTE.

Figure 2, MAC PDU format in LTE

Figure 3, MAC PDU format in NR

However, two protocol-design constraints prevent pipe-line processing realization as following:

- PHY Layer Limitation: At the physical layer of RX side, there are double CRC checks (i.e., CB CRC and TB CRC) for one TB and only TB-level delivery is supported. At the RX, the PHY layer firstly performs CB CRC check for each CB, then if all CBs passed their respective CB CRC checks, the PHY layer proceeds to perform a TB CRC check, and delivers the TB to MAC layer, only if TB CRC check is successful. That is to say, if the CB CRC check fails for even a single CB within the TB, the PHY layer doesn’t deliver the entire TB to MAC layer.

- MAC Layer Limitation: At the MAC layer, MAC PDU decoding is a sequential, chain-like process. If any MAC subPDU fails to be received, UE cannot find the boundaries of subsequent MAC subPDUs and subsequent decoding cannot be performed (i.e., this decoding process will be interrupted). Additionally, the boundaries of MAC subPDUs are not aligned with those boundaries of CBs. In this case, even if the CB-level delivery is implemented, for example, the successfully received CB2 is delivered to MAC layer, MAC layer cannot decode MAC subPDUs in CB2 if the reception of the preceding CB1 fails as shown in Figure 4, because MAC layer cannot find the boundary of the first MAC subPDU in CB2, which is contained in CB1, and thus cannot decode the subsequent MAC subPDU.

Figure 4, Relationship between MAC subPDU and CB

Due to the above two constraints, in NR, pipe-line processing for MAC subPDU cannot be achieved even if the interleaved structure of MAC PDU is supported. That is to say when some CBs in a TB were not successfully received, the MAC subPDUs in the successfully received CBs within the TB could not be parsed and processed. This results in increased service latency, especially for error tolerant services, for example, token communication and PDU sets encoded with AL-FEC for immersive communications. In addition, the UE needs to store the entire MAC PDU, which increases UE memory overhead.

Observation 10:	The intention of pipe-line processing in NR was not really achieved due to the double CRC checks and chain-like decoding of MAC PDU.

Proposal 4:	RAN2 to study the benefit and mechanisms for MAC PDU format enhancement.

2.2	SN assignment and retransmission

In the legacy NR, SN assignments are performed at both the RLC and PDCP layers in the L2 protocol.

- In the PDCP layer, the SN is used as input for ciphering and integrity protection algorithms. It is also used for duplicate detection in PDCP duplication, reordering in in-order delivery, and loss-less handover for AM DRB.

- For AM RLC, when a complete RLC SDU is transmitted, the SN assigned by the RLC is used for RLC status reporting and ARQ retransmission.

- For both UM and AM RLC, when a segmented RLC SDU is transmitted, the SN assigned by the RLC is used for reassembly.

However, as can be observed from Figure 1 above, there is a one-to-one correspondence between RLC and PDCP SDUs. Having both PDCP SN and RLC SN for a single SDU may lead to inefficiency in the system. On one hand, this results in increased Uu interface overhead. On the other hand, the transmitting/receiving device needs to manage packets separately at the PDCP and RLC layers, requiring two separate Tx/Rx windows at each layer, which may cause implementation complexity.

A straightforward approach to address this issue is to employ a shared SN. When PDCP and RLC reside within the same entity, the RLC SN and PDCP SN are inherently aligned for each data PDU. Therefore, transmitting only a single SN over the Uu interface is feasible and can simplify the protocol operation.

However, in the case of shared SN, supporting DC may present some challenges. For DC, the PDCP entity resides on the MN and splits the data to both the RLC on the MN and the RLC on the SN. With the unified SN, each RLC entity related to the split bearer will only receive a portion of the SNs, leading to SN gaps at the receiving RLC entity. To address these SN gaps, enhancements would be needed to avoid window stalling issues. For example, one potential method is to utilize the R19 RLC AM enhancement, which allows the Rx RLC to consider unreceived packets as successfully received when they are out-of-date.

Observation 11:	Sharing single SN between RLC and PDCP layers can reduce the overhead and complexity. Further study would be needed for its feasibility in case of DC.

Furthermore, the Tx/Rx window can be simplified as well. Currently, both PDCP and RLC receivers maintain separate receiving windows. At the PDCP layer, the window is primarily used for reordering, while the RLC window is used for reassembly and retransmission control. The separate receiving windows at the PDCP layer and the RLC layer can lead to inefficiencies. For example, when a packet is discarded by the PDCP layer due to the expiration of the corresponding reordering timer, the RLC layer is not aware of this and may continue to retransmit the packet. Additionally, with two separate windows, both PDCP and RLC need to maintain state variables and timers to manage the windows, which can increase some processing complexity.

The reordering and reassembly operations could have been performed under a single receiving window, as in the case of LTE RLC and PDCP. With some proper enhancements, a single window can provide flexible and unified management of the receiving operations, including reordering, reassembly, and retransmission control.

Observation 12:	Maintaining single Tx/Rx window can potentially reduce the complexity and improve the efficiency.

ARQ mechanism added on top of HARQ has been discussed under Section 2.1.3. In the legacy 5G, at the RLC layer, retransmission is performed at the RLC SDU or RLC SDU segment level to eliminate the residual error of HARQ retransmission. At the PDCP layer, retransmission is only performed at the SDU level only when PDCP is re-established, for example, during handover and PDCP data recovery. Although they served for different scenarios, it requires both PDCP and RLC to implement the retransmission mechanisms and buffer the packets, which will cause additional implementation complexity and memory consumption.

Currently, we do not see justification to remove either retransmission scheme. One possible study direction is to determine if retransmission in one layer could cover all purposes. This may reduce the complexity of the user plane protocol. However, additional aspects also need to be considered, such as the placement of the segmentation function and security during handover.

Observation 13:	Unified mechanism for RLC and PDCP retransmission may simplify the L2 protocol, while it needs to be further studied on other UP impacts.

Proposal 5:	RAN2 to study the mechanism to reduce the processing complexity of SN assignment, L2 reception window and retransmission.

2.3	Energy efficiency

Our considerations on energy efficiency design for 6G are detailed in the companion contribution [4]. In this document, we focus on the user plane functionality that enhances UE energy savings.

For time-domain energy-saving techniques, the fundamental principle is to reduce the PDCCH monitoring duration. Since Release 15, NR has introduced multiple power-saving features to achieve this objective, as summarized in the tables below.

| PS features | Description |
| --- | --- |
| DRX | The UE periodically wakes up to monitor the PDCCH during the on-duration timer, and the periodicity is controlled by the DRX cycle. |
| DCP | The UE periodically wakes up to check for DCP signals. Upon detecting a wake-up indication in the DCP, the UE starts the next on-duration timer and continues PDCCH monitoring throughout this timer period. If DCP is received indicating not to wake up, the UE omits the next on-duration timer and skips PDCCH monitoring accordingly. |
| LP-WUS | The UE periodically wakes up to detect LP-WUS signals using its low-power receiver. Upon successfully receiving an LP-WUS, the UE activates its main radio and begins PDCCH monitoring. For PDCCH monitoring control, two distinct approaches are defined: Option 1-1: When LP-WUS is detected, the UE initiates the DRX on-duration timer and monitors PDCCH throughout this timer period. Option 1-2: When LP-WUS is detected, the UE starts a dedicated PDCCH monitoring timer and performs PDCCH monitoring during this specific timer duration. |

Table3. Summary of UE power saving feature in time domain in 5G

In the CONNECTED state, C-DRX, which is inherited from LTE, serves as the basic UE power-saving feature and widely adopted in commercial networks. However, due to the dynamic and sporadic nature of data traffic, the legacy C-DRX mechanism—with its fixed on-duration—struggles to adapt to varying traffic patterns. As a result, the UE must wake up during every DRX cycle to monitor the PDCCH in on-duration periods, even when no downlink data is scheduled for it. To mitigate such unnecessary PDCCH monitoring, DCP and LP-WUS were introduced to reduce "blank" C-DRX Active Time. In Release 19, further enhancements were made through LP-WUS (i.e., LP-WUS Option 1-2), which offers greater flexibility. This approach not only improves UE energy efficiency but also reduces the latency for data transmission.

In RAN1#122bis meeting, RAN1 has agreed to study the DL WUS for 6G energy efficiency improvement as below:

Agreement Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects: - Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other) - Measurements and/or synchronization. - System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS. - RRC states - Other functionalities

Given the advantage of the DL WUS triggered on-demand PDCCH monitoring (i.e., like the DCP/LP-WUS mechanism in NR) over the legacy C-DRX, RAN2 can take it as the baseline for UE energy saving mechanism in 6G Day 1 and further study related aspects.

Proposal 6:	For discontinuously PDCCH monitoring, RAN2 to study DL WUS triggered PDCCH monitoring, i.e., not rely on DRX cycle and on-duration timer.

3	Conclusion

We summarize the observations and proposals in this contribution as follows:

Overall UP functions analysis:

Observation 1:	Header compression and data compression functionalities are useful for improving spectrum efficiency and network coverage, but also causes additional processing complexity.

Observation 2:	Both in-order delivery and out-of-order delivery are needed due to the requirements from TCP/UDP.

Observation 3:		HOL blockage might be an issue for multiple QUIC streams mapped to a single DRB that supports in-order delivery.

Observation 4:	ARQ mechanism ensures high reliability for data transmission. Further improvements can be considered to improve its flexibility.

Observation 5:	L2 duplication is useful to provide high reliability within short delay budget. But the resource consumption needs to be considered.

Observation 6:	Packet discard at the Tx side and duplicate detection at the Rx side are essential for buffer management.

Observation 7:	PDU set and PSI-based discard are built upon XR awareness, and could be further studied in coordination with SA2/4 and RAN1 progress for 6G immersive communication.

Observation 8:	Security functionalities can be studied in coordination with SA3 progress.

Proposal 1:	Support the following functionalities for 6GR UP:

- Header compression (RoHC and EHC) and uplink data compression (UDC)

- In-order and out-of-order delivery

- ARQ with R19 enhancements and duplication

- SDU discard at the Tx side and duplicate detection at the Rx side

- Segmentation and reassembly

Proposal 2:	Study the following functionalities for 6GR UP in coordination with the other groups:

- PDU-set and PSI-based discard in coordination with RAN1/SA2/SA4 progress

- Ciphering and integrity protection in coordination with SA3 progress

Processing complexity and memory requirement:

Proposal 3:	RAN2 to study the benefit and mechanism for PDCP concatenation.

Observation 9:	Not including the L field in the MAC subheader for the fixed-sized MAC CE saves only a small bit overhead but significantly increases the processing and implementation complexity.

Observation 10:	The intention of pipe-line processing in NR was not really achieved due to the double CRC checks and chain-like decoding of MAC PDU.

Proposal 4:	RAN2 to study the benefit and mechanisms for MAC PDU format enhancement.

SN assignment and retransmission:

Observation 11:	Sharing single SN between RLC and PDCP layers can reduce the overhead and complexity. Further study would be needed for its feasibility in case of DC.

Observation 12:	Maintaining single Tx/Rx window can potentially reduce the complexity and improve the efficiency.

Observation 13:	Unified mechanism for RLC and PDCP retransmission may simplify the L2 protocol, while it needs to be further studied on other UP impacts.

Proposal 5:	RAN2 to study the mechanism to reduce the processing complexity of SN assignment, L2 reception window and retransmission.

Energy efficiency:

Proposal 6:	For discontinuously PDCCH monitoring, RAN2 to study DL WUS triggered PDCCH monitoring, i.e., not rely on DRX cycle and on-duration timer.

4	References

[1]. R2#131bis Chair Note.

[2]. 3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[3]. 3GPP TR 38.914: " Study on 6G Scenarios and Requirements".

[4]. R2-2508519, Discussion on energy efficiency for network and UE, Huawei, HiSilicon