3GPP TSG-RAN WG2 Meeting #132						    R2-2508145

Dallas, US, 17th – 21st November 2025

Agenda item:	10.2.1

Source:			MediaTek Inc.

Title:	UE capability framework considerations for 6GR

Document for:	Discussion and Decision

- Introduction

As the agreements on UE capabilities in RAN2#131bis meeting [1]:

Agreements on UE capabilities for next meeting

- For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

- Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

- Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

In this contribution, we discuss the pain points, design considerations and R2 study directions.

- Discussion

  - Closing the loop of UE capability framework

Continue our appeals in [2], we take a different angle to look at the current 5G landscape and the potential risks of migrating to 6G in terms of UE capability framework.

Figure. 1: The loop of UE capability framework.

- Some high-level design principles from the 5G NR UE capability framework can be the starting assumptions for 6G CAP study, such as UE capability storage at CN such that they can be retrieved by RAN without additional radio interface overhead, source adapts to target principle, etc.

While the current loop of 5G UE capability framework is closed (as shown in the Figure.1) keeping the balance from the system operation perspective, the ever-growing UE capability information size leads to the recurring problems (as follows) threatening the balance:

- Memory consumption

- Signaling load and overhead (across the interfaces among different entities) with processing latency (at RRC layer)

- Interoperability issues

    - Network storage impacts

Memory consumption addresses both the network memory footprint for a single UE, the aggregated / accumulated memory footprint of all UEs in RRC_CONNECTED or RRC_INACTIVE states in the network side.

In 5G NR, this is one of practical deployment factor affecting the network deployment. But the most CN and RAN nodes had been virtualized or cloud-native nowadays so that we tend to believe that the memory consumption may not be a concern in practice. It is an essential start to know whether it is really a concern in the practical deployments and the survey results could steer the RAN2 direction and scope of studying the solutions.

- Considering the growing infrastructure cloudification trend, the UE capability storage cost (and side effects) caused by the large capability information size might not be a concern in practice.

- RAN2 to send LS to RAN3, SA2 and cc CT1 to check the RAN2 study assumption “Prioritize UE capability availability over conserving storage in the network” with them and ask for any feedback or confirmation on whether it could be the target principle throughout the 6G SI phase.

    - Capability request filter design concerns

It is also our observations and proposal [2] aiming for eliminating the concerns pertaining to signaling load and overhead and related processing latency. A lightweight UE capability reporting in 6GR should be targeted, of which a key enabler is capability request filter design.

A clear problem in 5G NR is that the way the filter has been used deviated from the purpose it was meant to serve. The filter was introduced to be used to reduce the UE capability reporting size: using the FreqBandList consisting of the deployed bands prioritized in the order of the list (i.e., the first-listed band is the highest one to be considered in composing the band combination capabilities).

However, in practice, other performance requirements (low signaling load over Uu; connection establishment delay avoidance) and design considerations (BC and FS consistency across different containers; filter-labeled capability updates to the CN storage), the network typically keeps the least filter setting which means ALL the deployed bands used in the PLMN are included for the UE to report capabilities. As a result, the UE capability reporting size is not really reduced by filtering so that the UEs can barely report a complete NR RAT container in the UECapabilityInformation to the network. But if the capability information is incomplete in the UE reporting, the effort of avoiding re-acquiring the UE capability as much as possible is in vain.

- Most 5G networks request a large number of (i.e., too many) bands in the filter, defeating the purpose of the filter. As a result, the UE capability reporting size is not well controlled. A similar approach would not translate well to control the capability size in 6G.

The use of the FreqBandList has been strongly tied to the whole UE capability framework (the aforesaid “closed loop”) so that the UE and network impacts would be significant if we replaced or dropped the current band-based method. With that in mind, we think the additional filtering in finer granularity level could be considered. Other filtering approach(es) should also be considered.

- RAN2 to study different filter mechanisms with fine-grained parameters to allow the network to acquire UE capabilities selectively in a way that effectively controls the capability reporting size, without impairing the usefulness of what is reported in view of the (local) RAN situation.

    - Interoperability issues

In the context of the side effects caused by the size explosion issue, the most common interoperability issue is handover failure because of:

- Incomplete UE capability information

Lack of essential capability parameters for being handed over to the target system.

- Oversized Source-to-Target Container

The UE capability is not successfully delivered during preparation phase.

We leave the other examples to the section 2.3 below for further discussion.

  - Dynamic change of UE capability

In this section we discuss the UE capability updating mechanism which is indispensable to the “closed loop” of the UE capability framework. The drawbacks of UAI mechanism were skipped given that RAN2 had discussed these aspects in the last (Prague) meeting. We also shared the pain points of UAI [2], quoted hereafter again.

- UAI had evolved into a broad framework with diverse parameters and lacks immediate effect transaction, making it unsuitable for urgent UE capability updates during RRC_CONNECTED.

- The sole UE-initiated capability updating procedure was suitable only when the UE is in no hurry to do capability change without concerning UX.

    - Use cases

Case 1: Thermal mitigation

This is the representative scenario that prompted the need for a basic RRC procedure for updating UE capability in RRC_CONNECTED [2]. This has been widely used in the field.

From UE perspective, the mitigation actions could be realized by any BC envelope reduction so that we think all the BC-related BB and RF capability parameters should be allowed to be updated over time or upon critical events (typ. overheating). This proposition originates from the fact that the UE has the best in-device knowledge to know which capability in what use case to be changed would have the best synergy gain.

Case 2: Power saving

Theoretically the best policy for saving UE battery life is to transmit the user data of full buffer traffic as quickly as possible. But it is not necessarily the case when delivering the data packets in a sparse traffic scenario. There is still power saving gain if the UE could cease partial transmission by underusing the modem circuits or sub-systems. Vendors have adopted such microengineering approach in the fierce market competition with picky appetite in UE power saving performance, but the limitations on UE capability updating restrict how much it can be used in practice.

Depending on the application(s) running in the UE at any one time, capabilities should be able to change dynamically which could help reducing battery consumption of the device. E.g. the UE could exploit full capabilities in case of a very demanding usage scenario e.g. XR gaming, and much less in case of just browsing the web or simply fetching background data (when the screen is off for instance). With devices such as smartphones that are increasingly versatile, the ability to adapt to the actual usage of the device can help reduce the demand on the battery.

Case 3: Resource sharing

The modern handheld UEs equipped with cellular and Wi-Fi modems has even worked more powerful than an entry-level desktop. However, how to make the wideband antenna to be best placed is still an engineering challenge due to the form factor restriction and interference mitigation design. The most typical case is that the cellular modem could share the same antennas with the Wi-Fi modem. Other RF and/or BB resource parameters could also be shared when designing a compact, cost-effective integrated solution.

From the lower layer (i.e., L2, PHY and RF) perspective, if the simultaneous reception and/or transmission among the multiple modems is needed, the UE internal coordination, arbitration and even preemption are the inevitable results of modem resource sharing. Therefore, the dynamic capability change mechanism will be needed as a necessity for doing UE-controlled capability coordination between both connections.

For high-layer application perspective, the MPTCP techniques have been supported in the mainstream mobile OS SDK for a long time and it provided a particular service type enabling that when Wi-Fi connection is too slow, the App will automatically switch to the cellular link. Sharing of radio components means that a change of operations on one modem (e.g., Wi-Fi) can affect the shared resources and require a change of capability on the other modem (e.g., cellular).

- There’re clear demands in implementing purpose-built features relying on the dynamic UE capability change mechanism however current standard restricts the potentials in practice.

Neither does the network predict what event is about to happen in the UE side. RAN2 should be able to commit now based on the inputs already seen, for further study in 6G SI:

- RRC will have an updating mechanism of UE capabilities without preliminary network permission during RRC_CONNECTED state.

    - Network impact considerations

Keeping the UE capability in the network side is two-fold: local cache in RAN nodes and long-term storage in CN nodes for serving different purposes like UE context retention, or UE capability retrieval, etc. These procedures are linked to the connection and mobility management in RRC_CONNECTED state, as well as RRC_INACTIVE state (in case the latter is carried forward into 6G).

Some key aspects and assumptions are listed as follows:

- The way to inform the changed capability parameters should be flexible and scalable for covering at least the existing use cases (e.g., all the L2, BB and RF related parameters).

- The path of propagating the changed UE capability should cover both RAN and CN given that we need to consider the deployment where no direct BS-BS connection.

- The new UE capability change mechanism will need to cover the lower layers capability parameters and will also need to have an interoperable RAN and CN side procedures to accommodate the context transfer and consistency requirements.

- RAN2 to further study how to accommodate for both RAN and CN cases with corresponding triggering methods of the capability change mechanism. Study how to coordinate a RAN updating among multiple BSs to handle mobility after the change.

- The network implementation should not be constrained and not be forced to a single approach option by the RRC signaling design. For example, for following what capability parameters are changed in said transaction, the network can either choose to track every change incrementally (by separate IEs), or just to consider all the changes signaled in the UE capability container (as a whole) from scratch like repeating what it does when entering the RRC_CONNECTED.

In addition, other use case that may require a change of other capabilities beyond the predefined parameters set described above (in the first bullet of this section). To allow directly signaling the changed capability parameter in the 6GR UE capability container and reuse the existing UE capability transfer procedure is unified, safer and future-proof.

- To allow the flexibility and the future-proofness, the changed capability parameters could be signaled either in separate IEs or in the 6GR UE capability container.

- RAN2 to study how to enable the UE to trigger a complete re-fetch of the capability.

By NR UAI practice, we understood that most networks can handle some BB and RF capabilities change during RRC_CONNECTED already. In other words, within such capability scope, the network can follow the change request from the UE and react to it accordingly. Yet we need to study a unified, scalable, and robust procedure to cater to a larger capability scope for the new change mechanism.

We also share the same concern from when we designed the UAI mechanism that we need to allow some processing and synchronization time for the intra-RAN or RAN-CN communication. But while important, it is not the most critical topic to discuss now; this can be addressed at a later stage once having a clearer guideline or principle for designing the new UE capability change mechanism.

- RAN2 to study how to manage the network behavior, with the understanding that the UE capability *will* change and the network will honor the UE choice to keep it connected or let it detach/reattach.

  - IODT issues in the field and solution direction

- As the NR system evolved, the features and the configurations were getting more and more complicated so that the most important and fundamental spirit of ensuring the backward compatibility was sometimes overlooked.

Ensuring interoperability, avoiding market fragmentation, and securing backward compatibility are important requirements that must met with 6G (the latter in subsequent releases post Rel-21). This is particularly important for the first release, as the baseline, starting from the 6G day-1. Any backward compatibility issue against the baseline release, should be brought to 3GPP for the resolution, instead of going to a proprietary solution.

    - Lesson learnt from an IODT issue

In short, when different number of MIMO layer is reported for multiple band entries in CA band combos and then the MIMO layer number is not explicitly configured by the network, ambiguity arises causing problems in preparing soft buffer according to RAN1 spec.

LTE case:

- LS on intra-band contiguous CA and intra-band non-contiguous CA (R2-153948)

- Reply LS for clarification on MIMO capability (R1-167914)

- Spec impacts

  - MIMO layer indication was needed for RI bit width determination and soft buffer calculation (36.331 CR1885r2 / 36.212 CR172r1; 36.331 CR1966r1; 36.331 CR1861r2 / 36.212 CR0175r2; 36.331 CR2343r1).

NR case:

- Reply LS on MIMO layer configuration (R1-1813952)

- Spec impacts

  - Clarification for setting of maxLayersMIMO in LTE during EN-DC operation (36.331 CR3740).

  - Per serving cell MIMO layer configuration (38.331 CR0543r2) + IOT bit (38.331 CR0295r4)

  - LBRM procedure update with new RRC parameter for maxRank MIMO layers (38.212 CR0005).

Though there are mandatory Rx diversity requirements for some NR high bands (specified in RAN4 specs), the MIMO layer capability parameter is still optional in the UECapabilityInformation. It is our observation that whether the network explicitly configures the downlink MIMO layer during the consecutive RRCReconfiguration transactions for operating CA and/or DC was implemented as a configurable feature, instead of a mandatory feature reacts upon whether the IOT bit is indicated or not.

We feel that the day-1 feature (with thoughtful design) is key to the homogeneous deployment and it would be very useful to avoid the possible interoperability issues in a non-homogeneous deployment.

- A day-2 solution even with a IOT bit tailored to the interoperability issue could not really solve the problem once and for all.

And this interoperability issue occurred not only in the SA network, but also in the NSA network, more specifically an EN-DC network consists of 2 different vendors.

- Vendor identification is useful but not the elixir of interoperability.

In a high-level view, the UE had been requested to report it’s full (and static) radio access capabilities to the network. For aiming to resolve the IODT problem, if any further capability related information is needed, then it would be very likely the ones from the network side.

Furthermore, when we refer to the “IODT problem”, it only means the interoperability issues over the Uu but never including the NVIOT (Network Vendor IOT) part. However, the network deployment and configuration are very flexible thus more complicated today and involve the NVIOT part as well. Providing only the vendor ID does not help resolving every IODT issue, e.g., the aforesaid max. MIMO layer configuration issue, in the field.

- RAN2 to study what network capability information is needed for resolving the identified IODT issues in addition to the network vendor ID. Other information or configuration as the solution is not precluded.

- Conclusion

In previous sections we made the following observations and proposals:

Observation 1	Some high-level design principles from the 5G NR UE capability framework can be the starting assumptions for 6G CAP study, such as UE capability storage at CN such that they can be retrieved by RAN without additional radio interface overhead, source adapts to target principle, etc.

Observation 2	Considering the growing infrastructure cloudification trend, the UE capability storage cost (and side effects) caused by the large capability information size might not be a concern in practice.

Observation 3	Most 5G networks request a large number of (i.e., too many) bands in the filter, defeating the purpose of the filter. As a result, the UE capability reporting size is not well controlled. A similar approach would not translate well to control the capability size in 6G.

Observation 4	UAI had evolved into a broad framework with diverse parameters and lacks immediate effect transaction, making it unsuitable for urgent UE capability updates during RRC_CONNECTED.

Observation 5	The sole UE-initiated capability updating procedure was suitable only when the UE is in no hurry to do capability change without concerning UX.

Observation 6	There’re clear demands in implementing purpose-built features relying on the dynamic UE capability change mechanism however current standard restricts the potentials in practice.

Observation 7	The new UE capability change mechanism will need to cover the lower layers capability parameters and will also need to have an interoperable RAN and CN side procedures to accommodate the context transfer and consistency requirements.

Observation 8	To allow the flexibility and the future-proofness, the changed capability parameters could be signaled either in separate IEs or in the 6GR UE capability container.

Observation 9	As the NR system evolved, the features and the configurations were getting more and more complicated so that the most important and fundamental spirit of ensuring the backward compatibility was sometimes overlooked.

Observation 10	A day-2 solution even with a IOT bit tailored to the interoperability issue could not really solve the problem once and for all.

Observation 11	Vendor identification is useful but not the elixir of interoperability.

Proposal 1	RAN2 to send LS to RAN3, SA2 and cc CT1 to check the RAN2 study assumption “Prioritize UE capability availability over conserving storage in the network” with them and ask for any feedback or confirmation on whether it could be the target principle throughout the 6G SI phase.

Proposal 2	RAN2 to study different filter mechanisms with fine-grained parameters to allow the network to acquire UE capabilities selectively in a way that effectively controls the capability reporting size, without impairing the usefulness of what is reported in view of the (local) RAN situation.

Proposal 3	RRC will have an updating mechanism of UE capabilities without preliminary network permission during RRC_CONNECTED state.

Proposal 4	RAN2 to further study how to accommodate for both RAN and CN cases with corresponding triggering methods of the capability change mechanism. Study how to coordinate a RAN updating among multiple BSs to handle mobility after the change.

Proposal 5	RAN2 to study how to enable the UE to trigger a complete re-fetch of the capability.

Proposal 6	RAN2 to study how to manage the network behavior, with the understanding that the UE capability *will* change and the network will honor the UE choice to keep it connected or let it detach/reattach.

Proposal 7	RAN2 to study what network capability information is needed for resolving the identified IODT issues in addition to the network vendor ID. Other information or configuration as the solution is not precluded.

- References

- R2-2508002	RAN2#131bis Meeting Report

- R2-2507147	On 6GR UE capability