3GPP TSG-RAN WG2 Meeting #132	R2-2508300

Dallas, USA, 17 – 21 November 2025

Agenda item:	10.3.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	6G QoS, QoE, and Service awareness

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

The envisioned QoS framework for 6G requires intelligence to deal with all the challenges and future scenarios foreseen in the coming decade. 5G QoS framework sets a good foundation for 6G QoS framework, and its extensions in the last 5G-A releases -initially for services like XR- can be beneficial beyond their original use cases.

Therefore, it is preferred to inherit the main design principles of the 5G QoS framework. Focusing on RAN-level QoS, 5G DRB concept, which defines the packet treatment on the wireless interface in terms of scheduling, queuing, shaping, etc., can be reused in 6G.

However, simply inheriting these 5G framework might not be sufficient. To position 6G as a truly QoE-driven generation, the focus for 6G shall be on how to actively and immediately enhance the user experience. The existing 5G QoE Measurement Collection (QMC) framework, enabling the RAN to read QoE metrics (RAN visible QoE (RVQoE)), is a good procedure for network optimization and benchmarking purposes. However, 6G should explore how to improve QoE awareness and management in the RAN, aiming to improve the levels of satisfaction rather than just passively collecting data.

Having this in mind, this contribution discusses potential area for enhancement from RAN2 perspective.

# 2	Discussions

## 2.1	Awareness

As we embark into the 6G era, new services and requirements from new applications need to be accommodated into 6G networks. New applications and services such as XR and AI exhibit different traffic patterns compared to eMBB and other traditional traffic types. XR services are a clear example of how application awareness (with the introduction of PDU sets, assistance information and DSR) can help optimizing QoS flow provisioning, and other RAN operations like UE power saving configurations.  Still, for 6G, XR use cases continue to evolve further, bringing more challenges to RAN. Furthermore, AI services are expected to become more relevant for 6G, with an explosive growth of AI applications and its integration in different types of devices. Currently in SA1 TR 22.870, different use cases and service requirements for 6G are studied, where AI is included as one of the key technologies. What makes AI services especially challenging is the unpredictability of the traffic [R2-2508237]. Given that RAN has scarce resources, it is crucial that RAN obtains the proper means to adequately manage its operations to provide the best possible user experience. However, it is observed that the visibility of the application requirements and characteristics is currently limited [RP-251125]. This is reflected by the following requirement from TSG-RAN, captured in TR 38.914:

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

Below are the few examples of service awareness information that can be leveraged in RAN:

1.	QoS requirements and related performance metrics preferences;

2.	Traffic characteristics e.g., direction, periodicity, burst size, burst arrival time, burst ending time, etc;

3.	Relationship/relevance of packets within the traffic flow e.g., burst, PDU Set information, etc;

4.	Application-level QoE metrics;

5.	UE assistance information and DSR.

Although most items will impact RAN2, we must acknowledge that they belong to areas handled by other WGs. For instance, traffic characteristics will be discussed by SA4, the provision of those by SA2 and RAN3... And thus, RAN2 should first focus on its area of expertise: the UE reporting to RAN.

Proposal 1: RAN2 initially focuses service awareness discussions on collecting information from the UE to help the RAN handling dynamic traffic behaviour.

## 2.2	Control of QoS flow

As identified in R2-2507615, one of a key 6G RAN capabilities is monitoring – which is referred to observability in RP-252227, driven by a diverse mix of services like XR and AI/sensing that would require stringent and highly granular QoS guarantees. Fulfilling these demanding requirements necessitates the network’s evolution to a more intelligent and predictive system. Therefore, in 6G QoS framework, traditional monitoring that checks for known problems using pre-defined metrics may be no longer sufficient.

Instead, a deeper level of observability would be needed to understand the real-time impact of radio conditions on data transfer. This specifically demands that the RAN can efficiently track the individual packets, noting that QoS flows may experience different delays based on their assigned priorities and scheduling. For instance, accurate per-flow delay measurement would be essential to make sure that the intended QoS differentiation is being delivered.

Proposal 2: RAN2 study for enhanced observability for 6G to enable accurate measurement of per QoS flow performance, including packet delay.

Up to 5G, LCP controls uplink transmission based on a priority and PBR. A key limitation, however, is that the current LCP logic lacks the means to not prioritize traffic that has already exceeded its configured PBR. This can lead to fairness issue, as this excess traffic continues to compete with other traffic while they should be treated as best effort.

A related issue could be the enforcement of MFBR. While MFBR is a key QoS parameter, its enforcement currently relies solely on the RAN’s scheduling decision: The gNB uses the QoS profile to prioritize and adjust scheduling so a QoS flow can stay between GFBR and MFBR. The UE, being unaware of this upper bound, cannot perform any necessary uplink traffic shaping (e.g., de-prioritizing, or applying an upper bound). This results in the UE making unnecessary transmission even beyond MFBR, leading to resource inefficiency.

In 6G, RAN should be able to directly configure a maximum bit rate on the UE, which not only enhances overall resource efficiency but also provides a more proactive way for QoS enforcement, complementing gNB side scheduling decision.

Proposal 3: For 6G, the LCP procedure should be enhanced to support and enforce a RAN-configurable upper bound, enabling direct MFBR enforcement at the UE side.

## 2.3	Congestion handling

The importance of UL congestion control has been highlighted during mass events, where UL traffic spikes stress not only the UL capacity but also the capacity of essential control channels like the PRACH, PUCCH, and PDCCH.

While NR has introduced several mechanisms for UL congestion control, such as the Recommended Bit Rate procedure and PSI-based SDU discard, a fundamental limitation remains: the procedures used for UL transmissions such as SR, BSR, LCP, and CG are not inherently congestion-aware. This may lead to inefficiencies in congested scenarios. For instance, mechanisms like the Recommended Bit Rate procedure only adjust the data rate; it does not prevent a UE from initiating transmission attempts. Similarly, PSI-based discard acts after UL data has already arrived in the buffer, thus failing to prevent the initial signaling overhead for transmission attempts.

Traditionally, the gNB has managed congestion by providing fewer UL grants in response to the scheduling requests. However, in severe congestion, such signalling from the UEs itself may become a significant burden on an already overloaded system. In our view, the root cause is that those UL transmission mechanisms are configured semi-statically via RRC and operate independently of the real-time congestion level. This static approach would lead to suboptimal behavior in severe congestion, where the network is forced to handle even low-priority when it would be far more beneficial to suspend them to guarantee the QoS for UEs and more important traffics.

Proposal 4: RAN2 study congestion-aware control of UL transmission procedures.

## 2.4	Control information handling

In uplink, MAC CEs carry various types of control information such as buffer status, power headroom, etc. With each release, new MAC CEs have been introduced to support new features, for instance, Beam, NR-unlicensed, sidelink, etc. This continuous expansion introduces two complexities: the assignment of LCIDs and handling of transmission priority.

Currently, the specification defines a fixed priority order for every MAC CE relative to all other MAC CEs and LCH data. While this provides a clear hierarchy for LCP, it lacks adaptability and cannot be adjusted to varying network condition or diverse service requirements. For instance, in some scenarios like severe congestion, the network may want to prioritize LCH data over less critical MAC CEs. Rel-19 priority adjustment somehow allows dynamic re-prioritization but only for LCH data.

The prioritization challenge could extend beyond the MAC layer. In PDCP and RLC, control information is also transmitted via control PDUs, e.g., Status Report. These control PDUs are currently prioritized over data PDUs, but only within the same RB. This kind of local prioritization would make sense when the control PDU is essential for that specific bearer. However, for system efficiency (e.g., to quickly move a stalled RLC window or efficiently prevent unnecessary retransmissions), it could be also beneficial to prioritize even a lower-priority RB’s control PDU over the high-priority RB’s data.

In addition, increase of MAC CEs has led to a shortage of available LCID values. As discussed in [R2-2507157], in Rel-16, eLCID was introduced to address this issue, but at the cost of extra one byte overhead in the MAC subheader. This also raised complicated discussions about which MAC CEs should continue using the LCID and which should rely on eLCID. A key problem is that many LCID or eLCID values remain reserved for the MAC CEs, and some appear unnecessarily reserved for the UE, given that the MAC CEs are typically only relevant when a specific feature is actually configured for the UE.

In 6G, the need of enhanced observability required by the network will likely increase the volume and variety of uplink control information, adding more challenges to the prioritization and resource allocation discussed above. Therefore, it would be important to design a scalable and flexible framework for handling of control information from the beginning.

Proposal 5: RAN2 study how to provide more flexible prioritization and efficient resource management for uplink control information, including MAC CEs and RLC/PDCP control PDUs.

# 3	Conclusion

In this contribution, we outlined potential areas for enhancements to achieve more improved QoS/QoE and focused areas for service awareness from RAN2 perspective, with the following proposals:

Proposal 1: RAN2 initially focuses service awareness discussions on collecting information from the UE to help the RAN handling dynamic traffic behaviour.

Proposal 2: RAN2 study for enhanced observability for 6G to enable accurate measurement of per QoS flow performance, including packet delay.

Proposal 3: For 6G, the LCP procedure should be enhanced to support and enforce a RAN-configurable upper bound, enabling direct MFBR enforcement at the UE side.

Proposal 4: RAN2 study congestion-aware control of UL transmission procedures.

Proposal 5: RAN2 study how to provide more flexible prioritization and efficient resource management for uplink control information, including MAC CEs and RLC/PDCP control PDUs.