3GPP TSG-RAN WG2 Meeting #132	R2-2508314

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.17

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on support of TDD mode for NB-IoT-NTN

WID/SID:	IoT_NTN_TDD - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At RAN #106 the work item on NB-IoT TDD NTN mode [1] was agreed with the following objective:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4]

In this contribution we discuss remaining issues as well as RIL N022 to support TDD NTN mode, which may impact RAN2 specification.

# 2	Discussion

## 2.1	SPS collision

At RAN2 #129bis it was agreed that when a SPS resource is in a non-U NB-IoT subframe, the UE postpones the SPS resource to the next valid UL subframe. Furthermore, at RAN1 #120 it was agreed that NPUSCH is mapped per current specifications, i.e. postponed to valid NB-IoT UL subframes.

RAN1 #120 agreement The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” from the specifications - NPUSCH is mapped as per current specifications. - NOTE: “non-U NB-IoT subframes” are subframes not contained within the set of U=8 usable consecutive uplink subframes in the TDD structure.

This is a good starting point to handle the sparse uplink resource availability for the individual UE, but from the network point of view it may still happen that multiple UEs postpone their semi-static uplink resource to the next valid uplink subframe and therefore there will be resource scarcity and in the worst-case scenario even collisions.

Observation 1: when multiple UEs postpone their SPS resource to the next valid UL subframe there may be collisions between different UEs.

One may argue that the postponement issue can be addressed by network implementation, for example, by allocating different UEs in different frequency domain resources.  However, since the NB-IoT has only 1 PRB (12 tone) when the subcarrier spacing is 15 kHz, it is not possible to allocate two UEs with different frequency domain resource if the UE is allocated with 12 tone transmission. It is worth noting that any UE having a SPS resource, which occurs within the time between a first UL occasion and the start of a second UL occasion, will all postpone the resource to the first valid UL subframe of the second UL occasion.

At RAN2 #131bis it was discussed the SPS interval can be a multiple of 90 ms. However, it is not clear how this solves the problem, since the “offset” of the interval is not adjustable in NB-IoT. The current MAC specification defines

## 5.10.2 Uplink After a Semi-Persistent Scheduling uplink grant is configured, the MAC entity shall: - if twoIntervalsConfig is enabled by upper layer: - set the Subframe_Offset according to Table 7.4-1. - else: - set Subframe_Offset to 0. - consider sequentially that the Nth grant occurs in the TTI for which: - subframe SPS is used: - (10 * SFN + subframe) = [(10 * SFNstart time + subframestart time) + N * semiPersistSchedIntervalUL + Subframe_Offset * (N modulo 2)] modulo 10240. - slot or subslot SPS is used: - (10 * SFN * sTTI_Number_Per_Subframe + subframe * sTTI_Number_Per_Subframe + sTTI_number) = [(10 * SFNstart time * sTTI_Number_Per_Subframe + subframestart time * sTTI_Number_Per_Subframe + sTTIStartTimeUl) + N * semiPersistSchedIntervalUL-sTTI+ Subframe_Offset * (N modulo 2) * sTTI_Number_Per_Subframe] modulo (10240 * sTTI_Number_Per_Subframe). Where SFNstart time, subframestart time and sTTIStartTimeUl are the SFN, subframe and sTTI_number, respectively, at the time the configured uplink grant were (re-)initialised. The sTTI_Number_Per_Subframe is 6 when subslot TTI is configued and 2 when slot TTI is configured for short TTI operation. sTTI_number refers to the index of the short TTI, i.e., index of subslot or slot within the subframe. … For NB-IoT UEs, BL UEs or UEs in enhanced coverage SFNstart time and subframestart time refer to SFN and subframe of the first transmission of PUSCH where configured uplink grant was (re-)initialized.

Thus the SFNstart time and subframestart time depend on the first NPUSCH transmission. This means that if the first NPUSCH transmission occurs (due to postponement) in the first subframe of the 8 consecutive subframes, any subsequent transmission will occur in the same first subframe of a later set of 8 consecutive subframes, and thus the collision issue is not resolved.

Observation 2: For NB-IoT UEs the subframestart time refers to the first transmission of the NPUSCH and a 90 ms SPS periodicity will therefore not resolve collisions on its own.

We propose RAN2 to discuss alternative solutions to handle the SPS postponement issue. We think one useful way forward can be to define that some SPS resources are not postponed to the next valid UL subframe (i.e. the first subframe of the 8 consecutive uplink subframes) but further offset to another valid UL subframe. For example, the network may configure a UL-SPS-postponed-offset subframe as part of the SPS configuration, which the UE applies if it postpones the SPS transmission. In this case the UE postpones the transmission to the configured subframe instead of the first subframe of the 8 consecutive uplink subframes. The UL-SPS-postponed-offset subframe can be in the range 1-8 to distribute the UEs across the 8 uplink subframes. Note, the use of a configured offset entails common UE and network understanding of which resource will be used. We provide a TP to MAC specification for this solution in the annex.

Proposal 1: RAN2 to define a SPS postponement offset which the UE applies (in case the SPS transmission gets postponed) to offset the SPS transmission to one of the 8 consecutive uplink subframes. The annex provides the TP.

## 2.2	RAN1 LS reply on OCC for IoT NTN TDD mode

At RAN1 #122bis, RAN1 made a reply (R1-2508110) to RAN2 regarding the use of OCC for IoT NTN TDD mode as follows:

Reply to RAN2: From RAN1’s perspective, introducing OCC for NB-IoT NTN TDD mode requires RAN1 specifications change. Therefore, RAN1 cannot confirm RAN2’s assumption “that OCC defined for NB-IoT NTN Rel-19 can be supported for NB-IoT NTN TDD mode without any RAN1 spec impact”.

Since both RAN1 and RAN2 are already in the release 19 maintenance phase we don’t consider it feasible to request RAN1 specification change to support OCC for IoT NTN TDD mode.

Proposal 2: Orthogonal Cover Codes are not supported for IoT NTN TDD mode.

## 2.3	RIL N022

For duration of NPUSCH segment transmission, RAN1 agreed the 2 and 4 ms segments are not applicable to IoT TDD NTN other than at the beginning of the transmission. In RIL N022, we propose to capture that in the field description for npusch-TxDuration.

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N022 | IoTTDD |  | Update the field description for npusch-TxDuration to reflect RAN1 agreement |  | Nokia (Ping Yuan) |  | V008 |

In RAN2-131bis discussion, the RIL N022 was not agreed, as RAN4 has been asked to confirm the RAN1 agreement referenced in LS R2-2507710/ R1-2508064. Consequently, company thinks RAN4’s confirmation is required before the field description can be updated. Therefore, we propose postponing RIL N022 until RAN4’s confirmation is available.

Proposal 3: Postpone RIL N022 until RAN4 confirms the RAN1 agreement on segment compensation.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: when multiple UEs postpone their SPS resource to the next valid UL subframe there may be collisions between different UEs.

Observation 2: For NB-IoT UEs the subframestart time refers to the first transmission of the NPUSCH and a 90 ms SPS periodicity will therefore not resolve collisions on its own.

Proposal 1: RAN2 to define a SPS postponement offset which the UE applies (in case the SPS transmission gets postponed) to offset the SPS transmission to one of the 8 consecutive uplink subframes. The annex provides the TP.

Proposal 2: Orthogonal Cover Codes are not supported for IoT NTN TDD mode.

Proposal 3: Postpone RIL N022 until RAN4 confirms the RAN1 agreement on segment compensation.

# Reference

[1] RP-243293, Iridum, “Revised WID on introduction of IoT-NTN TDD mode”, December 2024

# Annex: TP to 36.321

### 5.10.2 Uplink After a Semi-Persistent Scheduling uplink grant is configured, the MAC entity shall: - if twoIntervalsConfig is enabled by upper layer: - set the Subframe_Offset according to Table 7.4-1. - else: - set Subframe_Offset to 0. - consider sequentially that the Nth grant occurs in the TTI for which: - subframe SPS is used: - (10 * SFN + subframe) = [(10 * SFNstart time + subframestart time) + N * semiPersistSchedIntervalUL + Subframe_Offset * (N modulo 2)] modulo 10240. - slot or subslot SPS is used: - (10 * SFN * sTTI_Number_Per_Subframe + subframe * sTTI_Number_Per_Subframe + sTTI_number) = [(10 * SFNstart time * sTTI_Number_Per_Subframe + subframestart time * sTTI_Number_Per_Subframe + sTTIStartTimeUl) + N * semiPersistSchedIntervalUL-sTTI+ Subframe_Offset * (N modulo 2) * sTTI_Number_Per_Subframe] modulo (10240 * sTTI_Number_Per_Subframe). Where SFNstart time, subframestart time and sTTIStartTimeUl are the SFN, subframe and sTTI_number, respectively, at the time the configured uplink grant were (re-)initialised. The sTTI_Number_Per_Subframe is 6 when subslot TTI is configued and 2 when slot TTI is configured for short TTI operation. sTTI_number refers to the index of the short TTI, i.e., index of subslot or slot within the subframe. Except for NB-IoT, for TDD, the MAC entity is configured with semiPersistSchedIntervalUL shorter than 10 subframes, the Nth grant shall be ignored if it occurs in a downlink subframe or a special subframe. <omit part> The MAC entity shall clear the configured uplink grant immediately after first transmission of SPS confirmation MAC Control Element triggered by the SPS release. NOTE: Retransmissions for Semi-Persistent Scheduling can continue after clearing the configured uplink grant. For NB-IoT UEs, BL UEs or UEs in enhanced coverage SFNstart time and subframestart time refer to SFN and subframe of the first transmission of PUSCH where configured uplink grant was (re-)initialized. In the event of a resource conflict between multiple UL SPS configurations configured with Uplink Semi-Persistent Scheduling V-RNTI, the UE behaviour is undefined. In the event of a resource conflict in the same serving cell between the initial transmision within a configured grant bundle from multiple different UL SPS configurations configured with Uplink Semi-Persistent Scheduling C-RNTI, the UE behaviour is undefined. For IoT NTN TDD mode, if the UL SPS resource overlaps with non-U NB-IoT subframes, the UE shall postpone the UL SPS resource to the configured UL-SPS-postponed-offset subframe in the set of the next valid 8 UL subframes. For NB-IoT UEs, a configured uplink grant shall be used only for BSR or SPS confirmation transmission, and skipUplinkTxSPS is implicitly configured.