3GPP TSG-WG2 Meeting #132	R2-2508340

Dallas, USA, November 17 – 21, 2025

Agenda Item:	8.13.2

Source:	Ericsson

Title:	Discussion on RIL E049

Document for:	Discussion, Decision

# 1	Introduction

The RIL E049 and E053 are concerning the same issue, but aim for different solutions to address the same issue. In this paper, we discuss the RIL issues E049.

# 2	Discussion

## 2.1 RIL E049

This issue is described in the following

-- ASN1START

-- TAG-SL-L2RELAYUE-CONFIG-START

SL-L2RelayUE-Config-r17 ::=        SEQUENCE {

sl-RemoteUE-ToAddModList-r17       SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-RemoteUE-ToAddMod-r17    OPTIONAL,    -- Need N

sl-RemoteUE-ToReleaseList-r17      SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-DestinationIdentity-r16  OPTIONAL,    -- Need N

...,

[[

sl-U2U-RemoteUE-ToAddModList-r18   SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-U2U-RemoteUE-Config-r18   OPTIONAL,    -- Need N

sl-U2U-RemoteUE-ToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIdentity-r16   OPTIONAL     -- Need N

]]

}

SL-RemoteUE-ToAddMod-r17 ::=       SEQUENCE {

sl-L2IdentityRemote-r17            SL-DestinationIdentity-r16,

sl-SRAP-ConfigRelay-r17            SL-SRAP-Config-r17                                                      OPTIONAL,    -- Need M

...,

[[

sl-SRAP-ConfigRelay-ToAddModList-r19     SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-Config-ToAddMod-r19      OPTIONAL,      -- Need R

sl-SRAP-ConfigRelay-ToReleaseList-r19    SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-ConfigId-r19               OPTIONAL      -- Need R

]]

}

The ToAddModList and the ToReleaseList have wrong need code. The need code should be corrected as Need N instead of Need R.

The issue on the need code has been recently corrected in the rapporteur CR (CR5537).

Therefore, this paper focuses on the remaining aspects of the RIL 049.

- The issue of the need code for RIL 049 has been corrected in the rapporteur CR.

In addition, in the relay UE procedures on Remote UE release as captured in clause 5.3.5.15.2, upon reception of the sl-RemoteUE-ToReleaseList, the relay UE shall release configurations of each remote UE (and indirectly connected child UEs) whose SL-DestinationIdentity value included in the sl-RemoteUE-ToReleaseList. It is observed that the procedure texts on release configurations are missing (see texts highlighted in the following texts in yellow, exerted from the RRC spec v19.0.0).

##### 5.3.5.15.2	L2 U2N or U2U Remote UE Release

The L2 U2N Relay UE shall:

1>	if the release is triggered by reception of the sl-RemoteUE-ToReleaseList:

2>	for each SL-DestinationIdentity value included in the sl-RemoteUE-ToReleaseList:

3>	if the current UE has a PC5 RRC connection to a L2 U2N Remote UE with SL-DestinationIdentity:

4>	indicate upper layers to trigger PC5 unicast link release.

The L2 U2U Relay UE shall:

1>	if the release is triggered by reception of the sl-U2U-RemoteUE-ToReleaseList:

2>	for each SL-DestinationIdentity value included in the sl-U2U-RemoteUE-ToReleaseList:

3>	if the current UE has a PC5-RRC connection to a L2 U2U Remote UE with this SL-DestinationIdentity:

4>	release the configuration associated with the L2 U2U Remote UE.

- Upon reception of the sl-RemoteUE-ToReleaseList, the relay UE shall release configurations of each remote UE whose SL-DestinationIdentity value included in the sl-RemoteUE-ToReleaseList and its indirectly connected child UEs. It is observed that the procedure texts on release configurations are missing.

Therefore, it is recommended to include the procedure texts on releasing configurations of each remote UE (and its indirectly connected child UEs) whose SL-DestinationIdentity value included in the sl-RemoteUE-ToReleaseList.

- Include the procedure texts in clause 5.3.5.15.2 with respect to releasing configurations of each remote UE whose SL-DestinationIdentity value included in the sl-RemoteUE-ToReleaseList and its indirectly connected child UEs.

Further, in the relay UE procedures as captured in clause 5.3.5.15.3, there is no (clear/explicit) procedure texts on Addition/Modification of indirectly connected child UEs (whose sl-SRAP-ConfigId included in the sl-SRAP-ConfigRelay-ToAddModList or sl-SRAP-ConfigRelay-ToReleaseList). Thus, the behaviours of the relay UE to achieve delta signaling/configuration of indirectly connected child are unclear. (see texts highlighted in the following texts in yellow, exerted from the RRC rapporteur CR (CR 5537)).

##### 5.3.5.15.3	L2 U2N or U2U Remote UE Addition/Modification

The L2 U2N Relay UE shall:

1>	if no SRAP entity has been established:

2>	establish a SRAP entity as specified in TS 38.351 [66];

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is not part of the current UE configuration (L2 U2N Remote UE Addition):

2>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigRelay and sl-SRAP-ConfigRelay-ToAddModList if applicable;

2>	if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-ChannelPC5 is configured:

3>	release SL-RLC1, if established;

3>	associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-ChannelPC5 with SRB1;

2>	else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-ChannelPC5 is not configured)

3>	if SL-RLC1 is not established:

4>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 and associate it with the SRB1;

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is part of the current UE configuration (L2 U2N Remote UE modification):

2>	modify the configuration in accordance with the sl-SRAP-ConfigRelay and sl-SRAP-ConfigRelay-ToAddModList/ sl-SRAP-ConfigRelay-ToReleaseList if applicable;

Note 1: The network will not configure Intermediate U2N Relay UE’s direct child and its indirectly connected child UEs under this direct child with SL-RLC1 using both default and with dedicated configurations simultaneously.

The L2 U2U Relay UE shall:

1>	if no SRAP entity has been established:

2>	establish a SRAP entity as specified in TS 38.351 [66];

1>	for each target L2 U2U Remote UE indicated in sl-L2IdentityRemoteUE value included in the sl-U2U-RemoteUE-ToAddModList that is not part of the current UE configuration (target L2 U2U Remote UE Addition):

2>	for each source L2 U2U Remote UE indicated in sl-SourceUE-Identity in accordance with one entry of the sl-SourceRemoteUE-ToAddModList:

3>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigU2U;

1>	for each target L2 U2U Remote UE indicated in sl-L2IdentityRemote value included in the sl-U2U-RemoteUE-ToAddModList that is part of the current UE configuration (target L2 U2U Remote UE modification):

2>	for each source L2 U2U Remote UE indicated in sl-SourceUE-Identity included in the sl-SourceRemoteUE-ToReleaseList (source L2 U2U Remote UE Release):

3>	release the configuration associated with the source L2 U2U Remote UE;

2>	for the source L2 U2U Remote UE indicated in sl-SourceUE-Identity included in the sl-SourceRemoteUE-ToAddModList that is not part of the current UE configuration (source L2 U2U Remote UE Addition):

3>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigU2U;

2>	for the source L2 U2U Remote UE indicated in sl-SourceUE-Identity included in the sl-SourceRemoteUE-ToAddModList that is part of the current UE configuration (source L2 U2U Remote UE modification):

3>	modify the configuration in accordance with the sl-SRAP-ConfigU2U;

- In the relay UE procedures as captured in clause 5.3.5.15.3, there is no (clear/explicit) procedure texts on Addition/Modification of indirectly connected child UEs whose sl-SRAP-ConfigId included in the sl-SRAP-ConfigRelay-ToAddModList or sl-SRAP-ConfigRelay-ToReleaseList. Thus, the behaviours of the relay UE to achieve delta signaling/configuration of indirectly connected child are unclear.

Therefore, it is recommended include explicit procedure texts on Addition/Modification of indirectly connected child UEs whose sl-SRAP-ConfigId included in the sl-SRAP-ConfigRelay-ToAddModList or sl-SRAP-ConfigRelay-ToReleaseList in clause 5.3.5.15.3.

- In the relay UE procedures as captured in clause 5.3.5.15.3, include explicit procedure texts on Addition/Modification of indirectly connected child UEs whose sl-SRAP-ConfigId included in the sl-SRAP-ConfigRelay-ToAddModList or sl-SRAP-ConfigRelay-ToReleaseList.

We have also prepared corresponding TPs in section 5.

- Adopt TP1 captured in section 5.1 and TP2 captured in section 5.2.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The issue of the need code for RIL 049 has been corrected in the rapporteur CR.

Observation 2	Upon reception of the sl-RemoteUE-ToReleaseList, the relay UE shall release configurations of each remote UE whose SL-DestinationIdentity value included in the sl-RemoteUE-ToReleaseList and indirectly connected child UEs whose sl-SRAP-ConfigId included in the sl-SRAP-ConfigRelay-ToAddModLis. It is observed that the procedure texts on release configurations are missing.

Observation 3	In the relay UE procedures as captured in clause 5.3.5.15.3, there is no (clear/explicit) procedure texts on Addition/Modification of indirectly connected child UEs whose sl-SRAP-ConfigId included in the sl-SRAP-ConfigRelay-ToAddModList or sl-SRAP-ConfigRelay-ToReleaseList. Thus, the behaviours of the relay UE to achieve delta signaling/configuration of indirectly connected child are unclear.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Include the procedure texts in clause 5.3.5.15.2 with respect to releasing configurations of each remote UE whose SL-DestinationIdentity value included in the sl-RemoteUE-ToReleaseList and indirectly connected child UEs whose sl-SRAP-ConfigId included in the sl-SRAP-ConfigRelay-ToAddModList.

Proposal 2	In the relay UE procedures as captured in clause 5.3.5.15.3, include explicit procedure texts on Addition/Modification of indirectly connected child UEs whose sl-SRAP-ConfigId included in the sl-SRAP-ConfigRelay-ToAddModList or sl-SRAP-ConfigRelay-ToReleaseList.

Proposal 3	Adopt TP1 captured in section 5.1 and TP2 captured in section 5.2.

# 4	References

[1] RP-241609, “New WID on NR sidelink multi-hop relay”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17 – 20 June 2024.

# 5	Text proposals for RIL E049

## 5.1	TP1 in accordance with Proposal 1

The following texts are exerted from the TS 38.331 v19.0.0

##### 5.3.5.15.2	L2 U2N or U2U Remote UE Release

The L2 U2N Relay UE shall:

1>	if the release is triggered by reception of the sl-RemoteUE-ToReleaseList:

2>	for each SL-DestinationIdentity value included in the sl-RemoteUE-ToReleaseList:

3>	if the current UE has a PC5 RRC connection to a L2 U2N Remote UE with SL-DestinationIdentity:

4>	indicate upper layers to trigger PC5 unicast link release.

4 > release the configurations associated with the L2 U2N Remote UE and its indirectly connected child UEs.

The L2 U2U Relay UE shall:

1>	if the release is triggered by reception of the sl-U2U-RemoteUE-ToReleaseList:

2>	for each SL-DestinationIdentity value included in the sl-U2U-RemoteUE-ToReleaseList:

3>	if the current UE has a PC5-RRC connection to a L2 U2U Remote UE with this SL-DestinationIdentity:

4>	release the configuration associated with the L2 U2U Remote UE.

## 5.2	TP2 in accordance with Proposal 2

The following texts are exerted from the RRC rapporteur CR (CR CR5537)

##### 5.3.5.15.3	L2 U2N or U2U Remote UE Addition/Modification

The L2 U2N Relay UE shall:

1>	if no SRAP entity has been established:

2>	establish a SRAP entity as specified in TS 38.351 [66];

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is not part of the current UE configuration (L2 U2N Remote UE Addition):

2>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigRelay ;

2>	for each sl-SRAP-ConfigId included in the sl-SRAP-ConfigRelay-ToAddModList that is not part of the current UE configuration (L2 U2N indirectly connected child UE Addition):

3>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigRelay-r19;

2>	if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-ChannelPC5 is configured:

3>	release SL-RLC1, if established;

3>	associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-ChannelPC5 with SRB1;

2>	else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-ChannelPC5 is not configured)

3>	if SL-RLC1 is not established:

4>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 and associate it with the SRB1;

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is part of the current UE configuration (L2 U2N Remote UE modification):

2>	modify the configuration in accordance with the sl-SRAP-ConfigRelay;

2>	for each sl-SRAP-ConfigId included in the sl-SRAP-ConfigRelay-ToReleaseList that is part of the current UE configuration (L2 U2N indirectly connected child UE release):

3>	release the configuration associated with the L2 U2N indirectly connected child UE;

2>	for each sl-SRAP-ConfigId included in the sl-SRAP-ConfigRelay-ToAddModList that is part of the current UE configuration (L2 U2N indirectly connected child UE modification):

3>	modify the configuration in accordance with the sl-SRAP-ConfigRelay-19;

Note 1: The network will not configure Intermediate U2N Relay UE’s direct child and its indirectly connected child UEs under this direct child with SL-RLC1 using both default and with dedicated configurations simultaneously.

The L2 U2U Relay UE shall:

1>	if no SRAP entity has been established:

2>	establish a SRAP entity as specified in TS 38.351 [66];

1>	for each target L2 U2U Remote UE indicated in sl-L2IdentityRemoteUE value included in the sl-U2U-RemoteUE-ToAddModList that is not part of the current UE configuration (target L2 U2U Remote UE Addition):

2>	for each source L2 U2U Remote UE indicated in sl-SourceUE-Identity in accordance with one entry of the sl-SourceRemoteUE-ToAddModList:

3>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigU2U;

1>	for each target L2 U2U Remote UE indicated in sl-L2IdentityRemote value included in the sl-U2U-RemoteUE-ToAddModList that is part of the current UE configuration (target L2 U2U Remote UE modification):

2>	for each source L2 U2U Remote UE indicated in sl-SourceUE-Identity included in the sl-SourceRemoteUE-ToReleaseList (source L2 U2U Remote UE Release):

3>	release the configuration associated with the source L2 U2U Remote UE;

2>	for the source L2 U2U Remote UE indicated in sl-SourceUE-Identity included in the sl-SourceRemoteUE-ToAddModList that is not part of the current UE configuration (source L2 U2U Remote UE Addition):

3>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigU2U;

2>	for the source L2 U2U Remote UE indicated in sl-SourceUE-Identity included in the sl-SourceRemoteUE-ToAddModList that is part of the current UE configuration (source L2 U2U Remote UE modification):

3>	modify the configuration in accordance with the sl-SRAP-ConfigU2U;