3GPP TSG-RAN2 Meeting #132	R2-2508418

Dallas, USA Nov. 17th – 21st, 2025

Source: 			ZTE Corporation

Title: 	Discussion on RRM measurement event prediction

Agenda item:	9.3.3

Document for: 	Discussion and Decision

- Introduction

In the Rel-20 AI-Mob WI [1], the following objects are proposed:

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

In this contribution, we discuss the LCM functionality management for RRM measurement event prediction.

- Discussion

  - Principles

In the AI-Mob R19 SI, both indirect measurement event prediction and direct measurement event prediction are considered. For indirect measurement event prediction, the RRM measurement prediction is performed first to obtain prediction result(s). Afterwards, the event is determined based on the predicted results and/or actual results without further involvement of an AI/ML model. For direct measurement event prediction, the measurement event prediction is predicted via an AI/ML model directly. In our understanding, compared with direct measurement event prediction, indirect measurement event prediction is simpler and more straightforward. And based on the WID, whether direct measurement event prediction can be supported will be determined in RAN#112. So, for normative work on measurement event prediction, we shall prioritize the normative work on indirect measurement event prediction.

Proposal 1: RAN2 to prioritize the normative work on indirect measurement event prediction in AI-Mob.

As mentioned above, for indirect measurement event prediction, the event is determined based on the predicted results without further involvement of an AI/ML model. The same LCM framework can be applied for both RRM measurement prediction and measurement event prediction (including applicability reporting, inference configuration and reporting).

Proposal 2: The same LCM framework is applied for both measurement prediction and measurement event prediction (including applicability reporting, inference configuration and reporting).

The inference parameters in the RRM measurement prediction can be reused in the measurement event prediction. And the UE can know the inference configuration is for RRM measurement prediction or measurement event prediction via the value of reportType. Specifically, if the reportType is set to ‘periodical’, the inference configuration is for RRM measurement prediction; if the reportType is set to ‘eventTriggered’, the inference configuration is for measurement event prediction.

Proposal 3: The UE knows the inference configuration is for measurement prediction or measurement event prediction based on the reportType in ReportConfigNR. (i.e. for measurement prediction, the reportType is set to ‘periodical’; for measurement event prediction, the reportType is set to ‘eventTriggered’).

  - Event prediction specific aspects

In this part, we mainly discuss the aspects that specific to measurement event prediction.

Inference configuration:

In the Rel-19 SI, the following agreements are achieved:

RAN2 131: For RRM measurement event prediction, the following are included in the inference configuration: - The length of PW for the associated AIML model - Event-related information (event type and event-related parameters)

In the current RRC spec, the EventTriggeredConfig is used to provide event type (e.g. A1~A6) and event-related parameters. This configuration can be reused in event prediction.

Proposal 4: For measurement event prediction, the existing EventTriggeredConfig IE can be reused to indicate the event prediction related parameters (e.g. event type, threshold, offset, TTT…).

Inference report

For inference report, in Rel-19 SI, we have achieved the following agreements:

RAN2 131: For RRM measurement event prediction, the event prediction report can include: - RRM measurements (For normative phase to define what types of measurements) - Time-related information about the predicted event (e.g., the time/window of occurrence) For measurement event determined, when the UE is configured with temporal domain case B, and frequency domain prediction, UE reports measurement event by following an approach similar legacy procedure. This can be achieved without spec impact for measurement reporting. FFS for normative phase if additional information needs to be added

For measurement event prediction, the UE sends the inference report to the network when the event has not been triggered, so the UE needs to explicitly indicate when the event is predicted to occur in the inference report.

Proposal 5: For measurement event prediction, the UE explicitly indicates when the event is predicted to occur in the inference report.

For measurement event prediction, since the prediction window is sliding, the UE performs prediction multiple times. For example, as shown in the figure below, at time instance 1, the UE predicts that event will be triggered at time instance 6 and reports it to the network, then at time instance 2, the UE performs prediction and predicts that the event will be triggered. We are wondering whether the UE still needs to send inference results to the network? If the UE reports every time that the event is predicted (e.g. at time instance 1 and at time instance 2), the signaling overhead is increased. Therefore, RAN2 needs to further discuss how to avoid frequent report. For example, when the new prediction results are different from previous prediction results, the UE sends report to the network.

Figure: an example for measurement event prediction with case A

Observation 1: Since PW is sliding, there are multiple time instances at which the measurement event is predicted to be triggered. If the UE sends inference report every time a measurement event is predicted, the signaling overhead is increased.

Proposal 6: To minimize signaling overhead from frequent inference report due to sliding PW, RAN2 must specify the conditions for the UE to send updated inference results.

  - UE capability

Based on the WID, measurement event A1~A6 are in the scope of measurement event prediction. Considering the event is determined based on the prediction results from RRM measurement prediction, there is no need to introduce different UE capabilities to indicate the support of different event type, respectively. And, regarding RRM measurement prediction and measurement event prediction, although the same AI model can be used, the UE needs to post-process the predicted results in the measurement event prediction. So, we can introduce a single UE capability to indicate the support of measurement event prediction.

Proposal 7: Introduce a single UE capability for indicating the support of measurement event prediction, no differentiation of different event types.

With the UE capability for measurement event prediction, the support of different measurement event prediction use cases can be indicated via UE capability for measurement event prediction and UE capability for different RRM measurement prediction use case. Specifically:

- The support of measurement event prediction with temporal case A can be indicated via UE capability for temporal domain case A and UE capability for measurement event prediction.

- The support of measurement event prediction with temporal case B can be indicated via UE capability for temporal domain case B and UE capability for measurement event prediction.

- The support of measurement event prediction with frequency domain prediction can be indicated via UE capability for temporal frequency domain prediction and UE capability for measurement event prediction.

Observation 2: The network checks whether the UE supports measurement event prediction for a specific use cases (e.g. temporal-domain, frequency) based on the combination of the UE capability for measurement event prediction and the UE capability for the measurement prediction.

So, if the UE supports measurement event prediction, the UE shall be able to perform at least one of prediction scenarios (e.g. temporal case A, temporal case B, frequency domain prediction)

Proposal 8: A UE supporting measurement event prediction shall indicate the support of at least one measurement prediction scenario (e.g. temporal-domain case A, temporal-domain case B, frequency domain).

- Conclusion and proposals

In this contribution, we provide the following observations and proposals:

Observation 1: Since PW is sliding, there are multiple time instances at which the measurement event is predicted to be triggered. If the UE sends inference report every time a measurement event is predicted, the signaling overhead is increased.

Observation 2: The network checks whether the UE supports measurement event prediction for a specific use cases (e.g. temporal-domain, frequency) based on the combination of the UE capability for measurement event prediction and the UE capability for the measurement prediction.

Principles:

Proposal 1: RAN2 to prioritize the normative work on indirect measurement event prediction in AI-Mob.

Proposal 2: The same LCM framework is applied for both measurement prediction and measurement event prediction (including applicable reporting, inference configuration and reporting).

Proposal 3: The UE knows the inference configuration is for measurement prediction or measurement event prediction based on the reportType in ReportConfigNR. (i.e. for measurement prediction, the reportType is set to ‘periodical’; for measurement event prediction, the reportType is set to ‘eventTriggered’).

Event prediction specific aspects

Proposal 4: For measurement event prediction, the existing EventTriggeredConfig IE can be reused to indicate the event prediction related parameters (e.g. event type, threshold, offset, TTT…).

Proposal 5: For measurement event prediction, the UE explicitly indicates when the event is predicted to occur in the inference report.

Proposal 6: To minimize signaling overhead from frequent inference report due to sliding PW, RAN2 must specify the conditions for the UE to send updated inference results.

UE capability

Proposal 7: Introduce a single UE capability for indicating the support of measurement event prediction, no differentiation of different event types.

Proposal 8: A UE supporting measurement event prediction shall indicate the support of at least one measurement prediction scenario (e.g. temporal-domain case A, temporal-domain case B, frequency domain).

- Reference

- RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR