3GPP TSG-RAN2 Meeting #132	R2-2508434

Dallas, USA, 17 - 21 November 2025

Agenda Item:	8.4.1 Organizational

Source: 	Ericsson

Title:  	CN assigned subgroup ID for LP-WUS

Document for:	Discussion and Decision

# Introduction

In this contribution the SA2 questions concerning CN assigned subgroup ID are discussed [1,2].

# Discussion

## Introduction

RAN3 indicated that the CN-assigned subgroup ID range is 0..30 [1].

In case UEID-based subgrouping is not configured, then up to 31 CN-assigned subgroup IDs can be used:

LP-SubgroupConfig-r19 ::=        SEQUENCE {

lp-SubgroupsNumPerPO-r19         INTEGER (1.. maxNrofPagingSubgroupsLP-r19),

lp-SubgroupsNumForUEID-r19       INTEGER (1.. maxNrofPagingSubgroupsLP-r19) OPTIONAL,   -- Need S

...

}

maxNrofPagingSubgroupsLP-r19 INTEGER ::= 31   -- Maximum number of paging subgroups per paging occasion for LP-WUS

lpwus-PoNumPerLo-r19         ENUMERATED {po1, po2, po4}                         OPTIONAL,   -- Need R

But up to 31 subgroup IDs are available only when there is a 1:1 mapping of LO to PO. Up to 4 POs can be mapped to a single LO, in which case the available codepoints are shared between the 4 POs:

- in case 4 POs are mapped to one LO, subgroup ID 0..6 can be used

- in case 2 POs are mapped to one LO, subgroup ID 0..14 can be used

- in case 1 PO is mapped to one LO, subgroup ID 0..30 can be used

SA2 has the following questions concerning the CN-assigned subgroup ID:

a) 	What is the typical area that a PO-to-LO association configuration will apply to? (e.g., is the configuration uniform in a PLMN, different in every cell, etc, and is it mandatory for an LP-WUS capable gNB to support all 3 PO-to-LO association configurations?) This information will help SA2 to understand what kind of solution to propose.

b) 	Whether the paging PO-to-LO association configuration will be changed often, e.g., to save base station energy when the paging load is less?

c) 	Will RAN3 specify the relation between subgroup ID value range and the PO-to-LO association (e.g. based on RAN1 specification)? This could avoid SA2 having to refer to a complex part of TS 38.213.

The mapping of LO to PO(s), i.e. lpwus-PoNumPerLo-r19 IE with value range {po1, po2, po4}, is one of the LP-WUS configuration parameters in the gNB that influence the CN-assigned subgroup ID that can be allocated by CN:

- lp-SubgroupsNumPerPO-r19: 	The number of subgroups configured per PO

- lp-SubgroupsNumForUEID-r19: 	The number of subgroups configured for UEID-based subgroups

## PEI

With PEI it also has been discussed whether the configuration in RAN is homogeneous and whether signalling is needed to coordinate RAN and CN. It was agreed that all the cells in the registration area support the same number of CN-assigned subgroups [4]:

R2 assumes that All the cells within the registration area supports the same number of CN assigned subgroups, i.e. no remapping of CN assigned group ID to RAN subgroup ID (will revisit only if serious issues are found).

There is no signalling for this over NGAP interface, but this information could be handled using OAM signalling. This configuration is not expected to change, i.e. see also the discussion below.

## Answers to the SA2 questions

- What is the typical area that a PO-to-LO association configuration will apply to? (e.g., is the configuration uniform in a PLMN, different in every cell, etc, and is it mandatory for an LP-WUS capable gNB to support all 3 PO-to-LO association configurations?) This information will help SA2 to understand what kind of solution to propose.

Answer to a):

- The LO to PO mapping must be homogeneous at least within the same registration area. But in practice the same configuration in the complete PLMN may be used, because the registration area can be different between UEs. Furthermore the number of subgroups per PO and the number of UEID-based subgroups is also expected to be the same in the complete PLMN.

- The operator may decide on the LO to PO(s) mapping. A gNB supporting LP-WUS is assumed to be support all possible mappings.

b) 	Whether the paging PO-to-LO association configuration will be changed often, e.g., to save base station energy when the paging load is less?

Answer to b):

- The operator may have certain considerations about which LO to PO mapping to configure. For example the number of LP-WUS transmissions are reduced when 1:4 instead of a 1:1 mapping is used. But the false alarm rate increases when more POs use the same LO, i.e. less subgroups are available for each PO. The LO to PO(s) mapping is not expected to change once it is configured.

c) 	Will RAN3 specify the relation between subgroup ID value range and the PO-to-LO association (e.g. based on RAN1 specification)? This could avoid SA2 having to refer to a complex part of TS 38.213.

Answer to c):

- This is up to RAN3 to decide.

RAN2 provides the answers described above to SA2.

A draft LS is provided for further discussion and agreement in the Appendix.

# Summary

RAN2 is kindly asked to discuss the SA2 questions concerning the CN-assigned subgroup ID:

Proposal 1	RAN2 provides the answers described above to SA2.

# References

- R3-255941, LS on allocation of CN assigned subgroup ID for LP-WUS, RAN3, LS out, To: SA2, Cc: RAN2, RAN1, RAN3#129

- S2-2509834, Reply LS on allocation of CN assigned subgroup ID for LP-WUS, SA2, LS out, To: RAN3, RAN2, Cc: RAN1, SA2#171

- R2-2507939, Miscellaneous corrections on RRC for Rel-19 LP-WUS WUR, vivo(rapporteur), CR 38.331, RAN2#131-bis

- R2-2109094, [AT115-e][043][ePowSav] Paging Subgrouping, Nokia, Report, RAN2#115

# Appendix: TP draft LS

3GPP TSG-RAN2 Meeting #132	R2-25xxxxx

Dallas, USA, 17 - 21 November 2025

Title:	DRAFT Reply LS on CN assigned subgroup ID for LP-WUS

Response to:	LS (S2-2509834/R2-2508030) Reply LS on CN assigned subgroup ID for LP-WUS

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	Ericsson [TSG RAN WG2]

To:	SA2, RAN3

Cc:	RAN1

Contact Person:

#### Name:	Martin van der Zee

- E-mail Address:	martin.van.der.zee@ericsson.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 thanks SA2 for the LS on CN assigned subgroup ID for LP-WUS. RAN2 would like to provide the following answers to the questions:

- What is the typical area that a PO-to-LO association configuration will apply to? (e.g., is the configuration uniform in a PLMN, different in every cell, etc, and is it mandatory for an LP-WUS capable gNB to support all 3 PO-to-LO association configurations?) This information will help SA2 to understand what kind of solution to propose.

Answer to a):

- The LO to PO mapping must be homogeneous at least within the same registration area. But in practice the same configuration in the complete PLMN may be used, because the registration area can be different between UEs. Furthermore the number of subgroups per PO and the number of UEID-based subgroups is also expected to be the same in the complete PLMN.

- The operator may decide on the LO to PO(s) mapping. A gNB supporting LP-WUS is assumed to be support all possible mappings.

b) 	Whether the paging PO-to-LO association configuration will be changed often, e.g., to save base station energy when the paging load is less?

Answer to b):

- The operator may have certain considerations about which LO to PO mapping to configure. For example the number of LP-WUS transmissions are reduced when 1:4 instead of a 1:1 mapping is used. But the false alarm rate increases when more POs use the same LO, i.e. less subgroups are available for each PO. The LO to PO(s) mapping is not expected to change once it is configured.

c) 	Will RAN3 specify the relation between subgroup ID value range and the PO-to-LO association (e.g. based on RAN1 specification)? This could avoid SA2 having to refer to a complex part of TS 38.213.

Answer to c):

- This is up to RAN3 to decide.

2. Actions:

To SA2

ACTION: 	RAN2 kindly asks SA2 to take note of the answers provided by RAN2.

3. Date of Next TSG-RAN2 Meetings:

TSG RAN2 Meeting #133		9 – 13 February 2025		Gothenburg

TSG RAN2 Meeting #133-bis	13 – 17 April 2025		EU