3GPP TSG-RAN2 Meeting #132	R2-2508440

# Dallas, USA, Nov 17th – 21st, 2025

Agenda item:		8.11.2 (NR_duplex_evo)

Source:	LG Electronics Inc.

Title: 	Remaining MAC open issues on SBFD

Document for:	Discussion and Decision

1.	Introduction

In the post e-mail discussion on MAC open issues [1], following open issues are identified to be discussed in the next meeting:

Proposal easy to be agreed: [MAC-2] Proposal: RAN2 to send an LS to RAN1, requesting an update to the definition of the first PRACH occasions in TS 38.213, to let it inclusively cover the ROs for cases where tdd-UL-DL-ConfigurationCommon is not applied (e.g., FDD, SUL, TDD without tdd-UL-DL-ConfigurationCommon configured). Issues for further discussion with tdoc contributions in RAN2#132: - MAC-1: Whether/how to address the issue in case RO type switching and Msg1 repetition number fallback are triggered simultaneously. - MAC-2: Content to be captured in the LS for requesting RAN1 to update the definition of the first PRACH occasions in TS 38.213, to let it inclusively cover the ROs for cases where tdd-UL-DL-ConfigurationCommon is not applied.

This document discusses the highlighted open issue on SBFD.

2.	Discussion

[MAC-1] Dependency between RO type switch and Msg1 repetition number increment

In RAN2#131bis meeting [2], it was briefly discussed on whether RO type switch and the Msg1 repetition number increment can be performed simultaneously. However, it was postponed due to different views on whether this change is necessary.

R2-2507003 Discussion on residual issues for MAC spec Huawei, HiSilicon discussion Rel-19 NR_duplex_evo-Core Noted Postponed Proposal for MAC-x: Agree the TP of bypassing the immediate Msg1 repetition number fallback when the UE performs RO type fallback. Discussion - ZTE agree with the issue but want to refine the changes. - Samsung think this could be solved by NW configuration. LG E also think this not critical.

During the post e-mail discussion [1], it was proposed to avoid performing RO type switch and Msg1 repetition number increment at the same time, since the increment of Msg1 repetition number is not necessary due to performing RO type switch.

However, this issue can be avoided by the network configuration of number of RA attempt to perform RO type switch (i.e., preambleTransMaxRO-Type) and to perform the Msg1 repetition number increment (i.e., preambleTransMax-Msg1-Repetition), so performing both RO type switch and Msg1 repetition number increment is not usual case. In addition, even though the UE performs both RO type switch and Msg1 repetition number increment unnecessarily, there is no harm on RA procedure itself, e.g., reliability of RA preamble transmission.

Rather, if the Msg1 repetition number increment is not allowed at the time of RO type switch, the UE will lose an opportunity of Msg1 repetition number increment. For example, assume that RA partition is configured as follows:

- In SBFD RO:

  - Partition A1: [no feature]

  - Partition A2: [Msg1 repetition, RepNum = 2]

- In non-SBFD RO:

  - Partition B1: [no feature]

  - Partition B2: [Msg1 repetition, RepNum = 2]

  - Partition B3: [Msg1 repetition, RepNum = 4]

  - Partition B4: [Msg1 repetition, RepNum = 8]

Then, if the preambleTransMaxRO-Type = preambleTransMax-Msg1-Repetition (for non-SBFD RO) = n4 and the UE selects the partition A2 (SBFD RO, Msg1 repetition) for initial RA attempt, the UE will perform RO type switch when the UE fails RA attempts for 4 time (i.e., PREAMBLE_TRANSMISSION_COUNTER = 5). Then, if the Msg1 repetition number increment is not allowed at this time, the UE will perform the RA procedure using Msg repetition number of 4 (i.e., Partition B2). In this case, only one time of Msg1 repetition number increment is allowed and the UE will never allowed to perform Msg1 repetition with Msg1 repetition number 8, even though RA attempt is failed after the RO type switch. In other words, when the UE performs RO type switch and Msg1 repetition number increment is not allowed at this time, the UE will never use Partition B4, which may cause a reliability issue on RA preamble transmission.

Therefore, it is hard to say that it is beneficial to avoid performing RO type switch and Msg1 repetition number increment at the same time in all cases. However, since the WI is completed, further discussions on detailed UE behaviour case by case should be avoided, unless there is any critical issue on the current behaviour. Given that current behaviour does not have any big harm on RA procedure, it would be better to keep the current behaviour, i.e., performing RO type switch and Msg1 repetition number independently.

Proposal 1. [MAC-1] UE performs RO type switch and Msg1 repetition number increment independently. No spec change is needed.

[MAC-2] RO type for the cases where tdd-UL-DL-ConfigurationCommon is not applied

In current MAC specification [3], the SBFD-aware UE selects the RO type between the first RO (non-SBFD RO) and first RO (SBFD RO), after performing BWP operation and prior to RACH partition selection. Here, if the SBFD RO is not configured (i.e., if neither sbfd-RACH-SingleConfig nor sbfd-RACH-DualConfig is configured), the UE always selects first RO regardless of whether tdd-UL-DL-ConfigurationCommon is configured or not.

(…RO type selection for CFRA cases) 1> else if neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure and either sbfd-RACH-SingleConfig or sbfd-RACH-DualConfig is configured by RRC for the Random Access procedure (as specified in TS 38.331 [5]): (..selection between first RO and second RO for CBRA based on conditions) 1> else: 2> set the RO_TYPE to 1st-RO. 1> select the set of Random Access resources applicable to the current Random Access procedure according to clause 5.1.1b;

Based on the selected RO type, in clause 5.1.2 of TS 38.321, the UE selects the PRACH occasions.

1> else if an SSB is selected above: 2> if the set of Random Access resources associated with Msg1 repetition is selected for this Random Access procedure: (,,, Msg1 repetition case) 2> else: 3> determine the next available PRACH occasion from the PRACH occasions of the selected RO type corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, or ssb-SharedRO-MaskIndex if configured, or indicated by PDCCH, or indicated by the LTM Cell Switch Command MAC CE (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions of the selected RO type according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available PRACH occasion of the selected RO type corresponding to the selected SSB).

In other words, in following case, MAC entity shall select the PRACH occasions of the first RO for CBRA cases, since there would be no SBFD RO:

- Case 1) when the UE is operating in FDD cell;

- Case 2) when the UE is operating in TDD cell, but tdd-UL-DL-ConfigurationCommon is not configured in the cell; or

- Case 3) when the UE selects SUL carrier for this Random Access procedure.

On the other hand, in TS 38.213[4], first PRACH occasion is defined only for the case when tdd-UL-DL-ConfigurationCommon is configured. In other words, for aforementioned cases, the first PRACH occasion is not defined in the physical layer, even though MAC layer selects first RO and indicate the selected RO type to the lower layer.

Prior to initiation of the physical random access procedure, Layer 1 may receive from higher layers an indication to perform a random access procedure using: - first PRACH occasions each including only symbols that are indicated as uplink or flexible by tdd-UL-DL-ConfigurationCommon and considered as uplink for the random access procedure, or - second PRACH occasions, that are in RBs that are both in the active UL BWP and in the UL sub-band, and associated either only with SBFD symbols that include at least one SBFD symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon when the UE is provided either sbfd-RACHSingleConfig or sbfd-RACHDualConfig, or start from an SBFD symbol and end in a non-SBFD symbols when the UE is provided sbfd-RACHDualConfig and sbfd-RACHDualConfig-ValidROAcrossSymbolTypes The procedures in clause 8.1 apply separately for the first PRACH occasions and the second PRACH occasions. Only a Type-1 random access procedure is applicable for the second PRACH occasions.

In order to resolve the inconsistency between MAC specification and TS 38.213 on RO type selection when tdd-UL-DL-ConfigurationCommon is not applied (including the case for FDD cell, TDD cell without tdd-UL-DL-ConfigurationCommon, and SUL), it is straightforward to modify the definition of the first PRACH occasion, i.e. to define all ROs as first RO in these cases. Also note that it is common understanding in RAN2 perspective during the post e-mail discussion [1] as follows:

Question 2: If so, please indicate the preferred way-forward to resolve this issue, between: - Alt 1: Change MAC spec, to avoid introducing RO type selection and its impact for those cases where tdd-UL-DL-ConfigurationCommon is not applied (e.g., FDD, SUL, TDD without tdd-UL-DL-ConfigurationCommon configured) - Alt 2: Request RAN1 (e.g., LS) to update the definition of the first PRACH occasions in TS 38.213, to let it inclusively cover the ROs for those cases where tdd-UL-DL-ConfigurationCommon is not applied (e.g., FDD, SUL, TDD without tdd-UL-DL-ConfigurationCommon configured) Company; Need to address (Y/N); Alt 1 or 2; Comment if any. Samsung; Y; Alt 2; Prefer Alt 2 for spec conciseness and minimal spec change.. CATT; Y; Alt 2. ZTE; Y; Alt 2; We understand that ‘first PRACH occasion’in RAN1 spec should mean the legacy ROs, and ‘legacy ROs’ includes those in TDD carrier where tdd-UL-DL-ConfigurationCommon is configured or not configured. So it is better to let RAN1 clarify the relationship between first PRACH occasion’and legacy RO. Xiaomi; Y; Alt 2. Huawei, HiSilicon; Y; Alt2. LGE; Y; Alt 2; As we mentioned in Phase 1 discussion, Alt 2 is much clearer and simpler to solve the inconsistency issue. Also agree with ZTE that all legacy RO can be defined as first PRACH occasions.. Qualcomm; Y; Alt 2. Rapp summary: All the companies participated in this discussion support Alt 2, i.e., RAN2 to request RAN1 to update the definition of the first PRACH occasions in TS 38.213.

Meanwhile, during the RAN1#122bis meeting, it was discussed to modify RO definition to include the above case (i.e., when tdd-UL-DL-ConfigurationCommon is not applied) [5]. However, it was not agreed to change the definition of first RO, due to no consensus on whether it is necessary to cover the above cases.

Therefore, in order to clearly resolve the inconsistency issues on RO type selection for these cases with minimal changes, RAN2 should request RAN1 to update the definition of first PRACH occasion, to cover the cases where tdd-UL-DL-ConfigurationCommon is not applied.

Proposal 2. [MAC-2] Send an LS to RAN1, requesting an update to the definition of the first PRACH occasions in TS 38.213, to let it inclusively cover the ROs for cases where tdd-UL-DL-ConfigurationCommon is not applied (e.g., FDD, SUL, TDD without tdd-UL-DL-ConfigurationCommon configured).

3.	Conclusion

In this contribution, it is discussed about MAC issue for SBFD. This document includes following proposals.

Proposal 1. [MAC-1] UE performs RO type switch and Msg1 repetition number increment independently. No spec change is needed.

Proposal 2. [MAC-2] Send an LS to RAN1, requesting an update to the definition of the first PRACH occasions in TS 38.213, to let it inclusively cover the ROs for cases where tdd-UL-DL-ConfigurationCommon is not applied (e.g., FDD, SUL, TDD without tdd-UL-DL-ConfigurationCommon configured).

4.	Reference

[1] R2-2508163 Summary of Rel-19 SBFD MAC open issues for maintenance

[2] Draft Report of 3GPP TSG RAN WG2 meeting #131bis, Prague, Czech Republic, 13 - 17 October, 2025

[3] TS 38.321 v19.0.0

[4] TS 38.213 v19.1.0

[5] R1-2508018 Summary#2 on SBFD random access operation

Annex A - Draft LS

Title:	[Draft] LS on RO type where tdd-UL-DL-ConfigurationCommon is not applied

Response to:

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	LG electronics [to be RAN WG2]

To:	RAN WG1

Cc:

Contact person:	Hanseul Hong

hanseul.hong@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN2 has discussed inconsistency issue between MAC specification and TS 38.213 on RO type selection when tdd-UL-DL-ConfigurationCommon is not applied, and concluded that first PRACH occasions should inclusively cover the ROs for those cases where tdd-UL-DL-ConfigurationCommon is not applied (e.g., FDD, SUL, TDD without tdd-UL-DL-ConfigurationCommon configured).

RAN2 would like to request RAN1 to update the definition of the first PRACH occasions in TS 38.213 accordingly.

2	Actions

To RAN WG1

ACTION: RAN2 respectfully asks RAN1 to take the above information into to update the definition of first PRACH occasion.

3	Dates of next TSG RA1 WG2 meetings

TSG RAN WG2 Meeting #133	9th Feb. – 13th Feb. 2026	Gothenburg, SE

TSG RAN WG2 Meeting #133bis	13th Apr. – 17th Apr. 2026	TBD, Malta