3GPP TSG-RAN WG2#132	R2-2508463

Dallas, USA, Nov 17th – 21st 2025

Source: 	Fujitsu

Agenda item:	10.3.3.1

Title:	Discussion on data/model transfer

Document for:	Discussion and decision

Introduction

The SID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

The SID also describes the following objectives applied as a common function for 6GR CP/UP.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In RAN2#131bis meeting [2], RAN2 agreed the following.

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

Agreements on LCM Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

In this contribution, we would like to provide our views on data/model transfer and AI/ML aspects.

Discussion

Consideration of various types of data

In 5G, AI/ML has been studied and specified, but only AI for NW is specified, and the use cases are limited (e.g., Beam management, CSI feedback, positioning). However, in 6G, more use cases would be considered, and NW for AI might be introduced. All cases should be handled in one unified framework, so RAN2 should study the framework including a new data plane which is extendable to future use cases.

In 5G, the current protocol stacks have control plane (CP) and user plane (UP), and if either of these two planes is used for transferring data or models which are required for AI/ML, there are some issues described in [3]. For instance, if CP is used for model transfer, maximum size of RRC message would be a problem. On the other hand, if UP is used for model transfer, model management at gNB is difficult. Considering the unsuitability of CP and UP for AI/ML operation, RAN2 should study a new data plane which can connect the UE and the entities involved in the data transfer procedure. The new data plane can be used for transfer of AI/ML-related data which includes the AI/ML models. In addition, QoS management should be more flexible in this new data plane.

The same principle should be applied to Integrated Sensing and Communication. 6G network could provide sensing services by collecting sensing data, transmitting sensing data, processing sensing data, storing sensing data and exposing sensing results to the third application platform. The sensing data may be produced by a UE or a network node. Similarly, the transfer of sensing-related data can also be achieved by the new data plane.

Various new data services may be introduced while 6G is under development, but not all the services having large amount of or frequent data to be transferred can be well considered for the 6G first release. If the data transfer is discussed or designed with different structure to accommodate each different service, redundant effort will be introduced for every new service, and certain dependency among different services needs to be considered, e.g., relative priority, memory reservation, transfer tunnel allocation, therefore it will make deploying a new service not that easy. In contrast, by considering data of services only as data, a common data transfer design can accommodate all the data-based services, with common structure or procedure applied, it will be extendable to support a future service.

Proposal 1: RAN2 considers a common and future-proof data transfer design to transfer data of different services, e.g., AI/ML and sensing. In addition to legacy CP and UP, new data plane should be studied.

To make sure that data transfer design can be common and future-proof, the data transfer should consider how to address the service specific requirements. Data for an offline AI/ML model training may not be that crucial, but data for a real-time remote control may need to be reliable with low latency. Based on different priority levels, network or UE may schedule each resource by checking the critical data first, but when too many services have data pending, the priority-based control can be very complicated.

Therefore, we think service-based dedicated resources can be assigned to a service for data transfer according to the corresponding QoS requirement, or the dedicated resources can be assigned per service group, services in each service group have the same QoS requirements, to avoid inter-service dependency. The network may indicate the dedicated resources to UE per service or per service group, and UE may also distribute data of different services/service groups to different dedicated resources according to the QoS requirement or network indication. The dedicated resources can be logical resources, e.g., bearer-like resources, while it also can be physical resources, e.g., leverage scattered frequency domain resources with different bandwidth, data transfer for different services/service groups can be distributed to different bands or carriers.

Proposal 2: RAN2 considers using service-based dedicated resources to transfer data.

Data collection

When UE is in idle mode, it will periodically wake up to check paging based on Idle mode DRX cycle. UE may transfer back to connected mode according to the paging. Since there is no conflict with user traffic transmission, idle mode UE may be available to perform data collection operation, like monitoring and storing signal strength or quality of serving or neighbour cells. But when UE goes back to connected mode, the serving cell may have changed, corresponding collected data may not be valid or have meaning for new camped cell, and the collected data has to be discarded. Therefore, UE may need to stay in connected mode to collect data, and idle mode UE needs to be paged to transfer to connected mode before participating data collection.

During the last RAN2 meeting [2], requirements on data collection are agreed including minimizing impact to the UE battery, UE processing and memory utilization when UE performs data collection. UE data collection also needs to minimize impact to user traffic transmission and power saving features. When there is no valid paging message to the UE, the UE will keep sleeping to conserve power. If an idle UE being paged is allowed to transfer to connected mode to collect data, the power saving by UE will be interrupted and UE battery will be additionally consumed by the wake-up procedure, which is not aligned with the agreed data collection requirements. Therefore, we think RAN2 can make a clarification that network will not page an idle mode UE solely for data collection.

Proposal 3: UE in idle mode is not expected to be paged solely by data collection for NW-sided AI/ML model.

To further minimize the impact on UE battery and UE processing, the UE data collection behaviour should be limited to when there is data really needed, and UE starts the data collection only when a condition meets the requirements of the really needed data. For example, data collection events are introduced to indicate data collection start/stop conditions. And to minimize the impact on memory utilization, UE may only store the valid data by filtering the collected data according to the filter policy indicated to UE.

Proposal 4: UE-side data collection configuration should include data collection condition and data filter policy for really required data.

In 6G, various data-driven services may be deployed, and the data-driven services can make decisions based on large-scale data collection and analysis, e.g., AI/ML based service, sensing, QoE, and SON/MDT. AI/ML based service applies AI/ML models trained by collected data related to the service. Sensing system will collect environmental or device data for comprehensive analysis. QoE (Quality of Experience) metrics are derived from user interactions to evaluate and further enhance UE experience. SON (Self-Organizing Networks) and MDT (Minimization of Drive Tests) will collect performance data to further optimize network configuration. The statistical property of data is concerned other than the specific data from a unique UE for the large-scale data collection. Each large-scale data collection can apply common data collection condition and data filter policy for all the involved data from different UE. Therefore, RAN2 can consider a common data collection design for the large-scale data collection for AI/ML related data, sensing data, QoE, SON/MDT.

And considering services in 6G may be highly empowered by AI/ML, e.g., NW for AI, a large amount of data may be exchanged between UE and NW or data processing server for each service. Such data will be used to improve the experience of the corresponding service, but not to enhance the fundamental communication quality. With more and more demands on applying AI, defining the data collection for each service may not be practical anymore. The data used for NW for AI can be considered just as data and doesn’t need to be specially treated or decoded by the intermediate entity. From our perspective, the common data collection design may also accommodate the data of NW for AI or consider the AI/ML related data as including data of NW for AI.

Proposal 5: RAN2 considers a common data collection design for large-scale data collection, at least for sensing data, QoE, SON/MDT and AI/ML related data. FFS whether data of NW for AI can be considered as AI/ML-related data.

For UE side AI/ML model training, as a data producer, UE may start data collection based on existing configurations or require network to provide corresponding configurations to start data collection. UE may report the collected data just as data, with or without pre-processing by UE. In case the collected data can be directly transferred to the final training server, e.g., data is assembled as IP packet and delivered to the IP address of the final training server, without intermediate data consumer.

For network side AI/ML model training, the data collection can be originated by network (e.g., gNB). The collected data can be data which has already existed at the network side, or the network may trigger UE to generate data with corresponding data collection configurations indicated to the UE, like measurement configurations, data logging configurations, data reporting configurations. In this case, UE takes the role as data producer, and at least the network device who originates the data collection and provides data collection configurations has the opportunity to consume the collected data, including post-processing on the collected data. The collected data, with or without post-processing, can be further transferred to higher level network entities (e.g., CN, OAM) or data server (e.g., training centre) for further or final processing, so the higher level network entities and the data server will also consume the data. Therefore, when collecting UE side data for network side AI/ML model training, RAN2 needs to discuss how to make sure that user data privacy, anonymity and user consent are respected.

Proposal 6: RAN2 should discuss how to make sure that user data privacy, anonymity and user consent are respected, especially when collecting UE side data is for network-sided AI/ML model.

Model transfer/delivery requirements

About AI for NW, model transfer may be required to UE when two-sided model needs to be deployed or a UE sided model has been trained outside the UE but not yet been loaded, e.g., when UE would like to predict CSI and inference model is outside the UE. Model transfer can be done at the time when other user traffic is scarce if the model is not immediately required or the necessary model transfer is predicted in advance. To utilise network resource and energy efficiently, model transfer should be scheduled by NW depending on request from UE and this requires that model server is inside 3GPP network.

Considering NW for AI, model transfer is required e.g., when a UE is trying to perform image recognition and needs to get optimum model for the input image from the model server. Sizes of model for image recognition vary depending on model, number of parameters, and number of bits per parameters. According to [4], if downloading model needs to be finished in one second and if 8-bit parameters are used, required DL data rate is from 33.6 Mbps up to 1.1 Gbps. This is just an example, but model size can sometimes be too large to be transferred under some NW conditions. Model server within 3GPP network can select appropriate model under the current channel state considering requirements received from UE. This is good for NW because NW can avoid deteriorating other higher-priority traffic and it is good for UE as well because UE can get optimum inference result depending on the requirements e.g., if low latency is more important than precision, NW can provide UE a model with smaller size.

Proposal 7: Model transfer should be able to be scheduled by NW considering both NW and UE situations if the model server is within the 3GPP NW (i.e. not OTT server).

- For NW for AI use cases, size and type of delivered model can also be selected by NW based on network load at the time and requirements from UE.

Conclusion

In this paper we presented some views on Data/model transfer and AI/ML aspects and have come up with the following proposals:

Proposal 1: RAN2 considers a common and future-proof data transfer design to transfer data of different services, e.g., AI/ML and sensing. In addition to legacy CP and UP, new data plane should be studied.

Proposal 2: RAN2 considers using service-based dedicated resources to transfer data.

Proposal 3: UE in idle mode is not expected to be paged solely by data collection for NW-sided AI/ML model.

Proposal 4: UE-side data collection configuration should include data collection condition and data filter policy for really required data.

Proposal 5: RAN2 considers a common data collection design for large-scale data collection, at least for sensing data, QoE, SON/MDT and AI/ML related data. FFS whether data of NW for AI can be considered as AI/ML-related data.

Proposal 6: RAN2 should discuss how to make sure that user data privacy, anonymity and user consent are respected, especially when collecting UE side data is for network-sided AI/ML model.

Proposal 7: Model transfer should be able to be scheduled by NW considering both NW and UE situations if the model server is within the 3GPP NW (i.e. not OTT server).

- For NW for AI use cases, size and type of delivered model can also be selected by NW based on network load at the time and requirements from UE.

References

- RP-252912, Revised SID: Study on 6G Radio

- R2_131bis_ChairNotes_25-10-17_17-00_final

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- TR 22.874, Study on traffic characteristics and performance requirements for AI/ML model transfer in 5GS