3GPP TSG RAN WG2 #132			R2-2508527

Dallas, US, 17th-21st November, 2025

Source:	NTT DOCOMO, INC.

Title:	Views on 6G Energy Efficiency

Agenda Item:	10.3.3.2

Document for: 	Discussion and Decision

- Introduction

Revised SID Study on 6G Radio has been approved in RAN#109 [1]. At RAN2 #131-bis meeting, Energy Efficiency for 6G was discussed and the following was noted in Chair notes [2]:

- Design goal - Strive to develop solutions that consider network energy efficiency and UE power saving - For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

In this contribution, we provide our views on Energy Efficiency for 6G study.

- Discussion

2.1	Network energy efficiency

2.1.1	NES-related features

As stated in the SID, network energy saving is one of the largest objectives for 6GR design. We fully support this direction as an operator since it would be essential for us to provide our service with reasonable fee as an infrastructure, which requires energy consumption minimization.

NW energy saving related techniques have been investigated for multiple releases during NR as follows, and we think basically the WGs that led the corresponding NR features should continue to study 6G features. The exception would be that the design of SIB and multi-carrier operation because they would highly impact essential cell design from RAN2 perspective.

- RAN1

  - SSB-less SCell

  - On-demand SSB on SCell

  - On-demand SIB1

  - Power/spatial domain adaptation

  - SSB adaptation

  - PRACH adaptation

- RAN2-led

  - Cell DTX/DRX

  - Paging adaptation

- RAN4-led

  - SSB-less SCell

Proposal 1: RAN2 to study at least the following NES features

- Cell DTX/DRX mechanism with consideration of UE DRX

- Paging adaptation (energy efficiency paging design)

- On-demand SIB1/OSI and energy efficiency SIB design

- Consider multiple-carrier operation, including whether to keep CA framework with SSB-less, OD-SSB or not

However, we view that such techniques may not have sufficient potential to bring energy saving gain with smaller drawbacks for real field operation due to some reasons. One of the reasons is that those enhancements were required to be specified with minimum (or even zero) specification/operation impact on legacy behavior (i.e., Rel-15), which resulted in having marginal energy saving gains (and thus not much attractive for operators to deploy). Notable example is the introduction of NES in Rel-19, where extending legacy SSB was unable to achieve due to the impact on legacy UE behavior for initial access. Another reason is that NR specified several features which achieve very similar gain, and thus, even if they are deployed, it would be difficult for NW to develop and deploy these features efficiently due to the fragmentation.

2.1.2	RAN2 aspects related to SSB

From operational perspective, NW usually provides SSBs on a sync raster with 20 ms periodicities for PCell operation in NR. This is due to the assumption defined in legacy NR for IDLE mode UE expecting 20 ms periodicity reception of SSBs in initial cell selection. Also, in legacy operation, many of the common signal occasions are semi-statically configured and cannot be dynamically adapted according to situation. Based on the outcome of such legacy design in NR, NWs consume energy by providing common signals very often which are not always necessary for any UEs.

As discussed in our tdoc of initial access [3], the description of RAN2 specification for UE behavior for initial cell selection has not been touched since at least UTRA era. We believe 6G is a good time to reassess whether/how to capture more “practical” and “efficient” way for searching SSBs even in such a very beginning of initial access. For example, even when UE has just turned on and has no prior information by reception of any SSB and SIB1 in a certain system in advance, the UE can derive candidate frequency position(s) of SSB based on UE’s location and/or USIM information. This is because candidate frequency positions of SSB can be considered quite static information for a certain PLMN (country and operator), so such UE behavior can be treated as a default/specified one. Moreover, in the current specification, although UE’s assumption on SSB for PLMN selection are not defined, the same logic can be applicable, e.g., a UE can derive available PLMN (lists) based on its location and subscription information and then identify probable SSB frequency position(s).

Thus, we would like to emphasize the importance of study possible specification enhancement of SSB scanning during PLMN and initial cell selection for both UW and NW efficiency. At least, 6G design should not restrict too much SSB periodicity even if the SSB can be used even for very “initial” phase.

- For UE side, UE power consumption for scanning all frequencies can be reduced.

- For NW side, CD-SSB periodicity is no longer restricted by UE complexity and access latency for initial cell selection.

- E.g., it can be specified that UE always can be able to know candidate frequency position of SSB on sync-raster (without scanning) based on UE’s location and/or USIM information

2.1.3	Cell DTX/DRX

Cell DTX/DRX was introduced in Rel-18 where a UE is configured with a single Cell DTX/DRX pattern via RRC and activated/deactivated via RRC signaling or group-common DCI indication. During the non-active period, the UE does not require to monitor PDCCH, periodic/semi-persistent CSI-RS for CSI reporting, CSI-RS for mobility in connected mode, which reduces the NW and UE power consumption at a certain level. Since UE does not need to monitor PDCCH conveying DCI to inform activation or deactivation of Cell DTX/DRX, urgent transmission/reception cannot be instantly resumed (i.e., NW has to wait for the next active period to resume NES operation).

Still, UEs need to monitor other reference signals such as SSB, CSI-RS for tracking, RRM, RLM, and cell DTX/DRX is only applicable to UEs in RRC CONNECTED (i.e., UEs in IDLE/INACTIVE need to transmit and receive any signal/channel even during non-active period), which seems not the best way for NW/UE energy saving. On the other hand, not transmitting all the signals during non-active period may degrade UE performance. We should consider both aspects at the same time.

At first, we would like to emphasize that there is little difference between cell DTX and UE C-DRX with proper NW implementation. This is because, the function of cell DTX/DRX simply defines UE behaviour: a UE does not need to transmit or receive certain signal or channel. This behaviour is exactly the same as that which can be achieved by UE C-DRX. In our view, with proper NW implementation (e.g., aligning the active periods of multiple UEs by its configurations), NW can enter sleep mode during UEs’ non-active period. From the perspective of avoiding the definition of duplicated functions, we cannot accept that cell DTX/DRX will be a baseline feature without much justification over UE C-DRX with NW implementation.

However, if separation of functions into cell DTX and UE DRX is well justified, we should study which signals and channels can be turned off during non-active period of Cell DTX/DRX from scratch, including the possibility of applying this to RRC IDLE UEs.

Observation 1: Cell DTX/DRX has the following pain point

- There is little difference between cell DTX and UE C-DRX with proper NW implementation

- UE can be configured with only a single Cell DTX/DRX pattern via RRC

- DCI-based activation/deactivation cannot be received during non-active period of cell DTX

- Cell DTX/DRX is only applicable to UEs in RRC CONNECTED

- UE needs to monitor numbers of reference signals, such as SSB, CSI-RS for tracking, RRM, RLM even during non-active period

In this sense, we can consider two directions: one is to make cell DTX/DRX a meaningful function and deployable, the other is to make a single framework that can achieve UE and NW sleep duration.

Proposal 2: RAN2 to study the following directions for NES operation in RRC CONNECTED

- Direction 1: Keep cell DTX and UE DRX, and make cell DTX a meaningful function that cannot be achieved by UE DRX alone

- Direction 2: Integrate cell DTX and UE DRX into a single function that allows UE to not monitor and perform certain operation during non-active periods for both UE and NW energy saving

Some companies have proposed applying cell DTX/DRX mechanisms to the idle mode. At the same time, adaptation mechanisms have been specified separately in Rel-19 for common signal/channel for idle mode such as PRACH and paging. As both features fundamentally aim to prevent certain TX/RX during a “sleep” period, it is important to first determine which direction to pursue for the overall design. In general, we do not support having two overlapping approaches for essentially the same functionality in general.

Proposal 3: RAN2 to study the following directions for NES operation for RRC IDLE(/INACTIVE)

- Direction 1: Extend cell DTX/DRX to RRC IDLE MODE, e.g.,

  - Common signal/channel (e.g., CD-SSB, SIB, PRACH, paging) may not be TXed/RXed during non-active period

- Direction 2: Introduce adaptation mechanism and/or NES-friendly resource allocation for each common signal/channel for IDLE, e.g.,

  - NR Rel-19-like PRACH, paging adaptation mechanisms

- Note: Avoid supporting both of two directions that essentially provide the same functionality.

2.1.4	Multi-carrier operation in RRC CONNECTED

In NR Rel-15, for CA operation, NW can configure UE with multiple carriers as SCell(s) via RRC signaling, and later the SCell(s) can be activated or deactivated. The cell common information (e.g., SSB related parameters) of these SCell(s) are provided by the RRC signaling. While a SCell is activated, UE shall perform normal SCell operation such as monitoring PDCCH on SCell for data TX/RX and CSI reporting. In order to make sure the UE is ready to operate in the SCell, during that period, NW has to transmit always-on SSB on the SCell according to the RRC configurations. As cell common parameters provided by RRC configurations are semi-static, meaning that update of these parameters takes much time and signaling overhead, those always-on signal and channels should be TXed/RXed for a relatively long period.

Meanwhile, considering the major use-case of SCell, i.e., transferring larger data traffic while avoiding too much concentration of that traffic in a cell (i.e., PCell), it is not desirable that NW always transmit signal/channel for energy efficiency for both NW and UE in case the need of large traffic is not there. To improve energy efficiency, several enhancements were introduced in NR as follows:

SSB-less SCell operation

In NR Rel-15, SSB-less SCell operation is defined only in FR1 intra-band. This feature is beneficial for better energy efficiency in multi-carrier operation due to no SSB transmission in the SCell, which eventu-ally reduces NW energy consumption caused by the transmission for the SCell. In Rel-18, SSB-less SCell operation was extended to FR1 inter-band with some restrictions, such as, SSB-less SCell and reference cell need to be co-located, TRS or A-TRS needs to be configured on SCell, and restrictions on RTD, pow-er difference, QCL relationship between reference cell and SCell are defined.

On-demand SSB SCell operation

In NR Rel-19, mainly for the scenarios where SSB-less SCell operation is not supported, on-demand SSB was introduced so that SSB on SCell can be turned on/off dynamically without RRC signalling while the SCell is deactivated. The benefit of this feature is that, in the SCell that is not possible to turn off SSB completely, transmission of DL always-on signal/channel can be minimized, so NW can reduce energy consumption.

We believe that NW EE remains (or even more) important, so and hence the above features should continue to be taken into account as a baseline for efficient operation in muti-carriers. Based on the features, we can consider further details of enhancement to ease and broaden operation and applicable scenario, e.g., investigating the possibility of further relaxation of relationship of SSB-less SCell and reference cell, or co-existing support of on-demand SSB for SCell and PCell.

However, similar to the single carrier case, we see there are several levels of energy efficiency state for NW and/or UE on SCell as follows

- (UE side) Deactivated SCell + (NW side) On-demand SSB/SSB-less

- (UE side) Dormant SCell

- (UE side) Activated SCell with UE C-DRX + (NW side) Cell DTX/DRX on SCell

In 6G, while retaining the essence of the important NES features based on the CA framework introduced in 5G, we should carefully check the necessity of all features that might offer similar energy efficiency gain and similar advantage.

Proposal 4: RAN2 to study design of EE-friendly multi-carrier operation in RRC CONNECTED in 6G, considering whether/which the following EE states are supported or integrated:

- Opt1. (UE side) Deactivated SCell and (NW side) On-demand SSB/SSB-less

- Opt2. (UE side) Dormant SCell

- Opt3. (UE side) Activated SCell with UE C-DRX and (NW side) Cell DTX/DRX on SCell

2.2	UE power savings

Numerous techniques for UE power savings are specified in 5G as follows:

- RRC CONNECTED state

  - BWP adjustment, C-DRX including DCP and LP-WUS triggered PDCCH monitoring, SCell dormancy, Assistance TRS, RLM/BFD measurement relaxation, Cross-slot scheduling, UAI, PDCCH skipping, SSSG

- RRC IDLE/INACTIVE state

  - I-DRX including PEI, Paging subgrouping, and LP-WUS triggered paging monitoring, RRM measurement relaxation, LP-WUS based RRM relaxation/offloading, SDT

Among these specified techniques, the following list outlines those which we believe have a direct impact on RAN2 specifications and works.

- RRC CONNECTED state

  - C-DRX including DCP and LP-WUS triggered PDCCH monitoring, SCell dormancy, RRM/RLM/BFD measurement relaxation, UAI

- RRC IDLE/INACTIVE state

  - I-DRX including PEI, Paging subgrouping, and LP-WUS triggered paging monitoring, RRM measurement relaxation, LP-WUS based RRM relaxation/offloading, SDT

Observation 2:

- Following specified techniques for UE power savings in NR have RAN2 impact and should be discussed in RAN2:

  - RRC CONNECTED state

    - C-DRX including DCP and LP-WUS triggered PDCCH monitoring, SCell dormancy, RRM/RLM/BFD measurement relaxation, UAI

  - RRC IDLE/INACTIVE state

    - I-DRX including PEI, Paging subgrouping, and LP-WUS triggered paging monitoring, RRM measurement relaxation, LP-WUS based RRM relaxation/offloading, SDT

In the following, we show our view of each feature.

2.2.1 UE power savings in RRC CONNECTED state

- C-DRX related

In NR, UE performs tens of blind detections on PDCCH candidates in a slot and UE power consumption on PDCCH monitoring is observed as one of the dominant factors according to TR38.840. To effectively reduce UE power consumption, the DRX mechanism is essential for UE power savings in RRC CONNECTED state and should be supported in 6G. In NR, UE C-DRX was introduced for this purpose. Additionally, Cell DTX was introduced to reduce PDCCH transmissions by NW, consequently minimizing the need for UE PDCCH monitoring. However, as pointed out in Section 2.1.1, there is no significant functional difference between Cell DTX and UE C-DRX with proper NW implementation. In this sense, we can consider two directions: one is to make Cell DTX/DRX distinguished and deployable, the other is to make a single framework that can achieve UE and NW sleep duration as suggested in Proposal 2.

In addition to DRX, two features — Downlink Control Information for Power Saving (DCP) and Low Power Wake-Up Signal (LP-WUS) based wake-up mechanism — have been introduced. Both features aim to allow the UE to enter a sleep mode for a longer duration by skipping wake-up during certain On-duration periods. However, since the core purpose of DCP and LP-WUS is the same, they are functionally redundant. To avoid market fragmentation, it is preferable to standardize and implement only one of these features. We think the LP-WUS based wake-up mechanism is preferable as it offers greater power saving gain compared to DCP. For LP-WUS, UE may implement low-power radio and operates based on the LR in case of the smaller need of PDCCH monitoring. Once PDCCH monitoring is required, it is triggered by receiving the signalling which is compatible to LR, i.e., LP-WUS. By relying on LR, further power saving can be achieved. Meanwhile, since LP-WUS has to be compatible for LR, the signalling structure may have some drawbacks for the whole NW operation, such as limitations in coverage and increased resource overhead. If these drawbacks are significant from an operator's perspective, the benefit of using the LP-WUS/WUR operation solely for power saving may be negated. Therefore, the decision to introduce either DCP or the LP-WUS based wake-up mechanism should be justified and finalized after the LP-WUS design details are determined within RAN1 discussions.

- SCell dormancy

Spectrum utilization and aggregation is one of topics relevant to UE power savings. As discussed in [4], keeping signal/awareness of many bands/carriers would increase UE power consumption, but more sleep could result in worse user experienced communication. We need to strive to achieve a good balance between communication performance and UE power savings.

Fast SCell activation is one of the key features. From communication performance perspective, always activated SCells maximize data rate/latency performance. However, obviously, this is not realistic for UE power savings; from UE power savings perspective, SCell should be deactivated as much as possible. For the good balance of them, what is required is activating SCell immediately when much traffic happens. For 6GR, the study and introduction of a fast SCell activation feature is highly recommended.

In 5G NR, SCell dormancy was introduced for this purpose. Similar mechanism can be considered for 6GR, while just copies of them are not preferred. SCell dormancy in 5G NR highly correlates with BWP framework. For 6GR, we believe that BWP framework should be revisited for mitigating operational complexity as discussed in [4]. Instead of that, a potential approach is introducing SCell dormancy concept without relying on BWP framework.

On the other hand, as previously discussed in Section 2.1.4, several energy efficiency (EE) features for multi-carrier operation in the RRC CONNECTED state already exist. These features, such as Cell DTX/DRX, offer similar energy efficiency gains and advantages to SCell dormancy. Therefore, for 6GR, we should focus on studying the design of an EE-friendly multi-carrier operation in the RRC CONNECTED state without introducing redundant features.

- RRM/RLM/BFD measurement relaxation

Another potential power-consuming behaviour in RRC CONNECTED state would be measurements related, such as RRM, RLM and BFD. These features are essential for UE to prepare for several procedures, e.g., handover, RLF detection and beam failure detection. Meanwhile, those procedures may not frequently occur depending on the situation. For example, a stationary device (e.g., IoT, FWA) generally does not require mobility. Also, even for eMBB, in case the device is located at cell centre rather than cell edge, the need to prepare for mobility or link quality check could be small. In such situations, reducing measurements for RRM/RLM/BFD might help the UE reduce its power consumption further. In NR, there are some features defined for this purpose (e.g., RLM/BFD relaxation).

Having said that, despite the history in NR about specifying such RRM relaxation features, at this initial stage of 6GR SI, we would like to point out that it would be essential to check the practical need to pursue energy efficiency for the measurement related procedures. First, while we understand more efficient power consumption typically seems nice, we believe these measurements behaviours could be very important in some cases. For example, for moving UEs, it will experience handover frequently, and to achieve better experience through handover, frequent measurement would be rather needed. Also, as the related measurements are under NW configuration control in RRC CONNECTED state, even for e.g., non-moving UE, there may be no strong need of new features. Furthermore, power consumption caused by measurements may be less dominant than e.g., PDCCH monitoring, as it is not a blind procedure nor decoding. According to the analysis here, we may first need to identify how beneficial it is to pursue measurement-related energy efficiency in RRC CONNECTED state, and/or in which scenario they can be beneficial.

- UAI

In NR, a mechanism exists to allow the UE to report assistance information — carried on the UAI (UE Assistance Information) — to facilitate appropriate configurations for UE power savings. The following information can be reported by the UE via UAI to help the NW change configurations that affect UE power consumption: frequency, time, and spatial adaptation. We believe it is preferable to introduce a similar mechanism to 6G, as this would also be beneficial for UE power savings. However, the decision on whether to introduce UAI and its detailed design should be discussed under General Agenda Item in 10.2.

Proposal 5:

- For RRC CONNECTED state,

  - RAN2 is requested to study the following features, taking careful consideration to prevent feature duplication:

    - UE DRX mechanism with potential integration into Cell DTX. Whether to introduce DCP or LP-WUS should be justified after the LP-WUS design details are determined within RAN1 discussions.

    - SCell dormancy without relying on BWP framework in conjunction with other EE features designed for multi-carrier operation.

    - UAI for power savings. (Note: The decision on whether to introduce UAI itself in 6G should be discussed under the General Agenda.)

  - Deprioritize study for RRM/RLM/BFD in RRC CONNECTED, unless the practical need in terms of energy efficiency is observed.

2.2.2 UE power savings in RRC IDLE/INACTIVE state

- I-DRX related

In NR, for IDLE/INACTIVE state, power consumption caused by behaviours related to paging monitoring was identified as dominant. As a result, there are several features in NR to optimize power efficiency for paging procedures, such as I-DRX and PEI. In addition, to enjoy further power saving by relying on LR, LP-WUS operation can trigger paging PDCCH monitoring based on Rel-19 specifications.

Similar to NR, it is beneficial to support DRX mechanism for IDLE/INACTIVE state for 6G. However, when we look at real deployments, we don’t see many related features already deployed other than I-DRX. Also, it is a bit unclear whether/how power consumption in IDLE/INACTIVE state is significant among the whole procedures done by a UE, which can be one of the reasons why real field deployment of those features would be rather slow. Another thing which doesn’t look nice is that, despite the situation in reality, multiple features in the specification related to this seem to be colliding in terms of the expected gain. Therefore, in 6GR, to avoid too much complexity and too fragmented features, 3GPP should be more careful about specifying the features for paging monitoring related power saving in IDLE/INACTIVE.

Hence, in 6GR we believe it would be important to confirm if there is a clear justification to specify features which help to reduce power consumption related to paging in IDLE/INACTIVE states. Right now, from our perspective, unless e.g., huge power saving without significant drawbacks can be achieved, the motivation of this topic may not be sufficient to proceed with the standardization. Also, even if the justification is strong, we should pay much more attention to minimize the number of features to be specified for a single gain, e.g., must avoid having multiple features for one gain, such as PEI and LP-WUS.

- RRM measurement relaxation

RRM measurement is a dominant factor in terms UE power consumption in IDLE/INACTIVE states since UE typically performs serving cell measurement every one or two I-DRX cycle. However, from whole UE power savings perspective, power consumption of RRM measurement in IDLE/INACTIVE states may not be very critical. Given the situation, we believe pursuing further power saving for IDLE/INACTIVE state RRM measurement should be justified by a clear motivation. Moreover, in case it is well motivated, any solution shall achieve clear and practical power saving gain, together with very small (preferably zero) drawbacks. Followings are potential solutions on RRM measurement relaxation and can be studied as baseline:

- RRM offloading from MR to LR: LP-WUS offloading can be reused and enhanced

- RRM relaxation: low mobility, stationary or cell center relaxation can be reused and enhanced

- SDT

Basically, when UE in RRC IDLE/INACTIVE state requires communication with BS, it is necessary for the UE to do state transition to RRC CONNECTED state. Establishing connection between RAN/CN and UE consumes UE power. This is OK when much traffic happens as the corresponding communication will be performed efficiently.

Meanwhile, for some use cases, the amount of data traffic is quite small and correspondingly the number of data TXs for each communication occasion is also quite small. For such TXs only, establishing connection between RAN/CN and UE will not be reasonable from UE power savings and user experienced communication performance. Instead of that, completing the TXs without UE state transition could be beneficial for UE power savings as well as user experienced communication performance.

From that point of view, SDT was introduced in 5G NR. Efficient/effective SDT mechanism can be studied for 6GR as well. One possibility is that reuse of SDT defined in 5G NR may be sufficient, which also should not be precluded.

Proposal 6:

- For RRC IDLE/INACTIVE (if introduced) state, RAN2 is requested to study the following features, taking careful consideration to prevent feature duplication:

  - UE DRX mechanism with potential integration into Cell DTX.

  - RRM measurement relaxation including conditional RRM relaxation and RRM offloading from MR to LR.

  - SDT for UE power savings and user experienced communication performance, including whether SDT in 5G NR can be reused as it is.

- Conclusion

In this contribution, we provided the following observations and proposals of our views on Energy Efficiency for 6G study.

Proposal 1: RAN2 to study at least the following NES features

- Cell DTX/DRX mechanism with consideration of UE DRX

- Paging adaptation (energy efficiency paging design)

- On-demand SIB1/OSI and energy efficiency SIB design

- Consider multiple-carrier operation, including whether to keep CA framework with SSB-less, OD-SSB or not

Observation 1: Cell DTX/DRX has the following pain points

- There is little difference between cell DTX and UE C-DRX with proper NW implementation

- UE can be configured with only a single Cell DTX/DRX pattern via RRC

- DCI-based activation/deactivation cannot be received during non-active period of cell DTX

- Cell DTX/DRX is only applicable to UEs in RRC CONNECTED

- UE needs to monitor numbers of reference signals, such as SSB, CSI-RS for tracking, RRM, RLM even during non-active period

Proposal 2: RAN2 to study the following directions for NES operation in RRC CONNECTED

- Direction 1: Keep cell DTX and UE DRX, and make cell DTX a meaningful function that cannot be achieved by UE DRX alone

- Direction 2: Integrate cell DTX and UE DRX into a single function that allows UE to not monitor and perform certain operation during non-active periods for both UE and NW energy saving

Proposal 3: RAN2 to study the following directions for NES operation for RRC IDLE(/INACTIVE)

- Direction 1: Extend cell DTX/DRX to RRC IDLE MODE, e.g.,

  - Common signal/channel (e.g., CD-SSB, SIB, PRACH, paging) may not be TXed/RXed during non-active period

- Direction 2: Introduce adaptation mechanism and/or NES-friendly resource allocation for each common signal/channel for IDLE, e.g.,

  - NR Rel-19-like PRACH, paging adaptation mechanisms

- Note: Avoid supporting both of two directions that essentially provide the same functionality.

Proposal 4: RAN2 to study design of EE-friendly multi-carrier operation in RRC CONNECTED in 6G, considering whether/which the following EE states are supported or integrated:

- Opt1. (UE side) Deactivated SCell and (NW side) On-demand SSB/SSB-less

- Opt2. (UE side) Dormant SCell

- Opt3. (UE side) Activated SCell with UE C-DRX and (NW side) Cell DTX/DRX on SCell

Observation 2: Following specified techniques for UE power savings in NR have RAN2 impact and should be discussed in RAN2:

- RRC CONNECTED state

  - C-DRX including DCP and LP-WUS triggered PDCCH monitoring, SCell dormancy, RRM/RLM/BFD measurement relaxation, UAI

- RRC IDLE/INACTIVE state

  - I-DRX including PEI, Paging subgrouping, and LP-WUS triggered paging monitoring, RRM measurement relaxation, LP-WUS based RRM relaxation/offloading, SDT

Proposal 5:

- For RRC CONNECTED state,

  - RAN2 is requested to study the following features, taking careful consideration to prevent feature duplication:

    - UE DRX mechanism with potential integration into Cell DTX. Whether to introduce DCP or LP-WUS should be justified after the LP-WUS design details are determined within RAN1 discussions.

    - SCell dormancy without relying on BWP framework in conjunction with other EE features designed for multi-carrier operation.

    - UAI for power savings. (Note: The decision on whether to introduce UAI itself in 6G should be discussed under the General Agenda.)

  - Deprioritize study for RRM/RLM/BFD in RRC CONNECTED, unless the practical need in terms of energy efficiency is observed.

Proposal 6:

- For RRC IDLE/INACTIVE (if introduced) state, RAN2 is requested to study the following features, taking careful consideration to prevent feature duplication:

  - UE DRX mechanism with potential integration into Cell DTX.

  - RRM measurement relaxation including conditional RRM relaxation and RRM offloading from MR to LR.

  - SDT for UE power savings and user experienced communication performance, including whether SDT in 5G NR can be reused as it is.

References

- RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

- RAN2#131-bis meeting Chair Notes, RAN2 Chair (InterDigital)

- R2-2508533, “Views on 6G Initial Access”, NTT DOCOMO

- R1-2507814, “Discussion on overview of 6GR air interface”, NTT DOCOMO