3GPP TSG RAN WG2 #132			R2-2508533

Dallas, US, 17th-21st November, 2025

Source:	NTT DOCOMO, INC.

Title:	Views on 6G Initial Access

Agenda Item:	10.3.2.3

Document for: 	Discussion and Decision

- Introduction

Revised SID Study on 6G Radio has been approved in RAN#109 [1]. In this contribution, we provide our views on Initial and System Access for 6G study.

10.3.2.3 Initial and System Access and Others

Including contributions on initial and system access, including system information, paging, and aspects related to spectrum aggregation.etc. Contributions can include aspects related on on-demand SIB, SSBs, SI update mechanism, SIB1 size, area specific SIBs, etc.  and understanding of problems to address.

- Discussion

2.1	Initial access

PLMN/Initial cell selection

The first step to access a cellular system is when UE’s power is turned on. In the current NR, the UE behavior for the step is defined in TS38.304. The definition of initial cell selection is when UE has no prior knowledge of which RF channels are NR frequencies. Also, TS38.213 defines that a UE assumes SSB transmission with 20 ms during initial cell selection. This assumption seems to take care of UE complexity, that is, UE faces most burden to scan SSBs and find proper corresponding SIB1 without any prior knowledge of frequency position. NW does not anticipate when and where a UE which will perform initial cell selection appears, so NW has to transmit SSBs with 20ms periodicity for a cell/carrier operating as PCell, meaning NW cannot enter deep sleep mode.

As discussed in the first RAN1 meeting for 6GR, given that energy saving for network will be strongly motivated for 6GR as stated in the SID, SSB periodicity should be allowed to be longer than 5G NR. Since just prolonging default SSB periodicity of UE’ assumption might cause some impact on UE complexity and/or latency during initial cell selection, RAN1 and RAN4 are expected to work on candidate solutions to address the potential impact, e.g., reduce the number of defined sync-raster.

At the same time, the description of RAN2 specification for UE behavior for initial cell selection has not been touched since at least UTRA era. We believe 6G is a good time to reassess whether/how to capture more “practical” and “efficient” way for searching SSBs even in such a very beginning of initial access. For example, even when UE has just turned on and has no prior information by reception of any SSB and SIB1 in a certain system in advance, the UE can derive candidate frequency position(s) of SSB based on UE’s location and/or USIM information. This is because candidate frequency positions of SSB can be considered quite static information for a certain PLMN (country and operator), so such UE behavior can be treated as a default/specified one. Moreover, in the current specification, although UE’s assumption on SSB for PLMN selection are not defined, the same logic can be applicable, e.g., a UE can derive available PLMN (lists) based on its location and subscription information and then identify probable SSB frequency position(s).

Proposal 1:

- Study possible specification enhancement of SSB scanning during PLMN and initial cell selection for both UW and NW efficiency.

  - For UE side, UE power consumption for scanning all frequencies can be reduced

  - For NW side, CD-SSB periodicity is no longer restricted by UE complexity and access latency for initial cell selection.

  - E.g., it can be specified that UE always can be able to know candidate frequency position of SSB on sync-raster (without scanning) based on UE’s location and/or USIM information

IDLE MODE behavior

When the SSB is placed not on a sync raster, NW does not need to consider a UE performing initial cell selection. In such a scenario, it should be allowed for NW to transmit SSBs with much longer periodicity than the case for initial cell selection, e.g., 160 ms. This is because UEs already get information regarding the location of SSBs once they receive system information upon the completion of the cell selection.

In NR, PCell operation is associated with SSBs on sync-raster (according to TS38.300) and given that SSBs on sync-raster are used for initial cell selection as mentioned above, PCell is expected to transmit CD-SSB with 20ms periodicity and not to adapt its properties to avoid negative impact on UE in the initial phase. We believe that if such restriction that PCell has to be associated with on-sync raster CD-SSB is removed, NW will obtain more flexibility and energy saving gain as a whole in multi-carrier scenario over NR operation.

In such a case, the performance of UEs in that band cannot be guaranteed since they may need to monitor several RS samples for synchronization and measurement in some cases. With longer periodicity, UEs need to wake up long, e.g., before the paging is coming, which results in wasting UE energy. Therefore, in a real deployment, just making SSB longer periodicity may not be feasible due to potential degradation of e.g., IDLE mode behavior, such as cell reselection and paging reception.

Therefore, we think that in addition to the provision of always-on RS (AO-RS), on-demand RS (OD-RS) should be transmitted to help UE monitor RSs with relatively short periodicity. From that perspective, we should study further how to design a mechanism to provide dynamic RS periodicity adaptation and/or OD-RS transmission with less overhead. For OD-RS indications, we think the NW triggering mechanisms should be considered at least and UE triggering based OD-RS should be studied according to its necessity and criteria. Another study to be considered is how OD-RS can be used for UEs when OD-RS transmission occurs. For example, it should be studied whether/how UE can use such OD-RS to be the samples for RRM and how to define corresponding RRM requirements.

Proposal 2:

- Study on-demand SSB provision on PCell for IDLE/INACTIVE UEs, including

  - at least NW triggering mechanism, and UE triggering including its necessity and its criteria

  - RRM measurement mechanism that can be done within more confined time duration over NR using always-on SSB and potentially on-demand SSB for compensation of prolonged always-on SSB

2.2	System information

On-demand SIB

In the NR operation, the transmission of system information in an on-demand manner was supported, i.e., on-demand OSI from Rel-15 and on-demand SIB1 (OD-SIB1) from Rel-19. NR designed OSI as an on-demand native so the mechanism can be said to be well-specified. However, there seems some remaining room for enhancement in OD-SIB1, as the legacy NR design relies entirely on parameters from always-on SIB1 transmission.

For example, OD-SIB1 operations require another coordinated cell (cell A) to obtain UL WUS configurations for OD-SIB1 request to NES cell. From commercial point of view, deploying duplicated cells is generally decided to meet capacity demand. Therefore, it is unclear whether NES gain in such areas is the only critical scenario in terms of energy efficiency. For example, we understand low-load scenario is also important to pursue further NES, while the NR OD-SIB1 feature isn’t available. However, to realize OD-SIB1 without another coordinated cell, it is necessary to consider how the UE is provided with the corresponding configuration for UL trigger of OD-SIB1 (i.e., UL-WUS in NR). The parameters included in the UL WUS configurations in NR are significant in volume because there is a lot of flexibility in pointing out the resource grid and then PRACH resources used as UL WUS. These facts complicate both NW management and UE behavior, such as the coordination between cell A and NES cells for the OD-SIB1 operation. Additionally, in NR operation, a SIB1 request is always necessary before camping on a NES cell, even though a UE already has plenty of information about the NES cell.

From our perspective, to analyze whether OD-SIB1 concept is deemed necessary in 6GR, we should first study the feasibility of standalone OD-SIB1 operation with the lightened size of parameters that determines UL WUS, which corresponds to case #1 studied in Rel-19 NR. The beginning of 6G is the best opportunity to study such a standalone OD-SIB1 operation, since we can consider designing UL WUS, the necessary information and the container to deliver it from scratch, to enable this feature for all 6GR UEs. One potential direction can be to introduce pre-defined/pre-configured UL WUS configuration.

If such direction is not feasible, then we should allow a representative cell/carrier (cell A) to indicate information equivalent to SIB1/OSI of NES cells (which is similar to case #3 in Rel-19 OD-SIB1 study), and UEs to store such information during camping on an area represented by the cell A.

Proposal 3:

- Study whether/how standalone deployment of OD-SIB1 NES cell is achieved.

  - E.g., Introduce pre-defined/pre-configured UL WUS configurations to request OD-SIB1.

SI update mechanism

In NR, SI update is indicated via short message that is transmitted on paging occasion and updated SIB can be transmitted from next modification period. This implies that NW has to transmit multiple times short messages to inform all UEs in a cell. Moreover, in our understanding, NW should ensure proper alignment of cell common configuration between IDLE/INACTIVE UEs an RRC CONNECTED UEs. In order to inform the latter UEs of update of cell common configurations, exchange of multiple RRC dedicated signalling (RRCReconfiguration) would be needed. These are huge problem in terms of signalling overhead and energy consumption.

For NES operation in 6G, on-demand SSB on PCell and on-demand cell will be more attractive scenarios. Such operation have a possibility to cause more frequent update of properties of reference signal and cell which are typically cell common parameters in NR, so more efficient and dynamic mechanism for SI update should be studied.

Observation 1:

- SI update is indicated via short message that is transmitted on paging occasion and updated SIB can be transmitted only from next modification.

  - Multiple transmission of short messages consumes NW energy and UE energy which does not need to be indicated with the SI update at that time.

  - Maintaining necessary alignment of cell common parameters that are configured via SIB and RRC-dedicated message, causes a lot of signalling overhead (i.e., RRCReconfiguration and complete)

Proposal 4:

- Study a more efficient and dynamic mechanism to inform an update of cell common configuration for less signaling overhead and energy consumption.

Area specific SIBs

In NR, multiple cells can be treated as a certain area in terms of system information. SIB X which has same contents can be assigned with area scope ID and SIB1 indicates each area ID of one or multiple SIB X(s). By doing this, UE can omit re-acquisition of OSI within that area even upon mobility across cells. Especially for on-demand SI, NW can obtain more NES gain because a request by UEs to OSI will become less frequent. We believe that these functions have good potential to offer NW and UE ES in practical deployment. The potential remaining room to be addressed would be that NW has to ensure necessary alignment of all parameters of SIB X across multiple cells. It can be worth studying parameters defined in one SIB X that are partially same can be omitted to acquire and transmit.

Proposal 5:

- Study a more flexible area specific SI mechanism, e.g.,

  - When some parameters defined in one SIB X are same within an area, partial SIB X can be an area specific

2.3	Paging

In NR, paging frame (PF) and occasion (PO) of each UE is determined based on each UE ID, aiming at spread distribution in time domain and thus larger paging capacity in a cell is achieved. UE ID is assigned by core network such that it is unique among a certain area, i.e., RAN cannot control the UE ID assignment. The frequent transmission of paging would prevent NW from going to sleep mode and consume NW energy. Considering these factors, even if a few UEs camp on the cell, NW needs to wake up frequently and cannot enter longer sleep mode, depending on their UE IDs.

In Rel-19, paging adaptation mechanisms were introduced. The values of the parameters to define the number and time resources of PF within paging cycle (i.e., N) and PO (i.e., Ns) were extended (N = T/32, Ns= 8) such that PFs are confined within a cycle as possible. However, these Rel-19 features will offer limited NES gain due to limited extension of PF (e.g., N=T/64 was not supported) considering the signaling overhead of the parameter that configures the number and time positions of POs. For the study in 6G, since we can specify how to define these occasions considering potential concern (e.g., signaling overhead) from scratch, we would like to propose a mechanism to achieve further denser PF pattern than NR with increasing the number of POs into one PF for the paging capacity compensation.

Proposal 6:

- Study how to determine paging frame and paging occasion to improve both paging capacity and NW energy efficiency.

2.4	Spectrum aggregation

As discussed in our RAN1’s tdoc [2], in 5G, a lot of frequency bands are available to achieve e.g., comfortable user experience. However, it is difficult to say that 5G system can use all the frequency bands efficiently and effectively. In particular, in previous generations the operation on a cell is based on coupled DL and UL carriers defined as a band, which is inefficient and ineffective as DL and UL could have different requirements and limitations, such as different maximum Tx power and corresponding coverage, different amounts of traffic, different data rate demand, etc. For example, it is possible that a higher frequency is configured as PCell for a UE for offloading DL traffic from lower frequencies, assuming that the among of DL traffic is much larger than UL. Then, due to the coupling of DL and UL carriers, the higher frequency must be used for UL transmissions associated with PCell, and consequently, the UL performance will not be good.

SUL is a good concept to solve this kind of issues, and for 6G, it is desirable to define a more easily utilized and as a result widely implemented feature beyond 5G SUL.

Therefore, we propose to study DL/UL decoupling mechanism to utilize higher frequency more efficiently while ensuring UL coverage e.g., by allowing a cell consisting of DL carrier and UL carrier in different bands. It can be seen as simplification of 5G SUL since there is only lower frequency UL carrier for a cell and no dynamic switching between UL carriers within a cell. If utilizing wider UL carrier is also necessary when applicable e.g., for higher UL data rate, CA can be utilized.

As described in the section for initial access, we are considering multi-frequency layer scenario in which one frequency serves initial cell selection (entrance of a system) and other frequencies are in charge of other operation (e.g., for RRC CONNECTED, RACH procedure, SIB/paging reception in IDLE mode) to improve energy efficiency from a system perspective.

In this scenario, many UEs tend to select a carrier with a SSB on sync-raster, therefore access congestion in the carrier may occur. Note that the legacy mechanism for transitioning across inter-frequency can be done by using cell reselection procedure. However, the evaluation process for a transition is only triggered when measurement criteria of a serving cell are worse than the configured threshold. This would result in many UEs camping on a “stable and better, but not the best cell”. This is situation is not ideal for both NW and UE. Therefore, we believe that a new transition mechanism should be considered for 6G to resolve potential congestion before or during random access.

Proposal 7:

- Study transition mechanisms across multi-frequencies before and during random access.

  - E.g., Transition mechanism and criteria from a camping/serving cell to another cell across inter-frequency before (e.g., msg1 TX to another frequency) or during random access procedure (e.g., msg3 TX to another frequency)

- Conclusion

In this contribution, we provided the following observations and proposals of our views on initial and system access for 6G study.

Proposal 1:

- Study possible specification enhancement of SSB scanning during PLMN and initial cell selection for both UW and NW efficiency.

  - For UE side, UE power consumption for scanning all frequencies can be reduced.

  - For NW side, CD-SSB periodicity is no longer restricted by UE complexity and access latency for initial cell selection.

  - E.g., it can be specified that UE always can be able to know candidate frequency position of SSB on sync-raster (without scanning) based on UE’s location and/or USIM information

Proposal 2:

- Study on-demand SSB provision on PCell for IDLE/INACTIVE UEs, including

  - at least NW triggering mechanism, and UE triggering including its necessity and its criteria

  - RRM measurement mechanism that can be done within more confined time duration over NR using always-on SSB and potentially on-demand SSB for compensation of prolonged always-on SSB

Proposal 3:

- Study whether/how standalone deployment of OD-SIB1 NES cell is achieved.

  - E.g., Introduce pre-defined/pre-configured UL WUS configurations to request OD-SIB1.

Observation 1:

- SI update is indicated via short message that is transmitted on paging occasion and updated SIB can be transmitted only from next modification.

  - Multiple transmission of short messages consumes NW energy and UE energy which does not need to be indicated with the SI update at that time.

  - Maintaining necessary alignment of cell common parameters that are configured via SIB and RRC-dedicated message, causes a lot of signalling overhead (i.e., RRCReconfiguration and complete)

Proposal 4:

- Study a more efficient and dynamic mechanism to inform an update of “cell common configuration” for less signaling overhead and energy consumption.

Proposal 5:

- Study a more flexible area specific SI mechanism, e.g.,

  - When some parameters defined in one SIB X are same within an area, partial SIB X can be an area specific

Proposal 6:

- Study how to determine paging frame and paging occasion to improve both paging capacity and NW energy efficiency.

Proposal 7:

- Study transition mechanisms across multi-frequencies before and during random access.

  - E.g., Transition mechanism and criteria from a camping/serving cell to another cell across inter-frequency before (e.g., msg1 TX to another frequency) or during random access procedure (e.g., msg3 TX to another frequency)

References

- RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

- R1-2507814, “Discussion on overview of 6GR air interface”, NTT DOCOMO